# Credits: Produced

Gregory Walker, for the Digital Daguerreian Archive Project.

﻿The Project Gutenberg eBook of American Hand Book of the Daguerreotype

This ebook is for the use of anyone anywhere in the United States and

most other parts of the world at no cost and with almost no restrictions

whatsoever. You may copy it, give it away or re-use it under the terms

of the Project Gutenberg License included with this ebook or online

at www.gutenberg.org. If you are not located in the United States,

you will have to check the laws of the country where you are located

before using this eBook.

Title: American Hand Book of the Daguerreotype

Author: S. D. Humphrey

Release date: September 1, 1994 [eBook #167]

Most recently updated: July 3, 2020

Language: English

Credits: Produced by Gregory Walker, for the Digital Daguerreian Archive Project.

*** START OF THE PROJECT GUTENBERG EBOOK AMERICAN HAND BOOK OF THE DAGUERREOTYPE ***

Produced by Gregory Walker, for the Digital Daguerreian Archive Project.

[Illustration]

This etext was created by Gregory Walker, Austin, Texas, for the

Digital Daguerreian Archive Project.

Page numbers explicitly referred to in the text are marked at their

beginning by “[page ##]” on a separate line. I hope this etext

inspires a wider interest in the origins of photography and in the

modern practice of the Daguerreian Art.

[Updater’s note: In this version, the above page numbering convention

has been replaced by “{##}” sequences placed in line with the

surrounding text.]

AMERICAN HAND BOOK OF THE DAGUERREOTYPE

GIVING THE MOST APPROVED AND CONVENIENT METHODS FOR PREPARING THE

CHEMICALS, AND THE COMBINATIONS USED IN THE ART.

CONTAINING THE DAGUERREOTYPE, ELECTROTYPE, AND VARIOUS OTHER PROCESSES

EMPLOYED IN TAKING HELIOGRAPHIC IMPRESSIONS.

BY S. D. HUMPHREY

FIFTH EDITION

NEW YORK: PUBLISHED BY S. D. HUMPHREY 37 LISPENARD STREET

1858

Entered, according to Act of Congress, in the year 1858, by S. D.

HUMPHREY, In the Clerk’s Office of the District Court of the Southern

District of New York.

To J. GURNEY, WHOSE PROFESSIONAL SKILL, SCIENTIFIC ACCURACY, AND

ENERGETIC PERSEVERANCE, HAVE WON FOR HIM UNIVERSAL ESTEEM, THIS WORK IS

MOST RESPECTFULLY INSCRIBED.

PREFACE.

There is not an Amateur or practical Daguerreotypist, who has not felt

the want of a manual—Hand Book, giving concise and reliable information

for the processes, and preparations of the Agents employed in his

practice.

Since portraits by the Daguerreotype are at this time believed to be

more durable than any other style of “Sun-drawing,” the author has hit

upon the present as being an appropriate time for the introduction of

the Fifth Edition of this work. The earlier edition having a long since

been wholly; exhausted, the one now before you is presented.

The endeavor has been to point out the readiest and most approved

Methods of Operation, and condense in its pages; as much practical

information as its limits will admit. An extended Preface is

unnecessary, since the aim and scope of this work are sufficiently

indicated by the title.

S. D. HUMPHREY NEW YORK, 1858.

CONTENTS

# CHAPTER I.

Polishing the Daguerreotype Plate—Buffing the Plate—Coating the

Plate—Exposure of the Plate in the Camera—Position Developing the

Daguerreotype—Exposure to Mercury—Removing the Coating—Gilding or

fixing the Image—Coloring Daguerreotype, . . . . . 18

# CHAPTER II.

Coloring Back Grounds—Transparent ditto—Gilding Dissolvent—Solution for

removing Specks—Solarized Impression—To Purify Water—Cleaning

Mercury—Adhesive Paper—Black Stain for Apparatus—Sealing Wax for

Bottles—Rouge—Rotten Stone—Potassa Solution—Hyposulphite

Solution—Substitute for do.—Gilding Solution—Solution for increasing

the Brilliancy of the Daguerreotype—Bleaching Solution;—Cold

Gilding—Neutralizing Agents—Buff Dryer—Keeping Buffs in order—Cleaning

Buckskins—Reflector for taking Views, . . . . 52

# CHAPTER III.

Bromine and its Compounds—Iodine and its Compounds—Chlorine and its

Compounds—Cyanide of Potassium—Hyposulphite of Soda—Hyphosulphite of

Gold—Nitric Acid—Nitro-Muriatic Acid—Hydrochloric Acid—Hydrofluoric

Acid—Sulphuric Acid—Accelerating Substances—Liquid Sensitives—Dry

Sensitives, etc., etc., . . . . . 72

# CHAPTER IV.

Light—Optics—Solar Spectrum—Decomposition of Light—Light, Heat, and

Actinism—Blue Paper and Color for the Walls of the Operating

Room—Proportions of Light, Heat and Actinism composing a

Sunbeam—Refraction—Reflection—Lenses—Copying Spherical

Aberration—Chromatic Aberration, . . . 131

# CHAPTER V.

To make Plates for the Daguerreotype—Determining the Time of Exposure

in the Camera—Instantaneous Process for Producing

Daguerreotype—Galvanizing the Daguerreotype Plate—Silvering

Solution—Daguerreotype without Mercury—Management of Chemicals—Hints

and Cautions—Electrotyping—Crayon Daguerreotypes—Illuminated

Daguerreotypes—Natural Colors in Heliography—Multiplying Daguerreotypes

on one Plate—Deposit in Gilding—Practical Hints on the Daguerreotype, .

. . 149

# CHAPTER VI.

An Account of Wolcott and Johnson’s Early experiments in the

Daguerreotype, . . . 188

AMERICAN HAND-BOOK of THE DAGUERREOTYPE.

# CHAPTER I.

Polishing the Daguerreotype Plate—Buffing the Plate—Coating the

Plate—Exposure of the Plate in the Camera—Position—Developing the

Daguerreotype—Exposure to Mercury—Removing the Coating—Gilding or

fixing the Image—Coloring Daguerreotype.

Polishing the Daguerreotype Plate.—I shall endeavor to present to the

reader the process I have found productive of good and satisfactory

results, presenting the same in a clear and concise manner, so that any

one, by following the various manipulations given, will be enabled to

succeed. If there is any one part of the process in Daguerreotype in

which operators fail more than all others, it is in not properly

preparing the plate. It has truly been said that it would take a volume

to describe all the methods that have been suggested for polishing the

plate.

# I shall confine myself to the following description, which has been

successfully practised, also most generally adopted by our operators,

and I believe equal, if not superior to any other method, yet at the

same time it is not of so much importance what particular method is

employed, so that it be thoroughly and skillfully carried out.

There is a general tendency with beginners to slight this operation;

hence the necessity of adopting a system which precludes the

possibility of doing so. During many years’ study and practice in the

art, I have tried numerous methods and substances for the better

accomplishment of the end in view, and have finally settled upon the

following, as being (so far as experience allows me to Judge) the modus

operandi, best suited to all circumstances; under no condition would I

approve of a method less rigorous or precise.

The operator being provided with a bottle of finely prepared rotten

stone, cover the mouth of the bottle with a piece of thick paper, this

perforated with a pin so that the rotten stone can be dusted on the

plate. Fasten the plate on the holder, take the rotten stone (Becker’s

can always be depended upon), and dust on lightly until the surface is

freely covered; now drop on the plate’s surface a few drops of an

alcoholic solution.[1]

[1] This solution is composed of equal parts of alcohol and water, for

the summer, and in winter three parts alcohol to one of water; a few

drops of potassa solution may be added, and is known to have a decided

effect upon the plate.

Take a patch of Canton flannel; in order to prevent the moisture from

the hand it should have a thick, firm texture: with this rub the plate

in circles across, then back covering one-half of the former row of

circles in each crossing until you have gone over the plate and back to

the point of beginning, occupying at least half a minute in the

operation, for a small plate, and so in proportion for the other sizes.

Care should be observed to keep the patch wet with the alcoholic

solution forming a paste on the surface of the plate; the motion of the

hand should be brisk and free, not hurried, and the pressure about

equal to that of a pound weight. When the cotton is disposed to adhere

to the plate, and slip from under the finger, spread the fore and

middle fingers a little apart, then pressing down, bring them together

in such a manner as to form a fold in the cloth between them, by which

means you will hold it perfectly secure.

Avoid wetting the fingers, and should they perspire, wipe them often,

as the moisture penetrating the cotton and coming in contact with the

plate, would cause streaks it would be difficult to remove. I will here

remark that many operators use much more cotton flannel than there is

need of. I have found in my experience that a single patch, about one

and half inch square, will be better for cleaning a number of plates

than a new piece for every plate. This is the case for the wet, and for

the dryrubbing two or three pieces will be found to answer. Thus with

four or five cloths a dozen plates may be prepared.

Some operators use prepared cotton, and think it more convenient than

the flannel. This may be had prepared free from seeds and in a very

perfect state, if wished.

In going over the plate, great care should be observed, in touching its

surface as equally as possible. The greatest care should be taken

neither to touch the plate with the fingers, nor that part of the

cotton flannel which is to come in contact with its surface; take a

clean piece of flannel by one corner, snap it smartly to free it from

dust and loose fibres, lay it face-side upward, dust on a little fine

rotten stone; with this, polish around, or across, or in circles,

lightly and briskly, passing gradually over the whole surface of the

plate, as was done before with the wet. The plate should now exhibit a

bright, clear, uniform surface, with a strong metallic lustre,

perfectly free from any appearance of film; if not, the last polished

should be continued until the effect is obtained, and when once

obtained, the plate is ready for buffing.

Buffing the Plate.—There are a variety of ways and means employed in

this part of the operation. Some choose wheels, and others prefer the

ordinary hand-buff. I have been unable to detect any peculiar advantage

in the use of the wheel except in the facility of the operation; no

doubt, however, but there is a saving of time, particularly in the

preparation of the larger plates. For general use, we have not seen a

wheel better adapted for this purpose than the one patented by Messrs.

Lewis.

It is generally well to use a hand-buff before placing the plate on the

wheel; this is in order to prevent, as far as possible, the dust or

other substance that may be on the surface of the plate from coming in

contact with the cover of the wheel. I will here follow out the use of

the hand-buffs (two are necessary) as they are mostly used.

In the morning, before using the buffs, brush both as clean as

possible, in order to free them from dust; then with the blade of a

pair of shears, held perpendicular, rub the buffs from end to end; then

knock them both together in order to free them from all dust or other

substances, occasionally exposing them to the sun or to the fire.

With one of the buffs (reserving the finest and softest for the last

operation), powder its face with fine rouge and brush off slightly,

leaving only the finest particles in it. Every operator should have two

plate-holders; one for cleaning and one for buffing the plate; for when

using only one, the rotten stone is liable to get on the buff and

scratch the plate.

Rest the fingers of the left hand on the back of the buff, near the

farther end, with about the same pressure as in cleaning, while with

the right you bear on the handle to correspond, and give the buff a

free, easy, horizontal motion, passing it very nearly the whole length

over the plate each time. Continue this operation in such a manner that

the plate will on all parts of its surface have received an equal

amount of polish. This buff once well filled with polish, add but

little after, say a small quantity once in two or three plates. The

polish as well as the buffs must be kept perfectly dry.

The second buff should always be in the best order, and if this is the

case, but little polish after the first need be used. Much depends upon

the last finish of the surface of the plate, and as a fine impression

is desired in the same ratio, the operator must exercise care and skill

in this operation. Some buff the smaller plates on the hands, by

resting them on the fingers in such a manner that the buff cannot touch

them; some by holding the edges with thumb and little finger, with the

remaining fingers under, or on the back; and others buff on the holder.

When this last method is adopted, it requires the greatest caution to

prevent the dust from getting on the buff. The holder should be wiped

clean.

The plate frequently slips off or around, and the buff comes in contact

with the bed of the holder. When, however, the operator is so

unfortunate as to meet with this mishap, the utmost care must be

observed in thoroughly cleaning the buff cover before further buffing.

In this last buffing it may be continued as before, except without the

application of polish powder to the last buff. Examine the surface

occasionally, and buff more lightly towards the close of the operation,

using at last the mere weight of the buff. This last buffing should

occupy as long a time as the first.

The point to be aimed at is, the production of a surface of such

exquisite polish as to be itself invisible, like the surface of a

mirror. The secret of producing pictures discernible in any light, lies

in this: the more dark, deep and mirror-like the surface of the plate,

the more nearly do we approach to perfection.

In all cases, very light and long continued buffing is productive of

the greater success, since by that means a more perfect polish can be

obtained.

The question is often asked, why is it that the plates receive the

coating so unevenly? I will answer by saying that it may arise from two

causes: the first and most general cause is that those parts of the

plate’s surface which will receive the heaviest coating have been more

thoroughly polished, and the consequence is that it is more sensitive

to the chemical operation; second, and might perhaps be considered a

# part of the first, the heat of the plate may not be equal in all its

parts; this may arise from the heat caused by the friction in buffing.

It is a well known fact, with which every observing practitioner is

familiar, that a silver plate at a temperature of 45 deg. or less,

exposed to the vapors of iodine, is less sensitive and takes a longer

time to coat, than when it is at a temperature of 60 deg. or more.

Whenever a view is to be taken, or any impression which requires the

plate to be turned on the side, it should be buffed in the other

direction, so that the marks will always be horizontal, when the

picture is in position. With the finest possible polish, the plate is

ready for the coating box.

The question is often asked by operators, what is the state of the

plate when polished and allowed to stand for a time before using? To

meet this point we hare only to consider the silver and the power

acting upon it. Pure atmosphere does not act upon silver; but we do not

have this about in our operating rooms, as it is more or less charged

with sulphurated hydrogen, which soon tarnishes the surface of the

plate with a film of brown sulphurate. It is this that sometimes causes

the specks which appear on finishing the impression, and are a great

annoyance. Hence we see that the plate should be buffed just before

receiving the vapor of iodine. Mr Hunt gives his opinion of the use of

diluted nitric acid as the best solution for freeing, the surface of

the plate; he says:

“Numerous experiments on plated copper, pure silver plates, and on

silvered glass and paper, have convinced me that the first operation of

polishing with nitric acid, etc., is essential to the production of the

most sensitive surface. All who will take the trouble to examine the

subject, will soon be convinced that the acid softens the silver,

bringing it to a state in which it is extremely susceptible of being

either oxydized or iodized, according as the circumstances may occur of

its exposure to the atmosphere or the iodine.”

# I cannot see the objection to this solution; not, however, in general

use. Our operators do not find it of sufficient importance to the

success of their pictures to accept it, the alcoholic solution being in

its nature less objectionable.

# I will say here, that a plate submitted to only an ordinary polish is

found to contain numberless minute particles of the powder made use of.

Should the same plate be buffed for a long time, the polish will nearly

all disappear, leaving the cavities in the surface free for the action

of agents employed in subsequent operation. For this reason, I find

that great amount of polishing powder should not be applied to the last

buff, and it is obvious that three buffs can be employed to adventure;

the two last should not receive any polishing materials. I have

examined a plate that was considered to possess a fine finish, and

similar had produced good impressions; these same plates, when

subjected to a long and light buffing, would present a surface no finer

in appearance to the naked eye; but upon exposure to the solar

radiation, would produce a well-defined image in one fourth less time

than the plate without the extra buffing.

Coating the Plate.—For this purpose our mechanics and artists have

provided a simple apparatus called a coating-box, which is so arranged

as to be perfectly tight, retaining the vapor of the iodine or

accelerators, and at the same time allowing, by means of a slide, the

exposure of the plate to these vapors. They can readily be obtained by

application to any dealer, all of whom can furnish them.

The principal difficulty in coating the plate, is that of preserving

the exact proportion between the quantity of iodine and bromine, or

quick. It is here necessary to say, that hardly any two persons see

alike the same degree of color, so as to be enabled to judge correctly

the exact tint, i. e. what one might describe as light rose red, might

appear to another as bright or cherry red; consequently, the only rule

for the student in Daguerreotype, is to study what appears to him to be

the particular tint or shade required to aid him to produce the desired

result. Practise has proved that but a slight variation in the chemical

coating, of the Daguerreotype plate will very materially affect the

final result.

The operator will proportion the coating of iodine and bromine or

accelerators according to the strength and composition of the latter.

Experience proves that the common impressions, iodized to a rather

light yellow gold tint, and brought by the bromine to a very light,

rose color, have their whites very intense, and their deep shades very

black. It is also known that if you employ a thicker coating of iodine

and apply upon it a proportionate tint of bromine, so as to obtain a

deep rose tint, delineations will be less marked, and the image have a

softer tone. This effect has been obvious to everyone who has practised

the art. Thus I may observe that the light coatings produce strong

contrast of light and shade, and that this contrast grows gradually

less, until in the very heavy coating it almost wholly disappears. From

this it will readily be perceived that the middle shades are the ones

to be desired for representing the harmonious blending of the lights

and shades.

Then, if we examine, with respect to strength, or depth of tone, and

sharpness of impression, we see that the light coating, produces a very

sharp but shallow impression; while the other extreme gives a deep but

very dull one. Here, then, are still better reasons for avoiding either

extreme. The changes through which the plate passes in coating may be

considered a yellow straw color or dark orange yellow, a rose color

more or less dark in tint, or red violet, steel blue or indigo, and

lastly green. After attaining this latter color, the plate resumes a

light yellow tint, and continues to pass successively a second time,

with very few exceptions, through all the shades above mentioned.

# I will here present some excellent remarks upon this subject by Mr.

Finley. This gentleman says:

“It is well known to all who have given much attention to the subject,

that an excess of iodine gives the light portions of objects with

peculiar strength and clearness, while the darker parts are retarded,

as it were, and not brought out by that length of exposure which

suffices for the former. Hence, statuary, monuments, and all objects of

like character, were remarkably well delineated by the original process

of Daguerre; the plate being coated with iodine alone. An excess of

bromine, to a certain degree, has the opposite effect; the white

portions of the impression appearing of a dull, leaden hue, while those

which should be black, or dark, appear quite light. This being the

case, I conclude there must be a point between the two extremes where

light and dark objects will be in photogenic equilibrium. The great

object, therefore, is to maintain, as nearly as possible, a perfect

balance between the two elements entering into union to form the

sensitive coating of the plate, in order that the lights and shades be

truly and faithfully represented, and that all objects, whether light

or dark, be made to appear so far conformable to nature, as is

consistent with the difference in the photogenic energy of the

different colored rays of light. It is this nicely-balanced combination

which ensures, in the highest degree, a union of the essential

qualities of a fine Daguerreotype, viz., clearness and strength, with

softness and purity of tone.

“So far as I know, it is the universal practice of operators to judge

of the proportion of iodine and bromine in coating the plate, by two

standards of color the one fixed upon for the iodine, the other for the

additional coating of bromine. Now I maintain that these alone form a

very fallacious standard; first, because the color appears to the eye

either lighter or darker, according as there is more or less light by

which we inspect the coating; and secondly, because if it occur that we

are deceived in obtaining the exact tint for the first coating, we are

worse misled in obtaining the second, for if the iodine coating be too

light, then an undue proportion of bromine is used in order to bring it

to the second standard, and vice versa.”

The iodine box should be kept clean and dry. The plate immediately

after the last buffing, should be placed over the iodine, and the

coating will depend upon the character of the tone of the impression

desired. Coating over dry iodine to an orange color, then over the

accelerator, to a light rose, and back over iodine one sixth as long as

first coating, will produce a fine, soft tone, and is the coating

generally used for most accelerators. The plate iodized to a dark

orange yellow, or tinged slightly with incipient rose color, coated

over the accelerator to a deep rose red, then back over iodine

one-tenth as long as at first coating, gives a clear, strong, bold,

deep impression.

# I will here state a singular fact, which is not generally known to the

operator. If a plate, coated over the iodine to a rose red, and then

exposed to strong dry quick or weak bromine water, so that a change of

color can be seen, then recoated over the iodine twice as long as at

first coating, it will be found far more sensitive when exposed to the

light than when it has been recoated over the iodine one-fourth of the

time of the first coating.

Probably the best accelerating combination is the American compound

formerly known as “Gurney’s American compound,” or some of the

combinations of bromide of lime. The first is thought to possess

perhaps more uniformity in its action than any other combination I have

ever used.

The plate once coated should be kept excluded from the light by means

of the plate holder for the camera box.

# I will notice one of the principal causes having a tendency to prevent

the perfect uniformity of chemical action, between the iodine and

silver; hydrogen, or the moisture in the atmosphere, makes a very

perceptible barrier. This moisture may arise as the result of the cold,

from a want of friction in the buffing of the plate, which, coming in

contact with the warmer air, as a writer on this subject says:

“It is well known that as often as bodies, when cold, are exposed to a

warmer air, the humidity contained in them is condensed. It is to this

effect that we must attribute the difficulty experienced in operating

in most cases.” This is corroborated by the results experienced by our

operators. So it is seen that the plate should be of a temperature

above that of the atmosphere. Mr. Gurney submits his plates to a gentle

heat from a spirit lamp just before exposing them to the vapor of

iodine. Experience has convinced me that a plate heated to about 80

deg. before being exposed to iodine will present a far better defined

image than a plate at a temperature of 50 deg. I account for this by

noticing that, at a higher temperature, the plate throws off any larger

crystals that might otherwise be deposited, receiving only the finer,

thus producing a more perfect chemical combination of iodide of silver.

# I would call the attention of the operator to this point, as presenting

something of interest, and which may direct in a way of accelerating

the future operations.

That the presence of a film of moisture over the plate is a preventive

of uniform chemical action, may be readily understood from the fact

that iodine is almost insoluble in water, requiring seven thousand

parts of water to dissolve one of iodine, or one grain to a gallon of

water. Yet its affinities for silver and other substances are so

powerful as to prevent its existing in an insulated state, hence we can

account for the frequent occurrence of a plate presenting parts of an

image over its surface. It is quite evident that those parts of plate’s

surface covered with moisture are nothing like as sensitive to the

iodine as those parts perfectly free.

Exposure of the plate in the Camera, and Position.—The time of exposure

necessary to produce an image upon the Daguerreotype plate, can only be

determined by experiment, and requires a liberality of judgment to be

exercised on the part of the operator. The constant variation of the

light renders it impossible to lay down any exact rule upon this point.

Light is not alone to be considered; the amount of coating exercises a

deviating influence, also the subjects to be represented are not

equally photogenic, some requiring much longer time of exposure than

others. This may be easily observed by exposing the plate at the same

time to a plaster bust and a piece of black velvet, the first being a

much stronger reflector of light than the latter: the time necessary to

produce a well developed image of the velvet being about six times

longer than that required to produce an equally defined image of

plaster. The manner of judging correctly of the time is by the

appearance of impression after it has been developed by the mercurial

vapors. Should it present a deep blue or black appearance it is

solarized or over-timed. This sometimes is to an extent, that a perfect

negative is formed, the white being represented black, and the dark

light.

An object requiring the particular care and attention of the operator

is the proper focus. It is not unfrequently the complaint of sitters

that their hands are represented as being magnified and greatly out of

proportion with the general figure. This is the case also with the nose

and eyes, but in a less degree. As this cannot be wholly remedied, it

is desirous to come as near as possible, and in order to do this, it is

necessary to present the figure in such a position as to bring it as

nearly as possible upon the same plane by making all parts nearly at

equal distance from the lenses. This must be done by the sitter

inclining the head and bust formed to a natural, easy position, and

placing the hands closely to the body, thus preserving a propel

proportion, and giving a lively familiarity to the general impression.

It is not an uncommon fault among our less experienced operators to

give a front view of the face of nearly every individual, regardless of

any particular form, and this is often insisted upon by the sitter,[2]

who seems to think the truth of the picture exists principally in the

eyes staring the beholder full in the face.

[2] I might here picture some curious scenes experienced by our

operators Every one is familiar with a certain class of our community

whose ideas of the importance of a free and easy position of the body

are too closely confined with stays, attention to toilet, tightly

fitting dress coats and the like, to admit of being represented as if

nature had endowed them with least possible power of flexibility. To

such we would suggest the following, to be well learned and retained

in the mind while presenting themselves before the Daguerreotype

camera:

“The experience of one who has often been Daguerreotyped, is, to let

the operator have his own way.”

Nothing, in many instances, can be more out of place in a Daguerreotype

portrait than this, for let a man with a thin, long,

defeated-politician-face, be represented by a directly front view, we

have, to all appearances, increased the width of the face to such an

extent as to reveal it flat and broad, losing the characteristic point

by which it would be the most readily recognized. The method we should

adopt in taking the likeness of such an individual as above, would be

to turn the face from the camera, so as to present the end of the nose

and the prominence of the cheek bone equally distant from the lenses,

and then focusing on the corner of the eye towards the nose, we cannot

in many cases, fail to produce an image with the lips, chin, hair, eyes

and forehead in the minutest possible definition.

It should be the study of every operator to notice the effect of the

lights and shades while arranging the sitter, and at the same time be

very particular to give ease in the position.

No matter how successful the chemical effect may have been, should the

image appear stiff and monument-like, all is lost. “In the masterpiece,

grace and elegance must be combined.”

# I will here use the words of another, which are very true:

“So great is the difference in many faces, when inspected in opposite

directions, that one of the two views, however accurately taken, would

not communicate the likeness—it not being, the usually observed

characteristic form. When the right view of the head is obtained, it is

first necessary to consider the size of the plate it is to be taken on,

so as to form an idea of the proportion the head should bear to it. The

mind must arrange these points before we commence, or we shall find

everything, too large or too small for the happy proportion of the

picture, and the conveying of a just notion of the stature. The work

will have to be done over, and time sacrificed, if this is not attended

to. The adjustment of the head to the size of the plate (as seen from

the margin of the mat), is not to be taught: everyone must bring

himself, by scrutinizing practice, to mathematical accuracy; for

something will be discovered in every face which can be surmounted only

by experience.

“The eye nearest the camera, in a three-quarter-face, is placed in the

middle of the breadth of the plate; the chin, in a person of middle

stature, in the middle of the length, and higher according to the

proportional height of the person.”

In regard to the proper elevation of the camera, it may be here stated

that I have found it best in taking portraits where the hands are

introduced, to place the camera at about equal height with the eyes of

the sitter, in order to bring the face and hands equi-distant from the

tube. It will be found, if the above be followed, that by attaching a

string to the camera tube, and making a semi-circle, that the face and

hands of the sitter will occupy a corresponding distance, and the

consequence is that the impression will appear without the hands being

magnified. It has been found that a person with a freckly face can have

as fine, fair, and clear an impression as the most perfect complexion;

this may be done by the subject rubbing the face until it is very red.

The effect is to lessen the contrast, by giving the freckles and skin

the same color and the photogenic intensity of the red and yellow being

nearly the same, an impression can be produced perfectly clear.

When a child is to be taken, and there are doubts of its keeping still,

the operation may be accelerated by placing it nearer the window

bringing the screen nearer, and placing a white muslin cloth over the

head; this will enable you to work in one third of the usual time.

Should the person move, or the plate become exposed to the light, it

may be restored to its original sensitiveness by placing it over the

quick, one or two seconds.

Developing the Daguerreotype.—After the plate has been submitted to the

operation of the light, the image is still invisible. It requires to be

exposed to the vapors of heated mercury. It is not absolutely necessary

to apply artificial heat to the mercury to develop the image, for fair

proofs have been produced by placing a plate over the bath at the

ordinary temperature of the atmosphere. This plan, however, requires a

long time and cannot be adopted in practice, even if it were advisable.

The time more usually required in developing the image over the

mercurial vapors, is about two minutes, and the temperature is raised

to a point necessary to produce the desired effect in that time. This

point varies as indicated by different scales, but for the ordinary

scales it is not far from 90 deg. cen.

The mercury bath is accompanied with a centigrade thermometer, by which

the heat is regulated. Those furnished by the manufacturers are not

always correct, and it requires some experience to find the proper

degree on the scale.

# I would here remark that it is advisable, when placing the spirit lamp

under the bath, to so arrange it that the position of applied heat

should always be on the same point, viz., should the heat be directly

under the bulb containing the thermometer it would raise the mercury in

the tube to the point marked, and the temperature of that in the bath

would be far below what it should be; hence it is (where time is

followed for developing) that many failures occur. This is observed

more readily in the large baths made of thick iron, particularly upon

first heating. In practice I apply the heat as nearly as possible

between the centre of the bottom of the bath and the bulb containing

the mercury tube. It is advisable to keep the lamp lighted under the

bath from the time of commencing in the morning to the close of

business at night. By this means you have a uniformity of action, that

cannot be otherwise obtained.

It is well known to the experienced Daguerreotypist, that different

atmospheres have a decided effect upon the mercury in developing the

Daguerreotype. It will require a greater degree of heat for one

atmosphere than for another. Experience alone determines this little

difference.

In summer, on cloudy and stormy days, mercurial vapors rise more

readily and quickly than in the temperature of autumn or winter. From

60 degrees upwards towards the boiling point (660 deg.), the vapors of

mercury rise in greater abundance and collect in larger globules on

cold surfaces.

For various reasons I prefer a high temperature and short exposure. It

accelerates the process. It renders the lights of the picture more

strong and clear, while the deep shades are more intense. It gives a

finer lustre to the drapery. The solarized portions also are very

seldom blue, especially after gilding. If heated too high, however, the

light parts become of a dead, chalky white, and the shadows are injured

by numerous little globules of mercury deposited over them. Just the

right quantity of mercury leaves the impression of a transparent,

pearly white tone, which improves in the highest degree in gilding. To

mercurialize with exactness is a nice point. If there is reason to

suspect having timed rather short in the camera, reduce the time over

mercury in a corresponding proportion. A dark impression will be ruined

by the quantity of mercury which would only improve a light one.

If practicable, it is most expedient that the plate be submitted to the

action of mercury immediately on coming from the camera. I have

frequently, however, carried plates for miles in the plate-holders and

after exposing in the camera, brought them back to expose to mercury,

and obtained fair proofs; but for the reason before given, it is

advisable to carry along the bath, and bring out the impression on the

spot.

It is sometimes the practice of inexperienced operators to take the

plate off the bath and examine the impression by solar light. This plan

should be abandoned, as it is almost sure to produce a dense blue film

over the shadows.

This I am led to believe is occasioned by the action of light on the

yet sensitive portions of the plate, and made to appear only by

subsequent exposure to mercury, being equivalent to solarization.

There has been little said by our professors upon the subject of the

position of the plates while exposed to the mercurial vapour. Mr. Hunt,

in referring to this subject, says: “Daguerre himself laid much stress

upon the necessity of exposing the plate to the mercury at an angle of

about 45 deg.. This, perhaps, is the most convenient position as it

enables the operator to view the plate distinctly, and watch the

development of the design; but beyond this, I am satisfied there exists

no real necessity for angular position. Both horizontally and

vertically, I have often produced equally effective Daguerreotypes.” I

presume from the last sentence of Mr. Hunt, that he has confined his

experiments to the smaller sized plates. Hence he may not have thought

of the effect of the vertical exposure of a large plate.

In America this is a subject of no little importance. When an

impression is to be developed upon a plate fifteen by seventeen inches,

were we to use an angle of about 45 deg., it would be found to make a

perceptible difference in the appearance of the image. By examining the

wood tops of our baths as formerly made, it will be found that there is

a great variation in the distance from the mercury to the different

portions of the plate. By measuring one of these tops for the size

plate above mentioned, I find the distance to the nearest point between

the mercury and the plate, to be thirteen, and the middle point

sixteen, and the furthest point twenty-one and a half inches: by this

we see that one point of the plate is eight and a half inches further

from the mercury than the nearest point; even this is not the variation

there would necessarily be, were we to adopt the angle of 45 deg. as

urged by Daguerre.

Among our principal professors, the bevel top will not be found in use

where the large plates are used. Should any one feel desirous to test

more minutely the effect produced by a bevel top bath, I would suggest

to them to place a frame, so constructed as to hold three sixth size

plates, and fit it to the top of the bath, and so arrange it with

openings that the plates may be placed, one at the nearest point of the

mercury, the second midway, and the third to the greatest distance, and

by placing the plates over at one and the same time, the experimenter

will be enabled to judge if there exists a difference in the

developing. In speaking of the above, reference is had to baths to the

ordinary heights used by operators.

We will now proceed to examine the effect produced by mercurial vapor

upon the plate at different lengths of exposure. In some investigations

which I have made upon the appearance of the Daguerreotype impressions

when developed over mercury at 90 deg. C. (194 deg. F.), the following

was the result. Plates, coated and exposed to light in our usual manner

of operating, produced on exposure of

1/2 minute, whole impression, deep blue.

1 minute, ashy and flat; no shadows; linen, deep blue.

1 1/2 minute, coarse and spongy; shadows, muddy; drapery, dirty reddish

brown.

2 minutes, shallow or watery; shadows, yellowish; drapery, brown.

2 1/4 minutes, soft; face, scarcely white; shadows, neutral; drapery,

fine dark brown linen somewhat blue.

2 1/2 minutes, clear and pearly; shadows, clear and positive, of a

purple tint; drapery, jet black, with the dark shades slightly frosted

with mercury.

2 3/4 to 3 minutes, hard and chalky; shadows, harsh; drapery,

roughened, and misty with excess of mercury.

The foregoing results will be found general.

There are numerous opinions among our operators in regard to the

quantity of mercury necessary for a bath. As regards this, I need only

say, similar results occur when two pounds or two ounces are used, but

the quantity generally employed is about a quarter of a pound. I am of

the opinion that one ounce will answer as well as a larger quantity. I

know of no better proof in favor of a small quantity than that

presented in the following incident. Several years since, an operator

(Mr. Senter, of Auburn, N.Y.) of my acquaintance, was requested to go

several miles to take a Daguerreotype portrait of a deceased person. He

packed up his apparatus and proceeded over a rough road for some

distance to the house where he was to take the portrait, and arranging

his apparatus, with all the expedition which the occasion required,

after having everything in usual order (as was supposed), he proceeded

and took some ten or twelve very superior impressions. They were fine,

clear, and well developed. After taking the number ordered, he

proceeded to repack his apparatus, and to his surprise, when he took up

the bottle he carried the mercury in, he found it still filled, and

none in the bath, except only such particles as had adhered to the

sides, after dusting and being jolted for several miles over the rough

road. From this it will be seen that a very little mercury will suffice

to develop fine proofs. I saw some of the impressions referred to

above, and they were certainly well developed, and very superior

specimens of our art.

Removing the Coating.—After the impression has been developed over the

mercurial vapor, the next step is to remove the sensitive coating. For

this purpose the following solution is used:

Put about two ounces of hyposulphite of soda in a pint of water, which

should always be filtered before using. A convenient way of doing this

is to have two bottles, and a large funnel with a sponge pressed into

the neck of it; or, what is better, some filtering paper folded in it.

The solution in one bottle, the funnel is placed in the other, and the

picture held over it; when the solution is poured on the plate, it runs

from it into the filter, and is always ready for use.

It is best that the washing be done immediately on the plate coming

from the mercury bath. If allowed to stand long with the coating on, it

assumes a very dark tint—as the operation of the light continues,

though less active than while exposed in the camera, and destroys that

brightness which would otherwise have been obtained. It is preferable

to wash and gild a picture without it first being dried; yet when there

are doubts of its giving satisfaction, there would sometimes be a

saving by drying and getting the decision of the subject before

gilding, as this last injures the plate for another impression. First,

light your spirit-lamp, then with your plyers take the plate by the

lower right-hand corner, holding it in such a manner that the plyers

will form in a line with the upper left-hand corner; pour on, slowly,

the hyposulphite solution, slightly agitating the plate, until all the

coating is dissolved off; then rinse off with clean water, and if it is

not to be gilded, dry by holding the plate perpendicular with the

bottom left-hand corner lowest, and applying the blaze of the

spirit-lamp to the back, at the same time blowing gently downward on

the face of the plate.

The hyposulphite solution should be often filtered through a sponge,

and it will answer for a great number of washings. Yet it is observed

that the mercury collects in this solution in small globules; these

often come in contact with the plate, causing white spots, which spoil

the impression. They should be guarded against, and the solution

renewed. Again, in order to prevent streaks or scum on the surface of

the plate, it is necessary that the coating should be removed with a

good degree of uniformity. I find in practice that the hyposulphite of

soda in our market varies much as regards strength, and consequently

the rule to be adopted is to make a solution of sufficient strength to

remove the coating in about ten seconds. I am aware that it may be said

that this strong solution would have a tendency to injure the

impression by destroying in a measure the sharpness of outline. To meet

this, it need only to be said that the preventive is, to not let the

solution rest on the surface of the plate for a longer time than is

absolutely necessary, and then it should be drenched copiously with

water; hence a chemical action upon the image is prevented and the

general operation facilitated. This plan is adopted by our first

operators with the greatest success.

If the operator should allow the hyposulphite solution to run over the

plate unevenly, it is quite likely that white or blue streaks would

result. These it is impossible to remove without injury to the

impression. Some, in order to prevent this, breathe over the surface,

thus moistening it and putting it in a condition to receive the

solution with greater uniformity. The plate should be well washed with

water before gilding.

Gilding, or Fixing the Image.—The next process to be given is that for

fixing the image on the plate. This is done by precipitating a thin

film of gold over the surface and is productive of the most brilliant

effect when prepared immediately after the plate has been washed with

water after the application of the hyposulphite solution, and before

the plate has been allowed to dry. When, however, the plate has been

dried and allowed to stand for any time, before gilding, the

hyposulphite wash should be applied as at first, in order to destroy

any chemical coating that may have been formed on exposure of the plate

to the air. For gilding the larger plates, we have a gilding stand so

constructed that the plate can be put on a perfect level. In practice,

# I prefer holding the plate with nippers, fastened at one corner. Hold

the plate in the same manner as in removing the coating; pour on the

gilding, newly filtered, until the surface is wholly covered, and with

the blaze of the spirit lamp, at least three inches high, apply it to

the back of the plate, moving it about, that the surface may be heated

with as much uniformity as possible. Continuing this operation, the

surface will generally become covered with small yellow bubbles which

soon disappear, leaving the image clear and distinct.

It is advisable to make use of a lamp having a sufficiently strong

flame to produce the effect in a few minutes. If after a first heating,

it is found that the impression can admit of a greater degree of

intensity, it might be heated anew; but that is seldom necessary, and

often by trying to do too, well, the operator, if he persists in

heating certain parts of the plate, may find the liquid dry up just

above the flame, and inevitably cause a stain; [3] or else the blacks

are covered with a film, or even the coating of gold may suddenly

exfoliate, when small particles are detached from the plate. The

impression is then entirely spoiled, but the plate may be re-polished.

[3] This can be remedied, however, if it is immediately washed over

with the same solution that is on the plate, so that the surface shall

not become cool; continue for a short time to apply the lamp under,

and agitate the plate slightly, and it will soon be free from all

imperfections and give a fine clear tone.

It is not unfrequent that the surface assumes a dark, cloudy

appearance. This is generally the best sign that the gilding will bring

out the impression with the greatest degree of distinctness. Soon, the

clouds gradually begin to disappear, and, “like a thing of life” stands

forth the image, clothed with all the brilliancy and clearness that the

combined efforts of nature and art can produce. When in the operator’s

judgment the operation has arrived at the highest state of perfection,

rinse suddenly, with an abundance of clean water, and dry as before

described.

When an impression is dark, the gilding process may be longer

continued; but when light, it should be gilded quickly, as lengthening

the time tends to bleach the impression and make it too white. The

cause of this appears to be, that with a moderate heat the chlorine is

merely set free from the gold, and remaining in the solution, instead

of being driven off, with its powerful bleaching, properties, it

immediately acts upon the shades of the picture. A dark impression can

thus, by a low heat, long-continued, be made quite light. To procure

the best effect, then, heat suddenly with a large blaze, and judging it

to be at the maximum, cool as suddenly as possible.

When the hyposulphite of gold is used instead of the chloride, a less

heat should be employed.

Coloring Daguerreotypes.—Of all the so-called improvements in the

Daguerreotype, the coloring is the least worthy of notice. Yet the

operator is often, in fact most generally, called upon to hide an

excellent specimen under paint. I can conceive of nothing more perfect

in a Daguerreotype than a finely-developed image, with clearness of

lights and shadows, possessing the lively tone resulting from good

gilding. Such pictures, however, are not always had, and then color may

perform the part of hiding the imperfections. We present the following

method as given in Willat’s Manual:

“Daguerreotype portraits are now commonly met with beautifully colored;

but the coloring is a process requiring great care and judgment, and

many good pictures are spoiled in fruitless experiments. Several

different methods of coloring have been proposed. The simplest mode

appears to be that of using dry colors prepared in the following

manner: A little of the color required, very finely ground, is thrown

into a glass containing water, in which a few grains of gum arabic have

been dissolved. After standing a few moments, the mixture may be passed

through bibulous paper, and the residue perfectly dried for use. The

principal colors used are Carmine, Chrome Yellow, Burnt Sienna,

Ultramarine and White; boxes fitted with sets of colors properly

prepared, may be obtained of the dealers, and include Carmine, White,

Lilac, Sky Blue, Pink, Yellow, Flesh color, Orange, Brown, Purple,

Light Green, Dark Green and Blue. With a few colors, however, all the

rest may be made thus: Orange, by Yellow and Red; Purple, with Blue and

Red; Green, Blue and Yellow; Brown, with Umber, Carmine and Lamp Black;

Scarlet, Carmine and Light Red. While it is true that a little color

may relieve the dark metallic look of some Daguerreotypes, it must not

be concealed that the covering of the fine delicate outline and

exquisite gradations of tone of a good picture with such a coating, is

barbarous and unartistic.

“The prevaling taste is, however, decidedly for colored proofs, and the

following directions will assist the amateur in ministering to this

perverted taste, should he be so inclined. The coloring should commence

with the face, and the flesh tint must be stippled on (not rubbed) with

a small camel’s-hair brush, beginning from the centre of the cheek,

taking great care not to go over the outline of the face, and also not

to have too much color in the brush; the eyes and eyebrows must not be

touched with color. After the flesh color is applied, take a piece of

very soft cotton and pass it very gently backwards and forwards over

the face, so as to soften down the color, and then apply the carmine to

give the required tint. For men, the darker tints should predominate,

and for women the warmer. Very light hair may be improved by a slight

tint of brown, or yellow and brown, according to the color. In coloring

the drapery, the same care must be used. No rules can be laid down for

all the different colors required, and the amateur had better obtain

the assistance or advice of some one accustomed to the use of colors. A

little white with a dash of blue or a little silver, will improve white

linen, lace, etc. The jewelry may be touched with gold or silver from

the shells, moistened with distilled water, and laid on with a

fine-pointed sable-hair brush.

“Brilliants may be represented by picking the plate with the point of a

pin or knife.”

# CHAPTER II.

MISCELLANEOUS.

Coloring Back Grounds—Transparent ditto—Gilding Dissolvent Solution for

removing Specks—Solarized Impression—To Purify Water—Cleaning

Mercury—Adhesive Paper—Black Stain for Apparatus—Sealing Wax for

Bottles—Rouge—Rotten Stone—Potassa Solution—Hyposulphite

Solution—Substitute for do.—Gilding Solution—Solution for increasing

the Brilliancy of the Daguerreotype—Bleaching Solution;—Cold

Gilding—Neutralizing Agents—Buff Dryer—Keeping Buffs in order—Cleaning

Buckskins—Reflector for taking Views.

To Color Back-grounds—To obtain a properly colored back ground is a

matter of no little importance to the Daguerreotype operator. I had

nearly exhausted all patience, and tried the skill of painters to

obtain a back-ground that would be suitable to my purpose; but all to

no avail. At last I adopted the following method, and at a cost of

coloring of twenty-five cents, can now produce a back-ground far more

valuable than those which had cost five dollars before.

Take common earth paint, such as is used in painting roofs; mix this

with water to about the consistency of cream; then to four quarts of

this mixture add about one pint of glue water (common glue dissolved in

water, also about as thick as cream). This last will cause the paint to

adhere to the cloth, to which it is applied with a common white-wash

brush. By applying the brush on the coating while it is wet, it may be

so blended that not a line can be seen, and a perfectly smooth color of

any shade can be obtained. The shade of color I use is a light

reddish-brown. Tripoli, rotten-stone, or any earthy matter, may be

applied in the same manner.

Transparent or Invisible Back-ground.—I give this as originally

published in my System of Photography, 1849:

“Take a large woollen blanket with long nap, the longer and rougher it

is the finer will be the effect produced; stretch it on a frame of

sufficient size, and suspend the frame at the centre of the upper end

by a string fastened to a nail in the ceiling, from three to five feet

back of the sitter. Having arranged this, fasten another string to the

side of the frame, and while the operation is going on in the camera,

swing the back-ground from right to left, continuing this during the

whole time of sitting, and you have a clear “transparent” back-ground,

which throws the image out in bold relief, and renders the surface of

the plate invisible. If equalled at all it is only by atmospheric

back-ground. I consider it to be the best ever known, and think it

needs but to be tried to afford satisfactory proof that it is so.

Although used by few before, since the first edition of this work at

least two thirds of the operators have adopted its use; for any one can

at once understand the principle and the effect which it produces.”

It may be added that a motion imparted to to any back-ground where

softness is desired, produces an excellent effect.

Gilding Dissolvent.—To one quart of muriatic acid add as much oxide of

iron (common iron rust) as it will dissolve in two days. This may be

done by putting in the oxide in excess. It should be frequently shook,

and when wanted for bottling it should be allowed to stand in order to

settle. When this is done the solution may be poured off, and reduced

by adding to it an equal quantity of water; then it is ready for use.

This constitutes a gilding dissolvent now in our market.

Solution for Removing Specks.—There is probably no one cause of

complaint so general as “what makes those black specks?” There are

several causes which produce them, and probably the most general are

dust, rouge, or a spray of moisture on the plate. It this be the case,

there is no solution which can remove them, as they have prevented a

chemical action with the silver, and their removal would only expose

the surface of the plate which in itself would afford a contrast with

the impression. Another and less dangerous source of these specks is

organic matter contained in the solution employed in dissolving the

chemicals, or the water in washing. Much of the hyposulphite of soda in

market contains a sulphuret, which, coming in contact with the silver

surface, immediately causes oxidation. Such spots, as well also as most

all others found on the plate after it has been exposed in the camera,

can be removed by the following, solution: To one ounce of water add a

piece of cyanide of potassium the size of a pea; filter the solution

and apply by pouring it on the surface of the plate. In all cases the

plate should first be wet with water. Apply a gentle heat, and soon the

spots disappear, leaving the impression clear and free from all organic

matter.

In the absence of cyanide of potassium, a solution of pure hyposulphite

of soda will answer as a fair substitute.

To Redeem, a Solarized Impression.—The Daguerreotype plate, prepared in

the ordinary manner, should be exposed in the camera a sufficient time

to solarize the impression. Then, before it be exposed to the vapor of

mercury, expose it for a very brief period to the vapor of either

chlorine, bromine or iodine. Then expose over mercury, as usual. I have

produced singularly interesting results by this process.

To Purify Water.—Filter the water well, and then add about three drops

of nitric acid to the pint. This can be used as absolutely pure water,

but I would recommend the use of distilled water as preferable.

Cleaning Mercury.—Make a small bag of chamois skin, pour in the

mercury, and squeeze it through the leather. Repeat this several times,

and filter by means of a funnel made of paper, with a very small

aperture, through which it will escape and leave the particles of dust,

or other substances, in the paper. A paper with a pinhole through it

will answer as well, and it is less difficult to make.

Adhesive Paper.—Take gum arabic, four ounces, put it in a wide-mouthed

bottle and pour on water about one-third above the gum. Add half ounce

of isinglass, or fish glue, and a small piece of loaf sugar. Let all

dissolve, and spread over French letter paper, with a brush or piece of

sponge. If once spreading is not enough, perform the same operation a

second time.

Black Stain for Apparatus.—Dissolve gum shellac in alcohol, or procure

shellac varnish at the druggists’, stir in lampblack, and apply with a

sponge or bit of rag. This will adhere to metal, as well as wood, and

is used for the inside of camera, tubes, etc.

Sealing Wax for Bottles.—Melt together six parts rosin and one beeswax,

and add a small quantity of lampblack; or, if red is preferable, add

red lead. Common white wax is best, as most chemicals act less upon it.

When bottles containing bromine are to be sealed, it is well to grease

the stopper. This, however, only when the bottle is in frequent use,

for if it were to be sent by any conveyance it would be likely to fly

out.

Rouge.—The method employed by Lord Ross is probably unsurpassed in the

production of rouge. He has given his process as follows:

“I prepare the peroxide of iron by precipitation with water of ammonia,

from a pure dilute solution of sulphate of iron; the precipitate is

washed, pressed in a screw press till nearly dry, and exposed to a heat

which in the dark appears a dull, low red. The only points of

importance are, that the sulphate of iron should be pure, that the

water of ammonia should be decidedly in excess, and that the heat

should not exceed that I have described. The color will be a bright

crimson inclining to yellow. I have tried both potash and soda, pure,

instead of water of ammonia, but after washing with some degree of

care, a trace of the alkali still remained, and the peroxide was of an

ochrey color, till overheated, and did not polish properly.”

Care should be observed to apply rouge in a dry state to the surface of

the plate.

# I would remark, that so far as my experience has gone, I consider good

rouge fully equal to any other polishing, material for the last or

finishing polishing; consequently I shall not take up my space in

enumerating any of the great variety that find few advocates.

Why Rouge is to be preferred.—“Because it burnishes better, and because

it assists in fixing the layer of gold, rendering it less susceptible

of being removed in scales when heated too much.”

Rotten Stone.—“Purchase the best ground rotten stone of the druggist,

put a few ounces at a time in a wedgewood or porcelain mortar, with

plenty of clean rain water. This should have about forty drops of

nitric acid to the quart. Grind well, and after letting the mortar

stand two minutes, pour into a third. After remaining undisturbed eight

minutes, finally pour off into a fourth to settle. Rinse back the

sediment in the second and third, and grind over with a new batch.

Repeat the operation till you have all in the fourth vessel. Let this

stand several hours, and pour off the water very carefully. Set the

deposit in the sun, or by a stove to dry. When perfectly dry,

pulverize, and it is ready for use. With a little trouble you will

obtain in this way a much better article than can generally be bought

of dealers. For the last washing, alcohol, or a mixture of alcohol and

water, is preferable.”

Potassa Solution.—The use of a solution of potassa in the preparation

of the plate was suggested in the early history of the Daguerreotype.

It was thought to possess some peculiar property for improving the tone

of the impression. It is used for moistening the rotten stone in

polishing the plate, and may be prepared by putting about an ounce and

a half of alcohol in a close bottle, and add half a stick of caustic

potash. This will soon become of a deep red color. For use, fill your

small bottle, having a quill in the cork, with alcohol, and add a few

drops of the above, or enough to change it to a bright orange or

saffron color.

A Substitute for the Hyposulphite Solution.—M. DAGUERRE recommends the

use of a solution of salt water for removing the coating off the plate.

# I found this of some service at one time during my travels. My

hyposulphite bottle got broke and its contents lost, so as only to

leave enough for preparing gilding. I resorted to the use of salt

solution, and found it to answer well. Make a saturated solution of

salt in water. First wash the plate with clear water; then immerse it

in the saline solution, when it should be agitated, and the coating

will soon disappear. Another process with a salt solution of half the

strength of the above is very interesting and effectual. The plate

having been dipped into cold water, is placed in a solution of common

salt, of moderate strength; it lies without being acted upon at all;

but if it be now touched on one corner with a piece of zinc, which has

been scraped bright, the yellow coat of iodine moves off like a wave

and disappears. It is a very pretty process. The zinc and silver

forming together a voltaic pair, with the salt water intervening,

oxidation of the zinc takes place, and the silver surface commences to

evolve hydrogen gas; while this is in a nascent condition it decomposes

the film of iodide of silver, giving rise to the production of

hydriodic acid, which is very soluble in water, and hence instantly

removed.

This process, therefore, differs from that with hyposulphite. The

latter acts by dissolving the iodide of silver, the former by

decomposing it. It is necessary not to leave the zinc in contact too

long, or it deposits stains, and in large plates the contact should be

made at the four corners successively, to avoid this accident.

Gilding Solution.—To one pint of pure rain or distilled water add

fifteen grains of pure chloride of gold, and to another pint add sixty

grains of hyposulphite of soda. When dissolved, pour the gold solution

into the hyposulphite by small quantities, shaking well after each

addition. The soda solution must not be poured into the gold, as the

gold would be immediately decomposed, and the solution turn black, and

be unfit for use.

Some operators add muriate of potash and other substances, but these do

not possess any advantage except in cases where it is necessary to

bleach the solarized portions of the impression, and when such is the

case, chloride of sodium (common salt) is probably as effective and is

the most convenient. Add about a teaspoonful to two ounces of the

gilding.

Solution, for Increasing the Brilliancy of the Daguerreotype.—This

solution will have the effect to thoroughly cleanse the surface of the

gilded plate and excite a powerful influence on the general character

of the impression. To a solution of three ounces of water, in which is

dissolved a quarter of an ounce of cyanide of potassium, add one

teaspoonful of a solution containing six ounces of water and half an

ounce of each pure carbonate of potash, alum, common salt, gallic acid,

sulphate of copper, and purified borax. While the plate is wet, pour on

a little, and heat it with a powerful blaze. The effect will be quickly

produced, in from three to fifteen seconds. Rinse and dry, as in the

gilding.

Bleaching Solution.—Make a saturated solution of muriate of ammonia

(sal ammoniac) in pure water, and filter through paper. Reduce with an

equal quantity of water when used. When the linen or any other portion

of the impression is badly solarized, after removing the coating, rinse

with water; then pour this upon the surface in the same manner as the

gilding solution. If the solarization be very deep, apply the lamp

beneath, and warm the plate a trifle. Now pour off, and, without

rinsing, apply the gilding. The whole operation must be quickly

performed, or the chlorine soon attacks the shades of the picture. When

properly done, however, the solarized parts are restored to a clear,

transparent white.

Electro, or Cold Gilding.—This process I have adopted, and it produces

exceedingly beautiful impressions for the stereoscope, adding a great

charm to the pleasing effect of that instrument. It also possesses a

pretty and curious effect on views. It is easy of trial, and may be

used by dissolving one gramme of chloride of gold in half a litre of

ordinary water, and thirty grammes of hyposulphite of soda in another

half litre of similar water; then pour the solution of chloride of gold

into that of soda, by little and little, agitating it exactly as in M.

Fizeau’s preparation, of which there is but a variation.

When you wish to use it, pour some into a plate, or any other vessel of

the same kind, sufficient to cover the proof; then, after having added

to it a drop of ammonia, immerse the plate in it as soon as you take it

out of the mercury-box, after having wiped its back and edges, and

agitate the mixture quickly from right to left, so as to dissolve

rapidly the coating of iodide of silver as usual. As soon as the plate

appears white, cease all rapid motion, but continue to give it a slight

undulating one; for if it were allowed to remain still for only a few

minutes, the proof would be clouded. By little and little, the surface

of the plate takes a yellow tint, which darkens more and more,

approaching to bistre. You stop therefore, at the color you wish; and

when the proof has been washed and dried, in the manner previously

explained, it will be found to be fixed, without any stain, with a

limpid surface, and an extraordinary warm tone. If you were to augment

the proportions of the ammonia or chloride of gold, the operation would

progress much quicker, but then the middle of the proof would be always

much clearer than towards the border. The mixture may be used several

times without being renewed. It does not, however, give such a

beautiful color to the impression as when it is newly prepared. By

communicating to the vessel containing the solution a continual motion,

the impression, when once immersed, will be fixed. During that time,

and while attending to anything else, watch its color; and at the end

of ten minutes or a quarter of an hour, take it out of the bath and dry

it.

Agent for Neutralizing Bromine, Chlorine, and Iodine Vapors.—Aqua

ammonia, sprinkled about the chemical or coating room, will soon

neutralize all the vapor in the atmosphere of either chlorine, bromine,

or iodine. No operator should be without, at least, a six-ounce bottle

filled with ammonia. A little of its vapor about the camera-box has a

decided and happy effect. Burnt coffee, pulverized, has also the

property of destroying the vapors of the above chemicals, as also

almost any other agent employed about the Daguerreotype room. Its

deodorizing properties are such that if brought in contact with air

filled with the odor of decomposing meat, it will instantly destroy all

disagreeable smell. It can easily be used in the Daguerreotype room by

placing a little of the raw bean, finely pulverized, on an old plate,

and roasting it over the spirit-lamp.

Buff Dryer.—There are various methods for keeping buffs dry and free

from dust. Some place a sheet of iron against the wall at an angle

sufficient to put a lamp between it and the wall, and then let the buff

rest against the top of the sheet. By this method the buff is for its

full length close to the heated iron, and at the same time exposed to

the heated atmosphere and any dust that may be free. I would recommend

some arrangement by which the buff would be inclosed. I have found the

following to answer the purpose well, which is a box of sheet iron

twenty inches long, eight wide and five high, with one end left open

and the other closed; the cover is made of the same material, with the

edges bent over to go on and off. There are several wires running

through the centre of the sides, which it is necessary to cover with

cloth or paper to absorb all the moisture that may be made by applying

the heat, and the buffs are put in and taken out at the open end. In

order that the heat may be as nearly uniform as possible, an iron bar

one inch wide, eighteen inches long and one half inch thick, is so bent

that the centre is one quarter inch from the bottom of the box, and

that at least two inches of each end come in contact with the bottom;

this being riveted on the bottom, and a lamp with a small blaze applied

to the centre of the bar of iron. This will constitute one of the best

and cheapest buff dryers in use. It may be suspended from the wall by

placing wires around it, or it may stand upon legs. Perhaps a more

convenient plan is to place it under the workbench in a similar

position to a drawer. One precaution is necessary: when first heating

the dryer, apply but a very gentle heat. This will prevent an

accumulation of moisture, which would otherwise pass off in steam,

coming in contact with the buff, thus causing a dampness. Another

caution: never have the temperature of the air in the heater more than

ten degrees above that which surrounds it.

When wheels are used, they should be encased in a sheet iron or wood

case. All those made for our market are provided in this respect.

Keeping Buffs in Order.—This is one of the most important objects to

arrest the attention of the operator. Every buff is more or less liable

to get out of order by dust falling upon or coming in contact with the

polishing powder employed in cleaning the plate. The edge of every

plate should be thoroughly wiped and freed from any material that may

adhere while cleaning. I have adopted the following method, which

proves highly successful:

Rub the buff leather, holding the face down, with the sharp edge of a

pair of shears or a piece of glass. This brings out any portion of the

skin which may have become matted from any moisture, and also takes out

any substance imbedded in it, and prevents it from scratching. Then,

with a stiff brush, rub the buff well, and it will be found to work

well. This same process employ on wheels and hand buffs every morning,

or oftener, as occasion requires.

Preparing Buffs.—Two of these are necessary. That part of the stick to

be covered should be about eighteen or twenty inches long, and three

wide, and made crowning on the face from one end to the other, about

one half inch. Before covering, these are to be padded with two or

three thicknesses of Canton flannel. The buff should not be too hard,

but padded with flannel, so that by drawing it over the plate, it may

touch across the surface. The only proper material for buffs is

prepared buckskin; and if prepared in a proper manner, this needs

nothing but to be tacked upon the stick. There are several varieties of

wheels employed; the one most generally adopted is Lewis’ patent, which

consists of several varieties of wheels. Any operator can make a

suitable wheel on the same plan of a turning lathe.

To Clean Buckskins.—When the operator is compelled to purchase an

unprepared buckskin, the following is a good process for cleaning it:

There is always in the buckskin leather that is purchased, more or less

of an oily matter, which is acquired in its preparation, sometimes even

amounting, to a third of its weight. The following is the mode of

ridding it of this noxious ingredient: Dissolve, in about six or seven

quarts of filtered water, about five ounces of potash; when dissolved,

wash with the solution an ordinary buckskin; when it has been well

stirred in the liquid, the water becomes very soapy, owing to the

combination of the potash with the oily matters contained in the skin.

Throw away this solution and use some fresh water without potash and

rather tepid; change it several times until it remains quite limpid.

Then gently stretch the skin to dry in an airy shaded place. When

thoroughly dried, rub it well between the hands. It thus becomes very

pliant and velvet-like.

Reflectors for Taking Views.—There have been excellent cameras

introduced for taking views, but the time of exposure, which is

increased in proportion to the focal length, is considered an

objection; consequently many adhere to the old plan of using the

speculum, or rather, substitute a mirror. I now have one which I have

used for several years and find it equal to any article of the kind

have ever tried. One is easily made by a tin man, at a trifling

expense. Procure a piece of best plate looking-glass, two and a half by

five inches for a quarter, or four by eight for a half-sized camera;

put a piece of pasteboard of the same size on the back, to protect the

silvering, and stick around the edge in the same manner as in putting

up a picture. Take a sheet of tin for the large size, or a half sheet

for the other; place the glass crosswise in the centre; bend the ends

of the tin over the edge of the glass and turn them back so as to form

a groove to hold the glass, and still allow it to slide out and in.

These ends of the tin must be turned out flaring, that they may not

reflect in the glass.

Have a tin band about an inch wide made to fit close on the end of the

camera tube; place it on, and taking the tin containing the glass,

bring it to an angle of forty-five degrees with the tube, extending

nearly the whole length of the glass in front of the lenses; lap the

loose ends of the tin on each side of the tin rim, and having your

camera turned on the side to throw the view lengthwise, arrange the

exact angle by examining the image on the ground-glass. When you have

it exactly right, hold it while it is soldered fast to the band. Take

out your glass and stain the tin black, to prevent reflection.

# CHAPTER III.

CHEMICALS.

Bromine and its Compounds—Iodine and its Compounds—Chlorine and its

compounds—Cyanide of Potassium—Hyposulphite of Soda—Hyposulphite of

Gold—Nitric Acid—Nitro-Muriatic Acid—Hydrochloric Acid—Hydrofluoric

Acid—Sulphuric Acid—Accelerating Substances—Liquid Sensitives—Dry

Sensitives, etc., etc.

BROMINE.

An article so extensively used in the practice of the Daguerreotypic

art as Bromine, is deserving of especial attention, and accordingly

every person should endeavor to make himself familiar with its

properties and applications.

History.—This element was discovered in 1826 by M. Balard, in the

mother-liquor, or residue of the evaporation of sea-water. It is named

from its offensive odor (bromos, bad odor). In nature it is found in

sea-water combined with alkaline bases, and in the waters of many

saline springs and inland seas. The salt springs of Ohio abound in the

compounds of bromine, and it is found in the waters of the Dead Sea.

The only use which has been made of bromine in the arts is in the

practice of photography. It is also used in medicine In a chemical

point of view it is very interesting, from its similarity in

properties, and the parallelism of its compounds to chlorine and

iodine.

Dr. D. Alter, of Freeport, Pa., is the only American manufacturer, and

furnishes all of the “American Bromine.” Yet we understand much

purporting to be of German manufacture is prepared from that made in

Freeport. This is done by individuals in this city, who get well paid

for the deception.

For the successful application of bromine as an accelerating agent, we

are indebted to Mr. John Goddard of London, who at the time was

associated with Mr. John Johnson, now a resident of this city.

Preparation.—The mother-liquor containing bromides is treated with a

current of chlorine gas, which decomposes these salts, setting the

bromine free, which at once colors the liquid to a reddish brown color.

Ether is added and shaken with the liquid, until all the bromine is

taken up by the ether, which acquires a fine red color and separates

from the saline liquid.

{74}

Solution of caustic potash is then added to the ethereal solution,

forming bromide of potassium and bromate of potash. This solution is

evaporated to dryness, and the salts being collected are heated in a

glass retort with sulphuric acid and a little oxide of manganese. The

bromine is distilled, and is condensed in a cooled receiver, into a red

liquid.

Properties.—Bromine somewhat resembles chlorine in its odor, but is

more offensive. At common temperatures it is a very volatile liquid, of

a deep red color, and with a specific gravity of 3, being one of the

heaviest fluids known. Sulphuric acid floats on its surface, and is

used to prevent its escape. At zero it freezes into a brittle solid. A

few drops in a large flask will fill the whole vessel when slightly

warmed, with blood red vapors, which have a density of nearly 6.00, air

being one. It is a non-conductor of electricity, and suffers no change

of properties from heat, or any other of the imponderable agents. It

dissolves slightly in water, forming a bleaching solution.

Chloride of Bromine.—This as an accelerating agent is by many

considered superior {75} to the other Bromide combinations. It can be

readily prepared by passing a current of chlorine through a vessel

containing bromine. A mixture of two parts muriatic acid and one of

black oxide of manganese, should be put into a flask having a bent tube

to conduct the chlorine vapor into the bromine in another vessel. This

last vessel should also be supplied with a bent tube for conducting the

combined vapors with a third vessel or receiver. On the application of

the heat from a spirit lamp to the bottom of the flask, a current of

chlorine gas will be disengaged, and pass into the bromine, when it

readily combines, and gives off a vapor, which, when condensed in the

third vessel, forms a volatile yellowish-red liquid. It is best, even

at ordinary temperature, to place the receiver in an ice bath. For

manner of using, see farther on, under head of Accelerators.

Bromides.—A bromide treated with oil of vitriol, disengages

chlorohyadic acid; but vapors of bromine are constantly disengaged, at

the same time imparting a brown color to the gas. If the bromide be

treated with a mixture of sulphuric acid, and peroxide of manganese,

bromide is only disengaged. A solution of a bromide gives, with of

nitrate {76} silver, a light yellowish white precipitate of bromide of

silver, which is insoluble in an excess of acid, and readily dissolves

in ammonia. The precipitated bromide is colored by light like the

chloride, but is immediately tinged brown, while the chloride assumes

at first a violet hue. The bromides, in solution, are readily

decomposed and chloride being set free, colors the liquid brown.

In the whole range of heliographic chemicals there is probably not

another collection less understood and being so productive of

interesting investigation as the bromides.

Bromide of Iodine.—M. de Valicours furnishes us with the best method

for preparing this mixture:

“Into a bottle of the capacity of about two ounces, pour thirty or

forty drops of bromine, the precise quantity not being of importance.

Then add, grain by grain, as much iodine as the bromine will dissolve

till quite saturated. This point is ascertained when some grains of the

iodine remain undissolved. They may remain in the bottle, as they will

not interfere with the success of the preparation.

“The bromide of iodine thus prepared, from its occupying so small a

space, can very easily be carried, but in this state it is much too

concentrated to be used. When it is to be employed, pour a small

quantity, say fifteen drops, by means of a dropping-tube, into a bottle

containing about half an ounce of filtered river water. It will easily

be understood that the bromide of iodine can be used with a greater or

less quantity of water without altering the proportion which exists

between the bromine and iodine.”

This article forms a very good dry accelerator, and is by some persons

thought superior to all others, as it works with great uniformity, and

is less liable to scum the plate in coating at high temperatures, or

when the thermometer indicates a heat above 60 deg.

Bromide of Potassium—Is prepared by mixing bromine and a solution of

pure potass together, and evaporating to dryness; it crystallizes in

small cubes, and dissolves readily in water. This agent is extensively

employed in the paper and glass processes.

Bromide of Lime. This the principal accelerator used in the American

practice, and is the best of all dry combinations at present employed.

There are many reasons why the dry is advantageous; these are too

familiar to repeat.

“The bromide of lime may be produced by allowing bromine vapor to act

upon hydrate of lime for some hours. The most convenient method of

doing this is to place some of the hydrate at the bottom of the flask,

and then put some bromine into a glass capsule supported a little above

the lime. As heat is developed during the combination, it is better to

place the lower part of the flask in water at the temperature of about

50 deg. Fah.; the lime gradually assumes a beautiful scarlet color, and

acquires an appearance very similar to that of the red iodide of

mercury. The chloro-iodide of lime may be formed in the same manner; it

has a deep brown color. Both these compounds, when the vapor arising

from them is not too intense, have an odor analogous to that of

bleaching powder, and quite distinguishable from chlorine, bromine, or

iodine alone.”

Farther on, I have given, in connection with accelerators, a process I

adopt, which is far less tedious and equally reliable.

Bromide of Silver—May be formed by pouring an alkaline bromide into a

solution of nitrate of silver, in the shape of a white, slightly

yellowish precipitate, which is insoluble in water and nitric acid, but

readily dissolves in ammonia and the alkaline hyposulphites. Chlorine

easily decomposes bromide of silver, and transforms it into chloride.

M. Biot has expressed his opinion, that it is not possible to find any

substance more sensitive to light than the bromide of silver. This is

true to a certain extent, but in combination with deoxidizing agents,

other preparations have a decided superiority over the pure bromide of

silver.

Bromide of Gold—Is readily prepared by adding a little bromide to the

brown gold of the assayers, and allowing it to remain some time under

water, or assisting its action by a gentle heat. It forms a salt of a

bright crimson color, but in its general properties is precisely

similar to the chloride used in gilding.

Bromide of Magnesia—Is prepared in the same manner as bromide of lime.

This mixture is particularly adapted for hot climates, and is used in

this country by some few who regard its use as a valuable secret.

Bromide of Starch.—This preparation is much used, but not alone. It is

combined with lime by putting about one part in measure of starch to

four of lime. It is prepared by adding bromine to finely pulverized

starch, in the same manner as bromide of lime. (See Accelerators.)

Experiments with Bromine.—Place in a very flaring wine glass a few

drops (say ten) of bromine, then place a small piece of phosphorus

about one-twentieth of an inch in diameter. Place the latter on the end

of a stick from five to ten feet in length. So place it that the

phosphorus can be dropped into the glass, and in an instant combustion

giving a loud report will be the result.

b. Expose a daguerreotype plate to the vapor of bromine, it assumes a

leaden-grey color, which, blackens by light very readily. Exposing this

to mercury will not produce any decided action upon the lights. Immerse

it in the solution of the muriate of soda, and the parts unacted upon

by light becomes a jet black, while the parts on which the light has

acted will be dissolved off, leaving a clean coating of silver. This

will be a most decided black picture on a white ground.

c. Expose an impressioned plate, that has been sufficient time in the

camera to become solarized, to the vapors of bromine, and the

impression will be fully developed and exhibit no signs of

solarization. The exposure over the bromine most be very brief.

Chlorine or iodine will produce the same result. The latter is

preferable.

Again, should the impressioned plate be exposed too long over the vapor

of bromine, the impression would be rendered wholly insensitive to the

mercurial vapor. Hence this method is resorted to for restoring the

sensibility of the plate when there is reason to believe that the

impression would not be a desirable one; as, for example, if a likeness

of a child be wanted, and it had moved before the plate had been

sufficiently long exposed in the camera, the plate may be restored to

its original sensitiveness by re-coating over bromine, as above, thus

saving the time and labor of re-preparing the plate for the chemicals.

d. If by accident (we would not advise a trial to any extent of this),

you should inhale a quantity of the vapor of bromine, immediately

inhale the vapor of aqua ammonia, as this neutralizes the dangerous

effect of the bromine vapor. Every operator should be provided with a

bottle of ammonia, as a little sprinkled about the chemical room soon

disinfects it of all iodine or bromine vapor, and also tends to

facilitate the operation in the camera.

IODINE.

History of Iodine.—This is one of the simple chemical bodies which was

discovered in 1812 by M. Courtois, of Paris, a manufacturer of

saltpetre, who found it in the mother-water of that salt. Its

properties were first studied into by M. Gay Lussac. It partakes much

of the nature of chlorine and bromine. Its affinity for other

substances is so powerful as to prevent it from existing in an isolated

state. It occurs combined with potassium and sodium in many mineral

waters, such as the brine spring of Ashby-de-la-Zouche, and other

strongly saline springs. This combination exists sparingly in

sea-water, abundantly in many species of fucus or sea-weed, and in the

kelp made from them. It is an ingredient in the Salt Licks, saline, and

brine springs of this country, especially of those in the valley of the

Mississippi. It is sparingly found in fresh-water plants, as well also

in coal, and in combination with numerous other bodies.

Fermented liquors contain iodine; wine, cider, and perry are more

iodureted than the average of fresh waters. Milk is richer in iodine

than wine; independently of the soil, with which it varies, the

proportion of iodine in milk is in the inverse ratio of the abundance

of that secretion. Eggs (not the shell) contain much iodine. A fowl’s

egg weighing 50 gr. contains more iodine than a quart of cow’s milk.

Iodine exists in arable land. It is abundant in sulphur, iron, and

manganese ores, and sulphuret of mercury: but rare in gypsum, chalk,

calcareous and silicious earths. Any attempt to extract iodine

economically should be made with the plants of the ferro-iodureted

fresh waters. Most of the bodies regarded by the therapeutists as

pectoral and anti-scrofulous are rich in iodine.

It is probably to the application of this body that we owe the

discovery of the daguerreotype. There is no record of the precise date

when Daguerre commenced experimenting with iodine, but by the published

correspondence between him and M. Neipce, his partner, it was previous

to 1833. There is no doubt, however, that the first successful

application was made in 1838, as the discovery was reported to the

world early in January, 1839.

Preparation.—Iodine is mostly prepared from kelp, or the half vitrified

ashes of seaweed, prepared by the inhabitants of the western islands,

and the northern shores of Scotland and Ireland. It is treated with

water, which washes out all the soluble salts, and the filtered

solution is evaporated until nearly all the carbonate of soda and other

saline matters have crystallized out. The remaining liquor, which

contains the iodine, is mixed with successive portions of sulphuric

acid in a leaden retort, and after standing some days to allow the

sulphureted hydrogen, etc., to escape, peroxide of manganese is added,

and the whole gently heated. Iodine distills over in a purple vapor,

and is condensed in a receiver, or in a series of two-necked globes.

Properties.—Iodine is solid at the ordinary temperature, presenting the

appearance of dark-grey or purple spangles, possessing a high degree of

metallic lustre. It somewhat resembles plumbago, with which it is

sometimes diluted, particularly when it is fine. Operators should

endeavor to secure the larger crystals. It melts at 224.6 deg., forming

a brown or nearly black liquid. It boils at about 356 deg., and emits a

very deep violet colored vapor. It gives off a very appreciable vapor,

sufficient for all purposes of forming the iodide of silver on the

daguerreotype plate, at a temperature of 45 deg. or even lower. Iodine

crystallizes readily. Every operator has found upon the side of the jar

in his coating-box, perfectly regular crystals, deposited there by

sublimation.

Water dissolves but a small proportion of iodine, requiring 7000 parts

of water to dissolve one of iodine, {85} or one grain to the gallon of

water. Alcohol and ether dissolve it freely, as does a solution of

nitrate or hydrochlorate of ammonia and of iodides.

The density of solid iodine is 4.95; that of its vapor 8.716. It

greatly resembles chlorine and bromine in its combinations, but its

affinities are weaker. It does not destroy the majority of organic

substances, and vegetable colors generally resist its action. It

combines with several organic substances, imparting to them peculiar

colors. It colors the skin brown, but the stain soon disappears.

Chloride of Iodine—Is formed by passing chlorine into a bottle

containing some iodine. This can be readily done by pouring one ounce

and a half of muriatic acid upon a quarter of an ounce of powdered

black oxide of manganese, and heat it gradually in a flask, to which is

adapted a bent glass tube. This tube must connect with the bottle

containing the iodine, and the yellowish-green gas disengaged will

readily combine with the iodine, forming a deep red liquid, and the

operation is complete. The use of chloride of iodine will be referred

to in connection with the Accelerators.

{86}

Iodides.—The iodide treated with the oil of vitriol, instantly produces

a considerable deposit of iodine; and if the mixture be heated, intense

violent vapors are disengaged. The reaction is due to the decomposition

of oil of vitriol by iodohydric acid, water and sulphurous acid being

formed, and iodine set free. The iodides in solution are decomposed by

chlorine, iodine being precipitated, the smallest quantity of which in

solution is instantly detected by its imparting to starch an intensely

blue color.

Iodide of Potassium.[4]—This compound is easily made in the following

manner: Subject to a moderate heat a mixture of 100 parts of iodine, 75

of carbonate of potash, 30 of iron filings, and 120 parts of water.

This mass must be thoroughly dried and then heated to redness; the

resulting reddish powder is to be washed with water, and the solution

obtained filtered, and evaporated to dryness. It is found that 100

parts of iodine yield 135 parts of very white, but slightly alkaline,

iodide of potassium.

[4] I shall present the preparation of only a few iodides, and such as

are more intimately connected with the Daguerreotype.

Experiment.—On projecting dry pulverized iodide of potassium into fused

anhydrous phosphoric acid, a violent disengagement of iodine takes

place, attended by a transient ignition; fused hydrate of phosphoric

acid liberates iodine abundantly from iodide of potassium; this

reaction is accompanied by the phenomenon of flame and formation of a

considerable quantity of hydriodic acid.

Iodide of Mercury.—For the preparation of iodide of mercury, Dublanc

recommends to cover 100 grms. of mercury with 1 kilogrm. of alcohol, to

add 124 grms. of iodine gradually in portions of ten grms., and

agitating between each fresh addition, until the alcohol becomes

colorless again. After the addition of the last 4 grms. the alcohol

remains colored, the whole of the mercury having become converted into

iodide. The resulting preparation is washed with alcohol; it is

crystalline and of a hyacinth color.

Iodide of Silver.—This compound is formed upon every plate upon which a

Daguerreotype is produced. The vapor of iodine coming in contact with

the silver surface, forms an iodide which is peculiarly sensitive to

light.

The various colors produced are owing to the thickness of the coating,

and the maximum sensibility of the coating, as generally adopted, is

when it assumes a deep yellow, or slightly tinged with rose color.

This compound is largely employed in most photographic processes on

paper, and may be easily prepared by the following formula: By adding

iodide of potassium to a solution of nitrate of silver, a

yellowish-white precipitate of iodide of silver is obtained, which is

insoluble in water, slightly soluble in nitric acid, and soluble in a

small degree in ammonia, which properties seem easily to distinguish it

from the chloride and bromide of silver. Chlorine decomposes it and

sets the iodine free, and chlorohydric acid converts it into a

chloride. It fuses below a red heat. Although the effect of light on

the iodide is less rapid than on the chloride, the former sooner

turning black, assuming a brown tinge; but when in connection with

gallic acid and the ferrocyanate of potash, it forms two of the most

sensitive processes on paper.

Iodide of silver dissolves easily in a solution of iodide of potassium,

and the liquid deposits in evaporation crystals of a double iodide.

Iodide of Gold.—If a solution of potassium be added to a solution of

chloride of gold, a precipitate of iodide of gold takes place, soluble

in an excess of the precipitate. A little free potash should be added

to combine with any iodide that may chance to be set free by the

chloride of gold.

Iodide of Lime is prepared by adding iodine to hydrate of lime (which

will be referred to farther on) until the mixture assumes a light

yellow shade, when wanted for combinations with accelerators, or to a

dark brown when employed for the first coating. This latter mixture has

been sold in our market under the name of “Iodide of Brome.”

Iodide of Bromine.—(See page 76.)

Experiments with Iodine.—Place a plate which has been exposed in the

camera over the vapor of iodine for a very brief period, and it will

present the appearance of the impression having been solarized.

b. Upon a Daguerreotype plate, from which an impression has been

effaced by rubbing or otherwise, the picture may be made to reappear by

merely coating it over with iodine.

c. Place in a vessel a little water, into which put the smallest

possible quantity of free iodine and add a little starch, and the

liquid will instantly assume a blue color. Advantage is taken of this

fact in the laboratory to detect the presence of iodine in liquids. The

starch should be dissolved in boiling water and allowed to cool. There

are numerous other interesting experiments that can be performed by the

aid of iodine, but it is unnecessary here to consume more space.

CHLORINE.

History.—The Swedish chemist, Scheele, in 1774, while examining the

action of hydrochloric acid on peroxide of manganese, first noticed

this element. He called it dephlogisticated muriatic acid. It was

afterwards, by the French nomenclaturists, termed oxygenated muriatic

acid, conceiving it to be a compound of oxygen and muriatic acid. This

view of its notice was corrected by Sir H. Davy (in 1809), who gave it

the present name. In 1840-41, this gas vas employed for accelerating

the operation of light upon the iodized Daguerreotype plate. John

Goddard, Wolcott & Johnson, Claudet, Draper, Morse and others, were

among the first made acquainted with its use. Count Rumford, Ritter,

Scheele, Seebert and others, experimented with chlorine in regard to

its effect when exposed to the action of light in combination with

silver. In 1845, M. Edward Becquerel announced that he had “been

successful in obtaining, by the agency of solar radiations, distinct

impressions, of the colors of nature.”

On the 4th of March, 1851, Neipce, St. Victor, a former partner of

DAGUERRE, announced that he had produced “all the colors by using a

bath of bichloride of copper, and that a similar phenomenon occurs with

all salts of copper, mixed with chlorine.”

Preparation.—This is easily accomplished by putting about two parts of

hydrochloric (muriatic) acid on one of powdered black oxide of

manganese, and heating it gradually in a flask or retort, to which may

be adapted a bent glass tube. A yellowish-green gas is disengaged,

which being conducted through the glass tube to the bottom of a bottle,

can readily be collected, being much heavier than the air, displaces it

completely and the bottle is filled (which can be seen by the green

color); a greased stopper is tightly fitted to it, and another bottle

may be substituted.

In all experiments with chlorine, care should be taken not to inhale

the gas!

Properties.—Chlorine is a greenish-yellow gas (whence its name, from

chloros, green), with a powerful and suffocating odor, and is wholly

irrespirable. Even when much diluted with air, it produces the most

annoying irritation of the throat, with stricture of the chest and a

severe cough, which continues for hours, with the discharge of much

thick mucus. The attempt to breathe the undiluted gas would be fatal;

yet, in a very small quantity, and dissolved in water, it is used with

benefit by patients suffering under pulmonary consumption.

Under a pressure of about four atmospheres, it becomes a limpid fluid

of a fine yellow color, which does not freeze at zero, and is not a

conductor of electricity. It immediately returns to the gaseous state

with effervescence on removing the pressure.

Water recently boiled will absorb, if cold, about twice its bulk of

chlorine gas, acquiring its color and characteristic properties. The

moist gas, exposed to a cold of 32 deg., yields beautiful yellow

crystals, which are a definite compound of one equivalent of chlorine

and ten of water. If these crystals are hermetically sealed up in a

glass tube, they will, on melting, exert such a pressure as to liquefy

a portion of the gas, which is distinctly seen as a yellow fluid, not

miscible with the water which is present. Chlorine is one of the

heaviest of the gases, its density being 2.47, and 100 cubic inches

weighing 76.5 grains.

Chlorine Water.—This combination, which is used in conducting M.

Neipce’s process, can be readily prepared by conducting the gas into a

bottle containing distilled water. One part water dissolves two parts

of chlorine.

Chlorides.—The metallic chlorides are nearly all soluble in water; that

of silver and protochloride of mercury being the only exceptions. A

metallic chloride, treated with oil of vitriol, disengages chlorohydric

acid. Heated with a mixture of peroxide of manganese and sulphuric

acid, chlorine is given off, which is easily recognized by its odor and

other physical properties.

The chlorides dissolve in water; give with nitrate of silver, a white

precipitate, even in highly diluted solutions, becoming violet colored

and finally black when exposed to the light. The rapidity of the change

of color is proportioned to the intensity of the light. It is insoluble

in nitric acid, but readily soluble in ammonia; it fuses without

decomposition, forming, when cold, a tough, horny mass, and is reduced

by hydrogen and by fusion with carbonate of soda, or with resin.

Chloride of Bromine. (See page 74.)

Chloride of Iodine. (See page 85.)

Chloride of potassium.—or (Muriate of Potassa).—Dissolve half an ounce

of carbonate of potassa in water, and neutralize with muriatic acid.

Upon concentrating the solutions, cubic crystals will be obtained,

having a taste similar to common salt. They consist of potassium and

chloride, and when dissolved in water they may be regarded as muriate

of potassa.

Chloride of Lime.—Mix half an ounce of slacked lime (hydrate of lime)

with six ounces of water, and conduct into this milk of lime, with

frequent agitation, as much chlorine gas as will evolve from two ounces

of muriatic acid and half an ounce of black oxide of manganese. The

liquid clarifies by standing; may be regarded as a solution of chloride

of lime, and must be protected from the air and light. It may also be

made without putting in the water with the hydrate of lime, by merely

passing the chlorine into the hydrate of lime. This last is by some

used in preparations for accelerating the operation of taking

Daguerreotypes, but when used for this purpose it is in small

quantities.

Chloride of Calcium.—To one part of water add two parts of muriatic

acid, and add pieces of common chalk until effervescence ceases; then

filter through cotton cloth and evaporate it by placing it in all

earthen or porcelain dish, over a slow fire, to the consistency of a

syrup. When cooling, large prismatic crystals of chloride of calcium

are formed. These must be quickly dried by pressing between folds of

blotting paper and kept carefully excluded from the air, as it readily

attracts hydrogen. For most daguerreotype purposes, the syrup may be at

once evaporated to dryness. This is frequently placed in the iodine

coating box for the purpose of keeping the atmosphere dry. It is so

easily made that every operator can provide himself with it in a short

time, and at little expense.

Chloride of Gold.—Is prepared by dissolving gold in aqua regia, a

composition of one part of nitric to two parts of muriatic acid. Gold

foil is the best for our purposes; coin, however, answers, in most

cases, for the daguerreotype operator, as the alloy, being so slight is

not noticed in the gilding process. When the latter is used, it will

facilitate the operation to beat it out, forming a thin sheet, and then

cutting in small strips. Where purity is required, foil is better. The

gold is placed in three or four times its own weight of the above

acids. For this purpose, an evaporating dish is best (a common saucer

will do); a moderate heat may be applied to favor the action. The

mixture should be stirred often with a glass rod; care should be

observed not to apply too much heat, for at a temperature of about 300

deg. the chlorine would be expelled and leave a metallic precipitate,

which would require re-dissolving. Acid may at any time be added if

necessary to dissolve the gold, but it is advisable to add as little

excess as possible, as it would require more time to evaporate. After

all the gold has dissolved, and the liquid assumes a deep red color,

the solution should be allowed to cool, being stirred nearly all the

time. This salt is of a reddish-brown color. It is rarely we find in

our market good chloride of gold, as common, salt is used for the bulk;

and when the bottles are labelled “15 grains,” “20 grains,” nine-tenths

do not in reality contain exceeding five grains of chloride of gold.

The salt is mixed with the above solution when it is cooling, and gives

bright yellow crystals, which some of our uninformed operators conceive

to be the best quality.

Chloride of Silver.—(Oxide of Silver.)—Take any quantity of silver coin

or other silver, roll or hammer it thin; cut in small pieces. This in

order to save time. Put the silver in a glass or earthen vessel

(Florence flask is best); pour in nitric acid and water, about three

parts of the former to one of the latter. The operation of cutting up

the silver may be facilitated by applying a gentle heat. This blue

solution consists of oxide of silver and oxide of copper, both combined

with nitric acid. Should the operator wish a pure solution of silver,

which, however, is not always used, he may obtain it in the following

manner:

To separate the two metals contained in the above solution from each

other, put some bright copper coins into the solution and set it aside

in a warm place for three or four days, occasionally giving it a

circular motion. The separated laminae are pure silver, which is to be

digested with ammonia until it ceases to be colored blue. The silver,

after being washed and dried, is again dissolved in nitric acid, and

the liquid, diluted with water, is kept as solution, of silver.

Either of the above solutions (the one of oxide of silver and copper,

and the pure silver solution) may be prepared for use by putting them

in a bottle, with a quantity of water, and adding common fine salt, you

obtain a white curdy precipitate of chloride of silver. No matter how

much salt is used, provided enough be added to throw down all the

chloride of silver. This solution should be well agitated and then

allowed to stand for a few minutes; thus the white precipitate is in

the bottom of the bottle. When the water has become clear, pour it off

with care, leaving the sediment behind, then add a fresh quantity of

clean water, shake, let settle, and pour off as before. Repeat the same

for several times, and the excess of salt will disappear, leaving the

white precipitate, which may be drained of the water and dried in the

dark, and kept free from light and air.

CYANIDE OF POTASSIUM.

Cyanide of Potassium.—This important article is worthy the undivided

attention of every Daguerreotypist. I here give Mr. Smee’s process for

its preparation. This is from that author’s work entitled, “Electro

Metallurgy,” American edition:

“The cyanide of potassium, so often alluded to while treating of the

metallo-cyanides, may be formed in several ways. It may be obtained by

heating to a dull redness the yellow ferrocyanate of potash, in a

covered vessel, filtering and rapidly evaporating it. The objection to

this method, however, is that without great care the whole of the

ferrocyanate is not decomposed, a circumstance which much reduces its

value for electro-metallurgy. By boiling, however, the ignited residue

with spirits of wine this difficulty is said to be overcome, as the

ferrocyanate is absolutely insoluble in that menstruum, while the

cyanuret, at that heat, freely dissolves, and is as easily re-deposited

on cooling.

“There is, however, a much better process by which this salt may be

formed, viz. by simply transmitting hydrocyanic acid through potassium.

Although the modes of making this acid are very numerous, there is but

one which is likely to be employed on a very large scale, and that is

its formation from the yellow ferrocyanate by means of sulphuric acid.

This process is performed as follows: any given weight of the yellow

salt is taken and dissolved in about five times its weight of water;

this is placed in a retort, or some such analogous vessel, to which is

then added a quantity of strong sulphuric acid, twice the weight of the

salt, and diluted with three or four times its quantity of water. A

pipe is carried from the neck of the retort to the receiving bottle,

which should be kept as cool as possible.

“For small operations, those invaluable vessels, Florence flasks,

answer well: a bent tube being connected at one end to its month, the

other passing into the second vessel; heat should be cautiously applied

by means of an Argand lamp, a little vessel of sand being placed under

the flask, which helps the acid to decompose the salt. Prussic acid is

then generated and passes through the tube to the recipient vessel,

which is to be charged with liquor potassae.

“When the potash is saturated, the operation is completed. The Germans

recommend a strong, alcoholic solution of potassa to be used in the

second vessel, for in this case, the hydrocyanic or prussic acid

combines with the potassa, forming a hydrocyanate of potassa, or, the

water being abstracted, the cyanuret of potassium, which spontaneously

precipitates, on the saturation of the fluid, the cyanuret, being

insoluble in strong alcohol. The ferrocyanate of potash may be

considered as containing three equivalents of hydrocyanic acid, two of

potash and one of iron; but, unfortunately, we can only obtain half the

acid from the salt, owing to the formation of a compound during its

decomposition which resists the action of the acid. The decomposition

of this salt taking 2 equivalents or 426 grains (to avoid fractions)

would afford 3 equivalents or 81 grains of hydrocyanic, or prussic

acid, capable of forming 198 grains of cyanuret of potassium, while in

the retort there would remain 384 grains or 3 equivalents of bisulphate

of potash, and 1 equivalent or 174 grains of a peculiar compound, said

to contain 3 equivalents of cyanogen, 1 of potassium, and one of iron

(Pereira). It is manifest that, but for this later compound, we might

double the quantity of hydrocyanic acid from the yellow salt.”

The decomposition just described is the one usually received; but too

much reliance must not be placed on its accuracy, for the analysis of

the several compounds is too difficult for the results to be fully

admitted. The residue left in the retort speedily turns to one of the

blues, identical with, or allied to, Prussian blue. This is at best a

disagreeable process to conduct, for the hydrocyanic acid formed

adheres so strongly to the glass, that, instead of being freely given

off, bubbles are evolved suddenly with such explosive violence as

occasionally to crack the vessel. This may be remedied as far as

possible by the insertion of plenty of waste pieces of platinum—if

platinized, so much the better, as that facilitates the escape of the

gas. The heat should be applied to every part of the vessel, and the

flame should not be allowed to play upon one single part alone. Large

commercial operations are performed in green glass or stone-ware

retorts.

“Now for one word of advice to the tyro: Remember that you are working

with prussic acid; therefore, never conduct the process in a room, the

fumes being quite as poisonous as the solution of the acid itself;

moreover, have always a bottle of ammonia or chlorine by your side,

that should you have chanced to inhale more than is pleasant, it will

be instantly at hand to counteract any bad effects. It is stated by

Pereira, that a little sulphuric acid or hydroferrocyanic acid passes

to the outer vessel, but probably the amount would be of no consequence

for electro-metallurgy, otherwise, it might be as well to use a

Woulfe’s apparatus, and discard the salt formed in the first vessel. To

the large manufacturer it may be worth considering whether some other

metallo-cyanuret, formed in a similar manner to the ferrocyanuret,

might not be more advantageously employed, because the residue of the

process last described contains a large quantity of cyanogen which the

acid is unable to set free.

“There are other modes of procuring prussic acid, besides the one which

has been so tediously described; but these are found to be more

expensive. The only one which I shall now notice is the process by

which it is obtained from bicyanide of mercury. The bicyanide of

mercury itself is formed when peroxide of mercury is digested with

Prussian blue, the peroxide of mercury abstracting the whole of the

cyanogen from the blue, and leaving the oxides of iron at the bottom of

the vessel. The solution may be evaporated to dryness, and one part of

the salt dissolved in six of water; one part of muriatic acid, sp. gr.

1.15, is then added, and the solution distilled, when the whole of the

hydrocyanic acid passes over, and by being conducted into a solution of

potassa, as in the former process, forms cyanuret of potassium. This

process, though easier than the first described, is rather given as a

resource under peculiar circumstances than as one to be adopted by the

large manufacturer. The expense is the only objection, but in a small

quantity this cannot be a consideration.

“In giving this very rough outline of the general mode of forming

salts, the minutiae necessary for chemical work have altogether been

avoided, and those parts alone are entered upon which are more

immediately necessary for the electro metallurgist to know and practice

for himself. This will account for the long description of the cyanuret

of potassium, while the preparation of the equally important and even

more used acids, the sulphuric, muriatic, etc., commonly found in

commerce, are altogether neglected.

“In using solutions of cyanide of potassium, the workman should not

immerse his arms into them, otherwise it occasionally happens that the

solution produces very troublesome eruptions over the skin.”

HYPOSULPHITE OF SODA.

Hyposulphite of Soda.—This salt forms one of the important chemicals

for the Daguerreotype operator. Its application to this art is of an

interesting nature. It is used to dissolve the sensitive salt of silver

which remains unchanged during the exposure in the camera. It has the

property of readily dissolving the chloride, bromide and iodide of

silver. It should be pure and free from sulphuret of sodium; should

this last be present, it will cause brown spots of sulphurated silver

upon the Daguerreotype impression. This annoyance is a great source of

complaint from many operators, and ever will be, so long as it is

prepared by men who have no reputation to lose, and whose eyes are

blinded by the “Almighty Dollar.”

A good article may be prepared as follows:

“Mix one pound of finely pulverized carbonate of soda with ten ounces

of flowers of sulphur, and heat the mixture slowly in a porcelain dish

till the sulphur melts. Stir the fused mass, so as to expose all its

parts freely to the atmosphere, whereby it passes from the state of a

sulphuret, by the absorption of atmospheric oxygen, into that of a

sulphite, with the phenomenon of very slight incandescence. Dissolve in

water, filter the solution, and boil it immediately along with flowers

of sulphur. The filtered concentrated saline liquid will afford, on

cooling, a large quantity of pure and beautiful crystals of

hyposulphite of soda.”

Hyposulphite of Gold.—This compound salt is by a few considered

preferable to the chloride of gold, but our experience has induced us

to use the latter, believing we are enabled to produce a more brilliant

and warm-toned impression with it. When the hyposulphite of gold is

used in gilding, it requires less heat and a longer application, as

there is some danger of producing a glossy scum over some parts of the

surface of the plate. I prepare this salt as follows:

Dissolve one part chloride of gold and four parts hyposulphite of soda

in equal quantities of distilled water: pour the gold into the

hyposulphite solution, in the same manner as in mixing the gilding

solution; let it stand until it becomes limpid; filter and evaporate to

dryness. Re-dissolve and add a few grains of burnt alum.

After standing a few hours, filter and evaporate again. If not

sufficiently pure, repeat the crystallization until it is so. For

gilding, dissolve in water and use in the same manner as the common

gilding solution.

N.B.—The four following mixtures were employed in Neipce’s process in

his earliest experiments:

Aqueous Solution of Bichloride of Mercury.—Eight grains of bichloride

of mercury in 10,000 grains of distilled water.

Solution of Cyanide of Mercury.—A flask of distilled water is saturated

with cyanide of mercury, and a certain quantity is decanted, which is

diluted with an equal quantity of distilled water.

Acidulated White Oil of Petroleum.—This oil is acidulated by mixing

with it one tenth of pure nitric acid, leaving it for at least 48

hours, occasionally agitating the flask. The oil, which is acidulated,

and which then powerfully reddens litmus paper, is decanted. It is also

a little colored, but remains very limpid.

Solution of Chloride of Gold and Platinum.—In order not to multiply the

solutions, take the ordinary chloride of gold, used for fixing the

impressions, and which is composed of 1 gramme of chloride of gold and

50 grains of hyposulphate of soda, to a quart of distilled water.

With respect to chloride of platinum, 4 grains must be dissolved in 3

quarts of distilled water; these two solutions are mixed in equal

quantities.

Acids.—I shall not go into the preparations of the various acids

employed in the Daguerreotype. This would be useless to the operator,

as there are few, if any, that it would be advisable to prepare. It is

only necessary for the experimenter to be made acquainted with their

properties, and this in order to prevent any haphazard experiments,

which are too common among operators. Any person who may be desirous to

try an experiment, should first study the agents he wishes to employ.

By so doing much time and money will be saved; while the searcher after

new discoveries would rarely become vexed on account of his own

ignorance, or be obliged to avail himself of the experience of others

in any department of science.

Nitric Acid—Exists in combination with the bases, potash, soda, lime,

magnesia, in both the mineral and vegetable kingdoms, and is never

found insoluble. It has the same constituents as common air, but in

different proportions. The strongest nitric acid contains in every

pound, two and a quarter ounces of water. Pure nitric acid is

colorless, with a specific gravity of 1.5, and boiling at 248 deg.. It

is a most powerful oxidizing agent, and is decomposed with more or less

rapidity, by almost all the metals, to which it yields a portion of its

oxygen.

The nitric acid of commerce, is generally the article used by the

Daguerreotypist. This usually contains some chlorine and sulphuric

acid. It is obtained by the distillation of saltpetre with sulphuric

acid. It is employed in the Daguerreotype process for dissolving

silver, preparing chloride or oxide, nitrate of silver, [the former

used in galvanizing,] and in combination with muriatic acid for

preparing chloride of gold, used in gilding. It is also used by some

for preparing the plate.

Acidulated Solution.—This solution is used for cleaning the surface of

the Daguerreotype plate. It has the property of softening the silver,

and bringing it to a state in which it is very susceptible of being

either oxidized or iodized, hence it contributes to increase the

sensibility of the plate. The proportions are to one drop of acid add

from 15 to 20 drops of water, or make the solution about like sharp

vinegar to the taste.

Nitro-Muriatic Acid.—Aqua Regia is a compound menstruum invented by the

alchemists for dissolving gold. It is composed of colorless nitric acid

(aqua-fortis) and ordinary muriatic acid; the mixture is yellow, and

acquires the power of dissolving gold and platinum. These materials are

not properly oxidized; it nearly causes their combination with

chlorine, which is in the Muriatic acid.

Hydrochloric Acid (Muriatic Acid).—This acid forms a valuable addition

to the chemicals employed by the practical Daguerreotypist. This acid

is formed by acting upon common salt (which is chloride of sodium) by

concentrated sulphuric acid. The water of the acid is decomposed, and

its hydrogen combines with the chloride of the salt to form muriatic

acid, and this unites with the sulphuric acid to form sulphate of soda;

60 parts of common salt and 49 parts of concentrated sulphuric acid,

afford, by this mutual action, 37 parts of muriatic acid and 72 parts

of sulphate of soda. The muriatic acid of commerce has usually a

yellowish tinge, but when chemically pure it is colorless. The former

is commonly contaminated with sulphurous acid, sulphuric acid,

chlorine, iron, and sometimes with arsenic.

Muriatic acid, from the fact of the presence of the chlorine, is used

in the Daguerreotype process for dissolving gold, and in combination

with various accelerators. Its presence can be detected by ammonia. A

strip of paper dipped in this and waved to and fro will emit a thick

white smoke if the acid vapor be in the atmosphere. The ammonia

neutralizes the acid fumes. By reversing the experiment we can

determine whether vapor of ammonia be in the air, and also deprive

these suffocating and dangerous gases of their injurious properties,

and remove them from the air. Every Daguerreotype operator should be

furnished with, at least, a six ounce bottle of aqua ammonia. Its

operation is very nearly the same on bromine and iodine vapor.

Hydrofluoric Acid (Fluorohydric Acid).—This acid is used to form some

of the most volatile and sensitive compounds employed in the

Daguerreotype. It is one of the most dangerous bodies to experiment

with: it is volatile and corrosive, giving off dense white fumes in the

air. It combines with water with great heat. At 32 deg. it condenses

into a colorless fluid, with a density 1.069. It is obtained from

decomposition of fluorspar by strong sulphuric acid. It readily

dissolves the silica in glass, and consequently cannot be kept in a

vessel of that material. It is prepared and kept in lead. It is

employed in accelerators on account of its fluorine.

One small drop on the tongue of a dog causes death. The operator who

wishes to use it should pour some of the liquid for which he intends it

into a graduate, or other vessel, and then add the desired quantity of

acid. If by accident any of the spray should fall upon the skin, it

should at once be copiously drenched with water.

Sulphuric Acid.—There are two sorts of this acid: one is an oily,

fuming liquid; this is made in Nordhausen, in Saxony, and is commonly

called “Nordhausen sulphuric acid,” or oil of vitriol. The other which

is the kind used in connection with the Daguerreotype, is common

sulphuric acid. It is somewhat thinner, and when undiluted is not

fuming. This acid may be obtained in a solid and dry state, called

anhydrous sulphuric acid.

The common sulphuric acid is made by burning sulphur, which forms

sulphurous acid. To convert this into sulphuric acid and gain more

oxygen, nitric acid, which is rich in that body, is added. It forms a

limpid, colorless fluid, of a specific gravity of 1.8. It boils at 620

deg.; it freezes at 15 deg. It is acrid and caustic, and intensely acid

in all its characters, even when largely diluted.

Its attraction for basis is such that it separates or expels all other

acids, more or less perfectly, from their combinations. Its affinity

for water is such that it rapidly absorbs it from the atmosphere, and

when mixed with water much heat is evolved. It acts energetically upon

animal and vegetable substances, and is a poisonous, dangerous

substance to get on the skin. It is a powerful oxidizing agent; hence

its use in the galvanic battery, for which purpose it is mostly used by

the Daguerreotypist. The fumes of this being so much more offensive

than nitric acid, the latter is sometimes used. It is also employed in

some of the more sensitive accelerators.

ACCELERATING SUBSTANCES.

Remarks on the Accelerating substances Used in the Daguerreotype.—I

have now arrived at a point in this work, where the eye of the

Daguerreotype public will intently search for something new. This

search will prove in vain, at least so far as regards those who have

enjoyed and embraced the opportunities for studying the principles of

our art. Every experienced operator has in a degree become familiar

with the mechanical uses of all the agents employed, while I fear but

few understand the properties, and laws governing those properties,

which are so indispensable to produce an image impressed upon the

silver surface.

There are three substances which form the bases for producing a

Daguerreotype; silver, iodine and bromine. Each forms a separate body

which is indispensable to the operators success as the art is now

practiced in America. With these three, compounds of great variety are

formed.

The silver surface is first thoroughly cleaned and freed from all

organic matter, then exposed to vapor of iodine, producing an iodide of

silver. The plate upon which is this salt, is again exposed to the

vapor of bromine, forming a bromo-iodide of silver, a salt also.

As most of the various accelerators are compounds of bromine, with

either chlorine or fluorine combination, they partake somewhat of the

nature of these latter, giving results which can be detected by the

experienced operator. Thus muriatic acid is added for its chlorine,

which can generally be detected by the impression produced, being of a

light, soft, mellow tone, and in most cases presenting a brilliant

black to that colored drapery. Those who wish to experiment with agents

for accelerating substances, should first study to well understand

their peculiar nature and properties; as well, also, to endeavor to

find out what will be the probable changes they undergo in combination

as an accelerator. This should be done before making the experiments.

From the foregoing it will be seen that numerous compounds are formed

from the same basis, and, consequently, it would be a waste of time and

a useless appropriation to devote more of our space than is necessary

to give the principal and most reliable combination.

In America, the words “Quick” and “Quick Stuff,” are more generally

used for and instead of the more proper names, “Sensitives,” or

“Accelerators,” etc. As it has by use become common, I frequently use

it in this work.

Liquid Accelerator, No. 1.—This mixture was used by me in 1849, and is

given as it appeared in my “System of Photography,” published at the

above date:

Take pure rain or distilled water, one quart, filter through paper into

a ground stopper bottle, and add, for warm weather, one and a half

ounce chloride of iodine; or for cold, one ounce; then add one ounce

bromine, and shake well. Now with care not to allow the vapor to

escape, add drop by drop, thirty drops of aqua ammonia, shaking well at

each drop. Care must be taken not to add more at a time, as it evokes

too much heat. This mixed, in equal proportions with John Roach’s

quick, forms an excellent chemical combination. For this purpose, take

one and a half ounce of each, to which add ten ounces water, for warm

weather, or from six to seven for cold. Pour the whole into a large

box, and it will work from two to four months. I am now using (1849)

one charged as above which has been in constant use for three months,

and works uniformly well. The above is right for half or full size

boxes, but half of it would be sufficient for a quarter size box.

Coat to the first shade of rose over iodine, change to a deep rosy red

over quick, and black about one tenth the first.

# I would not now recommend the addition of “John Roach’s quick,” as I

believe equally good results can be produced without it. This liquid is

now used by many, and is very good for taking views.

Lime Water Quick.—This mixture is more used at present than all the

other liquids ever introduced. It produced the most uniform results,

giving the fine soft tone so characteristic in pictures produces from

accelerators containing chlorine. To one quart of lime water (this can

be had of any druggist) add one and a half ounce of pulverized alum.

This should be shook at intervals for twenty—four hours; then add one

ounce of chloride of iodine and three fourths ounce of bromine.

Lime Water.—This is easily prepared by putting lime into water, say a

piece of quick-lime about the size of an egg into one quart of water.

This should be shook occasionally for two or three days and allowed to

settle, when the water can be poured off and used.

Use.—To one part of quick add six parts of water; coat to a light

yellow over the iodine, to a rose color over the quick, and recoat

about one tenth. The above coating may be increased or diminished, it

matters not, so that there is not too much, and the proper proportions

are preserved. Some add to the above a small quantity of magnesia, say

about a teaspoonful to the quart of liquid.

Liquid Accelerator, No. 2.—The following was for a long time used by

one of the first houses in the United States, and probably was one of

the first liquids ever used. It produces a fine-toned picture, but is

not considered as sure as the lime water quick:

Take rain water one quart, add pulverized alum until it is a little

sour to the taste, and a small piece, say one half inch square, of

magnesia. Filter through paper, and add chloride of iodine one half

ounce, bromine sufficient to take it up, which is a little less than

half an ounce.

Charge with one of quick to six of water; coat over iodine to a soft

yellow, nearly, but not quite, bordering on a rose; over quick to a

dark purple, or steel, and back one sixth to one tenth.

Wolcott’s American Mixture.—Van Loan Quick.—This mixture was first

formed and used by T. Wolcott & Johnson and gained great celebrity for

its productions. I have now a bottle hermetically sealed that contains

about a half ounce of this mixture prepared in 1841 by John Johnson,

now a resident of this city, and the former partner of Mr. Wolcott. The

preparation of this mixture, as furnished by Mr. Johnson himself, is

given as follows:

“One part of bromine, eight parts of nitric acid, sixteen parts of

muriatic acid, water one hundred parts. This mixture should be allowed

to stand for several days; it improves by age.

“Use.—A few drops say, 6 to 12, of this mixture, should be put into

about 6 or 8 ounces of water; it will require frequent replenishing by

the addition of a few more drops. The plate should be coated over the

dry iodine to a red just bordering on a slate, and then exposed to the

mixture only sufficiently long to change the color. If this is not done

in less than six seconds it is not strong enough. Re-coat over the

iodine full one fourth as long as first coating.”

This exceedingly volatile compound is difficult to control from its

instability; it is but little used. The impressions successfully

produced by this mixture are very brilliant, and possess a pleasing

peculiarity.

DRY SENSITIVES.

Hydrate of Lime.—The operation by which water is combined with lime is

called slaking. Take a piece of quick lime, common lime used in mortar,

and immerse it in warm water for about fifteen seconds; then place it

in an iron or tin vessel. It will soon begin to swell, evolving a great

deal of heat and emitting steam, and soon falls into a fine powder,

hydrate of lime. This should be well stirred and allowed to cool, and

then bottled in order to prevent it from giving off the hydrate and

recovering the carbonic acid from the atmosphere. The last is

detrimental to its use with bromine, and is one cause of the complaint

that “it will not take bromine.” The hydrate of lime should, not be

dried over a heat, as has been supposed by many, for in that case the

hydrogen is expelled and it returns to a carbonate. It is advisable to

cool it in a damp place like a ground cellar. Much of the lime in our

market will not, except it be quite damp, combine with the bromine.

This is owing to impurities. Nothing is equal to oyster-shell lime,

which I use altogether.

Bromide of Lime.—In preparing large quantities of this, we adopt the

following method: Fill a four-quart bottle about two-thirds full of

hydrate of lime; pour into this about one or two ounces of bromine;

then shake well, add more of the bromine, shake well and let it stand

for a few hours, adding sufficient bromine to give it a fine red color.

It is better when kept in the large bottles, as it forms a more perfect

combination: in other words it improves by age.

Use.—Coat over the iodine to a rose red and then over this mixture to a

purple or slate; recoat over the first about one fourth as long as

first coating.

Gurneys American Compound.—Of this compound there are two combinations,

one for use, when the temperature of the atmosphere is above 65 or 70

deg., and the other at a lower temperature. The first is called No. 1,

the second No. 2.

No. 1 is prepared by placing hydrate of lime in a bottle, say to three

quarts of the hydrate of lime, add one ounce of pulverized burnt alum,

and as much chloride of lime as can be put on a quarter of a dollar,

and from 15 to 30 grains of dry pulverized iodine, or enough to change

the color of the hydrate of lime, to the slightest possible tinge of

yellow. There had better be less than carry the color to a deeper

shade. The object of using the iodine is to form a compound with

bromine that is not so volatile as the bromine itself. No matter how

little iodine is combined with the bromine, the vapors possess their

relative proportion; hence, only enough iodine to prevent “flaring,” or

as it is often termed a “scum-coating,” is used. The iodine should be

thoroughly combined with the lime, which will take about one or two

days. Should add bromine the same as in bromide of lime, until the

compound assumes a light red color.

No. 2 is prepared in the same manner as No. 1, except the addition of

the iodine, which is omitted.

Use.—No. 1. Coat over the iodine to a bright yellow color, then over

the compound, No. 1, to red color, recoat over iodine, about one sixth

as long, as the time occupied in first coating.

No. 2. Coat over iodine same as above, except recoat over the iodine

about one fourth to one half as long as first coating.

Dry Quick, No. 1.—Bromide of Lime and Starch.—The following compound

forms an excellent accelerator, and is used by many. It is claimed for

this preparation, that it will hold the bromine longer than others

where starch is not employed. As regards this claim we do not think it

can be substantiated. Our experience in practice has led us to the

conclusion that there is no great difference as respects durability,

but there is some little difference as regards the tone of the

impressions produced by its use.

To one quart of hydrate of lime add one quart of finely pulverized

starch. To this mixture add bromine, until it assumes a deep yellow or

pink color.

Starch may be added to any of the dry mixtures.

Use.—Coat over the iodine to a deep yellow, then over this quick to a

red color, recoat about one sixth of the time of first coating.

# I will here again remark, that the exact color of the coating is not

essentially provided a proper proportion is preserved.

# I have never seen it stated, though it be a fact worthy of note, that a

proportionate time for coating over the iodine and accelerator, will

not answer. For example: if a plate exposed to the vapor of iodine be

perfectly coated in sixteen seconds, and then exposed to an

accelerator, (not having iodine in its combination) receives its

coating in four seconds, it will be found that a proper proportionate

coating cannot be preserved by adopting, a proportion of time, but on

the contrary, the time will diminish; for exposure over the

accelerator, as in the above example, if it be desired to coat the

plate with twice as much iodine as in the above example, the time would

be, over iodine thirty-two seconds, and over the accelerator (to

possess a proper proportion) from six to seven seconds. Hence it is

that many inexperienced operators, when wishing to vary their usual

manner of coating, fail in producing a favorable result. They coat

calculating a proportion of time when they should not.

Dry Quick, No. 2.—Bromide of Lime and Magnesia.—To one quart of hydrate

of lime add one quart of magnesia, and mix them well together; add

bromine same as in preparing bromide of lime; coat the same as over dry

quick No. 1. This combination produces very uniform results, and is

worked with much success by beginners.

Chloro-Bromide of Lime.—To the bromide of lime add chloride of bromine

until the mixture becomes a pale yellow color, resembling sulphur. It

should be shook well, and enough of the chloride of bromine added to

bring the compound to a deep blood red color.

Use.—Coat over the iodine to a pink color, and then over the above to a

red, or just changing the color. It should be remembered that

accelerators containing chlorine do not admit of a great change of

color of coating on the plate.

Iodide of Starch.—This mixture can be employed for coating over in warm

weather, and prevent the flashing resulting at high temperatures. It

may be used the same as the iodide alone.

To six ounces of finely pulverized starch, add one fourth ounce of dry

iodine.

Use.—Same as the dry iodine alone.

The same combination may be made with lime, magnesia and other

substances.

Concentrated Solution of Iodine for First Coating.—It may appear

strange to some of our old operators that an aqueous solution of iodine

can be used for coating the plate and forming the iodide of silver. It

has long been a cry among most operators that it is impossible to

succeed when the iodine box contains dampness. Now this is a great

mistake, and we will here state that in all cases where dampness

appears upon a properly prepared Daguerreotype plate, it is the result

of a different temperature of the metal from the air which surrounds

it. Mr. Senter, of Auburn, was the first of our operators who used a

solution of iodine for coating the plate, and we several years since

saw his results, which would rival the production of any other

operator. A concentrated solution of iodine is prepared by putting into

a common bottle two thimblesful of hyposulphite of soda and a rather

larger quantity of iodine, so that there may be more than sufficient.

Add to it about 40 ounces of common water (heated to 60 or 70 degrees),

by little and little, moving, the bottle to warm it, for fear of

breaking. After shaking it a short time, the water is rapidly and

strongly colored. The solution should be poured into a bottle with a

ground stopper, and when cool used for iodizing.

A solution of sufficient strength can be made by moistening or just

covering the iodine with water.

Chloride of Iodine as an Accelerator.—This is probably one of the best

accelerators that can be used for coating the plate for taking views;

it works too slow, however, to meet the wants of the operating room,

yet its use was formerly, for a long time, adhered to by some of our

best professors. In producing views with this, we are successful in

obtaining well-developed impressions, with a depth of tone and richness

of appearance not to be met with in the productions of any other

substances. I give its use as furnished me by an old and experienced

operator, and published in Humphrey’s Journal, vol. i. p. 180:

“As the process of using chloride of iodine may be of interest to some

of our subscribers, I take pleasure in giving the following

manipulation. To one ounce of chloride of iodine add two ounces of

water; place this mixture in a coating-box, the same as quick stuff;

coat the plate with dry iodine to a light yellow, or lemon color; then

bring the coating to a deep pink over the chloride. The plate must be

recoated over the dry iodine.”

This combination has been very successfully used in one of our most

extensive establishments in this city, and the superiority of the

pictures produced by it was considered as an equivalent for the

additional time required to bring out the impressions.

Chlorine as an Accelerator.—I shall here refer to but a single

experiment in which I employed chlorine gas for coating the plate. I

was provided with a retort, the neck of which was fitted to the jar of

my coating-box, through a hole drilled for its reception. This was

fitted perfectly tight in my coating-box. I placed some pure undiluted

bromine water and the agents necessary for producing chlorine gas (in

small quantity) in the retort. The result was that my first experiment

produced an impression completely solarized in all its parts by an

exposure of four seconds of time, which would have required an exposure

of twenty seconds to produce a perfectly developed impression by the

usual process.

Another trial immediately produced one of the finest toned impressions

# I ever saw, perfectly developed in one second of time.

My next two or three experiments proved total failures. I was unable to

produce even a sign of an impression. By accident my retort was broken,

and not being in a locality convenient to obtain another, my

experiments were necessarily suspended.

My attention was not called to this subject again for several years,

when I noticed an account of some similar experiments by F. A. P.

Barnard and Dr. W. H. Harrington, the latter of whom is now of the firm

of Dobyns & Harrington, of New Orleans.

From reading this article, I found my own difficulties explained. Too

much of the chlorine gas was present in my coating jar. I would like to

see some of our enterprising operators investigate this combination.

It is a singular fact, that the vapors of bromine and chlorine

combining upon the iodide of silver, produce a more sensitive coating

than when the two are combined in solution, as in chloride of bromine

solution. Those having Humphrey’s Journal at hand, can refer to vol. i.

p. 142.

To use Bromine Water or other Accelerators in Hot Weather.—An excellent

plan for using bromine water is as follows:

Fill a two-ounce bottle quarter full of it, and then fill the bottle

with fine sand, which serves to preserve a low temperature; then place

the bottle in a porous cup, same as used in the battery; fill this also

with sand, and close the end with plaster of Paris. Place this in a

coating-box, and it will be found to act with great uniformity and be

quite permanent.

Bromide of Lime, another accelerator, can be used in the same manner,

except it is, only necessary, when a solid sensitive is used, to mix it

with the sand without placing it in a bottle. This method is employed

with great success by a few, who have regarded it as a secret worth

keeping.

A Combination, requiring the Use of only One Coating-box.—It is often

wondered by beginners, why some solution requiring only one coating

cannot be employed. This can be done, but the results are not so

satisfactory as when two or more are employed. Such an accelerator may

be produced by adding alcoholic solution of iodine to a solution of

chlorate of potash, until the latter will take up no more of the

former, and to each ounce, by measure of this solution, ten drops of a

saturated solution of bromide in water are added. The solution of

chlorate of potash is made by diluting, one part of a saturated

solution of the salt with ten parts of water. The use of the chlorate

is simply as a solvent of iodine.

Fats as Accelerators.—The use of fats, oils, or greasy substances, has

been one of the most emphatic prohibitions about the Daguerreotype

plate. Yet it has been proved that its presence in a small quantity

upon the silver surface has the effect of reducing the time of exposure

in the camera from two-thirds to three-fourths. An application may be

made as follows: Pour sweet oil, or rub beef or mutton fat, on a common

buff, which is free from all polishing powders. With this, buff a

well-cleaned plate, and it will leave a scum, which should be mostly

removed by using another buff, which should be clean. Coat the plate in

the usual manner, and the result will be a great reduction in the time

of exposure in the camera. The impression produced upon a plate so

prepared presents, when coming from the vapor of mercury, a grey,

scummy appearance, which, on the application of heat in gilding, does

not improve; hence its use is not generally adopted.

We have instituted some investigations upon this subject, and in the

present volume, we shall not refer to it further. Those wishing to

learn more fully the effect of light upon organic substances will find

Robert Hunt’s “Researches on Light” an invaluable work.

# CHAPTER IV.

LIGHT AND OPTICS.

Light—Optics—Solar Spectrum—Decomposition of Light—Light, Heat, and

Actinism—Blue Paper and Color for the Walls of the Operating

Room—Proportions of Light, Heat, and Actinism composing a

Sunbeam—Refraction—Reflection—Lenses—Copying Spherical

Aberration—Chromatic Aberration.

It is advisable that persons engaging in the Daguerreotype art should

have at least a little knowledge of the general principles of light and

optics. It is not the author’s design here to give a full treatise on

these subjects, but he only briefly refers to the matter, giving a few

facts.

It has been well observed by an able writer, that it is impossible to

trace the path of a sunbeam through our atmosphere without feeling a

desire to know its nature, by what power it traverses the immensity of

space, and the various modifications it undergoes at the surfaces and

interior of terrestrial substances.

Light is white and colorless, as long as it does not come in contact

with matter. When in apposition with any body, it suffers variable

degrees of decomposition, resulting in color, as by reflection,

dispersion, refraction, and unequal absorption.

To Sir I. Newton the world is indebted for proving the compound nature

of a ray of white light emitted from the sun. The object of this work

is not to engage in an extended theory upon the subject of light, but

to recur only to some points of more particular interest to the

photographic operator.

The decomposition of a beam of light can be noticed by exposing it to a

prism. If, in a dark room, a beam of light be admitted through a small

hole in a shutter, it will form a white round spot upon the place where

it falls. If a triangular prism of glass be placed on the inside of the

dark room, so that the beam of light falls upon it, it no longer has

the same direction, nor does it form a round spot, but an oblong

painted image of seven colors—red, orange, yellow, green, blue, indigo,

and violet. This is called the solar spectrum, and will be readily

understood by reference to the accompanying diagram, Fig. 1.

{133}

To those who are unacquainted with the theory of light (and for their

benefit this chapter is given), it may be a matter of wonder how a beam

of light can be divided.

[Illustration: Fig. 1]

This can be understood when I say, that white light is a bundle of

colored rays united together, and when so incorporated, they are

colorless; but in passing through the prism the bond of union is

severed, and the colored rays come out singly and separately, because

each ray has a certain amount of refracting (bending) power, peculiar

to itself. These rays always hold the same relation to each other, as

may be seen by comparing every spectrum or rainbow; there is never any

confusion or misplacement.

There are various other means of decomposing {134} white light besides

the prism, of which one of the principal and most interesting to the

Daguerreotypist is by reflection from colored bodies. If a beam of

white light falls upon a white surface, it is reflected without change;

but if it falls upon a red surface, only the red ray is reflected: so

also with yellow and other colors. The ray which is reflected

corresponds with the color of the object. It is this reflected

decomposed light which prevents the beautifully-colored image we see

upon the ground glass in our cameras.

[Illustration: Fig. 2]

A sunbeam may be capable of three divisions—LIGHT, HEAT, and ACTINISM;

the last causes all the chemical changes, and is the acting power upon

surfaces prepared to receive the photographic image. The accompanying

illustration, Fig. 2, will readily bring to the mind of the reader the

relation of these one to another, and their intensities in the

different parts of a decomposed sunbeam.

The various points of the solar spectrum are represented in the order

in which they occur between A, and B, this exhibits the limits of the

Newtonian spectrum, corresponding with Fig. 1. Sir John Herschel and

Seebeck have shown that there exists, beyond the violet, a faint violet

light, or rather a lavender to b, to which gradually becomes colorless;

similarly, red light exists beyond the assigned limits of the red ray

to a. The greatest amount of actinic power is shown at E opposite the

violet; hence this color “exerts” the greatest amount of influence in

the formation of the photographic image.

(Blue paper and blue color have been somewhat extensively used by our

Daguerreotype operators in their operating rooms and skylights, in

order to facilitate the operation in the camera. I fancy, however, that

this plan cannot be productive of as much good as thought by some, from

the fact, that the light falling upon the subject, and then reflected

into the camera, is, coming through colorless glass, not affected by

such rays as may be reflected from the walls of the operating room; and

even if it were so, I conceive that it would be injurious, by

destroying the harmony of shadows which might otherwise occur.) The

greatest amount of white light is at C; the yellow contains less of the

chemical power than any other portion of the solar spectrum. It has

been found that the most intense heat is at the extreme red, b.

Artificial lights differ in their color; the white light of burning

charcoal, which is the principal light from candles, oil and gas,

contains three rays—red, yellow, and blue. The dazzling light emitted

from lime intensely heated, known as the Drummond light, gives the

colors of the prism almost as bright as the solar spectrum.

If we expose a prepared Daguerreotype plate or sensitive paper to the

solar spectrum, it will be observed that the luminous power (the

yellow) occupies but a small space compared with the influence of heat

and chemical power. R. Hunt, in his Researches on Light, has presented

the following remarks upon the accompanying illustration:

[Illustration: Fig. 3]

“If the linear measure, or the diameter of a circle which shall include

the luminous rays, is 25, that of the calorific spectrum will be 42.10,

and of the chemical spectrum 55.10. Such a series of circles may well

be used to represent a beam from the sun, which may be regarded as an

atom of Light, surrounded with an invisible atmosphere of Heat, and

another still more extended, which possesses the remarkable property of

producing chemical and molecular change.

A ray of light, in passing obliquely through any medium of uniform

density, does not change its course; but if it should pass into a

denser body, it would turn from a straight line, pursue a less oblique

direction, and in a line nearer to a perpendicular to the surface of

that body. Water exerts a stronger refracting power than air; and if a

ray of light fall upon a body of this fluid its course is changed, as

may be seen by reference to Fig. 4.

[Illustration: Fig. 4]

It is observed that it proceeds in a less oblique direction (towards

the dotted line), and, on passing on through, leaves the liquid,

proceeding in a line parallel to that at which it entered. It should be

observed that at the surface of bodies the refractive power is exerted,

and that the light proceeds in a straight line until leaving the body.

The refraction is more or less, and in all cases in proportion as the

rays fall more or less obliquely on the refracting surface. It is this

law of optics which has given rise to the lenses in our camera tubes,

by which means we are enabled to secure a well-delineated

representation of any object we choose to picture.

When a ray of light passes from one medium to another, and through that

into the first again, if the two refractions be equal, and in opposite

directions, no sensible effect will be produced.

The reader may readily comprehend the phenomena of refraction, by means

of light passing through lenses of different curves, by reference to

the following diagrams:—

[Illustration: Fig. 5, 6, 7]

Fig 5 represents a double-convex lens, Fig. 6 a double-concave, and

Fig. 7 a concavo-convex or meniscus. By these it is seen that a

double-convex lens tends to condense the rays of light to a focus, a

double-concave to scatter them, and a concavo-convex combines both

powers.

If parallel rays of light fall upon a double-convex lens, D D, Fig. 8,

they will be refracted (excepting such as pass directly through the

centre) to a point termed the principal focus.

[Illustration: Fig. 8]

The lines A B C represent parallel rays which pass through the lens, D

D, and meet at F; this point being the principal focus, its distance

from the lens is called the focal length. Those rays of light which are

traversing a parallel course, when they enter the lens are brought to a

focus nearer the lens than others. Hence the difficulty the operator

sometimes experiences by not being able to “obtain a focus,” when he

wishes to secure a picture of some very distant objects; he does not

get his ground glass near enough to the lenses. Again, the rays from an

object near by may be termed diverging rays. This will be better

comprehended by reference to Fig. 9, where it will be seen that the

dotted lines, representing parallel rays, meet nearer the lenses than

those from the point A. The closer the object is to the lenses, the

greater will be the divergence. This rule is applicable to copying. Did

we wish to copy a 1/6 size Daguerreotype on a 1/16 size plate, we

should place it in such a position to the lenses at A that the focus

would be at F, where the image would be represented at about the proper

size. Now, if we should wish to copy the 1/6 size picture, and produce

another of exactly the same dimensions, we have only to bring it nearer

to the lenses, so that the lens D E shall be equi-distant from the

picture and the focus, i. e. from A to B. The reason of this is, that

the distance of the picture from the lens, in the last copy, is less

than the other, and the divergence has increased, throwing, the focus

further from the lens.”

[Illustration: Fig. 9]

These remarks have been introduced here as being important for those

who may not understand the principles of enlarging or reducing pictures

in copying.

# I would remark that the points F and A, in Fig. 9, are termed

“conjugate foci.”

If we hold a double-convex lens opposite any object, we find that an

inverted image of that object will be formed on a paper held behind it.

To illustrate this more clearly, I will refer to the following woodcut:

[Illustration: Fig. 10]

“If A B C is an object placed before a convex lens, L L, every point of

it will send forth rays in all directions; but, for the sake of

simplicity, suppose only three points to give out rays, one at the top,

one at the middle, and one at the bottom; the whole of the rays then

that proceed from the point A, and fall on the lens L L, will be

refracted and form an image somewhere on the line A G E, which is drawn

direct through the centre of the lens; consequently the focus E,

produced by the convergence of the rays proceding from A, must form an

image of A, only in a different relative position; the middle point of

# C being in a direct line with the axis of the lens, will have its image

formed on the axis F, and the rays proceeding from the point B will

form an image at D; so that by imagining luminous objects to be made up

of all infinite number of radiating points and the rays from each

individual point, although falling on the whole surface of the lens, to

converge again and form a focus or representation of that point from

which the rays first emerged, it will be very easy to comprehend how

images are formed, and the cause of those images being reversed.

“It must also be evident, that in the two triangles A G B and D G E,

that E D, the length of the image, must be to A B, the length of the

object, as G D, the distance of the image, is to G B, the distance of

the object from the lens.

It will be observed that in the last cut the image produced by the lens

is curved. Now, it would be impossible to produce a well-defined image

from the centre to the edge upon a plain surface; the outer edges would

be misty, indistinct, or crayon-like. The centre of the image might be

represented clear and sharp on the ground glass, yet this would be far

from the case in regard to the outer portions. This is called spherical

aberration, and to it is due the want of distinctness which is

frequently noticed around the edges of pictures taken in the camera. To

secure a camera with a flat, sharp, field, should be the object of

every operator; and, in a measure, this constitutes the great

difference in cameras manufactured in this country.

Spherical aberration is overcome by proper care in the formation of the

lens: “It can be shown upon mathematical data that a lens similar to

that given in the following diagram—one surface of which is a section

of an ellipse, and the other of a circle struck from the furthest of

the two foci of that ellipse—produces no aberration.

“At the earliest period of the employment of the camera obscura, a

double-convex lens was used to produce the image; but this form was

soon abandoned, on account of the spherical aberration so caused.

Lenses for the photographic camera are now always ground of a

concavo-convex form, or meniscus, which corresponds more nearly to the

accompanying diagram.”

[Illustration: Fig. 11]

Chromatic Aberration is another difficulty that opticians have to

contend with in the manufacturing of lenses. It will be remembered,

that in a former page (133) a beam of light is decomposed by passing

through a glass prism giving seven distinct colors—red, orange, yellow,

green, blue, indigo and violet.

Now, as has been said before, the dissimilar rays having an unequal

degree of refrangibility, it will be impossible to obtain a focus by

the light passing through a double-convex lens without its being

fringed with color. Its effect will be readily understood by reference

to the accompanying cut.

[Illustration: Fig. 8]

If L L be a double convex-lens, and R R R parallel rays of white light,

composed of the seven colored rays, each having a different index of

refraction, they cannot be refracted to one and the same point; the red

rays, being the least refrangible, will be bent to r, and the violet

rays, being the most refrangible, to v: the distance v r constitutes

the chromatic aberration, and the circle, of which the diameter is a l,

the place or point of mean refraction, and is called the circle of

least aberration. If the rays of the sun are refracted by means of a

lens, and the image received on a screen placed between C and o, so as

to cut the cone L a l L, a luminous circle will be formed on the paper,

only surrounded by a red border, because it is produced by a section of

the cone L a l L, of which the external rays L a L l, are red; if the

screen be moved to the other side of o, the luminous circle will be

bordered with violet, because it will be a section of the cone M a M l,

of which the exterior rays are violet. To avoid the influence of

spherical aberration, and to render the phenomena of coloration more

evident, let an opaque disc be placed over the central portion of the

lens, so as to allow the rays only to pass which are at the edge of the

glass; a violet image of the sun will then be seen at v, red at r, and,

finally, images of all the colors of the spectrum in the intermediate

space; consequently, the general image will not only be confused, but

clothed with prismatic colors.”

To overcome the difficulty arising from the chromatic aberration, the

optician has only to employ a combination of lenses of opposite focal

length, and cut from glass possessing different refrangible powers, so

that the rays of light passing through the one are strongly refracted,

and in the other are bent asunder again, reproducing white light.

To the photographer one of the most important features, requiring his

particular attention, is, that he be provided with a good lens. By the

remarks given in the preceding pages, he will be enabled, in a measure,

to judge of some of the difficulties to which he is occasionally

subjected. We have in this country but two or three individuals who are

giving their attention to the manufacture of lenses, and their

construction is such, that they are quite free from the spherical or

chromatic aberration.

# CHAPTER V.

To make Plates for the Daguerreotype—Determining the Time of Exposure

in the Camera—Instantaneous Process for Producing

Daguerreotype—Galvanizing the Daguerreotype Plate—Silvering

Solution—Daguerreotype without Mercury—Management of Chemicals—Hints

and Cautions—Electrotyping—Crayon Daguerreotypes—Illuminated

Daguerreotypes—Natural Colors in Heliography—Multiplying Daguerreotypes

on one Plate—Deposit in Gilding—Practical Hints on the Daguerreotype.

TO MAKE PLATES FOR THE DAGUERREOTYPE.

# I do not give the method employed by our regular plate manufacturers;

this is not important, as the operator could not possibly profit by it

from the fact of the great expense of manufacturing. The following will

be found practical:

Procure a well planished copper plate of the required size, and well

polish it, first with pumice stone and water, then with snake stone,

jewelers’ rouge. Plates can be purchased in a high state of preparation

from the engravers. Having prepared the copper-plate, well rub it with

salt and water, and then with the silvering powder. No kind answers

better than that used by clock-makers to silver their dial-plates. It

is composed of one part of well washed chloride of silver, five parts

of cream of tartar, and four parts of table salt. This powder must be

kept in a dark vessel, and in a dry place. For a plate six inches by

five, as much of this composition as can be taken up on a shilling is

sufficient. It is to be laid in the centre of the copper, and the

figures being wetted, to be quickly rubbed over every part of the

plate, adding occasionally a little damp salt. The copper being covered

with the silvering is to be speedily well washed in water, in which a

little soda is dissolved, and as soon as the surface is of a fine

silvery whiteness, it is to be dried with a very clean warm cloth. In

this state the plates may be kept for use. The first process is to

expose the plate to the heat of a spirit flame, until the silvered

surface becomes of a well-defined golden-yellow color; then, when the

plate is cold, take a piece of cotton, dipped in very dilute nitric

acid, and rub lightly over it until the white hue is restored, and dry

it with very soft clean cloths. A weak solution of the hydriodate of

potash, in which a small portion of iodine is dissolved, is now passed

over the plate with a wide camel’s hair brush. The silver is thus

converted, over its surface, into an ioduret of silver; and in this

state it is exposed to light, which blackens it. When dry, it is to be

again polished, either with dilute acid or a solution of carbonate of

soda, and afterwards with dry cotton, and the smallest possible portion

of prepared chalk: by this means a surface of the highest polish is

produced. The rationale of this process is, in the first place, the

heat applied dries off any adhering acid, and effects more perfect

union between the copper and silver, so as to enable it to bear the

subsequent processes. The first yellow surface appears to be an oxide

of silver with, possibly, a minute quantity of copper in combination,

which being removed leaves a surface chemically pure.

Another Method.—The best and simplest mode with which we are acquainted

is to divide an earthenware vessel with a diaphragm: one side should be

filled with a very dilute solution of sulphuric acid, and the other

with either a solution of ferroprussiate of potash, or muriate of soda,

saturated with chloride of silver. The copper plate, varnished on one

side, is united, by means of a copper wire, with a plate of zinc. The

zinc plate being immersed in the acid, and the copper in the salt, a

weak electric current is generated, which precipitates the silver in a

very uniform manner over the entire surface.

Another Method.—A piece of brass or of polished copper, brass is

preferred, is perfectly planished and its surface made perfectly clean.

A solution of nitrate of silver, so weak that the silver is

precipitated slowly, and a brownish color, on the brass, is laid

uniformly over it, “at least three times,” with a camel’s hair pencil.

After each application of the nitrate, the plate should be rubbed

gently in one direction, with moistened bitartrate of potassa, applied

with buff. This coat of silver receives a fine polish from peroxide of

iron and buff. Proofs are said to have been taken on it, comparable

with those obtained on French plates.

M. SOLIEL’S PROCESS FOR DETERMINING THE TIME OF EXPOSURE IN THE CAMERA.

M. Soliel has proposed the use of the chloride of silver to determine

the time required to produce a good impression on the iodated plate in

the camera. His method is to fix at the bottom of a tube, blackened

within, a piece of card, on which chloride of silver, mixed with gum or

dextrine, is spread. The tube thus disposed is turned from the side of

the object of which we wish to take the image, and the time that the

chloride of silver takes to become of a greyish slate color will be the

time required for the light of the camera to produce a good effect on

the iodated silver.

INSTANTANEOUS PROCESS FOR PROCURING DAGUERREOTYPES.

The following method of producing Daguerreotypes has by some been named

as above. Most experienced operators have been long acquainted with the

effect of the vapor of ammonia upon the chemically coated plate. I will

here insert Mr. W. H. Hewett’s plan of proceeding. This gentleman, in

referring to it (published in 1845), says:

“This improvement consists in using the vapor of ammonia, as an object

to accelerate the action of light upon the plate. The effect is

produced upon a simple iodized plate, but still more upon a plate

prepared in the ordinary way, with both iodine and bromine. By this

means, the author obtained impressions instantaneously in the sunshine,

and in five to ten seconds in a moderate light; and he hopes to be able

to take moving objects. It can be applied by exposing the prepared

plate over a surface of water, to which a few drops of ammonia have

been added (sufficient to make it smell of ammonia); or the vapor can

be introduced into the camera during the action. In fact, the presence

of ammonia, in the operating-room, appears to have a good effect, as it

also neutralizes the vapors of iodine and bromine that may be floating

about, and which are so detrimental to the influences of light upon the

plate.”

GALVANIZING THE DAGUERREOTYPE PLATE.

In consideration of the importance of galvanized plates, I shall

endeavor to give as plain and concise a manner of manipulation as

possible. For some time it was a question among the operators

generally, as to the beneficial result of electrotyping, the

Daguerreotype plate, but for a few years past our first operators have

found it a fact, that a well electro-silvered surface is the best for

producing a portrait by the Daguerreotype.

From my own experiments, I have found that a plate, by being

galvanized, can be rendered more sensitive to the operation of the

light in proportion of one to five, viz.: if a plate as furnished by

the market, be cleaned, polished, coated and exposed in the camera, if

the required time to freely develop an impression be ten seconds, a

similar plate prepared in like manner and galvanized, will produce an

equally well-defined image in eight seconds. In connection with this

subject, there is one fact worthy of notice; a plate with a very heavy

coating of pure silver, will not produce an equally developed image, as

a plate with a thinner coating, hence the thin coating, providing it

entirely covers the surface, is the best, and is the one most to be

desired. The experiment is plain and simple. Let the slate receive a

heavy or thick coating by the electrotype, then polish, coat, expose in

the usual manner, and the result will be a flat, ashy, indistinct

impression; when, on the other hand, the thin coating will produce a

bright, clear and distinct image, with all the details delineated.

The style of battery best for the purpose has been, and now is, a

question of dispute among operators; some preferring the Daniell

battery to Smee’s. Some claim the superiority of the first from its

uniformity of action; others, of the latter, for its strength. I

consider either good, and for the inexperienced would prefer the

Daniell. This is more simple in its construction, while it has

certainty in action. The more skillful electrotyper would prefer

Smee’s, and this is the one most generally in use. I would remark that

the plan of galvanizing plates should be followed by every operator,

and when once thoroughly tested, no one will abandon it.

SILVERING SOLUTION.

To any desired quantity of chloride of silver in water add, little by

little, cyanide of potassium, shaking well at each addition, until all

the cyanide is dissolved. Continue this operation, and add the cyanide,

until all the precipitate is taken up and held in solution.

This solution is now ready for the plate-cup. Enough water may be added

to cover any sized plate when held perpendicular in the cup. The

strength of the solution may be kept up by occasionally adding the

chloride of silver and cyanide of potassium. There should alway be a

very little excess of the cyanide.

The plate should be well cleaned and buffed, and the solution well

stirred before it is immersed. Care should be observed to keep the

solution clean, and allow no particle of dust to come in contact with

the surface of the plate. The plate is now to be attached to the pole

of the battery.

After remaining a short time, it assumes a blue color; take it out,

rinse freely with pure water, then dry with a spirit lamp, and it is

ready for buffing. Buff and coat in the usual manner. Some operators

are in the practice of immersing the plate in the solution and buffing

twice. This additional silvering is no improvement wherever there has

been a proper first coating.

Sometimes the operator is troubled with streaks or scum on the plate.

This may arise from three causes, all of which experience must teach

the experimenter to avoid; first, too great an excess of cyanide in the

solution; second, a lack of silver; third, the current too strong.

Another annoyance arises from the solution being dirty and the dirt

collecting on the surface. When this is the case, the dirt is sure to

come in contact with the surface of the plate as it is plunged into the

solution, and the result is a scum that it is difficult to dispose of.

This can be prevented only by frequent filtering. One thing should

always be borne in mind in electrotyping Daguerreotype plates—that in

order to secure a perfectly coated surface, the plate should be

perfectly cleaned. In this point, many who have tried the electrotype

process have failed, attributing their ill success to other than the

proper cause.

DAGUERREOTYPES WITHOUT MERCURY.

The following process possesses some interest, and is worthy a trial

from operators. M. Natterer, of Vienna, discovered a process for

obtaining proofs on iodized plates with the chloride of sulphur,

without the use of mercury. A plate of silver is iodized in the usual

manner, and then placed on the top of a vessel six or eight inches

high, having at the bottom, in a small cup, a few drops of chloride of

sulphur; it should remain exposed to the action of the vapor until the

sombre yellow color is changed to a red, after which it is brought to a

focus in the camera, where it is exposed to the light in the camera,

for about the time necessary to produce an ordinary daguerreotype. The

plate is then taken out and examined in the camera by the light of a

candle. It often occurs that no trace of the image is as yet

perceptible, but if the plate is heated by placing over a spirit lamp

the unprepared side, or if left for some time in the dark, or, lastly,

if exposed only a few seconds to a weak, dimmed light, the positive

picture then appears with all its shades. Of these three modes of

bringing out the image, the second is superior to the others.

MANAGEMENT OF CHEMICALS.

It is necessary, first of all, to know that you have a chemical which

is capable of producing good results when in skillful hands. For this

reason it is best to prepare your own quick, after some formula which

is known to be good. Those quick-stuffs which contain chloride of

iodine are noted for their depth of tone while they probably operate

with less uniformity than those which are destitute of it. For

operating under ordinary circumstances, especially with an inferior

light, probably no accelerator is more quick and sure than Wolcott’s.

It also produces a very fine, white pleasing picture, though lacking

that depth of impression so much to be desired. The dry quick operates

with surety, and its use is simple and easy, producing an impression

much like Wolcott’s. For those having a good and permanent light,

however, we would recommend a chemical giving more body to the

impression.

There is a class of accelerators called sensitives, claiming to work in

from three to ten seconds, which, however, will be found very little,

if any, more sensitive than this. We frequently work it with the

ordinary coating in twelve and fifteen seconds. The manner in which the

sensitives are worked is by coating very light. In this way, a flat,

shallow picture is obtained in a few seconds; and the same can be done

with any of the more volatile quicks.

It is a fact not generally known, that a plate coated in a light

chemical room is more sensitive than when coated in darkness. By

admitting a free, uniform light, and exposing the plate to it a few

seconds after coating, then timing short in the camera, a very light,

clear impression is obtained. The time in the camera is reduced in

proportion to the previous action of light. The shades, of course, are

destroyed, and the tone injured; still, for taking children, we have

succeeded better by this method than by the use of “sensitives.” The

discovery of this principle was accidental, while operating where the

direct ray s of the sun, entering the window just before sunset, fell

on the curtain of our dark room, rendering it very light within.

The selection of iodine is not unimportant. Reject, at once, that which

has anything like a dull, black, greasy appearance; and select that

which is in beautiful large crystalline scales, of a purple color, and

brilliant steel lustre.

Solarization, and general blueness of all the light parts of the

picture, were formerly great obstacles to success, though now scarcely

thought of by first-class artists. Beginners in the art, however, are

still apt to meet with this difficulty. It is occasioned by dampness in

the iodine box, which causes the plate to become coated with a

hydro-iodide of silver, instead of the iodide. The remedy is in drying

your iodine. If in summer, you can open your box and set it in sunshine

a few minutes; or if in winter, set it under a stove a short time. The

true method, however, is to dry it by means of the chloride of calcium.

It has such a remarkable affinity for water, that a small fragment

placed in the open air, even in the dryest weather, soon becomes

dissolved.

Take one or two ounces of this chemical, heat it in the drying bath, or

in a hot stove, to perfect dryness; place it in a small glass toy dish,

or large watch crystal, and set it in the centre of your iodine box.

Take this out and heat to dryness every morning. Adopt this process,

and with your mercury at a high temperature, you will never be troubled

with blue pictures.

Young operators are apt to impute all want of success in operating to

their chemicals, even though the cause is quite as likely to be

elsewhere. Failure is quite likely to occur from dampness in the buffs,

or in the polish; it is therefore necessary to be constantly on the

guard in this quarter. With a view to this, always scrape your buffs

with a dull knife, or with one blade of your shears, the first thing in

the morning, and after brushing them thoroughly, dry them, either in

the sun, by a stove, or in the buff-dryer. It is equally important that

the polish and the brush should be kept dry.

Want of success may arise from vapors of iodine or bromine in the

camera box, mercury bath, or even in the buffs. It is incredible how

small a quantity of these vapors will affect the effect of light when

coming in contact with the plate, after or during the exposure in the

camera. It is therefore necessary to be cautious not to mix chemicals,

nor open your boxes or bottles in your room, but take them out to do

it. Never hurry the operation through from lack of confidence in the

result. The fact of anything being out of order, forms no excuse for

slighting the process. If unsuccessful, do not pursue the same course

every trial, but vary with a view to detect the cause of the

difficulty.

In case of a long series of failures, institute a regular course of

investigation, after this manner, commencing where the trouble is most

likely to occur:

1. Are the plates well cleaned?

2. Is the iodine dry? If the impressions come out blue, you may rest

assured it is not. Take out the iodine, wipe and dry the box, and dry

the calcium.

3. Is the quick battery of the right strength? If dry, it must change

the plate in from six to fifteen seconds. If any of the chloride of

iodine class, it may vary from five seconds to a minute. Begin by

coating light, and increase on each trial, observing the effect. If the

light side of the picture seems loth to come out, and shows no contrast

with the dark side, it is to be inferred that your battery is too

strong, and must be reduced with water or set out in the open air for a

few minutes, with the lid off. If working an old battery, never renew

very strong, or it will work dark and heavy. A battery, to work well,

should be gradually losing strength, but never gaining. An old battery,

however, may be quickened up and made to work well for some time, by

adding five of six drops of sulphuric acid, repeating the quantity as

often as necessary, providing always that acid be not used in

manufacturing the quick.

4. Have the plates lost their sensitiveness by being many times exposed

to mercury? Clean and burn them; but if French plates, burn light, or

you spoil them.

5. Are the buff s dry and clean? Examine the plate critically after

buffing to detect any appearance of scum or film on the surface. If so,

the longer you buff the more it shows. Scrape and dry the buffs

thoroughly.

6. Is the mercury free from scum and dirt? If not, filter. Is it also

far enough from the coating boxes? Should be at least three feet, and

kept covered.

7. Is the mercury sufficiently heated? This is important. Long

exposure, however, will answer the same purpose.

8. Are your lenses clean, and in proper place?

9. Are the tablets in focus with the ground-glass? If you can attribute

the failure to none of these, mix a new box of some other kind of

quick, say the dry, for instance. If you fail in the same manner here,

take time, wash your buffs, overhaul all the chemicals, and start anew.

Do not be discouraged.

There is no day so dark but that the sun will shine again. We will

close with this brief summary of advice:

Clean your plates. Keep everything dry. Keep the mercury hot. Follow

these instructions carefully, and you must succeed.

HINTS AND CAUTIONS.

First of all, cleanliness should be observed. When there is dust or

dirt about your room, particularly about the work-bench, failures will

be frequent; for the smallest particles of rotten-stone, when allowed

to come in contact with the buffs, will produce scratches on the

surface of the plate, which very much injures the operation, and often

causes failures.

Dust flying about the room is injurious, if allowed to fall on the

plate, either before or after it has been coated, as it causes black

spots which cannot be removed.

The polished plate should not be allowed to come in contact with a

strong current of air, for it tends to oxidize the surface. Breathing

on the surface should also be avoided, for the same reason.

The plate should, in all cases, be buffed immediately before using, and

not allowed to stand any length of time. It should be held with the

polished face downward.

It is always best that the plate should be of the same temperature of

the atmosphere in the room.

Keep the camera and mercury-bath perfectly free from the vapors of

iodine and bromine; for the presence of the slightest degree of either

of the above will injure the impression in no small degree. As a

preventive, let the camera be exposed to the sun or fire for a few

minutes in the morning.

Filter your mercury often, to keep the surface free from film and dust.

The hyposulphite solution should be filtered through sponge every time

it is used.

The direct rays of light must not enter the camera in conjunction with

those reflected from the object; or the picture will be veiled, and the

color of the plate changed to a thick green.

If the plate be iodized only to a light-yellow, the result might be of

a bluish or grey tinge: and this is generally the case, when the quick

is new and strong, and there is an excess of it on the plate, and yet

not enough to form the bromide iodide of silver; in which case it would

wholly spoil the impression.

Your iodine will be found to operate more successfully, when the time

required for coating the plate does not fall short of fifteen seconds,

or exceed one minute.

Too quick coating can be avoided by using less iodine in your box. In

the summer months, when the weather is 80 deg. and over, one quarter of

an ounce, or even less, will work to advantage.

ELECTROTYPING.

# I am indebted to Mr. J. H. Fitzgibbons for the following process, which

he employed in producing the excellent specimens he exhibited at the

Crystal Palace:

“I shall endeavor to lay down in as comprehensive a manner as possible

the method by which I have been enabled to produce the most

satisfactory results. I use a Smee’s battery (another kind will do).

After filling the cell, of common size, nearly full with water; add

about quarter of an ounce of sulphuric acid. Mix this well, and let it

stand for about three hours, or until the action of the battery becomes

weak, when it is in order to work with a very uniform action. Put one

pound of sulphate of copper in one quart of water; stir it until the

sulphate of copper is all dissolved, and then add one half ounce of

sulphuric acid and a quarter of an ounce of nitric acid. This solution,

well mixed, should be filtered, and it is ready for use. It is very

important that the solution should be kept clean, clear, and free from

all foreign substance. The above quantity of this solution will be

found sufficient for electrotyping a dozen of the sixth-size plates.

When it is required to be strengthened, it is only necessary to add a

little of the sulphate of copper.

“With the battery prepared as above, and the solution of sulphate of

copper in a vessel of proper dimensions to receive your plate, connect

the galvanic current, and immerse the impressioned plate, letting it

remain until a thin film of copper has been formed, then the battery

can be strengthened, and the impression will be of sufficient thickness

to be removed in from eight to twelve hours. An old Daguerreotype plate

attached to the opposite pole of the battery (copper side towards the

face of the plate to be electrotyped), will answer the same purpose as

the silver-plate.

“The great difficulty in taking an electrotype impression, and

preserving the original, has been attributed to the battery being too

powerful. I am led to believe from practice that the principal

difficulty has been in the Daguerreotype plate itself, for if we use an

impression that has been taken but a few days, and taken in the usual

way, we will find it difficult to succeed without spoiling both the

copy and original, and so also with an old impression.

“I have found the most certain method to be as follows:—Coat the

Daguerreotype plate as usual, except use less of the accelerators, the

proportion of iodine coating being greater, of course the time of

exposure in the camera will be lengthened. Mercurialize it at about a

temperature requiring to develop the image, from six to eight minutes,

at least. Gilding the Daguerreotype has much to do towards producing a

good electrotype copy. This should be done by applying a little heat,

and gilding very slowly, giving a coating of gold with the greatest

possible uniformity. By this method, I have been enabled to produce any

number of proofs. I have produced a dozen from one impression, and it

remains as perfect as when first taken.

“By a little judgment and care the operator will be enabled to produce

the electrotype copy of the Daguerreotype plate without any difficulty.

The electrotype copy should be immediately put under a glass and sealed

in the same manner as the ordinary Daguerreotype.”

CRAYON DAGUERREOTYPES.

This process is patented in the United States, by J. A. Whipple, of

Boston, and of course no honorable person will use it for his own

benefit without purchasing a right.

A white back-ground is generally employed, the object being to blur the

lower portion of the plate, leaving the head of the subject in relief.

Every Daguerreotypist is familiar with the fact that a motion of any

body between the camera and the sitter will cause a “blur.” Cut a piece

of thin paper and scallop it, making a semicircle. This is kept

straight by means of a wire frame, and it is to be moved in front of

the lower part of the body of the sitter during the time of exposure of

the plate in the camera. Develop over mercury as usual, and the result

will be a crayon Daguerreotype.

Another method is to have a wheel with a hole cut through it of a

diameter of about 12 inches. This hole is so cut as to leave teeth

resembling those of a large saw. This wheel is so arranged that it can

be turned around, which should be done during the time of exposure in

the camera. It must be placed between the camera and the sitter, and at

such a distance from the camera as to allow such proportion of the body

of the sitter be seen upon the ground-glass as is desired. It will be

readily seen that by turning this wheel during the operation will

produce the same result as the paper being moved in the other method.

The teeth make the “blur.” The side of the wheel towards the camera may

be black, by which means the result will be a dark instead of a light

border.

ILLUMINATED DAGUERREOTYPES.

This process is also patented, and the remarks on the preceding subject

will apply in this case. The plate is prepared and exposed as in the

usual method of the Daguerreotype. A white back-ground is employed. Let

the head of the sitter come in the middle of the plate, and before

exposing it to the vapors of mercury, put a small mat or diaphragm,

having a small hole through it, over or directly on the surface of the

plate. This diaphragm should be bevelled, and the bevel should be

towards the surface of the plate; this, in order to prevent too sharp a

line on the impression. It will be readily seen that if an impressioned

plate so covered is placed over the mercury, it will be developed on

such portions only as are exposed. The principle is so familiar that

further explanations are unnecessary.

NATURAL COLORS IN HELIOGRAPHY.

This subject is worthy the attention of every operator. The following

process is so plain and easy of trial that any Daguerreotypist can try

it. This is as given by Mr. James Campbell, and was published in

Humphrey’s Journal of the Daguerreotype and Photographic Arts, vol. 5,

page 11. Mr. Campbell has done much to further the process announced by

M. Neipce, and his experiments have proved highly successful.

The following is submitted as worthy of trial:

“The proper preparation of the chloridated plate, to enable it to

receive colored impressions is an object of the first importance to

those wishing to experiment on it, and consequently requires particular

notice. The plate may be prepared by making it the positive pole of a

battery, and letting it at the same time be immersed in chlorine water.

The negative pole should be a slip of platinum. All the colors may be

produced from a plate so prepared if the chlorine and water are in the

right proportions; but generally one color or the other predominates,

according to the amount of chlorine in the liquid. By adding the

chlorides of strontian, uranium, potassium, sodium, iron, or copper to

the liquid, various effects may be produced, and these bodies will be

found to produce the same color on the plate that their flame gives to

alcohol.

“The honor of this discovery is due to M. Neipce. Copper gives a

variegated flame; hence many colors may be impressed on a plate

prepared with a solution of its chloride.

“M. Neipce recommends a solution of the mixed chlorides of copper and

iron, and it is with these, that I have been most successful. As the

chlorides of copper and iron are not much used in the arts, they are

not generally found for sale in the shops; and it may be well to

furnish those not much versed in chemistry with an easy method of

preparing them.

“They may be made directly from either metal by dissolving it in

hydrochloric acid; but they may be formed by a cheaper method, and by

which also the acid fumes are avoided.

“Sulphate of iron or copper, or both together, may be dissolved in

water and then neutralized with common crude potash, or its carbonate

or bicarbonate—known commonly as pearl ash and saleratus. If either of

the latter be used, there will be formed sulphate of potash and a

carbonate of the metal used, and there will also be a considerable

effervescence of carbonic acid, which will, if care is not taken, cause

the mixture to run over the vessel. After the copper or iron salt is

neutralized, which is known by its ceasing to effervesce, the carbonate

of the metal will settle slowly, and will at first nearly fill the

vessel. The supernatant fluid, which is sulphate of potash in solution,

may now be carefully poured off, and its place filled with water; this

operation should be repeated several times until the water which passes

off is tasteless. The carbonate of the metal rapidly changes to an

oxide by contact with the air, and it will generally be found, when it

is sufficiently washed, that it is at least half oxide. On adding

hydrochloric acid cautiously to the mixture, a chloric of the metal

will be formed, and carbonic acid will be evolved from the remaining

carbonate. The chloride formed is soluble; but as there are two

chlorides of these metals, and we wish to produce the one which

contains the most chlorine, it is best to add the acid cautiously until

the solution is decidedly acid. After filtering the solution, it is fit

for use; and it should be preserved in well-stoppered bottles. The

water used should be rain or distilled water.

“About one part of the mixed chlorides should be used to three or four

of water.

“The battery may be either Smee’s, Daniell’s, or Grove’s; if of either

of the former, it should be of two series; if of the latter, one cup is

sufficient.

“The plate on being immersed in the liquid, almost instantly takes a

violet color. It should be allowed to remain from two to five minutes,

according to the strength of the battery, and until it becomes nearly

black. It should now be carefully washed, and afterwards heated over a

spirit lamp until it takes a cherry-red color, and it is then ready for

exposure in the camera. Before speaking of exposing the plate, it may

be well to speak of some difficulties which the inexperienced operator

may find in preparing it. If the battery is not in good order, and a

sufficient current is not passed through the solution, the plate will

become coated—and apparently almost as well as when the battery is

working well—but on exposure it will give a negative picture, and but

little colored; while if the battery is in good order, the impression

is invariably positive.

“Sometimes on heating the plate after washing, the surface is covered

with spots or assumes a variegated appearance. This indicates that the

solution is impure, or that the plate have not been thoroughly washed

and are still contaminated with the soluble chlorides which are

contained in the solution.

“From the fact that the plate if prepared with positive electricity

gives a positive picture, while it prepared otherwise it gives a

negative, it is evident that electricity plays an important part in

this process. The same is true to some extent with the compounds formed

with iodine, bromine, and fluorine.

“On heating the plate, the brown coating of chloride melts into a

translucent enamel, and the heat should be withdrawn when a cherry-red

color is produced. It the heat is continued longer, the plate assumes a

lighter color, and becomes less sensitive; and the enamel will finally

scale off. To produce a picture by the ordinary process of M. Neipce,

unaccelerated, it should be exposed for from three to five hours to

sunlight in the camera, though pictures may be procured by contact, in

from fifteen to thirty minutes.”

MULTIPLYING DAGUERREOTYPES ON ONE PLATE.

# I have produced some interesting specimens of the Daguerreotypic art,

by exposing in the camera only a portion of the sensitive plate to the

action of light. When on the exposed portion an image is formed, then

taking the tablet into the dark room, change ends and expose the

sensitive portion, and produce another image, developing as usual. This

plan is adapted for taking likenesses for lockets. Two images can be

presented as sitting side by side, by covering half the plate with

black paper, and exposing as before. In this manner we have been

enabled to surprise persons by exhibiting their portrait on the same

plate with a stranger’s. Daguerreotypists must be cautious in

practicing this, as it might not be agreeable to the parties whose

likenesses are together, by the above process. It is impossible to

produce an impression without a line being seen where the edge of the

paper prevented the operation of the light.

# I have recently seen a fine specimen produced by another plan, which

far exceeds the above, there being no line, or any peculiarity denoting

two exposures. The specimen referred to, was a gentleman represented on

one plate by two full length portraits. This was produced by using a

black velvet for the background. The plate was exposed sufficient time

to produce one impression, and then the gentleman assumed another

position, and is repeated as looking at himself. From the fact that the

time required to develop black velvet being so much longer than that

for producing a portrait, we are enabled to produce the above

interesting results.

DEPOSIT IN GILDING.

Regarding specks from bad water, I would remark that gilding should be

made only with distilled water. Thus made, it produces very little

deposit, even by long keeping. It therefore preserves its original

strength, and works with great uniformity.

Every grain of deposit contains at least 7-10 its weight of gold,

easily discoverable by the blowpipe. Such gilding is continually

deteriorating, which with good chloride and distilled water may be

prevented. Distilled water should also be used for the hyposulphite.

and for cleaning plates. Any good, clear water may be afterwards used

for washing off, with equally good results. I am very rarely troubled

with specs, and deem this as the main reason.

With a portable still attached to a cooking stove, I obtain half a

gallon of water per hour, and with very little trouble. A small tin

retort or still connected with a Leibig’s condenser, would not add much

to the “traps” of the travelling operator, and save him many a

disreputable specimen.—T. J. BAILEY.—Humphrey’s Journal.

PRACTICAL HINTS ON THE DAGUERREOTYPE.

The following is from Humphrey’s Journal, vol. 5, and from the pen of

Dr. WM. HARRINGTON, one of the most able writers upon the subject of

the Daguerreotype in this country:

THE CAUSE OF THE DIFFICULTY THAT SOMETIMES OCCURS TO PREVENT THE

PRODUCTION OF A CLEAR IMPRESSION UPON A DAGUERREOTYPE PLATE.

Beyond all doubt this is traceable to dampness. Truly this is not a new

thought; but where does this dampness come from? How does it originate,

and where is it located? Generally it has been referred to a point

entirely remote from its real location.

This dampness exists particularly upon the surface of the plate; is

obviously derived immediately from the atmosphere; and is owing to a

certain relative temperature of the plate with the hygrometric

condition of the atmosphere.

Whenever this relation exists between the plate and atmosphere, a

precipitation of moisture takes place upon the surface of the plate,

which render all efforts at polishing impracticable. This interference

is not confined to the buffing operation alone, but sometimes is

discoverable even in the ordinary process of scouring. Every one at all

experienced in this art will remember that it is not always an easy

matter for him, by scouring, to bring his plate to the desired lustre.

All his efforts become unavailing; the more he rubs, the duller the

surface of his plate appears; and although he renews his cotton

repeatedly, still he is obliged to content himself with an

unsatisfactory finish.

This relative condition is not confined to any particular season of the

year, nor to any certain thermometric temperature; but may occur in

summer as well as in winter; the weather being warm or cold, wet or

dry, clear or cloudy, raining or shining. Under any of these

circumstances, if the relation of the plate and atmosphere be such as

to invite upon the plate a precipitation of humidity from the

atmosphere, the prospect of producing a clear impression is quite

problematical.

It is reasonable to expect this occurrence from the fact that metal is

a good radiator, and radiation reduces the temperature of a metallic

body below that of the atmosphere. Consequently, if this relative

condition happens, the result will be as I have stated.

Bodies may be colder than the atmosphere and yet derive no moisture

from it; while at the same time the driest atmosphere is not devoid of

moisture, but will part with it under certain conditions.

Assuming for granted that this relative condition between the plate and

atmosphere, disposing the former to receive the humidity of the latter,

constitutes the great obstacle the operator has to contend with in

producing, a clear proof upon the plate, the remedy naturally suggests

itself, and is very simple. It consists in merely heating the plate

above the temperature of the atmosphere, previous to polishing, and

retaining that temperature during the operation. Various measures might

be devised to effect the desired object; one of which consists of a

sheet-iron box, heated from the inside by a spirit-lamp, upon the top

of which are to be kept the plates ready to undergo the process of

being polished; the blocks of the swing or any other vice; or the iron

bed belonging to Lewis’s vice.

In cold weather, when it is necessary to keep a fire in the preparation

room, all of the above may be so arranged in the vicinity of the fire

as to receive the requisite degree of heat for the purpose specified.

This part of the subject, however, is left entirely for the ingenuity

of the operator. No matter by hat means he accomplishes the object; all

that is required is to heat the plate above the temperature of the

atmosphere and retain that heat during the process of polishing.

Since the adoption of this method, in connection with my partner, T. J.

Dobyns, even in this humid climate of ours, when everything in the room

is dripping with moisture, it has been attended with invariable

success.

CHOICE OF PLATES, ETC.

In the great catalogue of complaints made by operators, none is more

common than that alleged against the quality of plates in general use.

Although the greatest diversity of opinion exists upon this subject,

nevertheless the plates of every manufactory share in this universal

condemnation.

To be sure it cannot be denied but that this necessary article of

utility in the photographic art has undergone a sad deterioration in

quality owing to the increasing demand and great reduction in price—the

plates of the present day being by no means so heavily coated with

silver as formerly—but the complaint alluded to is not predicated so

much upon the thinness of silver as upon a mysterious something which

has conferred upon the plates the epithet of not good.

That this complaint is in a great measure groundless appears evident

from the fact that while, with the same brand of plates one operator

can work successfully, another encounters the greatest difficulty;

while one is able to produce beautifully clear and altogether

satisfactory results, the other labors under the troublesome annoyance

of innumerable specks, large dark insensitive patches and brown

map-like portions, together with divers other blemishes, sufficient to

prevent him from obtaining anything like a tolerable impression.

From this wide difference in the results of the two operators using

identically the same article, it is but reasonable to conclude that the

complaint is founded in error; while the inference is no more than

just, that the fault may be traced to a want of practical skill on the

# part of the complaining operator himself; rather than to the inferior

quality of the plates.

The question, then, whether the plates are unfit for use, or whether

those who pronounce them so understand how to use them, appears to be

satisfactorily answered. It therefore becomes a matter worthy of

investigation, to ascertain what superior judgment and skill one

operator possesses over another which enable him to work successfully a

quality of plate, pronounced by the other entirely useless.

Suppose we make a critical examination of one of the repudiated plates.

From its external appearance we have little hesitation in pronouncing

it to be French; indeed, this presumption is strongly corroborated by

the fact that it is ornamented upon one of its corners with a brand to

designate the manufactory from which it emanated.

Upon close inspection we cannot fail to notice a striking peculiarity

upon the surface; the roughness is very remarkable; the planishing

hammer has left amazingly visible indications of its busy work. One

would suppose the manufacturer intended the surface of the plate to

represent the undulations of the sea, instead of that smooth and level

character so strongly recommended by M. Daguerre.

Such a plate necessarily requires at the hand of the operator

considerable labor before the surface is in a proper condition to

receive a suitable polish from the buffer. The least reflection in the

world should teach any one that so long as the undulatory character

continues upon the surface of the plate, it is in a very imperfect

condition for buffing, because the buffer cannot touch every point

equally; the elevated portions alone receiving a high degree of polish

while the depressed portion, from their roughness acting as nuclei,

gather dust, rouge, and other foreign bodies, so detrimental to

sensitiveness. The secret of the superior judgment and skill of one

operator over another, is intimately connected with this point: his

success depends very much upon the first process of cleaning the plate.

Let us examine the manipulation of the complaining operator. He takes

one of these plates and gives it a careful scouring with rotten-stone

and alcohol or any other liquid preferred for this part of the

operation—that is, he gives it what he terms a careful scouring—very

gently indeed because, from the frequent trials he is in the habit of

making in the camera, he fears he will rub the silver entirely away

before he succeeds in obtaining a good impression. The dark patches,

specks, and granular appearance resulting entirely from the unevenness

of the surface of the plate, look like copper to him, and he is

surprised that he should have rubbed away the silver so soon,

particularly by such delicate handling.

The judgment and experience of the successful operator, however, teach

him that scouring injures a plate less than buffing. He knows that

unless the hammer marks be obliterated, he cannot by the buffer produce

a surface of uniform polish and sensitiveness, without which a fair

proof is extremely doubtful; he knows that the time employed in the

preliminary operation of cleaning the plate properly is economy.

There is a style of French plates in the market, denominated heavy,

which are truly excellent, if properly managed. Much patience, however,

is required to remove the marks of the hammer; but with tripoli and

alcohol the surface is readily cut down, and the plate is then

susceptible of a beautiful black lustre by polishing with the buffer.

The complaining operator could not succeed by his own method with one

of the plates; he would encounter all manner of clouds and other

unaccountable phenomena; he would imagine this plate entirely worn out

before it was half cleaned, and soon fix in his own estimation the

reputation of the heavy plate.

In making a choice of plates, therefore, it would appear to be a matter

of perfect indifference with an experienced operator what kind he would

use, except so far only as the labor required in cleaning them was to

be taken into consideration.

The distinction between a scale plate, a Scovill No. 1, S. F., heavy A,

star, crescent, eagle, or any other brand, consists in the superior

finish of some, and the thinness of the silver in the cheaper

qualities.

Consequently, let the complaining operator but employ the diligence

inculcated in this article, to clean his plate thoroughly, so as to

bring it to a perfectly even and level surface, and he will seldom be

troubled with specks, clouds, dark patches, and the host of other

obstacles which heretofore have tormented him.

# CHAPTER VI.

AN ACCOUNT OF WOLCOTT AND JOHNSON’S EARLY EXPERIMENTS, IN THE

DAGUERREOTYPE. BY JOHN JOHNSON.

[From Humphrey’s Journal, vol. ii 1851]

As a general thing, however perfect any invention may be deemed by the

inventor or discoverer, it falls to the lot of most, to be the subject

of improvement and advancement, and especially is this the case with

those new projects in science which open an untrodden field to the view

of the artisan. Such has been, in an eminent degree, the case with the

discovery first announced to the world by Mons. Jean Jaques Claude

Daguerre, of Paris, in the year 1839, and which excited unbounded

astonishment, curiosity and surprise. It may be questioned had any

other than Daguerre himself discovered a like beautiful combination,

whether the world would have been favored with details exhibiting so

much care, patience and perseverance as the Daguerreotype on its

introduction. Shortly after, these details reached the United States,

by Professor S. F. B. Morse, of New York, who was, at the time of the

discovery, residing in Paris. By this announcement, the whole

scientific corps was set in operation, many repeating the experiments,

following carefully the directions pointed out by Daguerre, as being

necessary to success. Among the number in the United States, was

Alexander S. Wolcott (since deceased) and myself; both of this city. On

the morning of the 6th day of October, 1839, I took to A. Wolcott’s

residence, a full description of Daguerre’s discovery, he being at the

time engaged in the department of Mechanical Dentistry, on some work

requiring his immediate attention, the work being promised at 2 P.M.

that day; having, therefore, no opportunity to read the description for

himself (a thing he was accustomed to do at all times, when

investigating any subject). I read to him the paper, and proposed to

him that if he would plan a camera (a matter he was fully acquainted

with, both theoretically and practically), I would obtain the materials

as specified by Daguerre. This being agreed to, I departed for the

purpose, and on my return to his shop, he handed me the sketch of a

camera box, without at all explaining in what manner the lens was to be

mounted. This I also undertook to procure. After 2, P.M., he had more

leisure, when he proceeded to complete the camera, introducing for that

purpose a reflector in the back of the box, and also to affix a plate

holder on the inside, with a slide to obtain the focus on the plate,

prepared after the manner of Daguerre. While Mr. Wolcott was engaged

with the camera, I busied myself in polishing the silver plate, or

rather silver plated copper; but ere reaching the end preparatory to

iodizing, I found I had nearly or quite removed the silver surface from

off the plate, and that being the best piece of sliver-plated copper to

be found, the first remedy at hand that suggested itself, was a

burnisher, and a few strips were quickly burnished and polished.

Meantime, the camera being finished, Mr. Wolcott, after reading for

himself Daguerre’s method of iodizing, prepared two plates, and placing

them in the camera, guessed at the required time they should remain

exposed to the action of the light; after mercurializing each in turn,

and removing the iodized surface with a solution of common salt two

successful impressions were obtained, each unlike the other!

Considerable surprise was excited by this result, for each plate was

managed precisely like the other. On referring to Daguerre, no

explanation was found for this strange result; time, however, revealed

to us that one picture was positive, and the other negative. On this

subject I shall have much to say during the progress of the work.

Investigating, the cause of this difference occupied the remainder of

that day. However, another attempt was agreed upon, and the

instruments, plates, etc., prepared and taken up into an attic room, in

a position most favorable for light. Having duly arranged the camera, I

sat for five minutes, and the result was a profile miniature (a

miniature in reality,) or a plate not quite three-eighths of an inch

square. Thus, with much deliberation and study, passed the first day in

Daguerreotype—little dreaming or knowing into what a labyrinth such a

beginning was hastening us.

[Description of apparatus represented on pages 192 and 199:]

A.—The Box—about 4 inches long by about 2 outside diameter. B.—The

Reflector soldered to a brass screw, and mounted in the rear of the

box.

c.—The slide to regulate the focus to the plate holder. d.—The standard

to the plate holder screwed to the slide. f.—The plate-holder frame

having two small ledges, * *, for the plate to rest upon.

{192}

g.—The plate resting upon the ledge., * *, and kept against the frame

by the spring h. The plates used were about 3/8 of an inch square.

A.—The window with the sashes removed.

B and C (p. 199) are large looking-glasses mounted as plain reflectors,

the lower one C having rotary motion upon the saddle, resting upon the

sill of the window in order to direct the rays of the sun upon the

reflector B, at any hour of the day—the vertical motion of the

reflector C being necessary, the sun varying in altitude so much during

the hours most favorable to the production of portraits. The reflector

# C was {193} kept up to the required position by the handle lever,

upright post and bolts. Reflector B was hinged at its upper end at the

top of the window frame, the only motion being necessary was that which

would reflect upon the sitter the incident rays from reflector C—the

reflector B being kept at the required angle by the connecting lever m,

etc. Suitable back-grounds were placed behind the sitter.

[Illustration: Fig. 12]

The reflector B and C, had frequently to be renewed, the heat of the

sun soon destroying their brilliance or power of reflecting, light,

before renewing them, however, we resorted to the springing of them, by

which means their power was increased for a period.

The camera or reflecting apparatus, invented by Mr. Wolcott, was, from

the nature of the case, better adapted at that day to the taking of

portraits from life, than any other instruments. After carefully

examining the camera described by Daguerre, and the time stated as

necessary to produce action for an image, it became evident to the mind

of Mr. Wolcott at once, that more light could be obtained (as the field

of view required was not large) by employing a reflector of short focus

and wide aperture, than from a lens arrangement, owing to spherical

aberration and other causes. Many experiments having been tried with

the small instrument figured (p. 199), a reflector for taking portraits

from life was determined on, having eight inches diameter, with twelve

inches focal distance for parallel rays; this was to admit plates of

two inches wide by two and a half long Mr. Wolcott having on hand

reflectors of the right diameter, for Newtonian telescopes, of eight

feet focal distance, resolved (as it was a matter of experiment) to

grind down or increase the curve for the focal distance before

named—this required time. In the mean time, many plans were pursued for

making good plates, and the means of finishing, them. As the completion

of the large reflector drew to a close, our mutual friend, Henry Fitz,

Jr., returned from England, whither he had been on a visit, and when he

heard what we were about, kindly offered his assistance; he being well

versed in optics, and having been before engaged with Mr. Wolcott, in

that and other business is offer was gladly accepted—Mr. Wolcott

himself having frequent engagement; to fill as operator in the details

of mechanical dentistry. Thus, by the aid of Mr. Fitz, the reflector

was polished, and experiments soon after tried on plates of two by tow

and a half inches, with tolerable success. Illness on my part quite

suspended further trial for nearly four weeks.

On my recovery, early in January, 1840, our experiments were again

resumed with improved results, so much so as to induce Mr. Wolcott and

myself to entertain serious thoughts of making a business of the taking

of likenesses from life, intending to use the reflecting apparatus

invented by Mr. Wolcott, and for which he obtained Letters Patent, on

the 8th day of May, 1840. Up to January 1st, 1840, all experiments had

been tried on an economical scale, and the apparatus then made, was

unfit for public exhibition; we resolved to make the instruments as

perfect as possible while they were in progress of manufacture.

Experiments were made upon mediums for protecting the eyes from the

direct light of the sun, and also upon the best form and material for a

back-ground to the likenesses. The length of time required for a

“sitting,” even with the reflecting apparatus, was such as to render

the operation anything but pleasant. Expedients were ever ready in the

hands of Wolcott: blue glass was tried and abandoned in consequence of

being, at that time, unable to procure a piece of uniform density and

surface: afterwards a series of thin muslin screens secured to wire

frames were prepared as a substitute for blue glass. The objections to

these screens, however, were serious, inasmuch as a multiplication of

them became necessary to lessen the intensity of the light sufficiently

for due protection to the eyes, without which, the likenesses, other

than profiles, were very unpleasant to look upon. Most of the

portraits, then of necessity were profiles formed upon back-grounds,

the lighter parts relieved upon black, and the darker parts upon light

ground; the back-ground proper being of light colored material with

black velvet so disposed upon the light ground, this being placed

sufficiently far from the sitter, to produce harmony of effect when

viewed in the field of the camera. Other difficulties presented

themselves seriously to the working of the discovery of Daguerre, to

portrait taking—one of which was the necessity for a constant and

nearly horizontal light, that the shaded portions of the portrait

should not be too hard, and yet, at the same time, be sufficiently well

developed without the “high light” of the picture becoming overdone,

solarized or destroyed. In almost all the early specimens of the

Daguerreotype, extremes of light and shade presented themselves, much

to the annoyance of the early operators, and seriously objectionable

were such portraits. To overcome this difficulty, Mr. Wolcott mounted,

with suitable joints, upon the top of his camera, a large looking-glass

or plane reflector, in such a manner that the light of the sun (as a

strong light was absolutely necessary), when falling upon the glass

could be directed upon the person in an almost horizontal direction.

Early in February, 1840, Mr. Johnson, Sen., (since deceased) sailed for

Europe with a few specimen likenesses taken with the instruments

completed as above, with the intention of patenting the invention. On

his arrival a joint arrangement was effected with Mr. Richard Beard, of

London, in patenting and working the invention in England. Up to

February, 1840, but few friends had been made acquainted with the

progress of the art in the hands of Mr. Wolcott and myself. From time

to time reports reached us from various sources of the success of

others, and specimens of landscapes, etc., were exhibited at Dr. James

R. Chilton’s laboratory, in Broadway, much to the gratification of the

numerous visitors and anxious expectants for this most wonderful

discovery. Dr. Chilton, Professor J. J. Mapes, Professor J. W. Draper.

Professor S. F. B. Morse, all of this city; Mr. Cornelius, Dr. Goddard

and others of Philadelphia; Mr. Southworth, Professor Plumbe, and

numerous others were early in the field; all, however, using the same

description of camera as that of Daguerre, with modification for light,

either by enlargement by lens and aperture for light, or by shortening

the focal distance.

At a conversational meeting of the Mechanics’ Institute, Professor J.

J. Mapes being present, a question was asked if any one present could

give information relative to portraiture from life by the

Daguerreotype. Mr. Kells, a friend of Mr. Wolcott and a scientific and

practical man (since deceased), at once marked out upon the

black-board, the whole as contrived by Mr. Wolcott. This gave publicity

to the invention of Mr. Wolcott. Shortly after, Professor Mapes, Dr.

Chilton, and many others, sat for their portraits, and were highly

gratified. Professor Morse also came and proposed to Mr. Wolcott to

join him in the working of the invention, etc.

From this time much interest was manifested by our friends in our

progress. Rooms were obtained in the Granite Buildings, corner of

Broadway and Chambers street, and fitted for business. The rooms being

small, it was soon found impracticable to use the arrangement of

looking-glass, as previously spoken of; a new plan became necessary, to

introduce which, the sashes were removed, {199} and two large

looking-glasses were mounted in proper frames, thus:—

[Illustration: Fig. 13]

Just in front, and between the sitter and {200} the reflector, upon a

proper stand, were used those paper muslin screen before described;

also screens of tissue paper. These screens, however, when they were

used, required so much time for a sitting, that some other medium, as a

protection to the eyes, became absolutely necessary. The most plausible

thing that suggested itself was blue glass; but, as this could not be

found, numerous were the expedients proposed by the friends of the art,

who from time to time visited our rooms. At the suggestion of Professor

Mapes (who is ever ready to assist those in perplexity), a trough of

plate glass s, about twenty-eight inches square in the clear, and from

three to four inches thick, was filled with a solution of ammonia

sulphate of copper, and mounted on the frame as in the sketch, which,

for a time, answered extremely well; soon, however, decomposition of

this solution became apparent from the increased length of time

required for a sitting, although to the eye of an observer, no visible

cause for such long sittings could be pointed out. Professor Mapes

being appealed to, suggested that to the above solution a little acid

be added which acted like a charm—shortening the time for a sitting

from six, eight, or ten minutes to that of about one. Decomposition,

however, would go on by the action of light and heat through the

solution. New solutions were tried, when the whole were finally

abandoned as being, too uncertain and troublesome. (The reflecting

apparatus R, was placed upon the stand as in the sketch, with a wedge

for elevating the camera, between it and the table, to obtain the image

properly upon the plate.) A quantity of blue window glass was next

obtained, and holes drilled through the corners of it, and several

sheets were wired together to increase the size, and, when complete,

was suspended from the ceiling in its proper place, and so arranged

that when a person was sitting, this sheet of glass could be moved to

and from, the object of which was to prevent shadows on the face of the

sitter produced from the uneven surface of the glass. This latter

contrivance was used until a perfect plate of glass was procured.

The number of persons desirous of obtaining, their miniatures, induced

many to entertain the idea of establishing themselves in the Art as a

profession, and numerous were the applications for information; many

persons paying for their portraits solely with the view of seeing the

manner of our manipulations, in order that they might obtain

information to carry on likeness-taking as a business.

The reflecting camera being a very troublesome instrument to make, and

difficulties besetting us from every source, but little attention could

be given to teaching others; and, indeed, as the facts seemed to be at

this time, we knew but little of the necessary manipulations ourselves.

In course of time, several established themselves. The first one, after

ourselves, who worked the discovery of Daguerre for portrait taking in

this city, was a Mr. Prosch; followed soon after by many others, in

almost all cases copying the reflecting arrangement for light, as

figured above, many using it even after we had long abandoned that

arrangement for a better one.

Innumerable obstacles to the rapid advance of the daguerreotype,

presented themselves almost hourly, much to the annoyance of ourselves,

and those dependent upon our movements for their advancement. Among the

most difficult problems of the day, was the procuring of good plates.

Messrs. Corduran & Co. were among the first to supply the trade; at

that early day, however, it was a very rare thing, to be able to

procure an even perfect surface, from the fact that a pure surface of

silver could scarcely be obtained, the manufacturers deeming it too

much trouble to prepare silver plated copper with pure silver—the

result was, that in attempting to polish perfectly such plated metal as

could be procured, the plates would become cloudy, or colored in spots,

from the fact of having more or less alloy, according as more or less

of the silver surface was removed in polishing the plate fit for an

impression. To explain more clearly, it was the practice of most silver

platers to use an alloy for silver-plating. In the reduction of the

ingot to sheet metal, annealing has to be resorted to, and acid pickles

to remove oxides, etc. The number of times the plated metal is exposed

to heat and acid in its reduction to the required thickness, produces a

surface of pure silver. The most of this surface is, however, so rough

as to be with difficulty polished, without in places removing entirely

this pellicle of pure metal, and exposing a polished surface of the

alloy used in plating. Whenever such metal was used, very unsightly

stains or spots frequently disfigured the portraits. The portrait, or

portion of it, developed upon the pure silver, being much lighter or

whiter than that developed upon the alloy; it therefore appeared that

the purer the silver, the more sensitive the plate became. Accordingly,

we directed Messrs. Scovills, of Connecticut, to prepare a roll of

silver-plated metal, with pure silver; it fortunately proved to be a

good article, but, unfortunately, a pound of this metal (early in 1840)

cost the round sum of $9. Like descriptions of metal, the same

gentlemen would be glad to furnish, at this time, for $4. Soon after

this, some samples of English plated metal, of a very superior quality,

came to our possession, and relieved us from the toil of making and

plating one plate at a time, an expedient we were compelled to resort

to, to command material to meet the pressing demands for portraits.

Having it now in our power to obtain good plated metal, a more rapid

mode of polishing than that recommended by Daguerre was attempted as

follows:

This metal was cut to the desired size, and having a pair of “hand

rolls” at hand, each plate, with its silvered side placed next to the

highly polished surface of a steel die, was passed and repassed through

the rolls many times, by which process a very smooth, perfect surface

was obtained. The plates were then annealed, and a number of plates

thus prepared were fastened to the bottom of a box a few inches deep a

foot wide, and eighteen inches long; this box was placed upon a table

and attached to a rod connected to the face plate of a lathe, a few

inches from its centre, so as to give the box a reciprocating motion. A

quantity of emery was now strewn over the plates, and the lathe set in

motion. The action produced wag a friction or rubbing of the emery over

the surface of the plates.

When continued for some time, a greyish polish was the result. Linseed,

when used in the same manner, gave us better hope of success, and the

next step resorted to was to build a wheel and suspend it after the

manner of a grindstone. The plates being secured to the inner side of

the wheel or case, and as this case revolved, the seeds would

constantly keep to the lower level, and their sliding over the surface

of the plates would polish or burnish their surfaces. This, with the

former, was soon abandoned; rounded shots of silver placed in the same

wheel were found not to perform the polishing so well as linseed.

Buff-wheels of leather with rotten-stone and oil, proved to be far

superior to all other contrivances; and, subsequently, at the

suggestion of Professor Draper, velvet was used in lieu of buff

leather, and soon superseded all other substances, both for lathe and

hand-buffs, and I would add, for the benefit of new beginners that

those who are familiar with its use, prefer cotton velvet. The only

requisite necessary is, that the buffs made of cotton velvet should be

kept dry and warm.

The greater number of operators, with whose practice I am familiar,

use, for polishing plates, prepared tripoli, imported from France, or

Browne’s rotten-stone. The former of these articles is very

objectionable, inasmuch as there is no positive certainty of being

enabled to procure or make the article of uniform grit—the nature of

the substance rendering, it impossible to reduce it to varying degrees

of evenness, by the well known process of washing, for that purpose,

and the burning of rotten-stone changes its chemical nature somewhat,

at the same time rendering, this invaluable article harsh and gritty.

And especially, no reliance can be placed upon burned rotten stone if

purchased from those who do not give very great attention and care to

its preparation; and the same remarks apply to rouge.

The best article for polishing Daguerreotype plates is rotten-stone,

such as can be procured in any town, prepared after the following

manner: Procure, say half a dozen wide-mouthed bottles, of suitable

dimensions, numbering each from one to six. Put into No. 1 about half a

pound of rotten-stone, and nearly fill the bottle with water. Then,

with a proper stick or spatule, mix well the rotten-stone and water;

after which, let No. 1 rest for, say one minute, then carefully pour

off into bottle No. 2 (or, what would be better, draw off by a syphon)

as much of the floating particles of rotten-stone as is suspended in

the water. Again fill bottle No. 1 with water, agitate it as before,

and decant it to bottle No. 2, care being taken to draw off only the

suspended particles of rotten-stone.

When a sufficient quantity of washings from bottle No. 1 is collected

into bottle No. 2, a similar process must be gone through, as above

stated, for No. 1; the difference being in the care required, and in

the time allowed between the stirring or mixing the rotten-stone and

water. The floating particles of rotten-stone, after four minutes’

subsiding, will be found fine enough for the finest Daguerreotype

polishing required.

A quantity of such washings may be collected in a large bottle, and

allowed to stand a few hours, when all the rotten-stone will have

settled. The water may be poured off and the rotten-stone put into an

evaporating dish, and while being dried, must be constantly stirred to

obtain an impalpable powder.

Further washings may in like manner be resorted to for finer qualities

of rotten-stone. In my practice, I have used the articles at two and

four minutes’ settling, and occasionally have prepared it after

standing for eight minutes. So fine a quality as this, however, is

seldom required. In using, rotten-stone, I mix with it, for polishing,

fine olive oil, until I obtain a thin paste—and the best of all methods

for polishing (well planished) Daguerreotype plates, is one like that

used for glass by lens polishers; that is, by using a disc or

buff-wheel, and having, a suitable holder by which to secure the plate,

and then by pressing the plate against the revolving buff, well

saturated with the mixed oil and rotten-stone, a very good surface is

obtained. A quantity of plates may be prepared in this way, and all the

adhering oil, etc., may be removed by a clean hand, or lathe buff,

after which each plate must be heated to the point necessary to burn

off the remaining oil great care being required not to overheat the

plate. A very slight excess of temperature will at once destroy all the

polish previously obtained. The test for ascertaining the right

temperature is at hand; the adhering oil will be driven from the plate

in the form of smoke when the right temperature is reached. The moment

the smoke ceases to rise from the plate, the heat must be removed, and

the plate quickly cooled upon a piece of iron.

A quantity of plates thus prepared may be kept on hand for any required

time, and the labor of one minute, with a lathe or hand-buff with dry

charcoal, or rather, prepared lampblack, will perfectly polish the

surface ready for indexing, etc. This lampblack also requires some care

in preparing. Take a small-size crucible, properly temper it by a slow

fire, that it may not be cracked after which, fill it with common

lampblack, cover it over with a piece of soap-stone, and again replace

it in the fire. Build a good hard coal fire around it continue the heat

for two or three hours, being careful not to raise the cover till the

crucible be quite cold. Pulverize when using it. It is very desirable

to keep this lampblack dry and warm. Some operators use much rouge I

would recommend the above in preference; but those who feel that they

cannot dispense with the use of rouge, had better try a large addition

of prepared lampblack to a small one of rouge, as this latter article,

unless great pains be taken in its preparation, will adhere and work

itself into the body of the surface, so that it cannot be removed

therefrom; and I have seen many specimens of Daguerreotype very much

injured in effect from this rouge tint disseminated throughout their

shaded features, at the same time that the whole general effect of such

pictures is that of a want of life. It is true that with the use of

rouge a very high degree of polish may be obtained, but probably not

higher than can be produced with many other substances of a less

objectionable nature.

From the announcement of the discovery by Daguerre to the beginning of

the year 1840, I am not aware of any attempt to lessen the time for the

action of an image, or an impression, other than that of the reflecting

camera invented by Mr. Wolcott. Early, however, in 1840, Mr. Wolcott

was desirous to be enabled to further shorten the time for a sitting,

and having some knowledge of bromine and its action, by request, Dr.

Chilton prepared a small quantity; but Mr. Wolcott did not succeed very

well with it, he having invariably used too much in combination with

iodine to produce that sensitive coating now well known to the

profession. Professor Morse, of this city, Dr. Goddard, of

Philadelphia, and others, in the years 1840 and 1841, were acquainted

with the use of bromine. N. Griffing, of this city, or myself, used

with tolerable success, iodine in large excess to nitric acid and

water; and, subsequently, to nitro muriatic acid (which reacted and

formed a peculiar chloride of iodine); this latter combination proved

to be preferable to simple iodine, at the same time somewhat more

sensitive, and was used by me in this city up to the time of my leaving

for London (October 1, 1840). On arriving in London, I instituted a

series of experiments in the various chemical combinations, solely with

the view to be enabled to obtain more speedily a portrait than it was

practicable to do with any known chemicals at that date. The high

latitude, and the winter season of the year rendering but a feeble

light at best, the greater the necessity for a more sensitive chemical

preparation to the shortening the time for a sitting. Near the

beginning of the year 1841, I discovered and practically applied,

chloride of iodine to great advantage, and, as far as memory serves me,

# I believe the first used in this country was some made and shipped,

Messrs. Harnden & Co., from London, to Mr. Wolcott, in New York.

About the same time, Mr. John Goddard, of London (who was associated

with myself), discovered a rather valuable combination of chemicals,

consisting of a mixture of iodine, bromine, iodus, and iodic acid, and

a proper combination of those bodies gave an action somewhat more

sensitive than chloride of iodine—but the “high lights” of the

portraits would become solarized or overdone, more frequently with this

combination than with the chloride of iodine. Throughout the year 1841,

# I used, with great success, chloride of iodine, applied as one

coating—occasionally in conjunction with Mr. Wolcott, attempting the

use of iodine, bromine, and chlorine, and at times with more or less

success. The difficulty of exactly combining, the three elements above

mentioned, in order to produce a certainty of result with harmony of

effect, was the work of many months, with great labor and study, the

slightest modification requiring a long, series of practical

experiments, a single change consuming, frequently, an entire day in

instituting comparisons, etc., etc.

Early in the year, 1842, I discovered a combination of chemicals (now

known in London as “Wolcott’s Mixture,” in hermetically sealed bulbs)

of exceeding uniform character, very sensitive to the action of light,

and specimens produced in 1842-3, with this combination, will bear

comparison with the best specimens produced at this late date.

About the same time, I discovered that however much overdone a

Daguerreotype might be, the means were at hand to save or redeem it. It

has long, since been known to operators, that if a plate be exposed to

light after being coated, unless it be again coated, a clear and

distinct picture could not be obtained upon the same plate without

first repolishing and recoating the same, care being taken that no

light fall upon the prepared surface. To prevent solarization, coat a

plate as usual, expose to the action of light any required time

(according to circumstances), say from quarter to one half more time

than would be required in the ordinary method of procedure; observe,

before putting the plate in the mercury box, place it over the vapor of

iodine, bromine, or chlorine, etc. (carefully excluding the light), for

a very brief period, great care being required to have the selected

vapor very much diluted with air, in order to success. Many experiments

will be required ere arriving at satisfactory results. Specimens now

unknown to general operators, for harmony of effect, have been, and may

again be produced by the method pointed out above. I have found the

best general effect, and the most certain result to follow from the use

of the vapor of chlorine—but this requires more than ordinary care. I

would, therefore, recommend the use of iodine. Thus: to a few grains of

iodine, add an ounce of warm water (which will become tinged with

iodine); when cold, to half a pint of pure water in a new and clean

coating box, put, of the above, fifty drops; stir and mix well this

small quantity of iodine in with the water; in ten minutes this box

will be ready for use. Great care and judgment will be required in the

application of this vapor to the plate; if the plate remain over the

vapor too long, the developed picture will have a faint and misty

appearance; if not exposed long enough, the “high light” will be

solarized. I have great hope of the ultimate use of this process, as it

is the only means yet discovered to be enabled to secure specimens of

extremes of light and shade, yet producing harmony of effect; and I

would call the attention of the profession to the fact, that a plate

may be exposed to the action of light for any length of time (a

thousand times longer than required to act for the lesser quantity of

mercury to deposit itself, or that amount necessary to form a perfect

specimen), and be restored by the application of any of the vapors

above mentioned, remarking that for extremes for solarization, denser

vapors will be required. Much remains to be done with this discovery to

the application of the Daguerreotype.

HUMPHREY’S JOURNAL OF THE DAGUERREOTYPE & PHOTOGRAPHIC ARTS.

The above-named publication is well known as the best and most valuable

one devoted to the Photographic Science in this country. Humphrey’s

Journal made its appearance November 1st, 1850, and consequently is the

first and oldest serial offered to the Photographic world.

The art of producing Portraits and Landscapes by means of Light, has

recently taken a new and enlivening impulse, which will in all

probability lead to important and interesting results. No practical

Daguerreotypist, Photographer, or amateur, should be without the means

at hand for securing all of the information upon this subject. Each

should be ready to receive and apply the improvements as they may be

developed. In order to accomplish this, it is a matter of great

importance to the Practitioner or Experimenter that he should have a

reliable medium through which he can obtain information. In what source

can the inquirer better place his confidence than in a regular Journal,

whose editor is literally a practical person, and familiar with the

manipulations necessary for producing Portraits upon “Daguerreotype

Plates,” and upon glass and paper? Such is the conductor of Humphrey’s

Journal.

This Journal is published once every two weeks, and contains all the

improvements relating to the Art, and is the only American Journal

whose editor is practically acquainted with the process for producing

Daguerreotypes, Ambrotypes, and Photographs. The first No. of Vol. X.

is dated May 1st, 1858. The terms (Two Dollars per annum) are trifling

compared with the vast amount of information furnished.

AMBROTYPES.—Humphrey’s Journal contains everything novel which appears

upon this subject, and has already presented more new, important, and

original matter than can be found in any other place.

Many are the letters we have received during the term of the last

volume, in which the writer has stated that a single number of

Humphrey’s Journal has contained information of more value to him than

“several times the amount paid for the entire volume.”

Our resources have grown up around us, and our facilities for

procuring, as well as distributing, all such facts and improvements as

will benefit as well as instruct all who have the progress of the Art

at heart, are as ample as they can well be made.

The future volumes will be abundantly furnished with original writings

from persons of standing in the scientific world; and the practical

Photographer will here find a full account of such improvements as may

from time to time develop themselves.

From the editor’s long practical experience in the Heliographic

Science, he will be enabled to present the subject in a plain, clear,

and concise manner.

Read what the Editors say of Humphrey’s Journal:—

“We have received a copy of a valuable Journal (Humphrey’s) published

in New York, which has reached the 18th number of Vol. VI.... We now

have the pleasure of quoting from our trans-atlantic

coadjutor.”—Liverpool Photographic Jour.

“Humphrey’s Journal is practical as well as scientific in

character.”—American Journal of Science and Arts.

“It treats the subject knowingly and with force.”—New York Tribune.

“It is both a popular and interesting publication.”—Mechanics’

Magazine.

“It is highly useful to all who practice ‘shadow catching.’”—Ohio State

Journal.

“The work is neatly gotten up, and contains many interesting varieties

in this new field of science.”—Times.

“It should be hailed and encouraged, not only by Daguerreotypists

themselves, but by every lover of Science and Art.”—The Democrat.

“We cannot too strongly urge all artists, and those persons who feel an

interest in the Heliographic Arts and Sciences, to take a copy of the

work.”—Sentinel.

“It is indicative of talent worthy of the important Art it is designed

to elevate.”—American.

“This Art is entitled to its own organ, which could not have fallen

into better hands than those of the editor of ‘Humphrey’s

Journal.’”—Transcript.

“It is a scientific work of interest and usefulness.”—Star of the

North.

“This Journal answers many points heretofore regarded as

inexplicable.”—Hudson River Chronicle.

“It is rich with interest.”—North American.

“It contains all the ‘Improvements.’”—Delta.

“It teaches us how to take our own portraits.”—Bee.

“It will cultivate a taste for Daguerreotypes.”—Commercial Advertiser.

“It should be in the hands of all.”—Reveille.

“It is the Daguerreotypist’s friend.”—London News.

“It should be found in every library.”—Evening Journal.

From some of our Subscribers

“Humphrey’s Journal has been the means of saving much time and money,

for by its instruction I have been enabled to produce some of the

finest Paper Pictures I have ever seen.” W. P.

“Don’t fail to send me the Journal, for I would not be without it for

five times the amount of subscription. It is the only publication I can

depend upon.” A. G. R.

“Your treatment of the humbugs and humbugging members of the

profession, is of the most valuable importance to us practical

Daguerreans. Go on. God speed! Here is the amount for the renewal of my

subscription.” E. F. S.

“How can any Operator afford to be without it?” L. L. H.

“Here are five dollars: send me Humphrey’s Journal to this amount. I

will not be without it.” M. S.

“It is my best friend.” J. E. W.

We might quote like commendatory extracts enough to more than ten times

fill this page.

Humphrey’s Journal contains 16 octavo pages of reading matter.

TERMS.

One copy per annum, in advance . . . . . .   $2 00

Three copies,  do.      do.    . . . . . .   $5 00

Six copies,    do.      do.    . . . . . .   $9 00

The thousands who read it cannot be induced to remain without it. All

who desire to keep up with the improvements should subscribe for a

copy.

Subscription price Two Dollars per year.

Don’t fail to become a subscriber. Address

S. D. HUMPHREY Office, 37 Lispenard Street, NEW YORK.

THE PRACTICAL MANUAL OF THE COLLODION PROCESS. BY S. D. HUMPHREY.

THIRD EDITION

This Edition contains all the Improvements in the Art made public up to

the day of publication, and gives complete Practical Instructions for

making Collodion Positives or Ambrotypes, Collodion Negatives,

Printing, etc., etc. The quick, great and unprecedented sale of the

first and second editions demonstrates, more than the strongest

language could possibly do, the extraordinary and increasing popularity

of this work. The Third Edition contains two hundred and sixteen 12mo.

pages, of a larger size and in smaller type than either of the

preceding editions, and is illustrated with numerous wood-cuts. It is

intended to be the best practical work extant; substantially bound in

cloth, price One Dollar; forwarded by mail (postage prepaid). Address

S. D. HUMPHREY, New York

A GUIDE TO PHOTOGRAPHY,

Containing simple and concise directions for obtaining Views,

Portraits, etc., by the chemical agency of Light, by W. H.

Thornthwaite, author of “Photogenic Manipulations,” etc. Illustrated

with numerous wood-cuts. The Book contains more than one hundred 12mo

pages, bound in board, and is sold at twenty-five cents per copy, or

five copies for one dollar. Address

S. D. HUMPHREY, New York

HOLMES, BOOTH & HAYDENS, MANUFACTURERS OF DAGUERREOTYPE CASES, PLATES,

CAMERAS, MATTINGS, PRESERVERS, ETC., ETC.

MANUFACTURERS AND IMPORTERS OF EVERY VARIETY OF AMBROTYPE, AND

PHOTOGRAPHIC GOODS,

Of the Best and most approved Quality.

ESPECIAL ATTENTION GIVEN TO THE FURNISHING OF EVERY ARTICLE USED IN THE

PRACTICE OF THE DAGUERREOTYPE

Depot 81 Chambers and 63 Reade St., New York.

Manufactory at Waterbury, Conn.

HOLMES, BOOTH & HAYDENS’ CAMERAS

From 1/4 to the Mammoth size. These CAMERAS are of the most superior

make, and all subjected to the most thorough test before being offered

for sale.

VIEW CAMERAS, made expressly for taking views: an entirely new article.

DAGUERREOTYPE PLATES, OF EVERY VARIETY, AND WE WOULD CALL PARTICULAR

ATTENTION TO THE

Wreath, and H. B. & H. Eagle 40 Plates, AS HAVING NO EQUALS IN THE

MARKET.

SOLE AGENTS FOR THE CELEBRATED H. B. AND N. P. FRENCH PLATES.

Always on hand a complete assortment of Ambrotype and Photographic

goods.

SOLE AGENTS FOR THE Patent solid glass corner Plate Holders.

All orders will receive prompt attention, and be forwarded with

dispatch.

HOLMES, BOOTH & HAYDENS, 81 Chambers, and 63 Reade St. New York.

SCOVILL MANUFACTURING CO.

MANUFACTURER OF DAGUERREOTYPE PLATES, MATTINGS, PRESERVERS, CASES,

APPARATUS, etc., etc.

Importers and Dealers in every description of Daguerreotype,

Photographic, AND AMBROTYPE GOODS,

The SCOVILL MANUFACTURING COMPANY flatter themselves that an experience

of nearly twenty years in the business and the most extensive variety

of the above Goods in the United States, entitle them to the

continuance of orders for the Domestic and Foreign trade, which will

receive the most careful attention.

Park Building, New York. Entrances—36 Park Row, 4 Beekman, and 141

Nassau Street.

SCOVILL MANUFACTURING CO.

Would call especial attention to their large variety of

CASES,

Embracing many Fancy Styles made only by themselves, and to which they

are constantly adding New Designs.

Union Cases, Of all sizes, with Riveted Hinges. AGENTS FOR THE SALE OF

# C. C. HARRISON’S CAMERAS,

with improvements, which we are now prepared to sell at reduced prices,

and warranted to give better satisfaction than ever before

DAGUERREOTYPE PLATES, H. B.—N. P.—Star and other brands PLATE GLASS,

embracing three-quarters white: Crown and all other varieties. We would

call particular attention to our Black Glass, made expressly for

Ambrotypes.

CHEMICALS, for the Daguerreotype and Photographic Art. Iodized and

Plain COLLODION. Gun Cotton, etc., etc. Tagliabue’s Collodiometres and

Actino-Hydrometres, for testing Chemicals.

APPARATUS OF EVERY VARIETY.

Gutta Percha, Porcelain, and Glassware of all kinds used in the Art.

A large assortment of Gilt Frames always on hand and made to order.

Ambrotype Shields, with solid corners of a new style.

All orders will meet with prompt attention

SCOVILL MANUFACTURING CO., PARK BUILDINGM New York 36 Park Row, 4

Beckman, and 141 Nassau Street.

HARDWICH’S PHOTOGRAPHIC CHEMISTRY AMERICAN EDITION.

A MANUAL of PHOTOGRAPHIC CHEMISTRY,

INCLUDING THE PRACTICE OF THE COLLODION PROCESS.

BY

T. FREDERICK HARDWICH,

LECTURER ON PHOTOGRAPHY IN KINGS COLLEGE, LONDON; LATE DEMONSTRATOR OF

CHEMISTRY IN KING’S COLLEGE.

Last Edition,

The above is the title of this new and valuable work. It is too well

known to need any further comment in this place. This volume contains

nearly 300 large duodecimo pages, bound in red cloth, $1.00. Copies to

be forwarded by mail, $1.18, postage pre-paid.

S. D. HUMPHREY, Photographic Book Publisher, 37 LISPENARD

STREET, NEW YORK,

N.B. Postage stamps taken.

*** END OF THE PROJECT GUTENBERG EBOOK AMERICAN HAND BOOK OF THE DAGUERREOTYPE ***

Updated editions will replace the previous one—the old editions will

be renamed.

Creating the works from print editions not protected by U.S. copyright

law means that no one owns a United States copyright in these works,

so the Foundation (and you!) can copy and distribute it in the United

States without permission and without paying copyright

royalties. Special rules, set forth in the General Terms of Use part

of this license, apply to copying and distributing Project

Gutenberg™ electronic works to protect the PROJECT GUTENBERG™

concept and trademark. Project Gutenberg is a registered trademark,

and may not be used if you charge for an eBook, except by following

the terms of the trademark license, including paying royalties for use

of the Project Gutenberg trademark. If you do not charge anything for

copies of this eBook, complying with the trademark license is very

easy. You may use this eBook for nearly any purpose such as creation

of derivative works, reports, performances and research. Project

Gutenberg eBooks may be modified and printed and given away—you may

do practically ANYTHING in the United States with eBooks not protected

by U.S. copyright law. Redistribution is subject to the trademark

license, especially commercial redistribution.

START: FULL LICENSE

THE FULL PROJECT GUTENBERG LICENSE

PLEASE READ THIS BEFORE YOU DISTRIBUTE OR USE THIS WORK

To protect the Project Gutenberg™ mission of promoting the free

distribution of electronic works, by using or distributing this work

(or any other work associated in any way with the phrase “Project

Gutenberg”), you agree to comply with all the terms of the Full

Project Gutenberg™ License available with this file or online at

www.gutenberg.org/license.

Section 1. General Terms of Use and Redistributing Project Gutenberg™

electronic works

1.A. By reading or using any part of this Project Gutenberg™

electronic work, you indicate that you have read, understand, agree to

and accept all the terms of this license and intellectual property

(trademark/copyright) agreement. If you do not agree to abide by all

the terms of this agreement, you must cease using and return or

destroy all copies of Project Gutenberg™ electronic works in your

possession. If you paid a fee for obtaining a copy of or access to a

Project Gutenberg™ electronic work and you do not agree to be bound

by the terms of this agreement, you may obtain a refund from the person

or entity to whom you paid the fee as set forth in paragraph 1.E.8.

1.B. “Project Gutenberg” is a registered trademark. It may only be

used on or associated in any way with an electronic work by people who

agree to be bound by the terms of this agreement. There are a few

things that you can do with most Project Gutenberg™ electronic works

even without complying with the full terms of this agreement. See

paragraph 1.C below. There are a lot of things you can do with Project

Gutenberg™ electronic works if you follow the terms of this

agreement and help preserve free future access to Project Gutenberg™

electronic works. See paragraph 1.E below.

1.C. The Project Gutenberg Literary Archive Foundation (“the

Foundation” or PGLAF), owns a compilation copyright in the collection

of Project Gutenberg™ electronic works. Nearly all the individual

works in the collection are in the public domain in the United

States. If an individual work is unprotected by copyright law in the

United States and you are located in the United States, we do not

claim a right to prevent you from copying, distributing, performing,

displaying or creating derivative works based on the work as long as

all references to Project Gutenberg are removed. Of course, we hope

that you will support the Project Gutenberg™ mission of promoting

free access to electronic works by freely sharing Project Gutenberg™

works in compliance with the terms of this agreement for keeping the

Project Gutenberg™ name associated with the work. You can easily

comply with the terms of this agreement by keeping this work in the

same format with its attached full Project Gutenberg™ License when

you share it without charge with others.

1.D. The copyright laws of the place where you are located also govern

what you can do with this work. Copyright laws in most countries are

in a constant state of change. If you are outside the United States,

check the laws of your country in addition to the terms of this

agreement before downloading, copying, displaying, performing,

distributing or creating derivative works based on this work or any

other Project Gutenberg™ work. The Foundation makes no

representations concerning the copyright status of any work in any

country other than the United States.

1.E. Unless you have removed all references to Project Gutenberg:

1.E.1. The following sentence, with active links to, or other

immediate access to, the full Project Gutenberg™ License must appear

prominently whenever any copy of a Project Gutenberg™ work (any work

on which the phrase “Project Gutenberg” appears, or with which the

phrase “Project Gutenberg” is associated) is accessed, displayed,

performed, viewed, copied or distributed:

This eBook is for the use of anyone anywhere in the United States and most

other parts of the world at no cost and with almost no restrictions

whatsoever. You may copy it, give it away or re-use it under the terms

of the Project Gutenberg License included with this eBook or online

at www.gutenberg.org. If you

are not located in the United States, you will have to check the laws

of the country where you are located before using this eBook.

1.E.2. If an individual Project Gutenberg™ electronic work is

derived from texts not protected by U.S. copyright law (does not

contain a notice indicating that it is posted with permission of the

copyright holder), the work can be copied and distributed to anyone in

the United States without paying any fees or charges. If you are

redistributing or providing access to a work with the phrase “Project

Gutenberg” associated with or appearing on the work, you must comply

either with the requirements of paragraphs 1.E.1 through 1.E.7 or

obtain permission for the use of the work and the Project Gutenberg™

trademark as set forth in paragraphs 1.E.8 or 1.E.9.

1.E.3. If an individual Project Gutenberg™ electronic work is posted

with the permission of the copyright holder, your use and distribution

must comply with both paragraphs 1.E.1 through 1.E.7 and any

additional terms imposed by the copyright holder. Additional terms

will be linked to the Project Gutenberg™ License for all works

posted with the permission of the copyright holder found at the

beginning of this work.

1.E.4. Do not unlink or detach or remove the full Project Gutenberg™

License terms from this work, or any files containing a part of this

work or any other work associated with Project Gutenberg™.

1.E.5. Do not copy, display, perform, distribute or redistribute this

electronic work, or any part of this electronic work, without

prominently displaying the sentence set forth in paragraph 1.E.1 with

active links or immediate access to the full terms of the Project

Gutenberg™ License.

1.E.6. You may convert to and distribute this work in any binary,

compressed, marked up, nonproprietary or proprietary form, including

any word processing or hypertext form. However, if you provide access

to or distribute copies of a Project Gutenberg™ work in a format

other than “Plain Vanilla ASCII” or other format used in the official

version posted on the official Project Gutenberg™ website

(www.gutenberg.org), you must, at no additional cost, fee or expense

to the user, provide a copy, a means of exporting a copy, or a means

of obtaining a copy upon request, of the work in its original “Plain

Vanilla ASCII” or other form. Any alternate format must include the

full Project Gutenberg™ License as specified in paragraph 1.E.1.

1.E.7. Do not charge a fee for access to, viewing, displaying,

performing, copying or distributing any Project Gutenberg™ works

unless you comply with paragraph 1.E.8 or 1.E.9.

1.E.8. You may charge a reasonable fee for copies of or providing

access to or distributing Project Gutenberg™ electronic works

provided that:

• You pay a royalty fee of 20% of the gross profits you derive from

the use of Project Gutenberg™ works calculated using the method

you already use to calculate your applicable taxes. The fee is owed

to the owner of the Project Gutenberg™ trademark, but he has

agreed to donate royalties under this paragraph to the Project

Gutenberg Literary Archive Foundation. Royalty payments must be paid

within 60 days following each date on which you prepare (or are

legally required to prepare) your periodic tax returns. Royalty

payments should be clearly marked as such and sent to the Project

Gutenberg Literary Archive Foundation at the address specified in

Section 4, “Information about donations to the Project Gutenberg

Literary Archive Foundation.”

• You provide a full refund of any money paid by a user who notifies

you in writing (or by e-mail) within 30 days of receipt that s/he

does not agree to the terms of the full Project Gutenberg™

License. You must require such a user to return or destroy all

copies of the works possessed in a physical medium and discontinue

all use of and all access to other copies of Project Gutenberg™

works.

• You provide, in accordance with paragraph 1.F.3, a full refund of

any money paid for a work or a replacement copy, if a defect in the

electronic work is discovered and reported to you within 90 days of

receipt of the work.

• You comply with all other terms of this agreement for free

distribution of Project Gutenberg™ works.

1.E.9. If you wish to charge a fee or distribute a Project

Gutenberg™ electronic work or group of works on different terms than

are set forth in this agreement, you must obtain permission in writing

from the Project Gutenberg Literary Archive Foundation, the manager of

the Project Gutenberg™ trademark. Contact the Foundation as set

forth in Section 3 below.

1.F.

1.F.1. Project Gutenberg volunteers and employees expend considerable

effort to identify, do copyright research on, transcribe and proofread

works not protected by U.S. copyright law in creating the Project

Gutenberg™ collection. Despite these efforts, Project Gutenberg™

electronic works, and the medium on which they may be stored, may

contain “Defects,” such as, but not limited to, incomplete, inaccurate

or corrupt data, transcription errors, a copyright or other

intellectual property infringement, a defective or damaged disk or

other medium, a computer virus, or computer codes that damage or

cannot be read by your equipment.

1.F.2. LIMITED WARRANTY, DISCLAIMER OF DAMAGES - Except for the “Right

of Replacement or Refund” described in paragraph 1.F.3, the Project

Gutenberg Literary Archive Foundation, the owner of the Project

Gutenberg™ trademark, and any other party distributing a Project

Gutenberg™ electronic work under this agreement, disclaim all

liability to you for damages, costs and expenses, including legal

fees. YOU AGREE THAT YOU HAVE NO REMEDIES FOR NEGLIGENCE, STRICT

LIABILITY, BREACH OF WARRANTY OR BREACH OF CONTRACT EXCEPT THOSE

PROVIDED IN PARAGRAPH 1.F.3. YOU AGREE THAT THE FOUNDATION, THE

TRADEMARK OWNER, AND ANY DISTRIBUTOR UNDER THIS AGREEMENT WILL NOT BE

LIABLE TO YOU FOR ACTUAL, DIRECT, INDIRECT, CONSEQUENTIAL, PUNITIVE OR

INCIDENTAL DAMAGES EVEN IF YOU GIVE NOTICE OF THE POSSIBILITY OF SUCH

DAMAGE.

1.F.3. LIMITED RIGHT OF REPLACEMENT OR REFUND - If you discover a

defect in this electronic work within 90 days of receiving it, you can

receive a refund of the money (if any) you paid for it by sending a

written explanation to the person you received the work from. If you

received the work on a physical medium, you must return the medium

with your written explanation. The person or entity that provided you

with the defective work may elect to provide a replacement copy in

lieu of a refund. If you received the work electronically, the person

or entity providing it to you may choose to give you a second

opportunity to receive the work electronically in lieu of a refund. If

the second copy is also defective, you may demand a refund in writing

without further opportunities to fix the problem.

1.F.4. Except for the limited right of replacement or refund set forth

in paragraph 1.F.3, this work is provided to you ‘AS-IS’, WITH NO

OTHER WARRANTIES OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT

LIMITED TO WARRANTIES OF MERCHANTABILITY OR FITNESS FOR ANY PURPOSE.

1.F.5. Some states do not allow disclaimers of certain implied

warranties or the exclusion or limitation of certain types of

damages. If any disclaimer or limitation set forth in this agreement

violates the law of the state applicable to this agreement, the

agreement shall be interpreted to make the maximum disclaimer or

limitation permitted by the applicable state law. The invalidity or

unenforceability of any provision of this agreement shall not void the

remaining provisions.

1.F.6. INDEMNITY - You agree to indemnify and hold the Foundation, the

trademark owner, any agent or employee of the Foundation, anyone

providing copies of Project Gutenberg™ electronic works in

accordance with this agreement, and any volunteers associated with the

production, promotion and distribution of Project Gutenberg™

electronic works, harmless from all liability, costs and expenses,

including legal fees, that arise directly or indirectly from any of

the following which you do or cause to occur: (a) distribution of this

or any Project Gutenberg™ work, (b) alteration, modification, or

additions or deletions to any Project Gutenberg™ work, and (c) any

Defect you cause.

Section 2. Information about the Mission of Project Gutenberg™

Project Gutenberg™ is synonymous with the free distribution of

electronic works in formats readable by the widest variety of

computers including obsolete, old, middle-aged and new computers. It

exists because of the efforts of hundreds of volunteers and donations

from people in all walks of life.

Volunteers and financial support to provide volunteers with the

assistance they need are critical to reaching Project Gutenberg™’s

goals and ensuring that the Project Gutenberg™ collection will

remain freely available for generations to come. In 2001, the Project

Gutenberg Literary Archive Foundation was created to provide a secure

and permanent future for Project Gutenberg™ and future

generations. To learn more about the Project Gutenberg Literary

Archive Foundation and how your efforts and donations can help, see

Sections 3 and 4 and the Foundation information page at www.gutenberg.org.

Section 3. Information about the Project Gutenberg Literary Archive Foundation

The Project Gutenberg Literary Archive Foundation is a non-profit

501(c)(3) educational corporation organized under the laws of the

state of Mississippi and granted tax exempt status by the Internal

Revenue Service. The Foundation’s EIN or federal tax identification

number is 64-6221541. Contributions to the Project Gutenberg Literary

Archive Foundation are tax deductible to the full extent permitted by

U.S. federal laws and your state’s laws.

The Foundation’s business office is located at 809 North 1500 West,

Salt Lake City, UT 84116, (801) 596-1887. Email contact links and up

to date contact information can be found at the Foundation’s website

and official page at www.gutenberg.org/contact

Section 4. Information about Donations to the Project Gutenberg

Literary Archive Foundation

Project Gutenberg™ depends upon and cannot survive without widespread

public support and donations to carry out its mission of

increasing the number of public domain and licensed works that can be

freely distributed in machine-readable form accessible by the widest

array of equipment including outdated equipment. Many small donations

($1 to $5,000) are particularly important to maintaining tax exempt

status with the IRS.

The Foundation is committed to complying with the laws regulating

charities and charitable donations in all 50 states of the United

States. Compliance requirements are not uniform and it takes a

considerable effort, much paperwork and many fees to meet and keep up

with these requirements. We do not solicit donations in locations

where we have not received written confirmation of compliance. To SEND

DONATIONS or determine the status of compliance for any particular state

visit www.gutenberg.org/donate.

While we cannot and do not solicit contributions from states where we

have not met the solicitation requirements, we know of no prohibition

against accepting unsolicited donations from donors in such states who

approach us with offers to donate.

International donations are gratefully accepted, but we cannot make

any statements concerning tax treatment of donations received from

outside the United States. U.S. laws alone swamp our small staff.

Please check the Project Gutenberg web pages for current donation

methods and addresses. Donations are accepted in a number of other

ways including checks, online payments and credit card donations. To

donate, please visit: www.gutenberg.org/donate.

Section 5. General Information About Project Gutenberg™ electronic works

Professor Michael S. Hart was the originator of the Project

Gutenberg™ concept of a library of electronic works that could be

freely shared with anyone. For forty years, he produced and

distributed Project Gutenberg™ eBooks with only a loose network of

volunteer support.

Project Gutenberg™ eBooks are often created from several printed

editions, all of which are confirmed as not protected by copyright in

the U.S. unless a copyright notice is included. Thus, we do not

necessarily keep eBooks in compliance with any particular paper

edition.

Most people start at our website which has the main PG search

facility: www.gutenberg.org.

This website includes information about Project Gutenberg™,

including how to make donations to the Project Gutenberg Literary

Archive Foundation, how to help produce our new eBooks, and how to

subscribe to our email newsletter to hear about new eBooks.