# British Airships, Past, Present, and Future

George Whale

﻿The Project Gutenberg eBook of British Airships, Past, Present, and Future

This ebook is for the use of anyone anywhere in the United States and

most other parts of the world at no cost and with almost no restrictions

whatsoever. You may copy it, give it away or re-use it under the terms

of the Project Gutenberg License included with this ebook or online

at www.gutenberg.org. If you are not located in the United States,

you will have to check the laws of the country where you are located

before using this eBook.

Title: British Airships, Past, Present, and Future

Author: George Whale

Release date: December 1, 1996 [eBook #762]

Most recently updated: January 1, 2021

Language: English

Credits: Produced by Dianne Bean.  HTML version by Al Haines.

*** START OF THE PROJECT GUTENBERG EBOOK BRITISH AIRSHIPS, PAST, PRESENT, AND FUTURE ***

Produced by Dianne Bean.  HTML version by Al Haines.

British Airships: Past, Present and Future

by

George Whale

(Late Major, R.A.F.)

# CHAPTER I

INTRODUCTION

# CHAPTER II

EARLY AIRSHIPS AND THEIR DEVELOPMENT TO THE PRESENT DAY

# CHAPTER III

BRITISH AIRSHIPS BUILT BY PRIVATE FIRMS

# CHAPTER IV

BRITISH ARMY AIRSHIPS

# CHAPTER V

EARLY DAYS OF THE NAVAL AIRSHIP SECTION--PARSEVAL AIRSHIPS,

ASTRA-TORRES TYPE, ETC.

# CHAPTER VI

NAVAL AIRSHIPS: THE NON-RIGIDS--

S.S. TYPE

COASTAL AND C STAR AIRSHIPS

THE NORTH SEA AIRSHIP

# CHAPTER VII

NAVAL AIRSHIPS: THE RIGIDS

RIGID AIRSHIP NO. 1

RIGID AIRSHIP NO. 9

RIGID AIRSHIP NO. 23 CLASS

RIGID AIRSHIP NO. 23 X CLASS

RIGID AIRSHIP NO. 31 CLASS

RIGID AIRSHIP NO. 33 CLASS

# CHAPTER VIII

THE WORK OF THE AIRSHIP IN THE WORLD WAR

# CHAPTER IX

THE FUTURE OF AIRSHIPS

# CHAPTER I

INTRODUCTION

Lighter-than-air craft consist of three distinct types: Airships, which

are by far the most important, Free Balloons, and Kite Balloons, which

are attached to the ground or to a ship by a cable.  They derive their

appellation from the fact that when charged with hydrogen, or some

other form of gas, they are lighter than the air which they displace.

Of these three types the free balloon is by far the oldest and the

simplest, but it is entirely at the mercy of the wind and other

elements, and cannot be controlled for direction, but must drift

whithersoever the wind or air currents take it.  On the other hand, the

airship, being provided with engines to propel it through the air, and

with rudders and elevators to control it for direction and height, can

be steered in whatever direction is desired, and voyages can be made

from one place to another--always provided that the force of the wind

is not sufficiently strong to overcome the power of the engines.  The

airship is, therefore, nothing else than a dirigible balloon, for the

engines and other weights connected with the structure are supported in

the air by an envelope or balloon, or a series of such chambers,

according to design, filled with hydrogen or gas of some other nature.

It is not proposed, in this book, to embark upon a lengthy and highly

technical dissertation on aerostatics, although it is an intricate

science which must be thoroughly grasped by anyone who wishes to

possess a full knowledge of airships and the various problems which

occur in their design.  Certain technical expressions and terms are,

however, bound to occur, even in the most rudimentary work on airships,

and the main principles underlying airship construction will be

described as briefly and as simply as is possible.

The term "lift" will appear many times in the following pages, and it

is necessary to understand what it really means.  The difference

between the weight of air displaced and the weight of gas in a balloon

or airship is called the "gross lift."  The term "disposable," or

"nett" lift, is obtained by deducting the weight of the structure,

cars, machinery and other fixed weights from the gross lift.  The

resultant weight obtained by this calculation determines the crew,

ballast, fuel and other necessities which can be carried by the balloon

or airship.

The amount of air displaced by an airship can be accurately weighed,

and varies according to barometric pressure and the temperature; but

for the purposes of this example we may take it that under normal

conditions air weighs 75 lb. per 1,000 cubic feet.  Therefore, if a

balloon of 1,000 cubic feet volume is charged with air, this air

contained will weigh 75 lb.  It is then manifest that a balloon filled

with air would not lift, because the air is not displaced with a

lighter gas.

Hydrogen is the lightest gas known to science, and is used in airships

to displace the air and raise them from the ground. Hydrogen weighs

about one-fifteenth as much as air, and under normal conditions 1,000

cubic feet weighs 5 lb.  Pursuing our analogy, if we fill our balloon

of 1,000 cubic feet with hydrogen we find the gross lift is as follows:

1,000 cubic feet of air weighs                75 lb.

1,000 cubic feet of hydrogen weighs            5 lb.

------

The balance is the gross lift of the balloon  70 lb.

It follows, then, that apart from the weight of the structure itself

the balloon is 70 lb. lighter than the air it displaces, and provided

that it weighs less than 70 lb. it will ascend into the air.

As the balloon or airship ascends the density of the air decreases as

the height is increased.  As an illustration of this the barometer

falls, as everyone knows, the higher it is taken, and it is accurate to

say that up to an elevation of 10,000 feet it falls one inch for every

1,000 feet rise.  It follows that as the pressure of the air decreases,

the volume of the gas contained expands at a corresponding rate.  It

has been shown that a balloon filled with 1,000 feet of hydrogen has a

lift of 70 lb. under normal conditions, that is to say, at a barometric

pressure of 80 inches.  Taking the barometric pressure at 2 inches

lower, namely 28, we get the following figures:

1,000 cubic feet of air weighs       70    lb.

1,000 cubic feet of hydrogen weighs   4.67 "

---------

65.33 lb.

It is therefore seen that the very considerable loss of lift, 4.67 lb.

per 1,000 cubic feet, takes place with the barometric pressure 2 inches

lower, from which it may be taken approximately that 1/30 of the volume

gross lift and weight is lost for every 1,000 feet rise.  From this

example it is obvious that the greater the pressure of the atmosphere,

as indicated by the barometer, the greater will be the lift of the

airship or balloon.

Temperature is another factor which must be considered while discussing

lift.  The volume of gas is affected by temperature, as gases expand or

contract about 1/500 part for every degree Fahrenheit rise or fall in

temperature.

In the case of the 1,000 cubic feet balloon, the air at 30 inches

barometric pressure and 60 degrees Fahrenheit weighs 75 lb., and the

hydrogen weighs 5 lb.

At the same pressure, but with the temperature increased to 90 degrees

Fahrenheit, the air will be expanded and 1,000 cubic feet of air will

weigh only 70.9 lb., while 1,000 cubic feet of hydrogen will weigh 4.7

lb.

The lift being the difference between the weight of the volume of air

and the weight of the hydrogen contained in the balloon, it will be

seen that with the temperature at 60 degrees Fahrenheit the lift is 75

lb. - 5 lb. = 70 lb., while the temperature, having risen to 90

degrees, the lift now becomes 70.9 lb. - 4.7 lb. = 66.2 lb.

Conversely, with a fall in the temperature the lift is increased.

We accordingly find from the foregoing observations that at the start

of a voyage the lift of an airship may be expected to be greater when

the temperature is colder, and the greater the barometric pressure so

will also the lift be greater.  To put this into other words, the most

favourable conditions for the lift of an airship are when the weather

is cold and the barometer is high.

It must be mentioned that the air and hydrogen are not subject in the

same way to changes of temperature.  Important variations in lift may

occur when the temperature of the gas inside the envelope becomes

higher, owing to the action of the sun, than the air which surrounds

it.  A difference of some 20 degrees Fahrenheit may result between the

gas and the air temperatures; this renders it highly necessary that the

pilot should by able to tell at any moment the relative temperatures of

gas and air, as otherwise a false impression will be gained of the

lifting capacity of the airship.

The lift of an airship is also affected by flying through snow and

rain.  A considerable amount of moisture can be taken up by the fabric

and suspensions of a large airship which, however, may be largely

neutralized by the waterproofing of the envelope. Snow, as a rule, is

brushed off the surface by the passage of the ship through the air,

though in the event of its freezing suddenly, while in a melting state,

a very considerable addition of weight might be caused.  There have

been many instances of airships flying through snow, and as far as is

known no serious difficulty has been encountered through the adhesion

of this substance.  The humidity of the air may also cause slight

variations in lift, but for rough calculations it may be ignored, as

the difference in lift is not likely to amount to more than 0.3 lb. per

1,000 cubic feet of gas.

The purity of hydrogen has an important effect upon the lift of an

airship.  One of the greatest difficulties to be contended with is

maintaining the hydrogen pure in the envelope or gasbags for any length

of time.  Owing to diffusion gas escapes with extraordinary rapidity,

and if the fabric used is not absolutely gastight the air finds its way

in where the gas has escaped.  The maximum purity of gas in an airship

never exceeds 98 per cent by volume, and the following example shows

how greatly lift can be reduced:

Under mean atmospheric conditions, which are taken at a temperature of

55 degrees Fahrenheit, and the barometer at 29.5 inches, the lift of

1,000 cubic feet of hydrogen at 98 per cent purity is 69.6 lb.  Under

same conditions at 80 per cent purity the lift of 1,000 cubic feet of

hydrogen is 56.9 lb., a resultant loss of 12.9 lb. per 1,000 cubic feet.

The whole of this statement on "lift" can now be condensed into three

absolute laws:

1. Lift is directly proportional to barometric pressure.

2. Lift is inversely proportional to absolute temperature.

3. Lift is directly proportional to purity.

AIRSHIP DESIGN

The design of airships has been developed under three distinct types,

the Rigid, the Semi-Rigid, and the Non-Rigid.

The rigid, of which the German Zeppelin is the leading example,

consists of a framework, or hull composed of aluminium, wood, or other

materials from which are suspended the cars, machinery and other

weights, and which of itself is sufficiently strong to support its own

weight.  Enclosed within this structure are a number of gas chambers or

bags filled with hydrogen, which provide the necessary buoyancy.  The

hull is completely encased within a fabric outer cover to protect the

hull framework and bags from the effects of weather, and also to temper

the rays of the sun.

The semi-rigid, which has been exploited principally by the Italians

with their Forlanini airships, and in France by Lebaudy, has an

envelope, in some cases divided into separate compartments, to which is

attached close underneath a long girder or keel.  This supports the car

and other weights and prevents the whole ship from buckling in the

event of losing gas.  The semi-rigid type has been practically

undeveloped in this country.

The non-rigid, of which we may now claim to be the leading builders, is

of many varieties, and has been developed in several countries.  In

Germany the chief production has been that of Major von Parseval, and

of which one ship was purchased by the Navy shortly before the outbreak

of war.  In the earliest examples of this type the car was slung a long

way from the envelope and was supported by wires from all parts.  This

necessitated a lofty shed for its accommodation as the ship was of

great overall height; but this difficulty was overcome by the

employment of the elliptical and trajectory bands, and is described in

the chapter dealing with No. 4.

A second system is that of the Astra-Torres.  This envelope is trilobe

in section, with internal rigging, which enables the car to be slung

very close up to the envelope.  The inventor of these envelopes was a

Spaniard, Senor Torres Quevedo, who manufactured them in conjunction

with the Astra Company in Paris.  This type of envelope has been

employed in this country in the Coastal, C Star, and North Sea

airships, and has been found on the whole to give good results.  It is

questionable if an envelope of streamline shape would not be easier to

handle, both in the air and on the landing ground, and at present there

are partisans of both types.

Thirdly, there is the streamline envelope with tangential suspensions,

which has been adopted for all classes of the S.S. airship, and which

has proved for its purpose in every way highly satisfactory.

Of these three types the rigid has the inherent disadvantage of not

being able to be  dismantled, if it should become compelled to make a

forced landing away from its base.  Even if it were so fortunate as to

escape damage in the actual landing, there is the practical certainty

that it would be completely wrecked immediately any increase occurred

in the force of the wind.  On the other hand, for military purposes, it

possesses the advantage of having several gas compartments, and is in

consequence less susceptible to damage from shell fire and other causes.

Both the semi-rigid and the non-rigid have the very great advantage of

being easily deflated and packed up.  In addition to the valves, these

ships have a ripping panel incorporated in the envelope which can

easily be torn away and allows the gas to escape with considerable

rapidity.  Innumerable instances have occurred of ships being compelled

to land in out-of-the-way places owing to engine failure or other

reasons; they have been ripped and deflated and brought back to the

station without incurring any but the most trifling damage.

Experience in the war has proved that for military purposes the large

rigid, capable of long hours of endurances and the small non-rigid made

thoroughly reliable, are the most valuable types for future

development.  The larger non-rigids, with the possible exception of the

North Sea, do not appear to be likely to fulfil any very useful

function.

Airship design introduces so many problems which are not met with in

the ordinary theory of structures, that a whole volume could easily be

devoted to the subject, and even then much valuable information would

have to be omitted from lack of space.  It is, therefore, impossible,

in only a section of a chapter, to do more than indicate in the

briefest manner a few salient features concerning these problems.  The

suspension of weights from the lightest possible gas compartment must

be based on the ordinary principles of calculating the distribution

loads as in ships and other structures.  In the non-rigid, the envelope

being made of flexible fabric has, in itself, no rigidity whatsoever,

and its shape must be maintained by the internal pressure kept slightly

in excess of the pressure outside.  Fabric is capable of resisting

tension, but is naturally not able to resist compression.  If the car

was rigged beneath the centre of the envelope with vertical suspensions

it would tend to produce compression in the underside of the envelope,

owing to the load not being fully distributed.  This would cause, in

practice, the centre portion of the envelope to sag downwards, while

the ends would have a tendency to rise. The principle which has been

found to be most satisfactory is to fix the points of suspension

distributed over the greatest length of envelope possible proportional

to the lift of gas at each section thus formed. From these points the

wires are led to the car.  If the car is placed close to the envelope

it will be seen that the suspensions of necessity lie at a very flat

angle and exert a serious longitudinal compression.  This must be

resisted by a high internal pressure, which demands a stouter fabric

for the envelope and, therefore, increased weight.  It follows that the

tendency of the envelope to deform is decreased as the distance of the

car from the gas compartment is increased.

One method of overcoming this difficulty is found by using the

Astra-Torres design.  As will be seen from the diagram of the North Sea

airship, the loads are excellently distributed by the several fans of

internal rigging, while external head resistance is reduced to a

minimum, as the car can be slung close underneath the envelope.

Moreover, the direct longitudinal compression due to the rigging is

applied to a point considerably above the axis of the ship.  In a large

non-rigid many of these difficulties can be overcome by distributing

the weight into separate cars along the envelope itself.

We have seen that as an airship rises the gas contained in the envelope

expands.  If the envelope were hermetically sealed, the higher the ship

rose the greater would become the internal pressure, until the envelope

finally burst.  To avoid this difficulty in a balloon, a valve is

provided through which the gas can escape.  In a balloon, therefore,

which ascends from the ground full, gas is lost throughout its upward

journey, and when it comes down again it is partially empty or flabby.

This would be an impossible situation in the case of the airship, for

she would become unmanageable, owing to the buckling of the envelope

and the sagging of the planes.  Ballonets are therefore fitted to

prevent this happening.

Ballonets are internal balloons or air compartments fitted inside the

main envelope, and were originally filled with air by a blower driven

either by the main engines or an auxiliary motor. These blowers were a

continual source of trouble, and at the present day it has been

arranged to collect air from the slip-stream of the propeller through a

metal air scoop or blower-pipe and discharge it into an air duct which

distributes it to the ballonets.

The following example will explain their functions:

An airship ascends from the ground full to 1,000 feet.  The ballonets

are empty, and remain so throughout the ascent.  By the time the

airship reaches 1,000 feet it will have lost 1/30th of its volume of

gas which will have escaped through the valves.  If the ship has a

capacity of 300,000 cubic feet it will have lost 10,000 cubic feet of

gas.  The airship now commences to descend; as it descends the gas

within contracts and air is blown into the ballonets.  By the time the

ground is reached 10,000 cubic feet of air will have been blown into

the ballonets and the airship will have retained its shape and not be

flabby.

On making a second ascent, as the airship rises the air must be let out

of the ballonet instead of gas from the envelope, and by the time 1,000

feet is reached the ballonets will be empty.  To ensure that this is

always done the ballonet valves are set to open at less pressure than

the gas valves.

It therefore follows in the example under consideration that it will

not be necessary to lose gas during flight, provided that an ascent is

not made over 1,000 feet.

Valves are provided to prevent the pressure in the envelope from

exceeding a certain determined maximum and are fitted both to ballonets

and the gaschamber.  They are automatic in action, and, as we have

said, the gas valve is set to blow off at a pressure in excess of that

for the air valve.

In rigid airships ballonets are not provided for the gasbags, and as a

consequence a long flight results in a considerable expenditure of gas.

If great heights are required to be reached, it is obvious that the

wastage of gas would be enormous, and it is understood that the Germans

on starting for a raid on England, where the highest altitudes were

necessary, commenced the flight with the gasbags only about 60 per cent

full.

To stabilize the ship in flight, fins or planes are fitted to the after

end of the envelope or hull.  Without the horizontal planes the ship

will continually pitch up and down, and without the vertical planes it

will be found impossible to keep the ship on a straight course.  The

planes are composed of a framework covered with fabric and are attached

to the envelope by means of stay wires fixed to suitable points, in the

case of non-rigid ships skids being employed to prevent the edge of the

plane forcing its way through the surface of the fabric.  The rudder

and elevator flaps in modern practice are hinged to the after edges of

the planes.

The airship car contains all instruments and controls required for

navigating the ship and also provides a housing for the engines.  In

the early days swivelling propellers were considered a great adjunct,

as with their upward and downward thrust they proved of great value in

landing.  Nowadays, owing to greater experience, landing does not

possess the same difficulty as in the past, and swivelling propellers

have been abandoned except in rigid airships, and even in the later

types of these they have been dispensed with.

Owing to the great range of an airship a thoroughly reliable engine is

a paramount necessity.  The main requirements are--firstly, that it

must be capable of running for long periods without a breakdown;

secondly, that it must be so arranged that minor repairs can be

effected in the air; and thirdly, that economy of oil and fuel is of

far greater importance to an airship than the initial weight of the

engine itself.

HANDLING AND FLYING OF AIRSHIPS

The arrangements made for handling airships on the ground and while

landing, and also for moving them in the open, provide scope for great

ingenuity.  An airship when about to land is brought over the aerodrome

and is "ballasted up" so that she becomes considerably lighter than the

air which she displaces. The handling party needs considerable

training, as in gusty weather the safety of the ship depends to a great

extent upon its skill in handling her.  The ship approaches the

handling party head to wind and the trail rope is dropped; it is taken

by the handling party and led through a block secured to the ground and

the ship is slowly hauled down.  When near the ground the handling

party seize the guys which are attached to the ship at suitable points,

other detachments also support the car or cars, as the case may be, and

the ship can then be taken into the shed.

In the case of large airships the size of the handling party has to be

increased and mechanical traction is also at times employed.

As long as the airship is kept head to wind, handling on the ground

presents little difficulty; on many occasions, however, unless the shed

is revolving, as is the case on certain stations in Germany, the wind

will be found to be blowing across the entrance to the shed.  The ship

will then have to be turned, and during this operation, unless great

discretion is used, serious trouble may be experienced.

Many experiments have been and are still being conducted to determine

the best method of mooring airships in the open.  These will be

described and discussed at some length in the chapter devoted to the

airship of the future.

During flight certain details require attention, and carelessness on

the pilot's part, even on the calmest of days, may lead to disaster.

The valves and especially the gas valves should be continually tested,

as on occasions they have been known to jam, and the loss of gas has

not been discovered until the ship had become unduly heavy.

Pressure should be kept as constant as possible.  Most airships work up

to 30 millimetres as a maximum and 15 millimetres as a minimum flying

pressure.  During a descent the pressure should be watched

continuously, as it may fall so low as to cause the nose to blow in.

This will right itself when the speed is reduced or the pressure is

raised, but there is always the danger of the envelope becoming

punctured by the bow stiffeners when this occurs.

HOUSING ACCOMMODATION FOR AIRSHIPS, ETC.

During the early days of the war, when stations were being equipped,

the small type of airship was the only one we possessed.  The sheds to

accommodate them were constructed of wood both for cheapness and speed

of construction and erection. These early sheds were all of very

similar design, and were composed of trestles with some ordinary form

of roof-truss.  They were covered externally with corrugated sheeting.

The doors have always been a source of difficulty, as they are

compelled to open for the full width of the shed and have to stand

alone without support.  They are fitted with wheels which run on guide

rails, and are opened by means of winches and winding gear.

The later sheds built to accommodate the rigid airship are of much

greater dimensions, and are constructed of steel, but otherwise are of

much the same design.

The sheds are always constructed with sliding doors at either end, to

enable the ship to be taken out of the lee end according to the

direction of the wind.

It has been the practice in this country to erect windscreens in order

to break the force of the wind at the mouth of the shed. These screens

are covered with corrugated sheeting, but it is a debatable point as to

whether the comparative shelter found at the actual opening of the shed

is compensated for by the eddies and air currents which are found

between the screens themselves. Experiments have been carried out to

reduce these disturbances, in some cases by removing alternate bays of

the sheeting and in other cases by substituting expanded metal for the

original corrugated sheets.

It must be acknowledged that where this has been done, the airships

have been found easier to handle.

At the outbreak of war, with the exception of a silicol plant at

Kingsnorth, now of obsolete type, and a small electrolytic plant at

Farnborough, there was no facility for the production of hydrogen in

this country for the airship service.

When the new stations were being equipped, small portable silicol

plants were supplied capable of a small output of hydrogen. These were

replaced at a later date by larger plants of a fixed type, and a

permanent gas plant, complete with gasholders and high pressure storage

tanks was erected at each station, the capacity being 5,000 or 10,000

cubic feet per hour according to the needs of the station.

With the development of the rigid building programme, and the

consequent large requirements of gas, it was necessary to reconsider

the whole hydrogen situation, and after preliminary experimental work

it was decided to adopt the water gas contact process, and plants of

this kind with a large capacity of production were erected at most of

the larger stations.  At others electrolytic plants were put down.

Hydrogen was also found to be the bye-product of certain industries,

and considerable supplies were obtained from commercial firms, the

hydrogen being compressed into steel cylinders and dispatched to the

various stations.

Before concluding this chapter, certain words must be written on

parachutes.  A considerable controversy raged in the press and

elsewhere a few months before the cessation of hostilities on the

subject of equipping the aeroplane with parachutes as a life-saving

device.  In the airship service this had been done for two years.  The

best type of parachute available was selected, and these were fitted

according to circumstances in each type of ship.  The usual method is

to insert the parachute, properly folded for use, in a containing case

which is fastened either in the car or on the side of the envelope as

is most convenient. In a small ship the crew are all the time attached

to their parachutes and in the event of the ship catching fire have

only to jump overboard and possess an excellent chance of being saved.

In rigid airships where members of the crew have to move from one end

of the ship to the other, the harness is worn and parachutes are

disposed in the keel and cars as are lifebuoys in seagoing vessels.

Should an emergency arise, the nearest parachute can be attached to the

harness by means of a spring hook, which is the work of a second, and a

descent can be made.

It is worthy of note that there has never been a fatal accident or any

case of a parachute failing to open properly with a man attached.

The material embodied in this chapter, brief and inadequate as it is,

should enable the process of the development of the airship to be

easily followed.  Much has been omitted that ought by right to have

been included, but, on the other hand, intricate calculations are apt

to be tedious except to mathematicians, and these have been avoided as

far as possible in the following pages.

# CHAPTER II

EARLY AIRSHIPS AND THEIR DEVELOPMENT TO THE PRESENT DAY

The science of ballooning had reached quite an advanced stage by the

middle of the eighteenth century, but the construction of an airship

was at that time beyond the range of possibility. Discussions had taken

place at various times as to the practicability of rendering a balloon

navigable, but no attempts had been made to put these points of

argument to a practical test.

Airship history may be said to date from January 24th, 1784.  On that

day Brisson, a member of the Academy in Paris, read before that Society

a paper on airships and the methods to be utilized in propelling them.

He stated that the balloon, or envelope as it is now called, must be

cylindrical in shape with conical ends, the ratio of diameter to length

should be one to five or one to six and that the smallest

cross-sectional area should face the wind.  He proposed that the method

of propulsion should be by oars, although he appeared to be by no means

sanguine if human strength would be sufficient to move them.  Finally,

he referred to the use of different currents of the atmosphere lying

one above the other.

This paper caused a great amount of interest to be taken in

aeronautics, with the result that various Frenchmen turned their

attention to airship design and production.  To France must be due the

acknowledgment that she was the pioneer in airship construction and to

her belongs the chief credit for early experiments.

At a later date Germany entered the lists and tackled the problems

presented with that thoroughness so characteristic of the nation.  It

is just twenty-one years ago since Count Zeppelin, regardless of public

ridicule, commenced building his rigid airships, and in that time such

enormous strides were made that Germany, at the outbreak of the war,

was ahead of any other country in building the large airship.

In 1908 Italy joined the pioneers, and as regards the semi-rigid is in

that type still  pre-eminent.  Great Britain, it is rather sad to say,

adopted the policy of "wait and see," and, with the exception of a few

small ships described in the two succeeding chapters, had produced

nothing worthy of mention before the outbreak of the great European

war.  She then bestirred herself, and we shall see later that she has

produced the largest fleet of airships built by any country and, while

pre-eminent with the non-rigid, is seriously challenging Germany for

the right to say that she has now built the finest rigid airship.

FRANCE

To revert to early history, in the same year in which Brisson read his

paper before the Academy, the Duke of Chartres gave the order for an

airship to the brothers Robert, who were mechanics in Paris.  This ship

was shaped like a fish, on the supposition that an airship would swim

through the air like a fish through water.  The gas-chamber was

provided with a double envelope, in order that it might travel for a

long distance without loss of gas.

The airship was built in St. Cloud Park; in length it was 52 feet with

a diameter of 82 feet, and was ellipsoidal in shape with a capacity of

30,000 cubic feet.  Oars were provided to propel it through the air,

experiments having proved that with two oars of six feet diameter a

back pressure of 90 lb. was obtained and with four oars 140 lb.

On July 6th in the same year the first ascent was made from St. Cloud.

The passengers were the Duke of Chartres, the two brothers Robert and

Colin-Hulin.  No valves having been fitted, there was no outlet for the

expansion of gas and the envelope was on the point of bursting, when

the Duke of Chartres, with great presence of mind, seized a pole and

forced an opening through both the envelopes.  The ship descended in

the Park of Meudon.

On September 19th the airship made a second ascent with the same

passengers as before, with the exception of the Duke.  According to the

report of the brothers Robert, they succeeded in completing an ellipse

and then travelled further in the direction of the wind without using

the oars or steering arrangements. They then deviated their course

somewhat by the use of these implements and landed at Bethune, about

180 miles distant from Paris.

In those days it was considered possible that a balloon could be

rendered navigable by oars, wings, millwheels, etc., and it was not

until the last decades of the nineteenth century, when light and

powerful motors had been constructed, that the problem became really

practical of solution.

During the nineteenth century several airships were built in France and

innumerable experiments were carried out, but the vessels produced were

of little real value except in so far as they stimulated their

designers to make further efforts.  Two of these only will be

mentioned, and that because the illustrations show how totally

different they were from the airship of to-day.

In 1834 the Compte de Lennox built an airship of 98,700 cubic feet

capacity.  It was cylindrical in form with conical ends, and is of

interest because a small balloon or ballonet, 7,050 cubic feet

contents, was placed inside the larger one for an air filling.  A car

66 feet in length was rigged beneath the envelope by means of ropes

eighteen inches long.  Above the car the envelope was provided with a

long air cushion in connection with a valve.  The intention was by

compression of the air in the cushion and the inner balloon, to alter

the height of the airship, in order to travel with the most favourable

air currents.  The motive power was 20 oar propellers worked by men.

This airship proved to be too heavy on completion to lift its own

weight, and was destroyed by the onlookers.

The next airship, the Dupuy de Lome, is of interest because the

experiments were carried out at the cost of the State by the French

Government.  This ship consisted of a spindle-shaped balloon with a

length of 112 feet, diameter of 48 1/2 feet and a volume of 121,800

cubic feet.  An inner air balloon of 6,000 cubic feet volume was

contained in the envelope.  The method of suspension was by means of

diagonal ropes with a net covering.  A rudder in the form of a

triangular sail was fitted beneath the envelope and at the after part

of the ship.  The motive power was double-winged screws 29 feet 6

inches diameter, to be worked by four to eight men.

On her trials the ship became practically a free balloon, an

independent velocity of about six miles per hour being achieved and

deviation from the direction of the wind of ten degrees.

At the close of the nineteenth century Santos-Dumont turned his

attention to airships.  The experiments which he carried out marked a

new epoch and there arose the nucleus of the airship as we know it

to-day.  Between the years 1898 and 1905 he had in all built fourteen

airships, and they were continually improved as each succeeding one

made its appearance.  In the last one he made a circular flight;

starting from the aerodrome of the aero club, he flew round the Eiffel

Tower and back to the starting point in thirty-one minutes on October

19th, 1902.  For this feat the Deutsch prize was awarded to him.

The envelopes he used were in design much nearer approach to a

streamline form than those previously adopted, but tapered to an

extremely fine point both at the both and stem.  For rigging he

employed a long nacelle, in the centre of which was supported the car,

and unusually long suspensions distributed the weight throughout

practically the entire length of the envelope.  To the name of

Santos-Dumont much credit is due.  He may be regarded as the originator

of the airship for pleasure purposes, and by his success did much to

popularize them.  He also was responsible to a large extent for the

development and expansion of the airship industry in Paris.

At a little later date, in 1902 to be precise, the Lebaudy brothers, in

conjunction with Julliot, an engineer, and Surcoup, an aeronaut,

commenced building an airship of a new type.  This ship was a semirigid

and was of a new shape, the envelope resembling in external appearance

a cigar.  In length it was 178 feet with a diameter of 30 feet and the

total capacity was 64,800 cubic feet.  This envelope was attached to a

rigid elliptical keel-shaped girder made of steel tubes, which was

about a third of the length of the ship.  The girder was covered with a

shirting and intended to prevent the ship pitching and rolling while in

flight.  A horizontal rudder was attached to the under side of this

girder, while right aft a large vertical rudder was fixed.

A small car was suspended by steel rods at a distance of 17 feet 9

inches from the girder, with a framework built up underneath to absorb

the shock on landing.

A 35 horse-power Daimler-Mercedes motor, weighing some 800 lb. without

cooling water and fuel, drove two twin-bladed propellers on either side

of the car.

In the year 1903 a number of experimental flights were made with this

ship and various details in the construction were continually

introduced.  The longest flight was 2 hours 46 minutes.  Towards the

end of that year, while a voyage was being made from Paris to Chalais

Meudon, the airship came in contact with a tree and the envelope was

badly torn.

In the following year it was rebuilt, and the volume was slightly

increased with fixed and movable planes added to increase the

stability.  After several trips had been made, the airship again on

landing came in contact with a tree and was burst.

The ship was rebuilt and after carrying out trials was purchased by the

French Army. The Lebaudy airship had at that time been a distinct

success, and in 1910 one was purchased for the British Government by

the readers of the Morning Post.

In the ten-ton Lebaudy the length of the keel framework was greatly

extended, and ran for very nearly the full length of the envelope.  The

disadvantage of this ship was its slowness, considering its size and

power, and was due to the enormous resistance offered by the framework

and rigging.

Airships known as the "Clement-Bayard" were also built about this time.

They were manufactured by the Astra Company in conjunction with

Monsieur Clement, a motor engineer.

In later days vessels were built by the Astra Company of the peculiar

design introduced by Senor Torres. These ships, some of which were of

considerable size, were highly successful, and we became purchasers at

a later date of several.

The Zodiac Company also constructed a number of small ships which were

utilized during the war for anti-submarine patrol.  It cannot be said,

however, that the French have fulfilled their early promise as airship

designers, the chief reason for this being that the airship is

peculiarly suitable for work at sea and the French relied on us to

maintain the commerce routes on the high seas and concentrated their

main efforts on defeating the Germans in the field, in which as all the

world acknowledges they were singularly successful and hold us under an

eternal obligation.

GERMANY

The progress and development of the airship in Germany must now be

considered; it will be seen that, although the production of

satisfactory ships was in very few hands, considerable success attended

their efforts in the early days of the twentieth century.

In 1812, Leppig built an airship at the cost of the State at Woronzowo

in Russia.  This was of the shape of a fish with a rigid framework

beginning at the height of the longitudinal axis.

The lower keel-shaped part of the same formed the car.  Two fans were

attached to the sides and a tail piece was provided behind to act as a

rudder.  The ship was inflated, but structural damage occurred during

this operation and rendered it incapable of flight.

In 1836, Georg Rebenstein, of Nurnburg, was considering the use of the

fall of inclined planes to obtain horizontal motion.

Nothing of importance was produced until a much later date, when in

1885 M. Wolf constructed an envelope of 26,500 cubic feet.  An engine

and propeller were fixed in a triangular framework in front of the

airship, supported by the steam pipe of a steam engine fixed under the

body of the envelope.  The framework lacked rigidity, and the envelope

tore during inflation and the airship failed to ascend.

In the following year Dr. Woelfert, of Berlin, produced a cigar-shaped

envelope, to which was attached rigidly a long bamboo framework

containing the car.  An 8 horse-power benzine Daimler motor drove a

twin-bladed aluminium propeller, and another propeller for vertical

movement was provided beneath the car.  Four trial flights were

attempted, but on each occasion the motor gave unsatisfactory results,

and Woelfert sought to improve it with a benzine vaporizer of his own

pattern.  This improvement was not a success, as during the last flight

an explosion took place and both Woelfert and an aeronaut named Knabe,

who was accompanying him, were killed.

In 1906, Major von Parseval experimented, in Berlin, with a non-rigid

type of airship.  His first ship had a volume of 65,200 cubic feet, but

owing to his system of suspensions, the car hung 27 feet 6 inches below

the envelope.  A Daimler engine was used, driving a four-bladed

propeller.  Owing to the great overall height of this ship, experiments

were made to determine a system of rigging, enabling the car to be

slung closer to the envelope, and in later types the elliptical rigging

girdle was adopted. His later ships were of large dimensions and proved

very satisfactory.  About the same time Major Gross also built airships

for the German aeronautical battalion.

It is, however, the rigid airship that has made Germany famous, and we

must now glance at the evolution of these ships with which we became so

familiar during the war.

The first rigid airship bearing any resemblance to those of the present

day was designed by David Schwartz, and was built in St. Petersburg in

1893.  It was composed of aluminium plates riveted to an aluminium

framework.  On inflation, the frame-work collapsed and the ship was

unusable.

In 1895 he designed a second rigid airship, which was built in Berlin

by Messrs. Weisspfennig and Watzesch. The hull framework was composed

of aluminium and was 155 feet long, elliptical in cross section, giving

a volume of 130,500 cubic feet.  It was pointed in front and rounded

off aft.  The car, also constructed of the same material, was rigidly

attached to the hull by a lattice framework, and the whole hull

structure was covered in with aluminium sheeting.  A 12 horse-power

Daimler benzine motor was installed in the car, driving through the

medium of a belt twin aluminium screw propellers; no rudders were

supplied, the steering being arranged by means of a steering screw

placed centrally to the ship above the top of the car.  Inflation took

place at the end of 1897 by a method of pressing out air-filled fabric

cells which were previously introduced into the hull. This operation

took three and a half hours.  On the day of the first flight trials

there was a fresh wind of about 17 miles per hour.  The airship

ascended into the air, but, apparently, could make little headway

against the wind.  During the trip the driving-belt became disengaged

from the propellers and the ship drifted at the mercy of the wind, but

sustained little damage on landing.  After being deflated, the hull

began to break up under the pressure of the wind and was completely

destroyed by the vandalism of the spectators.

In 1898 Graf F. von Zeppelin, inspired by the example of Schwartz, and

assisted by the engineers Kober and Kubler, conceived the idea of

constructing a rigid airship of considerable dimensions.  For this

purpose a floating shed was built on Lake Constance, near to

Friedrichshafen.  The hull was built of aluminium lattice-work girders,

and had the form of a prism of twenty-four surfaces with arch-shaped

ends.  In length it was 420 feet, with a diameter of 38 feet 6 inches,

and its capacity was 400,000 cubic feet.  The longitudinal framework

was divided by a series of rings, called transverse frames, into

seventeen compartments containing fabric gasbags.  The transverse

frames were fitted with steel wire bracings, both radial and chord, and

to strengthen the whole a triangular aluminium keel of lattice work was

used.  A vertical and horizontal rudder were fitted to the forward

portion of the ship, and aft another vertical rudder.  The whole

exterior of the ship was fitted with a fabric outer cover.

Two aluminium cars, each about 20 feet long, were rigidly attached to

the framework of the hull.  Each car was furnished with a 16

horse-power Daimler engine, driving two four-bladed screw propellers of

aluminium sheeting.  These propellers were situated on the side of the

hull at the centre of resistance. The transmission was supplied by

steel tubes with universal cross joints through the medium of bevel

gears.  Reversible driving arrangements were installed in the cars in

order that the ship could be driven backwards and forwards.  Electric

bells, telegraphs, and speaking tubes were also fitted, and it can be

seen that for general arrangements this airship was a long way ahead of

any built at that date.

The first flight was made on July 2nd, 1900.  The ship attained a speed

of 17 per hour, and the numerous technical details stood the tests

well.  The stability was considered sufficient, and the height of

flight could be altered by the horizontal rudder.  The landing on the

water was accomplished without difficulty, and could be regarded as

free from danger.  The faults requiring remedy were, firstly, the upper

cross stays, which buckled in flight owing to insufficient strength for

the length of the hull; secondly, the gasbags were not sufficiently

gastight and, thirdly, the power of the engines were not sufficient for

such a heavy ship.

This airship was broken up in 1902.

In 1905 the second ship of the series was completed.  She was of nearly

the same size as the previous ship, but the workmanship was much

superior.  Increased engine-power was also supplied, as in this

instance two 85 horse-power Mercedes engines were fitted. This ship was

destroyed by a storm while landing during the next year.

The third ship, which was completed in 1906, was the first Zeppelin

airship acquired by the Government, and lasted for a considerable time,

being rebuilt twice, first in 1908 and again in 1911.  She was slightly

larger than the previous two.

The building was continued, and up to the outbreak of war no fewer than

twenty-five had been completed.  It is impossible, in the space at our

disposal, to trace the career of all of them. Several came to an

untimely end, but as the years went by each succeeding ship proved more

efficient, and the first ship which was delivered to the Navy performed

the notable flight of thirty-one hours.

To revert, for a moment, once more to the earlier ships--the fourth was

wrecked and burned at Echterdingen in the same year in which she was

completed.  The fifth, which was the second military airship, was fitted

with two 110 horse-power engines and also came to a tragic end, being

destroyed by wind at Weilberg in 1910, and the following ship was burnt

at Baden in the same year.

The seventh ship was the first passenger airship of the series, and was

known as the Deutschland.  By this time the capacity had increased to

536,000 cubic feet, and she was propelled by three 120 horse-power

engines.  She also fell a victim to the wind, and was wrecked in the

Teutoberg Forest in 1910; and yet another was destroyed in the

following year at Dusseldorf.

The tenth ship to be completed was the passenger ship Schwaben; her

capacity was 636,500 cubic feet, and she had three 150 horse-power

engines.  This ship carried out her first flight in June, 1911, and was

followed four months later by the Victoria Luise.  The fourth passenger

airship was known as the Hansa. These three ships were all in

commission at the outbreak of war.

The first naval airship, L 1, mentioned above, was larger than any of

these.  The total length was 525 feet, diameter 50 feet, and cubic

contents 776,000 cubic feet.  Her hull framework in section formed a

regular polygon of seventeen sides, and was built up of triangular

aluminium girders.  The gasbags were eighteen in number.  This ship was

fitted with three 170 horse-power Maybach engines, which were disposed

as follows--one in the forward car, driving two two-bladed propellers;

two in the after car, each driving a single four-bladed propeller.  For

steering purposes she had six vertical and eight horizontal planes.

The total lift was 27 tons, with a disposable lift of 7 tons.  Her

speed was about 50 miles per hour, and she could carry fuel for about

48 hours.  Her normal crew consisted of fourteen persons, including

officers.

It will probably be remembered that the military Zeppelin Z III was

compelled to make a forced landing in France.  This ship was of similar

construction to L 1, but of smaller volume, her capacity being 620,000

cubic feet.  A trial flight was being carried out, and while above the

clouds the crew lost their bearings.  Descending they saw some French

troops and rose again immediately.  After flying for four hours they

thought they must be safely over the frontier and, running short of

petrol, made a landing--not knowing that they were still in France

until too late.  The airship was taken over by the French authorities.

Until the year 1916 the Zeppelin may be considered to have passed

through three stages of design.  Of the twenty-five ships constructed

before the war, twenty-four were of the first type and one of the

second.  Each type possessed certain salient features, which, for

simplicity, will be set out in the form of a tabulated statement, and

may be useful for comparison when our own rigid airships are reviewed.

Stage 1.

Long parallel portion of hull with bluff nose and tail.

External keel with walking way.

Box rudders and elevators.

Two cars.

Four wing propellers.

Stage 2.

Long parallel portion of hull with bluff nose, tail portion

finer than in Stage I

Internal keel walking way.

Box rudders and elevators.

Three cars, foremost for control only.

Four wing propellers.

Stage 3.

Shorter parallel portion of hull framework, bluff nose and

tapering tail.

Internal keel walking way.

Balanced monoplane rudders and elevators.

Three cars, foremost for control only.

Two foremost cars close together and connected by

a canvas joint to look like one car.

Four engines and four propellers.  One engine in forward

car driving pusher propeller.  Three engines in after

car driving two wing and one pusher propeller.

To the second stage belongs naval airship L 2, which was destroyed by

fire a month after completion in 1913.  In 1916 a fourth stage made its

appearance, of which the first ship was L 30, completed in May, and to

which the ill-fated L 33 belonged. This type is known as the

super-Zeppelin, and has been developed through various stage until L

70, the latest product before the armistice.  In this stage the

following are its main features:

Stage 4.

Short parallel portion of hull, long rounded bow and

long tapering stern. In all respects a good

streamline shape.

Internal keel walking way.

Balanced monoplane rudders and elevators.

Five cars.  Two forward (combined as in Stage 3),

one aft, and two amidships abreast.

Six engines and six propellers.  The after one of the

forecar and the sidecars each contain one engine

driving direct a pusher propeller.  The after car

contains three engines, two of which drive two wing

propellers; the third, placed aft, drives direct a

pusher propeller.

In this stage the type of girders was greatly altered.

A company known as the Schutte-Lanz Company was also responsible for

the production of rigid airships.  They introduced a design, which was

a distinct departure from Zeppelin or anyone else.  The hull framework

was composed of wood, the girders being built up of wooden sections.

The shape of these ships was much more of a true streamline than had

been the Zeppelin practice, and it was on this model that the shape of

the super-Zeppelin was based. These ships proved of use and took part

in raids on this country, but the Company was taken over by the

Government and the personnel was amalgamated with that engaged on

Zeppelin construction during the war.

ITALY

In 1908, Italy, stimulated by the progress made by other continental

nations, commenced experimental work.  Three types were considered for

a commencement, the P type or Piccolo was the first effort, then

followed the M type, which signifies "medium sized," and also the

semirigid Forlanini.

In the Forlanini type the envelope is divided into several compartments

with an internal rigid keel and to-day these ships are of considerable

size, the most modern being over 600,000 cubic feet capacity.  During

the war, Italian airships were developed on entirely dissimilar lines

to those in other countries.  Both we and our Allies, and to a great

extent the Germans, employed airships exclusively for naval operations;

on the other hand, the Italian ships were utilized for bombing raids in

conjunction with military evolutions.

For this reason height was of primary importance and speed was quite a

secondary consideration, owing to the low velocity of prevailing winds

in that country.  Flights were never of long duration compared with

those carried out by our airships.  Height was always of the utmost

importance, as the Italian ships were used for bombing enemy towns and

must evade hostile gunfire.  For this reason weight was saved in every

possible manner, to increase the height of the "ceiling."

In addition to the types already mentioned, three other varieties have

been constructed since the war--the Usuelli D.E. type and G class.  The

G class was a rigid design which has not been proceeded with, and, with

this single exception, all are of a semirigid type in which an

essentially non-rigid envelope is reinforced by a metal keel.  In the

Forlanini and Usuelli types the keel is completely rigid and assists in

maintaining the shape of the envelopes, and in the Forlanini is

enclosed within the envelope.  In the other types the keel is in

reality a chain of rigid links similar to that of a bicycle.  The form

of the envelope is maintained by the internal pressure and not by the

keel, but the resistance of the latter to compression enables a lower

pressure to be maintained than would be possible in a purely non-rigid

ship.

The M type ship is of considerable size, the P smaller, while the D.E.

is a small ship comparable to our own S.S. design.  The review of these

three countries brings the early history of airships to a conclusion.

Little of importance was done elsewhere before the war, though

Baldwin's airship is perhaps worthy of mention.  It was built in

America in 1908 by Charles Baldwin for the American Government.  The

capacity of the envelope was 20,000 cubic feet, she carried a crew of

two, and her speed was 16 miles per hour.  She carried out her trial

flight in August, 1908, and was accepted by the American military

authorities.  During the war both the naval and military authorities

became greatly interested in airships, and purchased several from the

French and English.  In addition to this a ship in design closely

resembling the S.S. was built in America, but suffered from the same

lack of experience which we did in the early days of airship

construction.

We must now see what had been happening in this country in those

fateful years before the bombshell of war exploded in our midst.

# CHAPTER III

BRITISH AIRSHIPS BUILT BY PRIVATE FIRMS

It has been shown in the previous chapter that the development of the

airship had been practically neglected in England prior to the

twentieth century.  Ballooning had been carried out both as a form of

sport and also by the showman as a Saturday afternoon's sensational

entertainment, with a parachute descent as the piece de resistance.

The experiments in adapting the balloon into the dirigible had,

however, been left to the pioneers on the Continent.

PARTRIDGE'S AIRSHIP

It appears that in the nineteenth century only one airship was

constructed in this country, which proved to be capable of ascending

into the air and being propelled by its own machinery. This airship

made its appearance in the year 1848, and was built to the designs of a

man named Partridge.  Very little information is available concerning

this ship.  The envelope was cylindrical in shape, tapering at each

end, and was composed of a light rigid framework covered with fabric.

The envelope itself was covered with a light wire net, from which the

car was suspended.  The envelope contained a single ballonet for

regulating the pressure of the gas.  Planes, which in design more

nearly resembled sails, were used for steering purposes.  In the car,

at the after end, were fitted three propellers which were driven by

compressed air.

Several trips of short duration were carried out in this airship, but

steering was never successfully accomplished owing to difficulties

encountered with the planes, and, except in weather of the calmest

description, she may be said to have been practically uncontrollable.

HUGH BELL'S AIRSHIP

In the same year, 1848, Bell's airship was constructed.  The envelope

of this ship was also cylindrical in shape, tapering at each end to a

point, the length of which was 56 feet and the diameter 21 feet 4

inches.  A  keel composed of metal tubes was attached  to the underside

of the envelope from which the car was suspended.  On either side of

the car screw propellers were fitted to be worked by hand.  A rudder

was attached behind the car.  It was arranged that trials should be

carried out in the Vauxhall Gardens in London, but these proved

fruitless.

BARTON'S AIRSHIP

In the closing years of the nineteenth century appeared the forerunners

of airships as they are to-day, and interest was aroused in this

country by the performances of the ships designed by Santos-Dumont and

Count Zeppelin.  From now onwards we find various British firms turning

their attention to the conquest of the air.

In 1903 Dr. Barton commenced the construction of a large non-rigid

airship.  The envelope was 176 feet long with a height of 43 feet and a

capacity of 235,000 cubic feet; it was cylindrical in shape, tapering

to a point at each end.  Beneath the whole length of the cylindrical

portion was suspended a bamboo framework which served as a car for the

crew, and a housing for the  motors supplying the motive power of the

ship. This framework was suspended from the envelope by means of steel

cables.  Installed in the car were two 50 horse-power Buchet engines

which were mounted at the forward and after ends of the framework.  The

propellers in themselves were of singular design, as they consisted of

three pairs of blades mounted one behind the other.  The were situated

on each side of the car, two forward and two aft.  The drive also

include large friction clutches, and each engine was under separate

control.

To enable the ship to be trimmed horizontally, water tanks were fitted

at either end of the framework, the water being transferred from one to

the other as was found necessary.

A series of planes was mounted at intervals along the framework to

control the elevation of the ship.

This ship was completed in 1905 and was tried at the Alexandra Palace

in the July of that year.  She, unfortunately, did not come up to

expectations, owing to the difficulty in controlling her, and during

the trial flight she drifted away and was destroyed in landing.

WILLOWS No. 1

From the year 1905 until the outbreak of war Messrs. Willows & Co. were

engaged on the construction of airships of a small type, and

considerable success attended their efforts.  Each succeeding ship was

an improvement on its predecessor, and flights were made which, in

their day, created a considerable amount of interest.

In 1905 their first ship was completed.  This was a very small

non-rigid of only 12,500 cubic feet capacity.  The envelope was made of

Japanese silk, cylindrical in shape, with rather blunt conical ends.  A

long nacelle or framework, triangular in section and built up of light

steel tubes, was suspended beneath the envelope by means of diagonally

crossed suspensions.

A 7 horse-power Peugeot engine was fitted at the after end of the

nacelle which drove a 10-feet diameter propeller.  In front were a pair

of swivelling tractor screws for steering the ship in the vertical and

horizontal plane.  No elevators or rudders were fixed to the ship.

WILLOWS No. 2

The second ship was practically a semi-rigid.  The envelope was over

twice the capacity of the earlier ship, being of 29,000 cubic feet

capacity.  This envelope was attached to a keel of bamboo and steel,

from which was suspended by steel cables a small car.  At the after end

of the keel was mounted a small rudder for the horizontal steering.

For steering in the vertical plane two propellers were mounted on each

side of the car, swivelling to give an upward or downward thrust.  A 30

horse-power J.A.P. engine was fitted in this case.  Several successful

flights were carried out by this ship, of which the most noteworthy was

from Cardiff to London.

WILLOWS No. 3

No. 2, having been rebuilt and both enlarged and improved, became known

as No. 3. The capacity of the envelope, which was composed of rubber

and cotton, was increased to 32,000 cubic feet, and contained two

ballonets.  The gross lift amounted to about half a ton.  As before, a

30 horse-power J.A.P. engine was installed, driving the swivelling

propellers.  These propellers were two-bladed with a diameter of 61

feet.  The maximum speed was supposed to be 25 miles per hour, but it

is questionable if this was ever attained.

This ship flew from London to Paris, and was the first British-built

airship to fly across the Channel.

WILLOWS No. 4

The fourth ship constructed by this firm was completed in 1912, and was

slightly smaller than the two preceding ships.  The capacity of the

envelope in this instance was reduced to 24,000 cubic feet, but was a

much better shape, having a diameter of 20 feet, which was gradually

tapered towards the stern.  A different material was also used,

varnished silk being tried as an experiment. The envelope was attached

to a keel on which was mounted the engine, a 35 horse-power Anzani,

driving two swivelling four-bladed propellers.  From the keel was

suspended a torpedo-shaped boat car in which a crew of two was

accommodated. Originally a vertical fin and rudder were mounted at the

stern end of the keel,  but these were later replaced by fins on the

stern of the envelope.

This ship was purchased by the naval authorities, and after purchase

was more or less reconstructed, but carried out little flying. At the

outbreak of war she was lying deflated in the shed at Farnborough.  As

will be seen later, this was the envelope which was rigged to the

original experimental S.S. airship in the early days of 1915, and is

for this reason, if for no other, particularly interesting.

WILLOWS No. 5

This ship was of similar design, but of greater capacity.  The

envelope, which was composed of rubber-proofed fabric, gave a volume of

50,000 cubic feet, and contained two ballonets.  A 60 horsepower engine

drove two swivelling propellers at an estimated speed of 38 miles per

hour.  She was constructed at Hendon, from where she made several short

trips.

MARSHALL FOX'S AIRSHIP

In the early days of the war an airship was constructed by Mr. Marshall

Fox which is worthy of mention, although it never flew. It was claimed

that this ship was a rigid airship, although from its construction it

could only be looked upon as a non-rigid ship, having a wooden net-work

around its envelope.  The hull was composed of wooden transverse frames

forming a polygon of sixteen sides, with radial wiring fitted to each

transverse frame.  The longitudinal members were spiral in form and

were built up of three-ply lathes.  A keel of similar construction ran

along the under side of the hull which carried the control position and

compartments for two Green engines, one of 40 horse-power, the other of

80 horse-power, together with the petrol, bombs, etc.

In the hull were fitted fourteen gasbags giving a total capacity of

100,000 cubic feet.  The propeller drive was obtained by means of a

wire rope.  The gross lift of the ship was 4,276 lb., and the weight of

the structure, complete with engines, exceeded this.

It became apparent that the ship could never fly, and work was

suspended.  She was afterwards used for carrying out certain

experiments and at a later date was broken up.

Apart from the various airships built under contract for the Government

there do not appear to be any other ships built by private firms which

were completed and actually flew.  It is impossible to view this lack

of enterprise with any other feelings than those of regret, and it was

entirely due to this want of foresight that Great Britain entered upon

the World War worse equipped, as regards airships, than the Central

Empires or any of the greater Allied Powers.

# CHAPTER IV

BRITISH ARMY AIRSHIPS

The French and German military authorities began to consider airships

as an arm of the Service in the closing years of the nineteenth

century, and devoted both time and considerable sums of money in the

attempt to bring them to perfection.  Their appearance in the British

Army was delayed for many years on account of the expense that would be

incurred in carrying out experiments.  In 1902, Colonel Templer, at

that time head of the Balloon Section, obtained the necessary sanction

to commence experiments, and two envelopes of gold-beaters skin of

50,000 cubic feet capacity were built.  With their completion the funds

were exhausted, and nothing further done until 1907.

NULLI SECUNDUS I

In 1907 the first complete military airship in England was built, which

bore the grandiloquent title of Nulli Secundus.  One of the envelopes

constructed by Colonel Templer was used: it was cylindrical in shape

with  spherical ends.  Suspended beneath the envelope by means of a net

and four broad silk bands was a triangular steel framework or keel from

which was slung a small car.  A 50 horsepower Antoinette engine was

situated in the forward part of the car which drove two metal-bladed

propellers by belts. At the after part of the keel were fitted a rudder

and small elevators, and two pairs of movable horizontal planes were

also fitted forward.  It is remarkable that no stabilizing surfaces

whatsoever were mounted.  The envelope was so exceedingly strong that a

high pressure of gas could be sustained, and ballonets were considered

unnecessary, but relief valves were employed.  The first flight took

place in September and was fairly successful.  Several were made

afterwards, and in October she was flown over London and landed at the

Crystal Palace.  The flight lasted 3 hours and 25 minutes, which

constituted at the time a world's record.  Three days later, owing to

heavy winds, the ship had to be deflated and was taken back to

Farnborough.

NULLI SECUNDUS II

In 1908 the old ship was rebuilt with several modifications.  The

envelope was increased in length and was united to the keel by means of

a covering of silk fabric in place of the net, four suspension bands

being again used.  A large bow elevator was mounted which made the ship

rather unstable.  A few flights were accomplished, but the ship proved

of little value and was broken up.

BABY

This little airship made its first appearance in the spring of 1909.

The envelope was fish-shaped and composed of gold-beater's skin, with a

volume of 21,000 cubic feet.  One ballonet was contained in the

envelope which, at first, had three inflated fins to act as

stabilizers.  These proved unsatisfactory as they lacked rigidity, and

were replaced after the first inflation by the ordinary type.  Two 8

horse-power 3-cylinder Berliet engines were mounted in a long car

driving a simple propeller, and at a later date were substituted by a

R.E.P. engine which proved most unsatisfactory.  During the autumn

permission was obtained to enlarge the envelope and fit a more powerful

engine.

BETA

Beta was completed in May, 1910.  The envelope was that of the Baby

enlarged, and now had a volume of 35,000 cubic feet.  The car was

composed of a long frame, having a centre compartment for the crew and

engines, which was the standard practice at that time for ships

designed by the Astra Company.  A 35 horse-power Green engine drove two

wooden two-bladed propellers by chains. The ship was fitted with an

unbalanced rudder, while the elevators were in the front of the frame.

This ship was successful, and in June flew to London and back, and in

September took part in the Army manoeuvres, on one occasion being in

the air for 7 3/4 hours without landing, carrying a crew of three.

Trouble was experienced in the steering, the elevators being situated

too near the centre of the ship to be really efficient and were

altogether too small.

In 1912, Beta, having been employed regularly during the previous year,

was provided with a new car having a Clerget engine of 45 horse-power.

In 1913 she was inflated for over three months and made innumerable

flights, on one occasion carrying H.R.H. the Prince of Wales as

passenger.  She had at that time a maximum speed of 35 miles per hour,

and could carry fuel for about eight hours with a crew of three.

GAMMA

In 1910 the Gamma was also completed.  This was a much bigger ship with

an envelope of 75,000 cubic feet capacity, which, though designed in

England, had been built by the Astra Company in Paris.  The car, as in

Beta, was carried in a long framework suspended from the envelope.

This portion of the ship was manufactured in England, together with the

machinery.  This consisted of an 80 horse-power Green engine driving

swivelling propellers, the gears and shafts of which were made by Rolls

Royce.  The engine drove the propeller shafts direct, one  from each

end of the crankshaft.

Originally the envelope was fitted with inflated streamline stabilizers

on either side, but at a later date these were replaced by fixed

stabilizing planes.  At the same time the Green engine was removed and

two Iris engines of 45 horse-power were installed, each driving a

single propeller.  There were two pairs of elevators, each situated in

the framework, one forward, the other aft.  In 1912, having been rigged

to a new envelope of 101,000 cubic feet capacity, the ship took part in

the autumn manoeuvres, and considerable use was made of wireless

telegraphy.

In a height reconnaissance the pilot lost his way, and running out of

petrol drifted all night, but was safely landed.  When returning to

Farnborough the rudder controls were broken and the ship was ripped.

In this operation the framework was considerably damaged.  When repairs

were being carried out the elevators were removed from the car

framework and attached to the stabilizing fins in accordance with the

method in use to-day.

CLEMENT-BAYARD

In 1910 it was arranged by a committee of Members of Parliament that

the Clement-Bayard firm should send over to England a large airship on

approval, with a view to its ultimate purchase by the War Office, and a

shed was erected at Wormwood Scrubs for its accommodation.  This ship

arrived safely in October, but was very slow and difficult to control.

The envelope, moreover, was of exceedingly poor quality and consumed so

much gas that it was decided to deflate it. She was taken to pieces and

never rebuilt.

LEBAUDY

About the same time, interest having been aroused in this country by

the success of airships on the Continent, the readers of the Morning

Post subscribed a large sum to purchase an airship for presentation to

the Government.  This was a large ship of 350,000 cubic feet capacity

and was of semi-rigid design, a long framework being suspended from the

envelope which supported the weight of the car.  It had two engines of

150 horse-power which developed a speed of about 32 miles per hour.

The War Office built a shed at Farnborough to house it, and in

accordance with dimensions given by the  firm a clearance of 10 feet

was allowed between the top of the ship and the roof of the shed.

Inconceivable as it may sound, the overall height of the ship was

increased by practically 10 feet without the War Office being informed.

The ship flew over and was landed safely, but on being taken into the

shed the envelope caught on the roof girders, owing to lack of

headroom, and was ripped from end to end.  The Government agreed to

increase the height of the shed and the firm to rebuild the ship.  This

was completed in March, 1911, and the ship was inflated again.  On

carrying out a trial flight, having made several circuits at 600 feet,

she attempted to land, but collided with a house and was completely

wrecked. This was the end of a most unfortunate ship, and her loss was

not regretted.

DELTA

Towards the end Of 1910 the design was commenced of the ship to be

known as the Delta, and in 1911 the work was put in hand.  The first

envelope was made of waterproofed silk.  This proved a failure, as

whenever the envelope was put up to pressure it invariably burst.

Experiments were continued, but no good resulting, the idea was

abandoned and a rubber-proofed fabric envelope was constructed of

173,000 cubic feet volume.  This ship was inflated in 1912.  The first

idea was to make the ship a semi-rigid by lacing two flat girders to

the sides of the envelope to take the weight of the car.  This idea had

to be abandoned, as in practice, when the weight of the car was

applied, the girders buckled.  The ship was then rigged as a non-rigid.

A novelty was introduced by attaching a rudder flap to the top

stabilizing fin, but as it worked somewhat stiffly it was later on

removed.  This ship took part in the manoeuvres of 1912 and carried out

several flights.  She proved to be exceedingly fast, being capable of a

speed of 44 miles per hour. In 1913 she was completely re-rigged and

exhibited at the Aero Show, but the re-designed rigging revealed

various faults and it was not until late in the year that she carried

out her flight trials.  Two rather interesting experiments were made

during these flights.  In one a parachute descent was successfully

accomplished; and in another the equivalent weight of a man was picked

up from the ground without assistance or landing the ship.

ETA

The Eta was somewhat smaller than the Delta, containing only 118,000

cubic feet of hydrogen, and was first inflated in 1913. The envelope

was composed of rubber-proofed fabric and a long tapering car was

suspended, this being in the nature of a compromise between the short

car of the, Delta and the long framework gear of the Gamma.  Her

engines were two 80 horse-power Canton-Unne, each driving one propeller

by a chain.  This ship proved to be a good design and completed an

eight-hour trial flight in September.  On her fourth trial she

succeeded in towing the disabled naval airship No. 2 a distance of

fifteen miles. Her speed was 42 miles per hour, and she could carry a

crew of five with fuel for ten hours.

On January 1st, 1914, the Army disbanded their Airship Section, and the

airships Beta, Gamma, Delta and Eta were handed over to the Navy

together with a number of officers and men.

# CHAPTER V

EARLY DAYS OF THE NAVAL AIRSHIP SECTION--PARSEVAL AIRSHIPS,

ASTRA-TORRES TYPE, ETC.

The rapid development of the rigid airships in Germany began to create

a considerable amount of interest in official circles.  It was realized

that those large airships in the future would be invaluable to a fleet

for scouting purposes.  It was manifest that our fleet, in the event of

war, would be gravely handicapped by the absence of such aerial scouts,

and that Germany would hold an enormous advantage if her fleet went to

sea preceded by a squadron of Zeppelin airships.

The Imperial Committee, therefore, decided that the development of the

rigid airship should be allotted to the Navy, and a design for Rigid

Airship No. 1 was prepared by Messrs. Vickers in conjunction with

certain naval officers in the early part of 1909.

As will be seen later this ship was completed in 1911, but broke in two

in September of that year and nothing more was done with her.  In

February, 1912, the construction of rigid airships was discontinued,

and in March the Naval Airship section was disbanded.

In September, 1912, the Naval Airship section was once more

reconstituted and was stationed at Farnborough.  The first requirements

were airships, and owing to the fact that airship construction was so

behindhand in this country, in comparison with the Continent, it was

determined that purchases should be made abroad until sufficient

experience had been gained by British firms to enable them to compete

with any chance of success against foreign rivals.

First a small non-rigid, built by Messrs. Willows, was bought by the

Navy to be used for the training of airship pilots.  In addition an

Astra-Torres airship was ordered from France.  This was a ship of

229,450 cubic feet capacity and was driven by twin Chenu engines of 210

horse-power each.  She carried a crew of six, and was equipped with

wireless and machine guns.  The car could be moved fore and aft for

trimming purposes, either by power or by hand.  This was, however, not

satisfactory, and was abandoned.

In April 1918, Messrs. Vickers were asked to forward proposals for a

rigid airship which afterwards became e known as No. 9. Full details of

the vicissitudes connected with this ship will be given in the chapter

devoted to Rigid Airships.

In July, approval was granted for the construction of six non-rigid

ships.  Three of these were to be of the German design of Major von

Parseval and three of the Forlanini type, which was a semi-rigid design

manufactured in Italy.  The order for the Parsevals was placed with

Messrs. Vickers and for the Forlaninis with Messrs. Armstrong.

The Parseval airship was delivered to this country and became known as

No. 4; a second ship of the same type was also building when war broke

out; needless to say this ship was never delivered.  At a later date

Messrs. Vickers, who had obtained the patent rights of the Parseval

envelope, completed the other two ships of the order.

The Forlanini ship was completing in Italy on the declaration of war

and was taken over by the Italians; Messrs. Armstrong had not commenced

work on the other two.  These ships, although allocated numbers, never

actually came into being.

PARSEVAL AIRSHIP No. 4

This airship deserves special consideration for two reasons; firstly,

on account of the active-service flying carried out by it during the

first three years of the war, and, secondly, for its great value in

training of the officers and men who later on became the captains and

crews of rigid airships.

The Parseval envelope is of streamline shape which tapers to a point at

the tail, and in this ship was of 300,000 cubic feet capacity.  The

system of rigging being patented, can only be described in very general

terms.  The suspensions carrying the car are attached to a large

elliptical rigging band which is formed under the central portion of

the envelope.  To this rigging band are attached the trajectory bands

which pass up the sides and over the top of the envelope, sloping away

from the centre at the bottom towards the nose and tail at the top.

The object of this is to distribute the load fore and aft over the

envelope.  These bands, particularly at the after end of the ship,

follow a curved path, so that they become more nearly vertical as they

approach the upper surface of the envelope. This has the effect of

bringing the vertical load on the top of the envelope; but a greater

portion of the compressive force comes on the lower half, where it

helps to resist the bending moment due to the unusually short

suspensions.  A single rudder plane and the ordinary elevator planes

were fitted to the envelope.  A roomy open car was provided for this

ship, composed of a duralumin framework and covered with duralumin

sheeting. Two 170 horse-power Maybach engines were mounted at the after

end of the car, which drove two metal-bladed reversible propellers.

These propellers were later replaced by standard four-bladed wooden

ones and a notable increase of speed was obtained.

Two officers and a crew of seven men were carried, together with a

wireless installation and armament.

This airship, together with No. 3, took part in the great naval review

at Spithead, shortly before the commencement of the war, and in

addition to the duties performed by her in the autumn of 1914, which

are mentioned later, carried out long hours of patrol duty from an east

coast station in the summer of 1917.  In all respects she must be

accounted a most valuable purchase.

PARSEVAL AIRSHIPS 5, 6 and 7

Parseval No. 5 was not delivered by Germany owing to the war, so three

envelopes and two cars were built by Messrs. Vickers on the design of

the original ship.  These were delivered somewhat late in the war, and

on account of the production of the North Sea airship with its greater

speed were not persevered with.  The dimensions of the envelopes were

somewhat increased, giving  a cubic capacity of 325,000 cubic feet.

Twin Maybach engines driving swivelling propellers were installed in

the car, which was completely covered in, but these ships were slow in

comparison with later designs, and were only used for the instruction

of officers and men destined for the crews of rigid airships then

building.

An experimental ship was made in 1917 which was known as Parseval 5; a

car of a modified coastal pattern with two 240 horse-power Renault

engines was rigged to one of envelopes.  During a speed trial, this

ship was calculated to have a ground speed of 50 to 53 miles per hour.

The envelope, however, consumed an enormous amount gas and for this

reason the ship was deflated and struck off the list of active ships.

This digression on Parseval airships has anticipated events somewhat,

and a return must now be made to earlier days.

Two more Astra-Torres were ordered from France, one known as No. 8,

being a large ship of 4,00,000 cubic feet capacity.  She was fitted

with two Chenu engines of 240 horse-power, driving swivelling

propellers.  This ship was delivered towards the end of the year 1914.

The second Astra was of smaller capacity and was delivered, but as will

be seen later, was never rigged, the envelope being used for the

original coastal ship and the car slung to the envelope of the ex-army

airship Eta.

On January 1st, 1914, an important event took place: the Army disbanded

their airship service, and the military ships together with certain

officers and men were transferred to the Naval Air Service.

Before proceeding further, it may be helpful to explain the system by

which the naval airships have been given numbers. These craft are

always known by the numbers which they bear, and the public is

completely mystified as to their significance whenever they fly over

London or any large town.  It must be admitted that the method is

extremely confusing, but the table which follows should help to

elucidate the matter.  The original intention was to designate each

airship owned by the Navy by a successive number.  The original

airship, the rigid Mayfly, was known as No. 1, the Willows airship No.

2, and so on.  These numbers were allocated regardless of type and as

each airship was ordered, consequently some of these ships, for example

the Forlaninis, never existed.  That did not matter, however, and these

numbers were not utilized for ships which actually were commissioned.

On the transfer of the army airships, four of these, the Beta, Gamma,

Delta and Eta, were given their numbers as they were taken over,

together with two ships of the Epsilon class which were ordered from

Messrs. Rolls Royce, but never completed.  In this way it will be seen

that numbers 1 to 22 are accounted for.

In 1915 it was decided to build a large number of small ships for

anti-submarine patrol, which were called S.S.'s or Submarine Scouts.

It was felt that it would only make confusion worse confounded if these

ships bore the original system of successive numbering and were mixed

up with those of later classes which it was known would be produced as

soon as the designs were completed.  Each of these ships was

accordingly numbered in its own class, S.S., S.S.P., S.S. Zero,

Coastal, C Star and North Sea, from 1 onwards as they were completed.

In the case of the rigids, however, for some occult reason the old

system of numbering was persisted in.  The letter R is prefixed before

the number to show that the ship is a rigid. Hence we have No. 1 a

rigid, the second rigid constructed is No. 9, or R 9, and the third

becomes R 23.  From this number onwards all are rigids and are numbered

in sequence as they are ordered, with the exception of the last on the

list, which is a ship in a class of itself.  This ship the authorities,

in their wisdom, have called R 80--why, nobody knows.

With this somewhat lengthy and tedious explanation the following table

may be understood:

No.  Type.                 Remarks.

1. Rigid          Wrecked, Sept. 24, 1911.

2. Willows        Became S.S. 1.

3. Astra-Torres   Deleted, May 1916.

4. Parseval       Deleted, July, 1917.

5. Parseval       Never delivered from Germany.

(Substitute ship built by Messrs. Vickers).

6. Parseval       Built by Messrs. Vickers.

7. Parseval       Built by Messrs. Vickers.

8. Astra-Torres   Deleted, May, 1916.

9. Rigid          Deleted, June, 1918.

10. Astra-Torres  Envelope used for C 1.

11. Forlanini     Never delivered owing to war.

12. Forlanini     Never delivered owing to war.

13. Forlanini     Never delivered owing to war.

14. Rigid         Never built.

15. Rigid         Never built.

16. Astra-Torres  See No. 8.

17. Beta          Transferred from Army.

Deleted, May, 1916.

18. Gamma         Deleted, May, 1916.

19. Delta         Deleted, May, 1916.

20. Eta           Transferred from the Army.

Fitted with car from No. 10.

Deleted May, 1916.

21. Epsilon       Construction cancelled May, 1916.

22. Epsilon       Construction cancelled May, 1916.

23. Rigid         23 Class.

24. Rigid         23 Class.

25. Rigid         23 Class.

26. Rigid         23 Class.

27. Rigid         23x Class.

28. Rigid         23x Class. Never completed.

29. Rigid         23x Class.

30. Rigid         23x Class. Never completed.

31. Rigid         31 Class.

32. Rigid         31 Class, building.

33. Rigid         33 Class.

34. Rigid         33 Class.

35. Rigid         Cancelled.

36. Rigid         Building.

37. Rigid         Building.

38. Rigid         Building.

39. Rigid         Building.

40. Rigid         Building.

80. Rigid         Building.

In August, 1914, Europe, which had been in a state of diplomatic

tension for several years, was plunged into the world war.  The naval

airship service at the time was in possession of two stations,

Farnborough and Kingsnorth, the latter in a half-finished condition.

Seven airships were possessed, Nos. 2, 3 and 4, and the four ex-army

ships--Beta, Gamma, Delta and Eta--and of these only three, Nos. 3, 4

and the Beta, were in any condition for flying.  Notwithstanding this,

the utmost use was made of the ships which were available.

On the very first night of the war, Nos. 3 and 4 carried out a

reconnaissance flight over the southern portion of the North Sea, and

No. 4 came under the fire of territorial detachments at the mouth of

the Thames on her return to her station.  These zealous soldiers

imagined that she was a German ship bent on observation of the dockyard

at Chatham.

No. 3 and No. 4 rendered most noteworthy service in escorting the

original Expeditionary Force across the Channel, and in addition to

this No. 4 carried out long patrols over the channel throughout the

following winter.

No. 17 (Beta) also saw active service, as she was based for a short

period early in 1915 at Dunkirk, and was employed in spotting duties

with the Belgian artillery near Ostend.

The Gamma and the Delta were both lying deflated at Farnborough at the

outbreak of the war, and in the case of the latter the car was found to

be beyond repair, and she was accordingly deleted. The Gamma was

inflated in January, 1915, and was used for mooring experiments.

The Eta, having been inflated and deflated several times owing to the

poor quality of the envelope, attempted to fly to Dunkirk in November,

1914.  She encountered a snowstorm near Redhill and was compelled to

make a forced landing.  In doing this she was so badly damaged as to be

incapable of repair, and at a later date was deleted.

No. 8, which was delivered towards the end of 1914, was also moored out

in the open for a short time near Dunkirk, and carried out patrol in

the war zone of the Belgian coast.

So ends the story of the Naval Airship Service before the war.

With the submarine campaign ruthlessly waged by the Germans from the

spring of 1915 and onwards, came the airship's opportunity, and the

authorities grasped the fact that, with development, here was the

weapon to defeat the most dangerous enemy of the Empire. The method of

development and the success attending it the following chapters will

show.

# CHAPTER VI

NAVAL AIRSHIPS.--THE NON-RIGIDS--S.S. TYPE

The development of the British airships of to-day may be said to date

from February 28th, 1915.  On that day approval was given for the

construction of the original S.S. airship.

At this time the Germans had embarked upon their submarine campaign,

realizing, with the failure of their great assaults on the British

troops in Flanders, that their main hope of victory lay in starving

Great Britain into surrender.  There is no doubt that the wholesale

sinking of our merchant shipping was sufficient to cause grave alarm,

and the authorities were much concerned to devise means of minimizing,

even if they could not completely eliminate the danger.  One proposal

which was adopted, and which chiefly concerns the interests of this

book, was the establishment of airship stations round the coasts of

Great Britain.  These stations were to be equipped with airships

capable of patrolling the main shipping routes, whose functions were to

search for submarines and mines and to escort shipping through the

danger zones in conjunction with surface craft.

Airship construction in this country at the time was, practically

speaking, non-existent.  There was no time to be wasted in carrying out

long and expensive experiments, for the demand for airships which could

fulfil these requirements was terribly urgent, and speed of

construction was of primary importance.  The non-rigid design having

been selected for simplicity in construction, the expedient was tried

of slinging the fuselage of an ordinary B.E. 2C aeroplane, minus the

wings, rudder and elevators and one or two other minor fittings,

beneath an envelope with tangential suspensions, as considerable

experience had been gained already in a design of this type.

For this purpose the envelope of airship No. 2, which was lying

deflated in the shed at Farnborough, was rushed post haste to

Kingsnorth, inflated and rigged to the fuselage prepared for it. The

work was completed with such despatch that the airship carried out her

trial flight in less than a fortnight from approval being granted to

the scheme. The trials were in every way most satisfactory, and a large

number of ships of this design was ordered immediately.  At the same

time two private firms were invited to submit designs of their own to

fulfil the Admiralty requirements.  One firm's design, S.S. 2, did not

fulfil the conditions laid down and was put out of commission; the

other, designed by Messrs. Armstrong, was sufficiently successful for

them to receive further orders.  In addition to these a car was

designed by Messrs. Airships Ltd., which somewhat resembled a Maurice

Farman aeroplane body, and as it appeared to be suitable for the

purpose, a certain number of these was also ordered.

About this period the station at Farnborough was abandoned by the Naval

Airship Service to make room for the expansion of the military

aeroplane squadrons.  The personnel and airships were transferred to

Kingsnorth, which became the airship headquarters.

The greatest energy was displayed in preparing the new stations, which

were selected as bases for the airships building for this

anti-submarine patrol.  Small sheds, composed of wood, were erected

with almost incredible rapidity, additional personnel was recruited,

stores were collected, huts built for their accommodation and that of

the men, and by the end of the summer the organization was so complete

that operations were enabled to commence.

The S.S., or submarine scout, airship proved itself a great success.

Beginning originally with a small programme the type passed through

various developments until, at the conclusion of the war, no fewer than

150 ships of various kinds had been constructed.  The alterations which

took place and the improvements effected thereby will be considered at

some length in the following pages.

S.S.B.E. 2C

The envelope of the experimental ship S.S. 1 was only of 20,500 cubic

feet capacity; for the active-service ships, envelopes of similar shape

of 60,000 cubic feet capacity were built.  The shape was streamline,

that is to say, somewhat blunt at the nose and tapering towards the

tail, the total length being 143 feet 6 inches, with a maximum diameter

of 27 feet 9 inches.

The gross lift of these ships with 98% pure gas at a temperature of 60

degrees Fahrenheit and barometer 30 inches, is 4,180 lb. The net lift

available for crew, fuel, ballast, armament, etc., 1,434 lb., and the

disposable lift still remaining with crew of two on board and full

tanks, 659 lb.

The theoretical endurance at full speed as regards petrol consumption

is a little over 8 hours, but in practice it is probable that the oil

would run short before this time had been reached.  At cruising speed,

running the engine at 1,250 revolutions, the consumption is at the rate

of 3.6 gallons per hour, which corresponds to an endurance of 16 1/2

hours.

With the engine running at 1,800 revolutions, a speed of 50.6 miles per

hour has been reached by one of these ships, but actually very few

attained a greater speed than 40 miles per hour.

The envelopes of S.S. airships are composed of rubber-proofed fabric,

two fabrics being used with rubber interposed between and also on the

inner or gas surface.  To render them completely gastight and as

impervious to the action of the weather, sun, etc., as possible, five

coats of dope are applied externally, two coats of Delta dope, two of

aluminium dope and one of aluminium varnish applied in that order.

One ripping panel is fitted, which is situated on the top of the

envelope towards the nose.  It has a length of 14 feet 5 inches and a

breadth of about 8 inches.  The actual fabric which has to be torn away

overlaps the edge of the opening on each side.  This overlap is sewn

and taped on to the envelope and forms a seam as strong and gastight as

any other portion of the envelope.  Stuck on this fabric is a length of

biased fabric 8 1/4 inches wide. These two strips overlap the opening

at the forward end by about three feet.  At this end the two strips are

loose and have a toggle inserted at the end to which the ripping cord

is tied. The ripping cord is operated from the car.  It is led aft from

the ripping panel to a pulley fixed centrally over the centre of the

car, from the pulley the cord passes round the side of the envelope and

through a gland immediately below the pulley.

The nose of the envelope is stiffened to prevent it blowing in. For

this purpose 24 canes are fitted in fabric pockets around the nose and

meet at a point 2 1/4 inches in front of the nose.  An aluminium

conical cap is fitted over the canes and a fabric nose cap over the

whole.

Two ballonets are provided, one forward and one aft, the capacity of

each being 6,375 cubic feet.  The supply of air for filling these is

taken from the propeller draught by a slanting aluminium tube to the

underside of the envelope, where it meets a longitudinal fabric hose

which connects the two ballonet air inlets.  Non-return fabric valves

known as crab-pots are fitted in this fabric hose on either side of

their junction with the air scoop.  Two automatic air valves are fitted

to the underside of the envelope, one for each ballonet.  The air

pressure tends to open the valve instead of keeping it shut and to

counteract this the spring of the valve is inside the envelope.  The

springs are set to open at a pressure of 25 to 28 mm.

Two gas valves are also fitted, one on the top of the envelope, the

other at the bottom.  The bottom gas valve spring is set to open at 30

to 35 mm. pressure, the top valve is hand controlled only.

These valves are all very similar in design.  They consist of two

wooden rings, between which the envelope is gripped, and which are

secured to each other by studs and butterfly nuts.  The valve disc, or

moving portion of the valve, is made of aluminium and takes a seating

on a thin india rubber ring stretched between a metal rod bent into a

circle of smaller diameter than the valve opening and the wooden ring

of the valve.  When it passes over the wooden ring it is in contact

with the envelope fabric and makes the junction gastight.  The disc is

held against the rubber by a compressed spring.

The valve cords are led to the pilot's seat through eyes attached to

the envelope.

The system of rigging or car suspension is simplicity itself and is

tangential to the envelope.  On either side there are six main

suspensions of 25 cwt. stranded steel cable known as "C" suspensions.

Each "C" cable branches into two halves known as the "B" bridles, which

in turn are supported at each end by the bridles known as "A."  The

ends of the "A" bridles are attached to the envelope by means of Eta

patches.  These consist of a metal D-shaped fitting round which the

rigging is spliced and through which a number of webbing bands are

passed which are spread out fanwise and solutioned to the envelope.  It

will thus be seen that the total load on each main suspension is

proportionally taken up by each of the four "A" bridles, and that the

whole weight of the car is equally distributed over the greater part of

the length of the envelope.  Four handling guys for manoeuvering the

ship on the ground are provided under the bow and under the stem.  A

group of four Eta patches are placed close together, which form the

point of attachment for two guys in each case.  The forward of these

groups of Eta patches forms the anchoring point.  The bridle,

consisting of 25 cwt. steel cable, is attached here  and connected to

the forepart of the skids of the car.  The junction of this bridle with

the two cables from the skids forms the mooring point and there the

main trail rope is attached.  This is 120 feet long and composed of

2-inch manilla.  This is attached, properly coiled, to the side of the

car and is dropped by a  release gear.  It is so designed that when the

airship is held in a wind by the trail rope the strain is evenly

divided between the envelope and the car.  The grapnel carried is

fitted to a short length of rope.  The other end of the rope has an

eye, and is fitted to slide down the main trail rope and catch on a

knot at the end.

For steering and stabilizing purposes the S.S. airship was originally

designed with four fins and rudders, which were to be set exactly

radial to the envelope. In some cases the two lower fins and rudders

were abandoned, and a single vertical fin and rudder fitted centrally

under the envelope were substituted.  The three planes are identical in

size and measure 16 feet by 8 feet 6 inches, having a gross stabilizing

area of 402 1/2 square feet.

They are composed of spruce and aluminium and steel tubing braced with

wire and covered by linen doped and varnished when in position.

The original rudders measured 3 feet by 8 feet 6 inches.  In the case,

however, of the single plane being fitted, 4-feet rudders are

invariably employed.  Two kingposts of steel tube are fitted to each

plane and braced with wire to stiffen the whole structure.

The planes are attached to the envelope by means of skids and stay

wires.  The skids, composed of spruce, are fastened to the envelope by

eight lacing patches.

The car, it will be remembered, is a B.E. 2C fuselage stripped of its

wings, rudders and elevators, with certain other fittings added to

render it suitable for airship work.  The undercarriage is formed of

two ash skids, each supported by three struts.  The aeroplane landing

wheels, axle and suspensions are abandoned.

In the forward end of the fuselage was installed a 75 horse-power air

cooled Renault engine driving a single four-bladed tractor propeller

through a reduction gear of 2 to 1. The engine is of the 8-cylinder V

type, weighing 438 lb. with a bore of 96 mm. and a stroke of 120 mm.

The Claudel-Hobson type of carburettor is employed with this engine.

The type of magneto used is the Bosch D.V.4, there being one magneto

for each line of cylinders.  In the older French Renaults the Bosch

H.L.8 is used, one magneto supplying the current to all the plugs.

Petrol is carried in three tanks, a gravity and intermediate  tank as

fitted to the original aeroplane, and a bottom tank placed underneath

the front seat of the car.  The petrol is forced by air pressure from

the two lower tanks into the gravity tank and is obtained by a hand

pump fitted outside the car alongside the pilot's seat.  The oil tank

is fitted inside the car in front of the observer.

The observer's seat is fitted abaft the engine and the pilot's seat is

aft of the observer.  The observer, who is also the wireless operator,

has the wireless apparatus fitted about his seat.  This consists of a

receiver and transmitter fitted inside the car, which derives power

from accumulator batteries.  The aerial reel is fitted outside the car.

During patrols signals can be sent and received up to and between 50

and 60 miles.

The pilot is responsible for the steering and the running of the

engine, and the controls utilized are the fittings supplied with the

aeroplane.  Steering is operated by the feet and elevating by a

vertical wheel mounted in a fore and aft direction across the seat.

The control wires are led aft inside the fairing of the fuselage to the

extreme end, whence they pass to the elevators and rudders.

The instrument board is mounted in front of the pilot.  The instruments

comprise a watch, an air-speed indicator graduated in knots, an aneroid

reading to 10,000 feet, an Elliott revolution counter, a Clift

inclinometer reading up to 20 degrees depression or elevation, a map

case with celluloid front.

There are in addition an oil pressure gauge, a petrol pressure gauge, a

glass petrol level and two concentric glass pressure gauges for gas

pressure.

The steering compass is mounted on a small wooden pedestal on the floor

between the pilot's legs.

The water-ballast tank is situated immediately behind the pilot's seat

and contains 14 gallons of water weighing 140 lbs.  The armament

consists of a Lewis gun and bombs.  The bombs are carried in frames

suspended about the centre of the undercarriage.  The bomb sight is

fitted near the bomb releasing gear outside the car on the starboard

side adjacent to the pilot's seat.  The Lewis gun, although not always

carried on the early S.S. airships, was mounted on a post alongside the

pilot's seat.

S.S. MAURICE FARMAN

For this type of S.S. the cars were built by Messrs. Airships Ltd.  In

general appearance they resemble the Maurice Farman aeroplane and were

of the pusher type; 60,000 and in later cases 70,000 cubic feet

envelopes were rigged to these ships, which proved to be slightly

slower than the B.E. 2C type, but this was compensated for owing to the

increased comfort provided for the crew, the cars being more roomy and

suitable for airship work in every way.

The system of rigging to all intents and purposes is the same in all

types of S.S. ships, the suspensions being adjusted to suit the

different makes of car.

In these ships the pilot sits in front, and behind him is the wireless

telegraphy operator; in several cases a third seat was fitted to

accommodate a passenger or engineer; dual rudder and elevator controls

are provided for the pilot and observer.

The engine is mounted aft, driving a four-bladed pusher propeller, with

the petrol tanks situated in front feeding the carburettors by gravity.

The engines used are Rolls Royce Renaults, although in one instance a

75 horse-power Rolls Royce Hawk engine was fitted, which assisted in

making an exceedingly useful ship.

S.S. ARMSTRONG WHITWORTH

The car designed by Messrs. Armstrong Whitworth is of the tractor type

and is in all ways generally similar to the B.E. 2C. The single-skid

landing chassis with buffers is the outstanding difference.  These cars

had to be rigged to 70,000 cubic feet envelopes otherwise the margin of

lift was decidedly small.  A water-cooled 100 horse-power Green engine

propelled the ship, and a new feature was the disposition of petrol,

which was carried in two aluminium tanks slung from the envelope and

fed through flexible pipes to a two-way cock and thence to the

carburettors. These tanks, which were supported in a fabric sling,

showed a saving in weight of 100 lb. compared with those fitted in the

B.E. 2C.

For over two years these three types of S.S. ships performed a great

# part of our airship patrol and gave most excellent results.

Owing to the constant patrol which was maintained whenever weather

conditions were suitable, the hostile submarine hardly dared to show

her periscope in the waters which were under observation.  In addition

to this, practically the whole of the airship personnel now filling the

higher positions, such as Captains of Rigids and North Seas, graduated

as pilots in this type of airship.  From these they passed to the

Coastal and onwards to the larger vessels.

As far as is known the height record for a British airship is still

held by an S.S.B.E. 2C, one of these ships reaching the altitude of

10,300 feet in the summer of 1916.

The Maurice Farman previously mentioned as being fitted with the Hawk

engine, carried out a patrol one day of 18 hours 20 minutes. In the

summer of 1916 one of the Armstrong ships was rigged to an envelope

doped black and sent over to France.  While there she carried out

certain operations at night which were attended with success, proving

that under certain circumstances the airship can be of value in

operating with the military forces over land.

S.S.P.

In 1916 the design was commenced for an S.S. ship which should have a

more comfortable car and be not merely an adaptation of an aeroplane

body.  These cars, which were of rectangular shape with a blunt nose,

were fitted with a single landing skid aft, and contained seats for

three persons.

The engine, a 100 horse-power water-cooled Green, was mounted on

bearers aft and drove a four-bladed pusher propeller.  The petrol was

carried in aluminium tanks attached by fabric slings to the axis of the

envelope.

Six of these ships were completed in the spring of 1917 and were quite

satisfactory, but owing to the success achieved by the experimental

S.S. Zero it was decided to make this the standard type of S.S. ship,

and with the completion of the sixth the programme of the S.S.P's was

brought to a close.

These ships enjoyed more than, perhaps, was a fair share of misfortune,

one was wrecked on proceeding to its patrol station and was found to be

beyond repair, and another was lost in a snowstorm in the far north.

The remainder, fitted at a later date with 75 horse-power Rolls Royce

engines, proved to be a most valuable asset to our fleet of small

airships.

S.S. ZERO

The original S.S. Zero was built at a south-coast station by Air

Service labour, and to the design of three officers stationed there.

The design of the car shows a radical departure from anything that had

been previously attempted, and as a model an ordinary boat was taken.

In shape it is as nearly streamline as is practicable, having a keel

and ribs  of wood with curved longitudinal members, the strut ends

being housed in steel sockets.  The whole frame is braced with piano

wire set diagonally between the struts.  The car is floored from end to

end, and the sides are enclosed with 8-ply wood covered with fabric.

Accommodation is provided for a wireless telegraphy operator, who is

also a gunner, his compartment being situated forward, amidships is the

pilot and abaft this seat is a compartment for the engineer.

The engine selected was the 75 horse-power water-cooled Rolls Royce, it

being considered to be the most efficient for the purpose.  The engine

is mounted upon bearers above the level of the top of the car, and

drives a four-bladed pusher propeller.

The car is suspended from an envelope of 70,000 cubic feet capacity,

and the system of rigging is similar to that in use on all S.S. ships.

The petrol is carried in aluminium tanks slung on the axis of the

envelope, identically with the system in use on the S.S.P's.   The

usual elevator planes are adopted with a single long rudder plane.

The speed of the Zero is about 45 miles per hour and the ship has a

theoretical endurance of seventeen hours; but this has been largely

exceeded in practice.

The original ship proved an immediate success, and a large number was

shortly afterwards ordered.

As time went on the stations expanded and sub-stations were added,

while the Zero airship was turned out as fast as it could be built,

until upwards of seventy had been commissioned.  The work these ships

were capable of exceeded the most sanguine expectations.  Owing to

their greater stability in flight and longer hours of endurance, they

flew in weather never previously attempted by the earlier ships.  With

experience gained it was shown that a large fleet of airships of

comparatively small capacity is of far more value for an anti-submarine

campaign than a lesser fleet of ships of infinitely greater capacity.

The average length of patrol was eight hours, but some wonderful

duration flights were accomplished in the summer of 1918, as the

following figures will show.  The record is held by S.S.Z. 39, with 50

hours 55 minutes; another is 30 hours 20 minutes; while three more vary

from 25 1/2 hours to 26 1/4.  Although small, the Zero airship has been

one of the successes of the war, and we can claim proudly that she is

entirely a British product.

S.S. TWIN

During the year 1917, designs were submitted for a twin-engined S.S.

airship, the idea being to render the small type of airship less liable

to loss from engine failure.  The first design proved to be a failure,

but the second was considered more promising, and several were built.

Its capacity is 100,000 cubic feet, with a length of 164 feet 6 inches,

and the greatest diameter 32 feet.

The car is built to carry five, with the engines disposed on gantries

on the port and starboard side, driving pusher propellers.  This type,

although in the experimental stage, is being persevered with, and the

intention is that it will gradually supplant the other S.S. classes.

It is calculated that it will equal if not surpass the C Star ship in

endurance, besides being easier to handle and certainly cheaper to

build.

"COASTAL" AND "C STAR" AIRSHIPS

The urgent need for a non-rigid airship to carry out anti-submarine

patrol having been satisfied for the time with the production of the

S.S. B.E. 2C type, the airship designers of the Royal Naval Air Service

turned their attention to the production of an airship which would have

greater lift and speed than the S.S. type, and, consequently, an

augmented radius of action, together with a higher degree of

reliability.  As the name "Coastal" or "Coast Patrol" implies, this

ship was intended to carry out extended sea patrols.

To obtain these main requirements the capacity of the envelope for this

type was fixed at 170,000 cubic feet, as compared with the 60,000 cubic

feet and, later, the 70,000 cubic feet envelopes adopted for the S.S.

ships.  Greater speed was aimed at by fitting two engines of 150

horse-power each, and it was hoped that the chances of loss owing to

engine failure would be considerably minimized.

The Astra-Torres type of envelope, with its system of internal rigging,

was selected for this class of airship; in the original ship the

envelope used was that manufactured by the French Astra-Torres Company,

and to which it had been intended to rig a small enclosed car.  The

ship in question was to be known as No. 10.  This plan was, however,

departed from, and the car was subsequently rigged to the envelope of

the Eta, and a special car was designed and constructed for the

original Coastal.  Coastal airship No. 1 was commissioned towards the

end of 1915 and was retained solely for experimental and training

purposes. Approximately thirty of these airships were constructed

during the year 1916, and were  allocated to the various stations for

patrol duties.

The work carried out by these ships during the two and a half years in

which they were in commission, is worthy of the highest commendation.

Before the advent  of later and more reliable ships, the bulk of

anti-submarine patrol on the east coast and south-west coast of England

was maintained by the Coastal.  On the east coast, with the prevailing

westerly and south-westerly winds, these airships had many long and

arduous voyages on their return from patrol, and in the bitterness of

winter their difficulties were increased ten-fold.  To the

whole-hearted efforts of Coastal pilots and crews is due, to a great

extent, the recognition which somewhat tardily was granted to the

Airship Service.

The envelope of the Coastal airship has been shown to be of 170,000

cubic feet capacity.  It is trilobe in section to employ the

Astra-Torres system of internal and external rigging.  The great

feature of this principle is that it enables the car to be slung much

closer to the envelope than would be possible with the tangential

system on an envelope of this size.  As a natural consequence there is

far less head resistance, owing to the much shorter rigging, between

the envelope and the car.

The shape of the envelope is not all that could have been desired, for

it is by no means a true streamline, but has the same cross section for

the greater part of its length, which tapers at either end to a point

which is slightly more accentuated aft.  Owing to the shape, these

ships, in the early days until experience had been gained, were

extremely difficult to handle, both on the landing ground and also in

the air.  They were extremely unstable both in a vertical and

horizontal plane, and were slow in answering to their rudders and

elevators.

The envelope is composed of rubber-proofed fabric doped to hold the gas

and resist the effects of weather.  Four ballonets are situated in the

envelope, two in each of the lower lobes, air being conveyed to them by

means of a fabric air duct, which is parallel to the longitudinal

centre line of the envelope, with transverse ducts connecting each pair

of ballonets.  In earlier types of the Coastal, the air scoop supplying

air to the air duct was fitted in the slip stream of the forward

engine, but later this was fitted aft of the after engine.

Six valves in all are used, four air valves, one fitted to each

ballonet, and two gas valves.  These are situated well aft, one to each

of the lower lobes, and are fitted on either side of the rudder plane.

A top valve is dispensed with because in practice when an Astra-Torres

envelope loses shape, the tendency is for the tail to be pulled upwards

by the rigging, with the result that the two gas valves always remain

operative.

Crabpots and non-return valves are employed in a similar manner to S.S.

airships.

The Astra-Torres system of internal rigging must now be described in

some detail.  The envelope is made up of three longitudinal lobes, one

above and two below, which when viewed end on gives it a trefoil

appearance.  The internal rigging is attached to the ridges formed on

either side of the upper lobe, where it meets the two side lobes.  From

here it forms a V, when viewed cross sectionally, converging at he

ridge formed by the two lobes on the underside of the envelope which is

known as the lower ridge.

To the whole length of the top ridges are attached the internal rigging

girdles and also the lacing girdles to which are secured the top and

side curtains.  These curtains are composed of ordinary unproofed

fabric and their object is to make the envelope keep its trilobe shape.

They do not, however, divide the ship into separate gas compartments.

The rigging girdle consists of a number of fabric scallops through

which run strands of Italian hemp.  These strands, of which there are a

large number, are led towards the bottom ridge, where they are drawn

together and secured to a rigging sector.  To these sectors the main

external rigging cables are attached.  The diagram shows better than

any description this rigging system.

Ten main suspensions are incorporated in the Coastal envelope, of which

three take the handling guys, the remaining seven support the weight of

the car.

The horizontal fins with the elevator flaps, and the vertical fin with

the rudder flap, are fixed to the ridges of the envelope.

The car was evolved in the first instance by cutting away the tail

portion of two Avro seaplane fuselages and joining the forward portions

end on, the resulting car, therefore, had engines at either end with

seating accommodation for four.  The landing chassis were altered,

single skids being substituted for the wider landing chassis employed

in the seaplane.  The car consists of four longerons with struts

vertical and cross, and stiffened with vertical and cross bracing

wires.  The sides are covered with fabric and the flooring and fairing

on the top of the car are composed of three-ply wood.  In the later

cars five seats were provided to enable a second officer to be carried.

The engines are mounted on bearers at each end of the car, and the

petrol and oil tanks were originally placed adjoining the engines in

the car.  At a later date various methods of carrying the petrol tanks

were adopted, in some cases they were slung from the envelope and in

others mounted on bearers above the engines.

Wireless telegraphy is fitted as is the case with all airships. In the

Coastal a gun is mounted on the top of the envelope, which is reached

by a climbing shaft passing through the envelope, another mounting

being provided on the car itself.

Bombs are also carried on frames attached to the car.  Sunbeam engines

originally supplied the motive power, but at a later date a 220

horse-power Renault was fitted aft and a 100 horse-power, Berliet

forward. With the greater engine power the ship's capabilities were

considerably increased.

Exceedingly long flights were achieved by this type of ship, and those

exceeding ten hours are far too numerous to mention.  The most noteworthy

of all gave a total of 24 1/4 hours, which, at the time, had only once

been surpassed by any British airship.

Towards the end of 1917, these ships, having been in commission for

over two years, were in many cases in need of a complete refit.

Several were put in order, but it was decided that this policy should

not be continued, and that as each ship was no longer fit for flying it

should be replaced by the more modern Coastal known as the C Star.

The record of one of these ships so deleted is surely worthy of special

mention.  She was in commission for 2 years 75 days, and averaged for

each day of this period 3 hours 6 minutes flying. During this time she

covered upwards of 66,000 miles.  From this it will be seen that she

did not pass her life by any means in idleness.

"C STAR" AIRSHIP

After considerable experience had been gained with the Coastal, it

became obvious that a ship was required of greater capabilities to

maintain the long hours of escort duty and also anti-submarine patrols.

To meet these requirements it was felt that a ship could be

constructed, not departing to any extent from the Coastal, with which

many pilots were now quite familiar, but which would show appreciable

improvement over its predecessor.

The design which was ultimately adopted was known as the C Star, and

provided an envelope of 210,000 cubic feet, which secured an extra ton

and a quarter in lifting capacity.  This envelope, although of the

Astra-Torres type, was of streamline form, and in that respect was a

great advance on the early shape as used in the Coastal.  It is to all

intents and purposes the same envelope as is used on the North Sea

ships, but on a smaller scale.  An entirely new type of fabric was

employed for this purpose.  The same model of car was employed, but was

made more comfortable, the canvas covering for the sides being replaced

by three-ply wood.  In all other details the car remained entirely the

same. The standard power units were a 100 horse-power Berliet forward

and a Fiat of 260 horse-power aft.  The petrol tanks in this design

were carried inside the envelope, which was quite a new departure.

These airships may be considered to have been successful, though not

perhaps to the extent which was expected by their most ardent admirers.

With the advent of the S.S. Twin it was resolved not to embark on a

large constructional programme, and when the numbers reached double

figures they were no longer proceeded with.  Notwithstanding this the

ships which were commissioned carried out most valuable work, and, like

their prototypes, many fine flights were recorded to their credit.

Thirty-four and a half hours was the record flight for this type of

ship, and another but little inferior was thirty hours ten minutes.

These flights speak well for the endurance of the crews, as it must be

borne in mind that no sleeping accommodation is possible in so small a

car.

The Coastal airship played no small part in the defeat of the

submarine, but its task was onerous and the enemy and the elements

unfortunately exacted a heavy toll.  A German wireless message received

in this country testified to the valiant manner in which one of these

ships met with destruction.

THE "NORTH SEA" AIRSHIP

The North Sea or N.S. airship was originally designed to act as a

substitute for the Rigid, which, in 1916, was still a long way from

being available for work of practical utility.  From experience gained

at this time with airships of the Coastal type it was thought possible

to construct a large Non-Rigid capable of carrying out flights of

twenty-four hours' duration, with a speed of 55 to 60 knots, with

sufficient accommodation for a double crew.

The main requirements fall under four headings:

1. Capability to carry out flights of considerable duration.

2. Great reliability.

3. The necessary lift to carry an ample supply of fuel.

4. Adequate arrangements to accommodate the crew in comfort.

If these could be fulfilled the authorities were satisfied that ships

possessing these qualifications would be of value to the Fleet and

would prove efficient substitutes until rigid airships were available.

The North Sea, as may be gathered from its name, was intended to

operate on the east coasts of these islands.

The first ship, when completed and put through her trials, was voted a

success, and the others building were rapidly pushed on with.  When

several were finished and experience had been gained, after long

flights had been carried out, the North Sea airship suffered a partial

eclipse and people were inclined to reconsider their favourable

opinion.  Thus it was that for many months the North Sea airship was

decidedly unpopular, and it was quite a common matter to hear her

described as a complete failure.  The main cause of the prejudice was

the unsatisfactory design of the propelling machinery, which it will be

seen later was modified altogether, and coupled with other improvements

turned a ship of doubtful value into one that can only be commended.

The envelope is of 360,000 cubic feet capacity, and is designed on the

Astra-Torres principle for the same reasons as held good in the cases

of the Coastal and C Star.  All the improvements which had been

suggested by the ships of that class were incorporated in the new

design, which was of streamline shape throughout, and looked at in

elevation resembled in shape that of the S.S. airship. Six ballonets

are fitted, of which the total capacity is 128,000 cubic feet,

equivalent to 35.5 per cent of the total volume.  They are fitted with

crabpots and non-return valves in the usual manner.

The rigging is of the Astra-Torres system, and in no way differs from

that explained in the previous chapter. Nine fans of the internal

rigging support the main suspensions of the car, while similar fans

both fore and aft provide attachment for the handling guys.  Auxiliary

fans on the same principle support the petrol tanks and ballast bag.

Four gas and six air valves in all are fitted, all of which are

automatic.

Two ripping panels are embodied in the top lobe of the envelope.

The N.S. ship carries four fins, to three of which are attached the

elevator and rudder flaps.  The fourth, the top fin, is merely for

stabilizing purposes, the other three being identical in design, and

are fitted with the ordinary system of wiring and kingposts to prevent

warping.

The petrol was originally carried in aluminium tanks disposed above the

top ridges of the envelope, but this system was abandoned owing to the

aluminium supply pipes becoming fractured as the envelope changed shape

at different pressures.  They were then placed inside the envelope, and

this rearrangement has given every satisfaction.

To the envelope of the N.S. is rigged a long covered-in car.  The

framework of this is built up of light steel tubes, the rectangular

transverse frames of which are connected by longitudinal tubes, the

whole structure being braced by diagonal wires.  The car, which tapers

towards the stern, has a length of 85 feet, with a height of 6 feet.

The forward portion is covered with duralumin sheeting, and the

remainder with fabric laced to the framework.  Windows and portholes

afford the crew both light and space to see all that is required.  In

the forward portion of the car are disposed all the controls and

navigating instruments, together with engine-telegraphs and voice

pipes.  Aft is the wireless telegraphy cabin and sleeping accommodation

for the crew.

A complete electrical installation is carried of two dynamos and

batteries for lights, signalling lamps, telephones, etc.  The engines

are mounted in a power unit structure separate from the car and reached

by a wooden gangway supported by wire cables. This structure consists

of two V-shaped frameworks connected by a central frame and by an

under-structure to which floats are attached.  The mechanics'

compartment is built upon the central frame, and the engine controls

are operated from this cabin.

In the original power units two 250 horse-power Rolls Royce engines

were fitted, driving propellers on independent shafts through an

elaborate system of transmission.  This proved to be a great source of

weakness, as continual trouble was experienced with this method, and a

fracture sooner or later occurred at the universal joint nearest to the

propeller.  When the modified form of ship was built the whole system

of transmission was changed, and the propellers were fitted directly on

to the engine crankshafts.

At a later date 240 horse-power Fiat engines were installed, and the

engineers' cabin was modified and an auxiliary blower was fitted to

supply air to the ballonets for use if the engines are not running.

In the N.S. ship as modified the car has been raised to the same level

as the engineers' cabin, and all excrescences on the envelope were

placed inside.  This, added to the improvement effected by the

abolition of the transmission shafts, increased the reliability and

speed of the ship, and also caused a reduction in weight.

The leading dimensions of the ship are as follows: length, 262 feet;

width, 56 feet 9 inches; height, 69 feet 3 inches.  The gross lift is

24,300 lb.; the disposable lift, without crew, petrol, oil, and

ballast, 8,500 lb.  The normal crew carried when on patrol is ten,

which includes officers.

As in the case of the Coastal, a gun is mounted on the top of the

envelope, which is approached by a similar climbing shaft, and guns and

bombs are carried on the car.

These ships have become notorious for breaking all flying records for

non-rigid airships.  Even the first ship of the class, despite the

unsatisfactory power units, so long ago as in the summer of 1917

completed a flight of 49 hours 22 minutes, which at the time was the

record flight of any British airship.  Since that date numerous flights

of quite unprecedented duration have been achieved, one of 61 1/2 hours

being particularly noteworthy, and those of upwards of 30 hours have

become quite commonplace.

Since the Armistice one of these ships completed the unparalleled total

of 101 hours, which at that date was the world's record flight, and

afforded considerable evidence as to the utility of the non-rigid type

for overseas patrol, and even opens up the possibility of employing

ships of similar or slightly greater dimensions for commercial purposes.

N.S. 6 appeared several times over London in the summer months of 1918,

and one could not help being struck by the ease with which she was

steered and her power to remain almost stationary over such a small

area as Trafalgar Square for a quite considerable period.

The flights referred to above were not in any way stunt performances to

pile up a handsome aggregate of hours, but were the ordinary flying

routine of the station to which the ships were attached, and most of

the hours were spent in escorting convoys and hunting for submarines.

In addition to these duties, manoeuvres were carried out on occasions

with the Fleet or units thereof.

From the foregoing observations it must be manifest that this type of

ship, in its present modified state, is a signal success, and is

probably the best large non-rigid airship that has been produced in any

country.

For the purposes of comparison it will be interesting to tabulate the

performances of the standard types of non-rigid airships. The leading

dimensions are also included in this summary:

Type            S.S. Zero    S.S. Twin     Coastal     North

Star        Sea

Length            143' 0"      165' 0"     218' 0"     262' 0"

Overall width      32' 0"       35' 6"      49' 3"      56' 9"

Overall height     46' 0"       49' 0"      57' 6"      69' 3"

Hydrogen capacity

(cubic feet)      70,000      100,000     210,000     360,000

Gross lift (lb.)    4,900        7,000      14,500      24,300

Disposable

lift (lb.)         1,850        2,200       4,850       8,500

Crew                    3            4           5          10

Lift available

for fuel and

freight (lb.)      1,370        1,540       4,050       6,900

Petrol consumption

at full speed

(lb. per hour)     3.6          7.2        18.4        29.8

Gals. per hour      0.36         0.72        2.05         3

# CHAPTER VII

NAVAL AIRSHIPS.--THE RIGIDS--RIGID AIRSHIP No. 1

The responsibility for the development the Rigid airship having been

allotted to the Navy, with this object in view, in the years 1908 and

1909 a design was prepared by Messrs. Vickers Ltd., in conjunction with

certain naval officers, for a purely experimental airship which should

be as cheap as possible.  The ship was to be known as Naval Airship No.

1, and though popularly called the Mayfly, this title was in no way

official.  In design the following main objects were aimed at:

1. The airship was to be capable of carrying out the duties of an

aerial scout.

2. She was to be able to maintain a speed of 40 knots for

twenty-four hours, if possible.

3. She was to be so designed that mooring to a mast on the water

was to be feasible, to enable her to be independent of her

shed except for docking purposes, as in the case with surface

vessels.

4. She was to be fitted with wireless telegraphy.

5. Arrangements were to be made for the accommodation of the crew

in reasonable comfort.

6. She was to be capable of ascending to a height of not less

than 1,500 feet.

These conditions rendered it necessary that the airship should be of

greater dimensions than any built at the time, together with larger

horse-power, etc.

These stipulations having been settled by the Admiralty, the Admiralty

officials, in conjunction with Messrs. Vickers Ltd., determined the

size, shape, and materials for the airship required.  The length of the

ship was fixed at approximately 500 feet, with a diameter of 48 feet.

Various shapes were considered, and the one adopted was that

recommended by an American professor named Zahm.  In this shape, a

great proportion of the longitudinal huff framework is parallel sided

with curved bow and stern portions, the radius of these curved portions

being, in the case of the bow, twice the diameter of the hull, and in

the case of the stern nine times the same diameter. Experiments proved

that the resistance of a ship of this shape was only two-fifths of the

resistance of a ship of the same dimensions, having the 1 1/2 calibre

bow and stern of the Zeppelin airships at that time constructed.

A considerable difference of opinion existed as to the material to be

chosen for the construction of the hull.  Bamboo, wood, aluminium, or

one of its alloys, were all considered.  The first was rejected as

unreliable.  The second would have been much stronger than aluminium,

and was urged by Messrs. Vickers.  The Admiralty, however, considered

that there was a certainty of better alloys being produced, and as the

ship was regarded as an experiment and its value would be largely

negatived if later ships were constructed of a totally different

material, aluminium or an alloy was selected.  The various alloys then

in existence showed little advantage over the pure metal, so pure

aluminium was specified and ordered.  This metal was expected to have a

strength of ten tons per square inch, but that which arrived was found

to be very unreliable, and many sections had, on test, only half the

strength required.  The aluminium wire intended for the mesh wiring of

the framework was also found to be extremely brittle.  A section of the

framework was, however, erected, and also one of wood, as a test for

providing comparisons.  In the tests, the wooden sections proved,

beyond all comparison, the better, but the Admiralty persisted in their

decision to adopt the metal.

Towards the end of 1909 a new aluminium alloy was discovered, known as

duralumin.  Tests were made which proved that this new metal possessed

a strength of twenty-five tons per square inch, which was over twice as

strong as the nominal strength of aluminium, and in practice was really

five times stronger.  The specific gravity of the new metal varied from

2.75 to 2.86, as opposed to the 2.56 of aluminium.  As the weights were

not much different it was possible to double the strength of the ship

and save one ton in weight.  Duralumin was therefore at once adopted.

The hull structure was composed of twelve longitudinal duralumin

girders  which ran fore and aft the length of the ship and followed the

external shape.  The girders were secured to a steel nose-piece at the

bow and a pointed stern-piece aft.  These girders, built of duralumin

sections, were additionally braced wherever the greatest weights

occurred.  To support these girders in a thwartship direction a series

of transverse frames were placed at 12 feet 6 inches centres throughout

the length of the ship, and formed, when viewed cross-sectionally, a

universal polygon of twelve sides.  For bracing purposes mesh wiring

stiffened each bay longitudinally, so formed by the junction of the

running girder and the transverse frames, while the transverse frames

between the gasbags were stiffened with radial wiring which formed

structure similar to a wheel with its spokes.  The frames where the

gondolas occurred were strengthened to take the addition weight, while

the longitudinals were also stiffened at the bow and stern.

Communication was provided between the gondolas by means of an external

keel which was suspended from extra keel longitudinals. In this design

the keel was provided for accommodation purposes only, and in no way

increased the structural stability of the ship as in No. 9 and later

ships.  This keel, triangular in section, widened out amidships to form

a space for a cabin and the wireless compartment.  The fins and

rudders, which were adopted, were based entirely on submarine

experience, and the Zeppelin method was ignored.  The fins were fitted

at the stern of the ship only, and comprised port and starboard

horizontal fins, which followed approximately the shape of the hull,

and an upper and lower vertical fin.  Attached to these fins were box

rudders and elevators, instead of the balanced rudders first proposed.

Auxiliary rudders were also fitted in  case of a breakdown of the main

steering gear abaft the after gondola.  Elevators and rudders were

controlled from the forward gondola and the auxiliary rudders from the

after gondola.

The gasbags were seventeen in number and were twelve-sided in section,

giving approximately a volume of 663,000 cubic feet when completely

full.  Continental fabric, as in use on the Zeppelin airships, was

adopted, although the original intention was to use gold-beater's skin,

but this was abandoned owing to shortage of material.  These bags were

fitted with the Parseval type of valve, which is situated at the top,

contrary to the current Zeppelin practice, which had automatic valves

at the bottom of the bags, and hand-operated valves on the top of a few

bags for control purposes.  Nets were laced to the framework to prevent

the bags  bulging through the girders.

The whole exterior of the hull was fitted with an outer cover; Zeppelin

at this time used a plain light rubber-proofed fabric, but this was not

considered suitable for a ship which was required to be moored in the

open, as in wet weather the material would get saturated and

water-logged.  Various experiments were carried out with cotton, silk

and ramie, and, as a result, silk treated with Ioco was finally

selected.  This cover was laced with cords to the girder work, and

cover-strips rendered the whole impervious to wet.  Fire-proofed fabric

was fitted in wake of the gondolas for safety from the heat of the

engines.

Two gondolas, each comprising a control compartment and engine-room,

were suspended from the main framework of the hull. They were shaped to

afford the least resistance possible to the air, and were made of

Honduras mahogany, three-ply where the ballast tanks occurred, and

two-ply elsewhere.  The plies were sewn together with copper wire.  The

gondolas were designed to have sufficient strength to withstand the

strain of alighting on the water.  They were suspended from the hull by

wooden struts streamline in shape, and fitted with internal steel-wire

ropes; additional wire suspensions were also fitted to distribute the

load over a greater length of the ship.  The engines were carried in

the gondolas on four hollow wooden struts, also fitted internally with

wire.  The wires were intended to support the gondolas in the event of

the struts being broken in making a heavy landing.

Two engines were mounted, one in each gondola, the type used being the

8-cylinder vertical water-cooled Wolseley developing a horse-power of

160.  The forward engine drove two wing propellers through the medium

of bevel gearing, while the after engine drove a single large propeller

aft through 4 gear box to reduce the propeller revolutions to half that

of the engine.  The estimated speed of the ship was calculated to be 42

miles per hour, petrol was carried in tanks, fitted in the keel, and

the water ballast tanks were placed close to the keel and connected

together by means of a pipe.

No. 1 was completed in May, 1911. She had been built at Barrow in a

shed erected on the edge of Cavendish Dock.  Arrangements were made

that she should be towed out of the shed to test her efficiency at a

mooring post which had been prepared in the middle of the dock.  She

was launched on May 22nd in a flat calm and was warped out of the shed

and hauled to the post where she was secured without incident.  The

ship rode at the mooring post in a steady wind, which at one time

increased to 36 miles per hour, until the afternoon of May 25th, and

sustained no damage whatever.  Various  engine trials were carried out,

but no attempt was made to fly, as owing to various reasons the ship

was short of lift.  Valuable information was, however, gained in

handling the ship, and much was learnt of her behaviour at the mast.

More trouble was experienced in getting her back into the shed, but she

was eventually housed without sustaining any damage of importance.

Owing to the lack of disposable lift, the bags were deflated and

various modifications were carried out to lighten the ship, of which

the principal were the removal of the keel and cabin entirely, and the

removal of the water-trimming services.  Other minor alterations were

made which gave the ship, on completion, a disposable lift of 3.21

tons.  The transverse frames between the gasbags were  strengthened,

and a number of broken wires were replaced.

On September 22nd the ship was again completed, and on the 24th she was

again to be taken out and tested at the mooring post. Unfortunately,

while being hauled across the dock, the framework of the ship

collapsed, and she was got back into the shed the same day.

Examination showed that it was hopeless to attempt to reconstruct her,

and she was broken up at a later date.  The failure of this ship was a

most regrettable incident, and increased the prejudice against the

rigid airship to such an extent that for some time the Navy refused to

entertain any idea of attempting a second experiment.

RIGID AIRSHIP No. 9

Rigid Airship No. 1 having met with such a calamitous end, the

authorities became rather dubious as to the wisdom of continuing such

costly experiments.  Most unfortunately, as the future showed and as

was the opinion of many at the time, rigid construction in the

following year 1912 was ordered to be discontinued.  This decision

coincided with the disbanding of the Naval Air Service, and for a time

rigid airships in this country were consigned to the limbo of

forgetfulness.  After the Naval Air Service had been reconstituted, the

success which attended the Zeppelin airships in Germany could no longer

be overlooked, and it was decided to make another attempt to build a

rigid airship in conformity with existing Zeppelin construction.  The

first proposals were put forward in 1913, and, finally, after eleven

months delay, the contract was signed.  This airship, it has been seen,

was designated No. 9.

No. 9 experienced numerous vicissitudes, during the process of design

and later when construction was in progress.  The contract having been

signed in March, 1914, work on the ship was suspended in the following

February, and was not recommenced until July of the same year.  From

that date onwards construction was carried forward; but so many

alterations were made that it was fully eighteen months before the ship

was completed and finally accepted by the Admiralty.

The ship as designed was intended "to be generally in conformity with

existing Zeppelin construction," with the following main requirements

stipulated for in the specification:

1. She was to attain a speed of at least 45 miles per hour at the

full power of the engines.

2. A minimum disposable lift of five tons was to be available for

movable weights.

3. She was to be capable of rising to a height of 2,000 feet

during flight.

The design of this ship was prepared by Messrs. Vickers, Ltd., and as

it was considered likely that owing to inexperience the ship would

probably be roughly handled and that heavy landings might be made, it

was considered that the keel structure and also the cars should be made

very strong in case of accidents occurring.  This, while materially

increasing the strength of the ship, added to its weight, and coupled

with the fact that modifications were made in the design, rendered the

lift somewhat disappointing.  The hull structure was of the "Zahm"

shape as in No. 1, a considerable portion being parallel sided, while

in transverse section it formed a 17-sided polygon. In length it was

526 feet with a maximum diameter of 53 feet.  The hull framework was

composed of triangular duralumin girders, both in the longitudinal and

transverse frames, while the bracing was carried out by means of high

tensile steel wires and duralumin tubes. Attached to the hull was a

V-shaped keel composed of tubes with suitable wire bracings, and in it

a greater part of the strength of the structure lay.  It was designed

to withstand the vertical forces and bending moments which resulted

from the lift given by the gasbags and the weights of the car and the

cabin.  The keel also provided the walking way from end to end of the

ship, and amidships was widened out to form a cabin and wireless

compartment.

The wiring of the transverse frames was radial and performed similar

functions to the spokes of a bicycle wheel.  These wires could be

tightened up at the centre at a steel ring through which they were

threaded and secured by nuts.

In addition to the radial wires were the lift wires, which were led to

the two points on the transverse frames which were attached to the

keel; on the inflation of the gasbags, the bags themselves pressed upon

the longitudinal girders on the top of the ship, which pressure was

transferred to the transverse frames and thence by means of the several

lift wires to the keel.  In this way all the stresses set up by the gas

were brought finally to the keel in which we have already said lay the

main strength of the ship.

The hull was divided by the transverse frames into seventeen

compartments each containing a single gasbag.  The bags were composed

of rubber-proofed fabric lined with gold-beater's skin to reduce

permeability, and when completely full gave a total volume of 890,000

cubic feet.  Two types of valve were fitted to each bag, one the

Parseval type of valve with the pressure cone as fitted in No. 1, the

other automatic but also controlled by hand.

To distribute the pressure evenly throughout the upper longitudinal

frames, and also to prevent the gasbags bulging between the girders,

nets were fitted throughout the whole structure of the hull.

The whole exterior of the ship was fitted with an outer cover, to

protect the gasbags and hull framework from weather and to render the

outer surface of the ship symmetrical and reduce "skin friction" and

resistance  to the air to a minimum.  To enable this cover to be easily

removed it was made in two sections, a port and starboard side for each

gasbag.  The covers were laced to the hull framework and the

connections were covered over with sealing strips to render  the whole

weathertight.

The system of fins for stabilizing purposes on No. 9 were two--vertical

and horizontal. The vertical fin was composed of two parts, one above

and the other below the centre line of the ship.

They were constructed of a framework of duralumin girders, covered over

with fabric.  The fins were attached on one edge to the hull structure

and wire braced from the other edge to various positions on the hull.

The horizontal fins were of similar design and attached in a like

manner to the hull.  Triplane rudders and biplane elevators of the box

type were fitted in accordance with the German practice of the time.

Auxiliary biplane rudders were fitted originally abaft the after car,

but during the first two trial flights they proved so very

unsatisfactory that it was decided to remove them.

Two cars or gondolas were provided to act as navigating compartments

and a housing for the engines, and in design were calculated to offer

the least amount of head resistance to the wind.  The cars were

composed of duralumin girders, which formed a flooring, a main girder

running the full length of the car with a series of transverse girders

spaced in accordance with the main loads.  From each of these

transverse girders vertical standards with a connecting piece on top

were taken and the whole exterior was covered with duralumin plating.

The cars were suspended in the following manner.  Two steel tubes

fitting into a junction piece at each end were bolted to brackets at

the floor level at each end of the transverse girders.  They met at an

apex above the roof level and were connected to the tubing of the keel.

In addition, to distribute the weight and prevent the cars from

rocking, steel wire suspensions were led to certain fixed points in the

hull.

Each car was divided into two parts by a bulkhead, the forward portion

being the control compartment in which were disposed all instruments,

valve and ballast controls, and all the steering and elevating

arrangements.  Engine-room telegraphs, voice pipes and telephones were

fitted up for communication from one part of the ship to the other.

The keel could be reached by a ladder from each car, thus providing

with the climbing shaft through the hull access to all parts of the

ship.

The original engine equipment of No. 9 was composed of four

Wolseley-Maybach engines of 180 horse-power each, two being installed

in the forward car and two in the after car. As the ship was deficient

in lift after the initial flight trials had been carried out, it was

decided to remove the two engines from the after car and replace them

with a single engine of 250 horse-power; secondly, to remove the

swivelling propeller gear from the after car and substitute one

directly-driven propeller astern of the car.  This as anticipated

reduced the weight very considerably and in no way lessened the speed

of the ship.

The forward engines drove two four-bladed swivelling propellers through

gear boxes and transmission shafts, the whole system being somewhat

complicated, and was opposed to the Zeppelin practice at the time which

employed fixed propellers.

The after engine drove a large two-bladed propeller direct off the main

shaft.

The petrol and water ballast were carried in tanks situated in the keel

and the oil was carried in tanks beneath the floors of the cars.

The wireless cabin was situated as before mentioned in a cabin in the

keel of the ship, and the plant comprised a main transmitter, an

auxiliary transmitter and receiver and the necessary aerial for

radiating and receiving.

No. 9 was inflated in the closing days of 1916, and the disposal lift

was found to be 2.1 tons under the specification conditions, namely,

barometer 29.5 inches and temperature 55 degrees Fahrenheit.  The

contract requirements had been dropped to 3.1 tons, which showed that

the ship was short by one ton of the lift demanded.  The flight trials

were, however, carried out, which showed that the ship had a speed of

about 42 1/2 miles per hour.

The alterations previously mentioned were afterwards made, the bags of

the ship were changed and another lift and trim trial was held in

March, 1917, when it was found that these had had the satisfactory

result of increasing the disposable lift to 3.8 tons or .7 ton above

the contract requirements, and with the bags 100 per cent full gave a

total disposable lift of 5.1 tons.

Additional trials were then carried out, which showed that the speed of

the ship had not been impaired.

For reference purposes the performances of the ship are tabulated below.

Speed:

Full            45 miles per hour

Normal   = 2/3  38    "  "  "

Cruising = 1/3  32    "  "  "

Endurance:

Full     18 hours =   800 miles

Normal   26   "   = 1,000  "

Cruising 50   "   = 1,600  "

No. 9 having finished her trials was accepted  by the Admiralty in Mar.

1917, and left Barrow,  where she had been built, for a patrol station.

In many ways she was an excellent ship, for it must be remembered that

when completed she was some years out-of-date judged by Zeppelin

standards.  Apart from the patrol and convoy work which she

accomplished, she proved simply invaluable for the training of officers

and men selected to be the crews of future rigid airships.  Many of

these received their initial training in her, and there were few

officers or men in the airship service who were not filled with regret

when orders were issued that she was to be broken up.  The general

feeling was that she should have been preserved as a lasting exhibition

of the infancy of the airship service, but unfortunately rigid airships

occupy so much space that there is no museum in the country which could

have accommodated her. So she passed, and, except for minor trophies,

remains merely a recollection.

RIGID AIRSHIP No. 23 CLASS

After the decision had been made in 1915 that work on No. 9 should be

restarted, the Admiralty determined that a programme of rigid airships

should be embarked upon, and design was commenced.

Several ships of the same class were, ordered, and the type was to be

known as the 23 class.  Progress on these ships, although slow, was

more rapid than had been the case with No. 9, and by the end of 1917

three were completed and a fourth was rapidly approaching that state.

The specification, always ambitious, laid down the following main

stipulations.

(1) The ship is to attain a speed of at least 55 miles per hour

for the main power of the engines.

(2) A minimum of 8 tons is to be available for disposable weights

when full.

(3) The ship must be capable of rising at an average rate of not

less than 1,000 feet per minute, through a height of 3,000

feet starting from nearly sea level.

As will be seen later this class of ship, although marking a certain

advance on No. 9 both as regards workmanship and design, proved on the

whole somewhat disappointing, and it became more evident every day that

we had allowed the Germans to obtain such a start in the race of

airship construction as we could ill afford to concede.

We may here state that all of the ships of this class which had been

ordered were not completed, the later numbers being modified into what

was known as the 23 X class; four in all of the 23 class were built, of

which two--Nos. 23 and 26--were built by Messrs. Vickers, Ltd., at

Barrow, No. 24 by Messrs. Wm. Beardmore and Co., at Glasgow, and No. 25

by Messrs. Armstrong, Whitworth and Co., at Selby, Yorkshire.

In many respects the closest similarity of design exists between No. 9

and No. 23, especially in the hull, but it will be of interest to

mention the salient differences between the two ships.

The length of the hull, which in No. 9 was 520 feet, was increased in

No. 23 to 535 feet, and the number of gasbags from seventeen to

eighteen.  This gave a total volume of 997,500 cubic feet compared with

890,000 cubic feet in No. 9, with a disposable lift under specification

conditions of 5.7 tons as opposed to 3.8 tons.

The longitudinal shape of No. 23 is a modified form of "Zahm" shape,

the radius of the bow portion being twice the diameter of the parallel

portion, while the stern radius is three times the same diameter.

In design the hull framework is almost a repetition of No. 9,

particularly in the parallel portion, the same longitudinal and

transverse frames dividing the hull into compartments, with tubes

completely encircling the section between each main transverse frame.

The system of wiring the hull is precisely the same in both the ships,

and nets are employed in the same way.

The triangular section of keel is adhered to, but its functions in No.

23 are somewhat different.  In No. 9 it was intended to be sufficiently

strong to support all the main vertical bending moments and shearing

forces, but in No. 23 it was primarily intended to support the

distributed weights of water ballast, petrol tanks, etc., between the

main transverse frames.  Unlike No. 9, the keel is attached to the main

transverse frames only. The cabin and wireless cabin are disposed in

the keel in the same manner, and it also furnishes a walking way for

the total length of the ship.

The stabilizing fins, both vertical and horizontal, are similar to

those attached to No. 9, but the system of rudders and elevators is

totally different. In place of the box rudders and elevators in No. 9,

single balanced rudders and elevators are attached to the fins; they

have their bearing on the outboard side on the external girders of the

fins, which are extended for the purpose.  The elevators and rudders

are composed of a duralumin framework, stiffened by a kingpost on

either side with bracing wires.

The bags, eighteen in number, are made of rubber-proofed fabric lined

with gold-beater's skin.  It is interesting to note that the number of

skins used for the bags of a ship of this class is approximately

350,000.  The system of valves is entirely different from that in No.

9. The Parseval type of valve with the pressure cone at the bottom of

the bag is omitted, and in the place of the two top valves in the

former ship are a side valve of the Zeppelin type entirely automatic

and a top valve entirely hand controlled.  The side valve is set to

blow off at a pressure of from 3 to 5 millimetres.  The outer cover was

fitted in the same manner as in No. 9.  Two cars or gondolas, one

forward, the other aft, each carry one engine provided with swivelling

propellers and gears.  They are enclosed with sides and a fireproof

roof, and are divided into two compartments, one the navigating

compartment, the other the engine room.  The cars are in all respects

very similar to those of No. 9, and are suspended from the hull in a

similar manner.  The remaining two engines are carried in a small

streamline car situated amidships, which has just sufficient room in it

for the mechanics to attend to them. Originally this car was open at

the top, but it was found that the engineers suffered from exposure,

and it was afterwards roofed in.

The engine arrangements in this ship were totally different to those of

No. 9, four 250 horse-power Rolls Royce engines being installed in the

following order.  Single engines are fitted in both the forward and

after cars, each driving two swivelling four-bladed propellers.  In the

centre car two similar engines are placed transversely, which drive

single fixed propellers mounted on steel tube outriggers through

suitable gearing.

The engines are the standard 12 cylinder V-type Rolls Royce which will

develop over 300 brake horse-power at full throttle opening.

The engine is water cooled, and in the case of those in the forward and

after cars the original system consisted of an internal radiator

supplied by an auxiliary water tank carried in the keel.  It was found

on the flight trials that the cooling was insufficient, and external

radiators were fitted, the internal radiator and fan being removed.  In

the case of the centre car no alteration was necessary, as external

radiators were fitted in the first instance.

The engines are supported by two steel tubes held by four brackets

bolted to the crank case, these being carried by twelve duralumin tubes

bolted to the bearers and transverse frames of the car respectively.

The drive from the engine is transmitted through a universal joint to a

short longitudinal shaft, running on ball bearings.  This shaft gears

into two transverse shafts, which drive the propellers through the

medium of a gear box to the propeller shafts, making five shafts in all.

The engines in the centre car being placed transversely the

transmission is more direct, the engines driving the propellers through

two gear wheels only.  The propeller gear box is supported by steel

tube outriggers attached by brackets to the framework of the car.  The

petrol is carried in a series of tanks situated beneath the keel

walking way, and are interconnected so that any tank either forward or

aft can supply any engine, by this means affording assistance for the

trimming of the ship.

Four-bladed propellers are used throughout the ship.

Water ballast is carried in fabric bags also situated beneath the keel

walking way, and a certain amount is also carried beneath the floor of

the car.

Engine-room telegraphs, swivelling propeller telegraphs, speaking tubes

and telephones, with a lighting set for the illumination of the cars

and keel, were all fitted in accordance with the practice standard in

all rigid airships.

The lift and trim trials taken before the initial flight trials showed

that the ship possessed a disposable lift under standard conditions of

5.7 tons.  The original disposable lift demanded by the specification

was 3 tons but this was reduced by 2 tons owing to the machinery

weights being 2 tons in excess of the estimate.  Since then these

weights had been increased by another half-ton, making a total of 2 1/2

tons over the original estimate.  It was evident that with so small a

margin of lift these ships would never be of real use, and it was

decided to remove various weights to increase the lift and to

substitute a wing car of a similar type to those manufactured for the R

33 class for the heavy after car at present in use.

R 23 carried out her trials without the alteration to the car, which

was effected at a later date, and the same procedure was adopted with R

24 and R 25.  In the case of R 26, however, she had not reached the

same stage of completion as the other two ships, and the alterations

proposed for them were embodied in her during construction.  The

gasbags were of lighter composition, all cabin furniture was omitted

and the wing car was fitted in place of the original after  car.  This

wing car is of streamline shape with a rounded bow and tapered stern.

The lower portion is plated with duralumin sheets and the upper part is

covered with canvas attached to light wooden battens to give the

necessary shape.  This effected a very considerable reduction in

weight. The original 250 horse-power Rolls Royce engine was installed,

now driving a single large two-bladed propeller astern.  A test having

been taken, it was found that the disposable lift under standard

conditions was 6.28 tons. It was therefore decided that all the ships

of the class should be modified to this design when circumstances

permitted.  Speed trials were carried out under various conditions of

running, when it was found that the ship possessed a speed of 54 1/4

miles per hour with the engines running full out.

To summarize the performances of these ships as we did in the case of

No. 9, we find:

Speed:

Full               54 miles per hour

Normal   =2/3      48   "  "  "

Cruising =1/3      33   "  "  "

Endurance:

Full       18 hours = 1,000 miles

Normal     26   "   = 1,250  "

Cruising   50   "   = 1,900  "

The production of the rigid airship during the war was always

surrounded with a cloak of impenetrable mystery.  Few people, except

those employed on their construction or who happened to live in the

immediate vicinity of where they were built, even knew of their

existence, and such ignorance prevailed concerning airships of every

description that the man in the street hailed a small non-rigid as "the

British Zeppelin" or admired the appearance of R 23 as "the Silver

Queen."  The authorities no doubt knew their own business in fostering

this ignorance, although for many reasons it was unfortunate that

public interest was not stimulated to a greater degree.  In the summer

months of 1918, however, they relented to a certain extent, and R 23

and one of her sister ships were permitted to make several flights over

London to the intense delight of thousands of its inhabitants, and a

certain amount of descriptive matter appeared in the Press.

From that time onwards these large airships have completely captured

the popular imagination, and many absurd rumours and exaggerations have

been circulated regarding their capabilities. It has been gravely

stated that these airships could accomplish the circuit of the globe

and perform other feats of the imagination. It must be confessed that

their merits do not warrant these extravagant assertions.  The fact

remains, however, that R 23 and her sister ship R 26 have each carried

out patrols of upwards of 40 hours duration and that, similarly to No.

9, they have proved of the greatest value for training airship crews

and providing experience and data for the building programme of the

future.  At the present time highly interesting experiments are being

carried out with them to determine the most efficient system of mooring

in the open, which will be discussed at some length in the chapter

dealing with the airship of the future.

RIGID AIRSHIP 23 X CLASS

During the early days of building the airships of the 23 class, further

information was obtained relating to rigid airship construction in

Germany, which caused our designers to modify their views.  It was

considered a wrong policy to continue the production of a fleet of

ships the design of which was becoming obsolete, and accordingly within

ten months of placing the order for this class a decision was reached

that the last four ships were to be altered to a modified design known

as the 23 X class. As was the case with the ships of the preceding

class when nearing completion, they were realized to be out of date,

and special efforts being required to complete the ships of the 33

class and to release building space for additional larger ships, the

construction of the second pair was abandoned.

The main modification in design was the abolition of the external keel,

and in this the later Zeppelin principles were adopted. This secured a

very considerable reduction in structural weight with a corresponding

large expansion of the effective capabilities of the ship.

It has been seen that the purpose of the keel in No. 9 was to provide a

structure sufficiently strong to support all the main vertical bending

moments and shearing forces, and that in No. 23 this principle was

somewhat different, in that the keel in this ship was primarily

intended to support the distributed weights of petrol, water, ballast,

etc., between the transverse frames.

In this later design, namely, the 23 X class, it was considered that

the weights could be concentrated and suspended from the radial wiring

of the transverse frames and that the keel, incorporated in the design

of the former ships, could be dispensed with.

For all practical purposes, apart from the absence of the keel, the 23

# X class of airship may be regarded as a slightly varied model of the 23

class.  The main dimensions are nearly the same, and the general

arrangement of the ship is but little changed. The loss of space owing

to the introduction of the internal corridor is compensated by a

modification of the shape of the bow, which was redesigned with a

deeper curve.  The hull structure was also strengthened by utilizing a

stronger type of girder wherever the greatest weights occur.  In these

strengthened transverse frames the girders, while still remaining of

the triangular section, familiar in the other ships, are placed the

opposite way round, that is, with the apex pointing outwards.

The walking way is situated at the base of the hull passing through the

gas chambers, which are specially shaped for the purpose.  The corridor

is formed of a light construction of hollow wooden struts and duralumin

arches covered with netting.

In all other leading features the design of the 23 class is adhered to;

the gasbags are the same, except for the alteration due to the internal

corridor, and the system of valves and the various controls are all

highly similar.

The arrangement of gondolas and the fitting of engines in all ways

corresponds to the original arrangement of R 23, with the exception

that they were suspended closer to the hull owing to the absence of the

external keel.  The substitution of the wing car of the 33 class for

the original after gondola,  carried out in the modifications undergone

by the ships of the 23 class, was not adopted in these ships, as the

wireless compartment installed in the keel in the former was fitted in

the after gondola in the latter.

The disposable lift of these ships under standard conditions is 7 1/2

tons, which shows considerable improvement on the ships of the former

classes.

Summarizing as before, the performances appear as under--

Speed:

Full            56 1/2 miles per hour

Normal          53       "  "  "

Cruising        45       "  "  "

Endurance:

Normal          19 hours = 1,015 miles

Cruising        23 1/2 " = 1,050   "

The two ships  of this class, which were commissioned, must be regarded

within certain limits as most satisfactory, and are the most successful

of those that appeared and were employed during the war.  Escort of

convoys and extended anti-submarine patrols were carried out, and

certain valuable experiments will be attempted now that peace has

arrived.

In spite of the grave misgivings of many critics, the structure without

the keel has proved amply strong, and no mishap attended this radical

departure on the part of the designers.

RIGID AIRSHIP No. 31 CLASS

The airship known as R 81 was a complete deviation from any rigid

airship previously built in this country.  In this case the experiment

was tried of constructing it in wood in accordance with the practice

adopted by the Schutte-Lanz Company in Germany.

It must be frankly acknowledged that this experiment resulted in

failure. The ship when completed showed great improvement both in

shape, speed and lifting capacity over any airship commissioned in this

country, and as a whole the workmanship exhibited in her construction

was exquisite.  Unfortunately, under the conditions to which it was

subjected, the hull structure did not prove durable, and to those

conditions the failure is attributed. Under different circumstances it

may be hoped that the second ship, when completed, will prove more

fortunate.

In length R 31 was 615 feet, with a diameter of 66 feet, and the

capacity was 1 1/2 million cubic feet.

In shape the hull was similar to the later types of Zeppelin, having a

rounded bow and a long, tapering stern. The longitudinal and transverse

frames were composed of girders built up of three-ply wood, the whole

structure being braced in the usual manner with wire bracings.  It had

been found in practice with rigid airships that, if for any reason one

gasbag becomes much less inflated than those adjacent to it, there is

considerable pressure having the effect of forcing the radial wires of

the transverse frames towards the empty bag.  The tension resulting in

these wires may produce very serious compressive strain in the members

of the transverse frames, and to counteract this action an axial wire

is led along the axis of the ship and secured to the centre point of

the radial wiring.  This method, now current practice in rigid airship

construction, was introduced for the first time in this ship.

As will be seen from the photograph, the control and navigating

compartment of the ship is contained in the hull, the cars in each case

being merely small engine rooms.  These small cars were beautifully

made of wood of a shape to afford the least resistance to the air, and

in number were five, each housing a single 250 horse-power Rolls Royce

engine driving a single fixed propeller.  Here we see another decided

departure from our previous methods of rigid airship construction, in

that for the first time swivelling propellers were abandoned.  R 31

when completed carried out her trials, and it was evident that she was

much faster than previous ships.  The trials were on the whole

satisfactory and, except for a few minor accidents to the hull

framework and fins, nothing untoward occurred.

At a later date the whole ship was through fortuitous circumstances

exposed to certain disadvantageous conditions which rendered her

incapable of further use.

R 33 CLASS

September 24th, 1916, is one of the most important days in the history

of rigid airship design in this country; on this date the German

Zeppelin airship L 33 was damaged by gunfire over London, and being hit

in the after gasbags attempted to return to Germany.  Owing to lack of

buoyancy she was forced to land at Little Wigborough, in Essex, where

the crew, having set fire to the ship, gave themselves up.  Although

practically the entire fabric of the ship was destroyed, the hull

structure most fortunately remained to all intents and purposes intact,

and was of inestimable value to the design staff of the Admiralty, who

measured up the whole ship and made working drawings of every part

available.

During this year other German rigid airships had been brought down,

namely L 15, which was destroyed at the mouth of the Thames in April,

but which was of an old type, and from which little useful information

was obtained; and also the Army airship L.Z. 85, which was destroyed at

Salonica in the month of May.  A Schutte-Lanz airship was also brought

down at Cuffley, on September 2nd, and afforded certain valuable

details.

All these ships were, however, becoming out of date; but L 33 was of

the latest design, familiarly called the super-Zeppelin, and had only

been completed about six weeks before she encountered disaster.

In view of the fact that the rigid airships building in this country at

this date, with the exception of the wooden Schutte-Lanz ships were all

based on pre-war designs of Zeppelin airships, it can be readily

understood that this latest capture revolutionized all previous ideas,

and to a greater extent than might be imagined, owing to the immense

advance, both in design and construction, which had taken place in

Germany since 1914.

All possible information having been obtained, both from the wreck of

the airship itself and from interrogation of the captured crew,

approval was obtained, in November of the same year, for two ships of

the L 33 design to be built; and in January, 1917, this number was

increased to five.

It was intended originally that these ships should be an exact

facsimile of L 33; but owing to the length of time occupied in

construction later information was obtained before they were completed,

both from ships of a more modern design, which were subsequently

brought down, and also from other sources.  Acting on this information,

various improvements were embodied in R 33 and R 34, which were in a

more advanced state; but in the case of the three other ships the size

was increased, and the ships, when completed, will bear resemblance to

a later type altogether.

As a comment on the slowness of construction before mentioned, the fact

that while we in this country were building two ships on two slips,

Germany had constructed no fewer than thirty on four slips, certainly

affords considerable food for reflection.

The two airships of this class having only just reached a state of

completion, a detailed description cannot be given without making

public much information which must necessarily remain secret for the

present.  Various descriptions have, however, been given in the daily

and weekly Press, but it is not intended in the present edition of this

book to attempt to elaborate on anything which has not been already

revealed through these channels.

It is regrettable that so much that would be of the utmost interest has

to be omitted; but the particulars which follow will at any rate give

sonic idea of the magnitude of the ship and show that it marks a

decided departure from previous experiments and a great advance on any

airship before constructed in Great Britain.

It is also a matter for regret that these two ships were not completed

before the termination of hostilities, as their capabilities would

appear to be sufficient to warrant the expectations which have been

based on their practical utility as scouting agents for the Grand Fleet.

In all its main features the hull structure of R 33 and R 34 follows

the design of the wrecked German Zeppelin airship L 33. The hull

follows more nearly a true streamline shape than in the previous ships

constructed of duralumin, in which a great proportion of the total

length was parallel-sided.  The Germans adopted this new shape from the

Schutte-Lanz design and have not departed from this practice.  This

consists of a short parallel body with a long rounded bow and a long

tapering stem culminating in a point.  The overall length of the ship

is 643 feet with a diameter of 79 feet and an extreme height of 92 feet.

The type of girders in this class has been much altered from those in

previous ships.  The hull is fitted with an internal triangular keel

throughout practically the entire length.  This forms the main corridor

of the ship, and is fitted with a footway down the centre for its

entire length.  It contains water ballast and petrol tanks, bomb

stowage and crew accommodation and the various control wires, petrol

pipes and electric leads are carried along the lower part.

Throughout this internal corridor runs a bridge girder, from which the

petrol and water ballast tanks are supported.  These tanks are so

arranged that they can be dropped clear of the ship.

Amidships is the cabin space with sufficient room for a crew of

twenty-five. Hammocks can be slung from the bridge girder before

mentioned.

In accordance with the latest Zeppelin practice, monoplane rudders and

elevators are fitted to the horizontal and vertical fins.

The ship is supported in the air by nineteen gasbags which give a total

capacity of approximately two million cubic feet of gas. The gross lift

works out at approximately 59 1/2 tons, of which the total fixed weight

is 33 tons, giving a disposable lift of 26 1/2 tons.

The arrangement of cars is as follows:  At the forward end the control

car is slung, which contains all navigating instruments and the various

controls.  Adjoining this is the wireless cabin, which is also fitted

for wireless telephony.  Immediately aft of this is the forward power

car containing one engine, which gives the appearance that the whole is

one large car.

Amidships are two wing cars each containing a single engine. These are

small and just accommodate the engine with sufficient room for

mechanics to attend to them.  Further aft is another larger car which

contains an auxiliary control position and two engines.

It will thus be seen that five engines are installed in the ship; these

are all of the same type and horse-power, namely, 250 horse-power

Sunbeam.  R 33 was constructed by Messrs. Armstrong Whitworth Ltd.,

while her sister ship R 34 was built by Messrs. Beardmore on the Clyde.

In the spring of 1918, R 33 and R 34 carried out several flight trials,

and though various difficulties were encountered both with the engines

and also with the elevator and rudder controls, it was evident that,

with these defects remedied, each of these ships would prove to be

singularly reliable.

On one of these trials made by R 34, exceedingly bad weather was

encountered, and the airship passed through several blinding

snowstorms; nevertheless the proposed flight of some seventeen hours

was completed, and though at times progress was practically nil owing

to the extreme force of the wind, the station was reached in safety and

the ship landed without any contretemps. This trial run having been

accomplished in weather such as would never have been chosen in the

earlier days of rigid trial flights, those connected with the airship

felt that their confidence in the vessel's capabilities was by no means

exaggerated.

The lift of the ship warranted a greater supply of petrol being carried

than there was accommodation for, and the engines by now had been

"tuned up" to a high standard of efficiency. Accordingly it was

considered that the ship possessed the necessary qualifications for a

transatlantic flight.  It was, moreover, the opinion of the leading

officers of the airship service that such an enterprise would be of

inestimable value to the airship itself, as demonstrating its utility

in the future for commercial purposes.

Efforts were made to obtain permission for the flight to be attempted,

and although at first the naval authorities were disinclined to risk

such a valuable ship on what appeared to be an adventure of doubtful

outcome, eventually all opposition was overcome and it was agreed that

for the purposes of this voyage the ship was to be taken over by the

Air Ministry from the Admiralty.

Work was started immediately to fit out the ship for a journey of this

description.  Extra petrol tanks were disposed in the hull structure to

enable a greater supply of fuel to be carried, a new and improved type

of outer cover was fitted, and by May 29th, R 34 was completed to the

satisfaction of the Admiralty and was accepted.  On the evening of the

same day she left for her station, East Fortune, on the Firth of Forth.

This short passage from the Clyde to the Forth was not devoid of

incident, as soon after leaving the ground a low-lying fog enveloped

the whole country and it was found impossible to land with any degree

of safety.  It having been resolved not to land until the fog lifted,

the airship cruised about the north-east coast of England and even came

as far south as York.  Returning to Scotland, she found the fog had

cleared, and was landed safely, having been in the air for 21 hours.

The original intention was that the Atlantic flight should be made at

the beginning of June, but the apparent unwillingness of the Germans to

sign the Peace Treaty caused the Admiralty to retain the ship for a

time and commission her on a war footing. During this period she went

for an extended cruise over Denmark, along the north coast of Germany

and over the Baltic.  This flight was accomplished in 56 hours, during

which extremely bad weather conditions were experienced at times.  On

its conclusion captain and crew of the ship expressed their opinion

that the crossing of the Atlantic was with ordinary luck a moral

certainty.  Peace having been signed, the ship was overhauled once more

and made ready for the flight, and the day selected some three weeks

before was July 2nd.

A selected party of air-service ratings, together with two officers,

were sent over to America to make all the necessary arrangements, and

the American authorities afforded every conceivable facility to render

the flight successful.

As there is no shed in America capable of housing a big rigid, there

was no alternative but to moor her out in the open, replenish supplies

of gas and fuel and make the return journey as quickly as possible.

On July 2nd, at 2.38 a.m. (British summer time), R 34 left the ground

at East Fortune, carrying a total number of 30 persons. The route

followed was a somewhat northerly one, the north coast of Ireland being

skirted and a more or less direct course was kept to Newfoundland.

From thence the south-east coast of Nova Scotia was followed and the

mainland was picked up near Cape Cod.

From Cape Cod the airship proceeded to Mineola, the landing place on

Long Island.  All went well until Newfoundland was reached. Over this

island fog was encountered, and later electrical storms became a

disturbing element when over Nova Scotia and the Bay of Fundy.  The

course had to be altered to avoid these storms, and owing to this the

petrol began to run short.  No anxiety was occasioned until on

Saturday, July 5th, a wireless signal was sent at 3.59 p.m. asking for

assistance, and destroyers were dispatched immediately to the scene.

Later messages were received indicating that the position was very

acute, as head winds were being encountered and petrol was running

short.  The airship, however, struggled on, and though at one time the

possibility of landing at Montauk, at the northern end of Long Island,

was considered, she managed after a night of considerable anxiety to

reach Mineola and land there in safety on July 6th at 9.55 a.m.

(British summer time).  The total duration of the outward voyage was

108 hours 12 minutes, and during this time some 3,136 sea miles were

covered.  R 34 remained at Mineola until midnight of July 9th according

to American time.  During the four days in which she was moored out

variable weather was experienced, and in a gale of wind the mooring

point was torn out, but fortunately, another trail rope was dropped and

made fast, and the airship did not break away.

It was intended that the return should be delayed until daylight, in

order that spectators in New York should obtain a good view of the

airship, but an approaching storm was reported and the preparations

were advanced for her immediate departure.  During the last half-hour

great difficulty was experienced in holding the ship while gassing was

completed.

At 5.57 a.m. (British summer time) R 34 set out on her return voyage,

steering for New York, to fly over the city before heading out into the

Atlantic.  She was picked up by the searchlights and was distinctly

visible to an enormous concourse of spectators.  During the early part

of the flight a strong following wind was of great assistance, and for

a short period an air speed of 83 miles per hour was attained.  On the

morning of July 11th the foremost of the two engines in the after car

broke down and was found to be beyond repair.  The remainder of the

voyage was accomplished without further incident.  On July 12th at

noon, a signal was sent telling R 34 to proceed to the airship station

at Pulham in Norfolk as the weather was unfavourable for landing in

Scotland.  On the same day at 8.25 p.m., land was first sighted and the

coast line was crossed near Clifden, county Galway, at 9 p.m.  On the

following morning, July 13th, at 7.57 a.m. (British summer time), the

long voyage was completed and R 34 was safely housed in the shed,

having been in the air 75 hours 3 minutes.

Thus a most remarkable undertaking was brought to a successful

conclusion.  The weather experienced was by no means abnormally good.

This was not an opportunity waited for for weeks and then hurriedly

snatched, but on the preordained date the flight was commenced.  The

airship enthusiast had always declared that the crossing of the

Atlantic presented no insuperable difficulty, and when the moment

arrived the sceptics found that he was correct. We may therefore assume

that this flight is a very important landmark in the history of aerial

transport, and has demonstrated that the airship is to be the medium

for long-distance travel. We may rest assured that such flights,

although creating universal wonder to-day, will of a surety be accepted

as everyday occurrences before the world is many years older.

# CHAPTER VIII

THE WORK OF THE AIRSHIP IN THE WORLD WAR

The outbreak of war found us, as we have seen, practically without

airships of any military value.  For this unfortunate circumstance

there were many contributory causes.  The development of aeronautics

generally in this country was behind that of the Continent, and the

airship had suffered to a greater extent than either the seaplane or

the aeroplane.  Our attitude in fact towards the air had not altered so

very greatly from that of the man who remarked, on reading in his paper

that some pioneer of aviation had met with destruction, "If we had been

meant to fly, God would have given us wings."  Absurd as this sounds

nowadays, it was the opinion of most people in this country, with the

exception of a few enthusiasts, until only a few years before we were

plunged into war.

The year 1909 saw the vindication of the enthusiasts, for in this

summer Bleriot crossed the Channel in an aeroplane, and the first

passenger-carrying Zeppelin airship was completed.  Those who had

previously scoffed came to the conclusion that flying was not only

possible but an accomplished fact, and the next two years with their

great aerial cross-country circuits revealed the vast potentialities of

aircraft in assisting in military operations.  We, therefore, began to

study aeronautics as the science of the future, and aircraft as an

adjunct to the sea and land forces of the empire.

The airship, unfortunately, suffered for many reasons from the lack of

encouragement afforded generally to the development of aeronautics.

The airship undoubtedly is expensive, and one airship of size costs

more to build than many aeroplanes.  In addition, everything connected

with the airship is a source of considerable outlay.  The shed to house

an airship is a most costly undertaking, and takes time and an

expenditure of material to erect, and bears no comparison with the

cheap hangar which can be run up in a moment to accommodate the

aeroplane.  The gas to lift the airship is by no means a cheap

commodity.  If it is to be made on the station where the airship is

based, it necessitates the provision of an expensive and elaborate

plant. If, on the other hand, it is to be manufactured at a factory,

the question of transport comes in, which is a further source of

expense with costly hydrogen tubes for its conveyance.

Another drawback is the large tract of ground required for an

aerodrome, and the big airship needs a large number of highly-trained

personnel to handle it.

A further point always, raised when the policy of developing the

airship was mooted is its vulnerability.  It cannot be denied that it

presents a large target to artillery or to the aeroplane attacking it,

and owing to the highly inflammable nature of hydrogen when mixed with

air there can be no escape if the gas containers are pierced by

incendiary bullets or shells.

Another contributing factor to the slow development of the airship was

the lack of private enterprise.  Rivalry existed between private firms

for aeroplane contracts which consequently produced improvements in

design; airships could not be produced in this way owing to the high

initial cost, and if the resulting ships ended in failure, as many were

bound to do, there would be no return for a large outlay of capital.

The only way by which private firms could be encouraged to embark on

airship building was by subsidies from the Government, and at this time

the prevalent idea of the doubtful value of the airship was too strong

for money to be voted for this purpose.

To strengthen this argument no demand had either been made from those

in command of the Fleet or from commanders of our Armies for airships

to act as auxiliaries to our forces.

The disasters experienced by all early airships and most particularly

by the Zeppelins were always seized upon by those who desired to

convince the country what unstable craft they were, and however safe in

the air they might be were always liable to be wrecked when landing in

anything but fine weather. Those who might have sunk their money in

airship building thereupon patted themselves upon the back and rejoiced

that they had been so far-seeing as to avoid being engaged upon such a

profitless industry.

Finally, all in authority were agreed to adopt the policy of letting

other countries buy their experience and to profit from it at a later

date.  Had the war been postponed for another twenty years all might

have been well, and we should have reaped the benefit, but most

calamitously for ourselves it arrived when we were utterly unprepared,

and having, as we repeat, only three airships of any military value.

With these three ships, Astra-Torres (No. 3), Parseval (No. 4) and

Beta, the Navy did all that was possible.  At the very outbreak of war

scouting trips were made out into the North Sea beyond the mouth of the

Thames by the Astra and Parseval, and both these ships patrolled the

Channel during the passage of the Expeditionary Force.

The Astra was also employed off the Belgian coast to assist the naval

landing party at Ostend, and together with the Parseval assisted in

patrolling the Channel during the first winter of the war.

The Beta was also sent over to Dunkirk to assist in spotting for

artillery fire and locating German batteries on the Belgian coast.  Our

airships were also employed for aerial inspection of London and other

large towns by night to examine the effects of lighting restrictions

and obtain information for our anti-aircraft batteries.

With the single exception of the S.S. ship, which carried out certain

manoeuvres in France in the summer of 1916, our airships were confined

to operations over the sea; but if we had possessed ships of greater

reliability in the early days of the war, it is conceivable that they

would have been of value for certain purposes to the Army.  The Germans

employed their Zeppelins at the bombardment of Antwerp, Warsaw, Nancy

and Libau, and their raids on England are too well remembered to need

description. The French also used airships for the observation of

troops mobilizing and for the destruction of railway depots.  The

Italians relied entirely at the beginning of the war on airships,

constructed to fly at great heights, for the bombing of Austrian troops

and territory, and met with a considerable measure of success.

When it was decided, early in 1915, to develop the airship for

anti-submarine work difficulties which appeared almost insuperable were

encountered at first.  To begin with, there were practically no firms

in the country capable of airship production.  The construction of

envelopes was a great problem; as rubber-proofed fabric had been found

by experiment to yield the best results for the holding of gas, various

waterproofing firms were invited to make envelopes, and by

whole-hearted efforts and untiring industry they at last provided very

excellent samples. Fins, rudder planes, and cars were also entrusted to

firms which had had no previous experience of this class of work, and

it is rather curious to reflect that envelopes were produced by the

makers of mackintoshes and that cars and planes were constructed by a

shop-window furnisher.  This was a sure sign that all classes of the

community were pulling together for the good of the common cause.

Among other difficulties was the shortage of hydrogen tubes, plants,

and the silicol for making gas.

Sufficient sheds and aerodromes were also lacking, and the airships

themselves were completed more quickly than the sheds which were to

house them.

The lack of airship personnel to meet the expansion of the service

presented a further obstacle.  To overcome this the system of direct

entry into the R.N.A.S. was instituted, which enabled pilots to be

enrolled from civil life in addition to the midshipmen who were drafted

from the Fleet.  The majority of the ratings were recruited from civil

life and given instruction in rigging and aero-engines as quickly as

possible, while technical officers were nearly all civilians and

granted commissions in the R.N.V.R.

A tremendous drawback was the absence of rigid airships and the lack of

duralumin with which to construct them.

Few men were also experienced in airship work at this time, and there

was no central airship training establishment as was afterwards

instituted.  Pilots were instructed as occasion permitted at the

various patrol stations, having passed a balloon course and undergone a

rudimentary training at various places.

To conclude, the greatest of all difficulties was the shortage of money

voted for airship development, and this was a disadvantage under which

airships laboured even until the conclusion of hostilities.

We have seen previously how the other difficulties were surmounted and

how our airships were evolved, type by type, and the measure of success

which attended them.  It is interesting to recall that five years ago

we only possessed three ships capable of flying, and that during the

war we built upwards of two hundred, of which no fewer than 103 were

actually in commission on the date of the signing of the Armistice.

The work carried out by our airships during the war falls under three

main headings:

1. Operations with the fleet or with various units.

2. Anti-submarine patrol and searching for mines.

3. Escort of shipping and examination duties.

With regard to the first heading it is only permissible at present to

say very little; certain manoeuvres were carried out in connection with

the fleet, but the slow development of our rigid airships prohibited

anything on a large scale being attempted.  The Germans, on the other

hand, made the fullest use of their Zeppelins for scouting purposes

with the high seas fleet.  Responsible people were guilty of a grave

mistake when speaking in public in denouncing the Zeppelin as a useless

monster every time one was destroyed in a raid on this country. The

main function of the Zeppelin airship was to act as an aerial scout,

and it carried out these duties with the utmost efficiency during the

war.  It is acknowledged that the German fleet owed its escape after

the Battle of Jutland to the information received from their airships,

while again the Zeppelin was instrumental in effecting the escape of

the flotilla which bombarded Scarborough in 1916.

Very probably, also, the large airship was responsible for the success

which attended the U boats during their attack on the cruisers

Nottingham and Falmouth, and also at the Hogue disaster.

Various experiments were carried out in towing airships by cruisers, in

refuelling while in tow and changing crews, all of which would have

borne good fruit had the war lasted longer.

An exceedingly interesting experiment was carried out during the

closing stages of the war by an airship of the S.S. Zero type. At this

period the German submarines were gradually extending their operations

at a greater distance from our coasts, and the authorities became

concerned at the prospect that the small type of airship would not

possess sufficient endurance to carry out patrol over these increased

distances.  The possibility was considered of carrying a small airship

on board a ship which should carry out patrol and return to the ship

for refuelling purposes, to replenish gas, and change her crew.  To

test the feasibility of this idea S.S. Z 57 carried out landing

experiments on the deck of H.M.S. Furious, which had been adapted as an

aeroplane carrier.  S.S. Z 57 came over the deck and dropped her trail

rope, which was passed through a block secured to the deck, and was

hauled down without difficulty.  These experiments were continued while

the ship was under weigh and were highly successful.  No great

difficulty was encountered in making fast the trail rope, and the

airship proved quite easy to handle.  The car was also lowered into the

hangar below the upper deck, the envelope only remaining on the upper

level, and everything worked smoothly.  If the war had continued there

is no doubt that some attempt would have been made to test the

practical efficiency of the problem.

Anti-submarine patrol was the chief work of the airship during the war,

and, like everything else, underwent most striking changes.  Submarine

hunting probably had more clever brains concentrated upon it than

anything else in the war, and the part allotted to the airship in

conjunction with the hunting flotillas of surface craft was carefully

thought out.

In the case of a suspected submarine in a certain spot, all surface and

air craft were concentrated by means of wireless signals at the

appointed rendezvous.  It is in operations of this kind that the

airship is so superior to the seaplane or aeroplane, as she can hover

over a fixed point for an indefinite period with engines shut off.  If

the submarine was located from the air, signals were given and depth

charges dropped in the position pointed out.  Incidents of this kind

were of frequent occurrence, and in them the value of the airship was

fully recognized.

The most monotonous and arduous of the airship's duties was the routine

patrol.  The ship would leave her shed before dawn and be at the

appointed place many miles away from land.  She then would carry out

patrol, closely scanning the sea all round, and investigating any

suspicious object.  For hours this might last with nothing seen, and

then in the gathering darkness the ship would make her way home often

against a rising wind, and in the winter through hail and snow.  Bombs

were always carried, and on many occasions direct hits were observed on

enemy submarines.  A sharp look-out was always kept for mines, and many

were destroyed, either by gunfire from the airship herself or through

the agency of patrol boats in the vicinity.  This was the chief work of

the S.S. ships, and was brought to a high pitch of perfection by the

S.S. Zero.  These ships proved so handy that they could circle round an

object without ever losing sight of it, and yet could be taken in and

out of sheds in weather too bad to handle bigger ships.

The hunting of the submarine has been likened to big-game hunting, and

certainly no one ever set out to destroy a bigger quarry. It needs the

same amount of patience and the same vigilance.  Days may pass without

the opportunity, and that will only be a fleeting one: the

psychological moment must be seized and it will not brook a moment's

delay.  The eye must be trained to pick up the minutest detail, and

must be capable of doing this for hour after hour.  For those on

submarine patrol in a small ship there is not one second's rest.  As is

well known, the submarine campaign reached its climax in April, 1917.

In that month British and Allied shipping sustained its greatest

losses. The value of the airship in combating this menace was now fully

recognized, and with the big building programme of Zero airships

approved, the housing accommodation again reached an acute stage.

Shortage of steel and timber for shed building, and the lack of labour

to erect these materials had they been available, rendered other

methods necessary.  It was resolved to try the experiment of mooring

airships in clearings cut into belts of trees or small woods.

A suitable site was selected and the trees were felled by service

labour.  The ships were then taken into the gaps thus formed and were

moored by steel wires to the adjacent trees.  Screens of brushwood were

then built up between the trees, and the whole scheme proved so

successful that even in winter, when the trees were stripped of their

foliage, airships rode out gales of over 60 miles per hour.  The

personnel were housed either in tents or billeted in cottages or houses

in the neighbourhood, and gas was supplied in tubes as in the earlier

days of the stations before the gas plants had been erected.

This method having succeeded beyond the most sanguine expectations,

every station had one or more of these sub-stations based on it, the

airships allocated to them making a periodical visit to the parent

station for overhaul as required. Engineering repairs were effected by

workshop lorries, provided that extensive work was not required.

In this way a large fleet of small airships was maintained around our

coasts, leaving the bigger types of ships on the parent stations, and

the operations were enabled to be considerably extended.  Of course,

certain ships were wrecked when gales of unprecedented violence sprung

up; but the output of envelopes, planes and cars was by this time so

good that a ship could be replaced at a few hours' notice, and the cost

compared with building of additional sheds was so small as to be

negligible.

From the month of April, 1917, the convoy system was introduced, by

which all ships on entering the danger zones were collected at an

appointed rendezvous and escorted by destroyers and patrolboats.  The

airship was singularly suitable to assist in these duties.  Owing to

her power of reducing her speed to whatever was required, she could

keep her station ahead or abeam of the convoy as was necessary, and

from her altitude was able to exercise an outlook for a far greater

distance than was possible from the bridge of a destroyer.  She could

also sweep the surface ahead of the approaching convoy, and warn it by

wireless or by flash-lamp of the presence of submarines or mines.  By

these timely warnings many vessels were saved.  Owing to the position

of the stations it was possible for a convoy to be met by airships west

of the Scilly Isles and escorted by the airships of the succeeding

stations right up the Channel.  In a similar manner, the main shipping

routes on the east coast and also in the Irish Sea were under constant

observation.  The mail steamers between England and Ireland and

transports between England and France were always escorted whenever

flying conditions were possible.  For escort duties involving long

hours of flying, the Coastal and C Star types were peculiarly suitable,

and at a later date the North Sea, which could accompany a convoy for

the length of Scotland.  Airships have often proved of value in

summoning help to torpedoed vessels, and on occasions survivors in open

boats have been rescued through the agency of patrolling airships.

Examination duties are reckoned among the many obligations of the

airship.  Suspicious-looking vessels were always carefully scrutinized,

and if unable to give a satisfactory answer to signals made, were

reported to vessels of the auxiliary patrol for closer examination.

Isolated fishing vessels always were kept under close observation, for

one of the many ruses of the submarine was to adopt the disguise of a

harmless fishing boat with masts and sails.

The large transports, conveying American troops who passed through

England on their way to France, were always provided with escorting

airships whenever possible, and their officers have extolled their

merits in most laudatory terms.

Our rigid airships also contributed their share in convoy work,

although their appearance as active units was delayed owing to slowness

in construction.

A disturbing feature to the advocate of the large airship, has been the

destruction of raiding Zeppelins by heavier-than-air machines, and the

Jeremiahs have not lost this opportunity of declaring that for war

purposes the huge rigid is now useless and will always be at the

complete mercy of the fast scouting aeroplane.  There is never any

obstacle in this world that cannot be surmounted by some means or

other.  On the one hand there is helium, a non-inflammable gas which

would render airships almost immune to such attacks.  On the other

hand, one opinion of thought is that the rigid airship in the future

will proceed to sea escorted by a squadron of scouting aeroplanes for

its defence, in the same way that the capital ship is escorted at sea

by destroyers and torpedo boats.  This latter idea has been even

further developed by those who look into the future, and have conceived

the possibility of a gigantic airship carrying its own aeroplanes for

its protection.

To test the possibility of this innovation, a small aeroplane was

attached to one of our rigid airships beneath the keel. Attachments

were made to the top of the wings and were carried to the main

framework of the hull.  The release gear was tested on the ground to

preclude the possibility of any accident, and on the day appointed the

airship was got ready for flight.  While the airship was flying, the

pilot of the aeroplane was in his position with his engine just ticking

over.  The bows of the airship were then inclined upwards and the

release gear was put into operation.  The pilot afterwards said that he

had no notion that anything had been done until he noticed that the

airship was some considerable height above him. The machine made a

circuit of the aerodrome and landed in perfect safety, while no trouble

was experienced in any way in the airship.  Whether this satisfactory

experiment will have any practical outcome the future alone can say,

but this achievement would have been considered, beyond all the

possibilities of attainment only a few years ago.

Since the Armistice several notable endurance flights were accomplished

by ships of the North Sea class, several voyages being made to the

coast of Norway, and quite recently a trip was carried out all round

the North Sea.

The weather has ceased to be the deterrent of the early days. Many will

no doubt remember seeing the North Sea airship over London on a day of

squalls and snow showers, and R 34 encountered heavy snow storms on the

occasion of one of her flight trials, which goes to prove that the

airship is scarcely the fair-weather aircraft as maintained by her

opponents.

Throughout the war our airships flew for approximately 89,000 hours and

covered a distance of upwards of two and a quarter million miles.  The

Germans attempted to win the war by the wholesale sinking of our

merchant shipping, bringing supplies and food to these islands, and by

torpedoing our transports and ships carrying guns and munitions of war.

They were, perhaps, nearer to success than we thought at the time, but

we were saved by the defeat of the submarine.  In the victory won over

the underseas craft the airship certainly played a prominent part and

we, who never suffered the pinch of hunger, should gratefully remember

those who never lost heart, but in spite of all difficulties and

discouragement, designed, built, maintained and flew our fleet of

airships.

# CHAPTER IX

THE FUTURE OF AIRSHIPS

With the signing of the Armistice on November 11th, 1918, the airship's

work in the war was practically completed and peace reigned on the

stations which for so many months had been centres of feverish

activity.  The enemy submarines were withdrawn from our shipping routes

and merchant ships could traverse the sea in safety except for the

occasional danger of drifting mines. "What is to be the future of the

airship?" is the question which is agitating the minds of innumerable

people at the present moment.

During the war we have built the largest fleet of airships in the

world, in non-rigids we have reached a stage in design which is

unsurpassed by any country, and in rigid airships we are second only to

the Germans, who have declared that, with the signing of the peace

terms, their aircraft industry will be destroyed.  Such is our position

at the present moment, a position almost incredible if we look back to

the closing days of the year 1914. Are we now to allow ourselves to

drift gradually back to our old policy of supineness and negligence as

existed before the war? Surely such a thought is inconceivable; as we

have organized our airship production for the purposes of war, so shall

we have to redouble our efforts for its development in peace, if we

intend to maintain our supremacy in the air.

Unless all war is from henceforth to cease, a most improbable

supposition when the violence of human nature is considered, aircraft

will be in the future almost the most important arm. Owing to its

speed, there will not be that period of waiting for the concentration

and marching of the armies of the past, but the nation resolved on war

will be able to strike its blow, and that a very powerful and terrible

one, within a few hours of the rupture of negotiations.  Every nation

to be prepared to counter such a blow must be possessed of adequate

resources, and unless the enormous expense is incurred of maintaining

in peace a huge establishment of aircraft and personnel, other methods

must be adopted of possessing both of these available for war while

employed in peace for other purposes.

From the war two new methods of transportation have emerged--the

aeroplane and the airship.  To the business man neither of these is at

the present juncture likely to commend itself on the basis of cost per

ton mile.  When, however, it is considered that the aeroplane is faster

than the express train and the airship's speed is double that of the

fastest merchant ship, it will be appreciated that for certain

commercial purposes both these mediums for transport have their

possibilities.  The future may prove that in the time to come both the

airship and the aeroplane will become self-supporting, but for the

present, if assisted by the Government, a fair return may be given for

the capital laid out, and a large fleet of aircraft together with the

necessary personnel will always be available for military purposes

should the emergency arise.  The present war has shown that the

merchant service provided a valuable addition both of highly-trained

personnel and of vessels readily adapted for war purposes, and it

appears that a similar organization can be effected to reinforce our

aerial navies in future times of danger.

In discussions relative to the commercial possibilities of aircraft, a

heated controversy always rages between advocates of the airship and

those of the heavier-than-air machine, but into this it is not proposed

to plunge the reader of this volume.  The aeroplane is eminently

adapted for certain purposes, and the greatest bigot in favour of the

airship can hardly dispute the claims of this machine to remain

predominant for short-distance travel, where high speed is essential

and the load to be carried is light.  For long distance voyages over

the oceans or broken or unpopulated country, where large loads are to

be carried, the airship should be found to be the more suitable.

The demand for airships for commercial purposes falls under three main

headings, which will be considered in some detail.  It will be shown to

what extent the present types will fill this demand, and how they can

be developed in the future to render the proposed  undertakings

successful.

1. Pleasure.

2. A quick and safe means of transport for passengers.

3. A quick commercial service for delivering goods of reasonable

weight from one country to another.

1. Pleasure.--In the past, men have kept mechanically-driven means of

transport such as yachts, motor cars, and motor boats for their

amusement, and to a limited extent have taken recreation in the air by

means of balloons.  For short cruises about this country and round the

coast a small airship, somewhat similar to the S.S. Zero, would be an

ideal craft.  In cost it would be considerably less than a small yacht,

and as it would only be required in the summer months, it would be

inflated and moored out in the open in a park or grounds and the

expense of providing a shed need not be incurred.  For longer

distances, a ship of 150,000 cubic feet capacity, with a covered-in car

and driven by two engines, would have an endurance of 25 hours at a

cruising speed of 45 miles per hour.  With such a ship voyages could

easily be made from the south coast to the Riviera or Spain, and

mooring out would still be possible under the lee of a small wood or to

a buoy on the water.

Possibilities also exist for an enterprising firm to start a series of

short pleasure trips at various fashionable seaside resorts, and until

the novelty had worn off the demand for such excursions will probably

be far in excess of the supply.

2. Passenger transport.--In the re-organization of the world after this

devastating war the business man's time will be of even more value than

it was before.  This country is largely bound up with the United States

of America in business interests which necessitate continual visits

between the two countries. The time occupied by steamer in completing

this journey is at present about five days.  If this time can be cut

down to two and a half days, no doubt a large number of passengers will

be only too anxious to avail themselves of this means of travel,

providing that it will be accomplished in reasonable safety and

comfort. The requirements for this purpose are an aerial liner capable

of carrying a hundred passengers with a certain quantity of luggage and

sufficient provisions for a voyage which may be extended over the

specified time owing to weather conditions. The transatlantic service

if successful could then be extended until regular passenger routes are

established encircling the globe.

3. Quick commercial service for certain types of goods.--Certain mails

and parcels are largely enhanced in value by the rapidity of transport,

and here, as in the passenger service outlined above, the airship

offers undoubted facilities.  As we have said before, it is mainly over

long distances that the airship will score, and for long distances on

the amount carried the success of the enterprise will be secured.  For

this purpose the rigid airship will be essential.  There are certain

instances in which the non-rigid may possibly be profitably utilized,

and one such is suggested by a mail service between this country and

Scandinavia.  A service is feasible between Newcastle and Norway by

airships of a capacity of the S.S. Twin type.  These ships would carry

700 lb. of mails each trip at about 4d. per ounce, which would reduce

the time of delivering letters from about two and a half to three days

to twenty-four hours.

A commercial airship company is regarded in this country as a new and

highly hazardous undertaking, and it seems to be somewhat overlooked

that it is not quite the novel idea so many people imagine.  Before the

war, in the years 1910 to 1914, the Deutsche Luftfahrt Actien

Gesellschaft successfully ran a commercial Zeppelin service in which

four airships were used, namely, Schwaben, Victoria Luise, Hansa and

Sachsan.  During this period over 17,000 passengers were carried a

total distance of over 100,000 miles without incurring a single fatal

accident. Numerous English people made trips in these airships,

including Viscount Jellicoe, but the success of the company has

apparently been forgotten.

We have endeavoured to show that the non-rigid airship has

potentialities even for commercial purposes, but there is no doubt

whatever that the future of the airship in the commercial world rests

entirely with the rigid type, and the airships of this type moreover

must be of infinitely greater capacity than those at present in

existence, if a return is to be expected for the capital invested in

them.  General Sykes stated, in the paper which he read before the

London Chamber of Commerce, "that for commercial purposes the airship

is eminently adapted for long-distance journeys involving non-stop

flights.  It has this inherent advantage over the aeroplane, that while

there appears to be a limit to the range of the aeroplane as at present

constructed, there is practically no limit whatever to that of the

airship, as this can be overcome by merely increasing the size.  It

thus appears that for such journeys as crossing the Atlantic, or

crossing the Pacific from the west coast of America to Australia or

Japan, the airship will be peculiarly suitable."

He also remarked that, "it having been conceded that the scope of the

airship is long-distance travel, the only type which need be considered

for this purpose is the rigid.  The rigid airship is still in an

embryonic state, but sufficient has already been accomplished in this

country, and more particularly in Germany, to show that with increased

capacity there is no reason why, within a few years' time, airships

should not be built capable of completing the circuit of the globe and

of conveying sufficient passengers and merchandise to render such an

undertaking a paying proposition."

The report of the Civil Aerial Transport Committee also states that,

"airships are the most suitable aircraft for the carrying of passengers

where safety, comfort and reliability are essential."

When we consider the rapid development of the rigid airship since 1914,

it should not be insuperable to construct an airship with the

capabilities suggested by General Sykes.  In 1914, the average

endurance of the Zeppelin at cruising speed was under one day and the

maximum full speed about 50 miles per hour.  In 1918, the German L 70,

which is of 2,195,000 cubic feet capacity, the endurance at 45 miles

per hour has risen to 7.4 days and the maximum full speed to 77 miles

per hour.  The "ceiling" has correspondingly increased from 6,000 feet

to 23,000 feet.

The British R 38 class, at present building, with a capacity of

approximately 2 3/4 million cubic feet has an estimated endurance at 45

miles per hour of 211 hours or 8.8 days, which is 34 hours greater than

the German L 70 class.  It is evident that for a ship of this calibre

the crossing of the Atlantic will possess no difficulty, and as an

instance of what has already been accomplished in the way of a

long-distance flight the exploit of a Zeppelin airship based in

Bulgaria during the war is sufficiently remarkable.  This airship in

the autumn of 1917 left the station at Jamboli to carry twelve tons of

ammunition for the relief of a force operating in German East Africa.

Having crossed the Mediterranean, she proceeded up the course of the

Nile until she had reached the upper waters of this river. Information

was then received by wireless of the surrender of the force, and that

its commander, Von Lettow, was a fugitive in the bush.  She thereupon

set out for home and reached her station in safety, having been in the

air 96 hours, or four days, without landing.

It is therefore patent that in R 33 and R 34 we possess two airships

which can cross to America to-morrow as far as actual distance is

concerned, but various other conditions are necessary before such

voyages can be undertaken with any prospects of commercial success.

The distance between England and America must be roughly taken as 3,000

miles.  It is not reasonable for airship stations to be situated either

in the inaccessible extreme west of Ireland or among the prevailing

fogs of Newfoundland.

Weather conditions must also be taken into account; head winds may

prevail, rendering the forward speed of the ship to be small even with

the engines running full out.  In calculations it is considered that

the following assumptions should be made:

1. At least 75 per cent additional petrol to be carried as would

be necessary for the passage in calm air, should unfavourable

weather conditions be met.  This amount could be reduced to 50

per cent in future airships with a speed of upwards of 80

miles per hour.

2. About a quarter of the total discharge able lift of the ship

should be in the form of merchandise or passengers to render

the project a reasonable commercial proposition.

We will consider the commercial loads that can be carried by the German

airship L 70 and our airships R 33 and R 38 under the conditions given

above.  Two speeds will be taken for the purposes of this comparison:

normal full speed, or about 60 miles per hour, and cruising speed of 45

miles per hour.

# L 70.--At 60 miles per hour a distance of 3,000 miles will be

accomplished in 50 hours.

Fuel consumption about

13 tons  + 9.75 tons (additional for safety)  = 22.75 tons.

Available lift for fuel and freight          = 27.8 tons.

Fuel carried                                 = 22.75 "

------------

Balance for freight                          =  5    "  about.

-----------

At 45 miles per hour, distance will be

accomplished in 66.6 hours.

Fuel consumption about

10 tons + 7.5 tons additional                = 17.5 tons.

Available lift                               = 27.8 tons

Fuel carried                                 = 17.5  "

------------

Balance for freight                          = 10  "  about.

------------

R. 33.--At 60 miles per hour.

Fuel consumption

14.25 tons + 10.68 tons additional           = 24.93 tons.

Lift available for fuel and freight          = 21.5  tons.

Fuel carried                                 = 24.93  "

------------

Minus balance                                = 3. 43  "

------------

At 45 miles per hour.

Fuel consumption

9.66 tons  + 7.23 tons                        (17 tons approx.)

Lift available for fuel and freight          = 21.5 tons.

Fuel carried                                 = 17    "

------------

Balance for freight                          = 4.5   "

------------

R. 38.-Estimated only.  At 60 miles per hour.

Fuel consumption

20 tons + 15 tons additional                 = 35 tons.

Lift available for fuel and freight          = 42 tons.

Fuel carried                                 = 35  "

------------

Balance for freight                          =  7  "

------------

At 45 miles per hour.

Fuel consumption 12 tons + 9 tons additional = 21 tons.

Lift available for fuel and freight          = 42  "

Fuel carried                                 = 21  "

------------

Balance for freight                          = 21  "

------------

It will thus be seen that at the faster speed small commercial loads

can be carried by L 70 and R 38 and not at all in the case of R 33,

that is assuming, of course, that the extra fuel is carried, of which

75 per cent of the total does not appear at all excessive in view of

the weather continually experienced over the Atlantic.

At the cruising speed the loads naturally increase but still, in L 70,

and more particularly in R 33, they are too small to be considered

commercially.  In R 38, however, the load that can be carried at

cruising speed is sufficient to become a commercial proposition.

From this short statement it is evident that, by a comparatively small

increase in volume, the lifting capacity of an airship is enormously

increased, and it is in this subject that the airship possesses such

undoubted advantage over the aeroplane.  In the heavier-than-air

machine there is no automatic improvement in efficiency resulting from

greater dimensions.  In the airship, however, this automatic

improvement takes place in a very marked degree; for example, an

airship of 10,000,000 cubic feet capacity has five times the lift of

the present 2,000,000 cubic feet capacity rigid, but the length of the

former is only 1.7 times greater, and therefore the weight of the

structure only five times greater (1.7); that is, the weight of the

structure is directly proportional to the total lift.  Having seen that

the total lift varies as the cube of the linear dimensions while air

resistance, B.H.P.--other things being equal--vary as the square of the

linear dimensions, it follows that the ratio "weight of machinery/total

lift" decreases automatically.

In comparing the different methods of transport for efficiency, the

resistance or thrust required is compared as a percentage of the total

weight.  The result obtained is known as the "co-efficient of tractive

resistance."  Experiments have shown that as the size of the airship

increases, the co-efficient of tractive resistance decreases to a

marked extent; with a proportionate increase in horse-power it is

proportionally more economical for a 10,000,000 cubic feet capacity

rigid to fly at 80 miles per hour than for a 2,000,000 cubic feet

capacity to fly at 60 miles per hour.

As the ratio "weight structure/total lift" is in airships fairly

constant, it follows that the ratio "disposable lift/total lift"

increases with the dimensions.

It is therefore obvious that increased benefits are obtained by

building airships of a larger size, and that the bigger the ship the

greater will be its efficiency, providing, of course, that it is kept

within such limits that it can be handled on the ground and manoeuvred

in the air.

The proportion of the useful lift in a large rigid, that is the lift

available for fuel, crew, passengers, and merchandise, is well over 50

per cent when compared with the gross lift. When the accompanying table

is studied it will be seen that with airships of large capacity the

available lift will be such that considerable weights of merchandise or

passengers can be carried.

Capacity in       Gross Lift    Length    Diameter

cubic feet        in tons      in feet    in feet

2,000,000            60.7         643       79

3,000,000            91.1         736       90.4

4,000,000           121.4         810       99.5

5,000,000           151.8         872      107.2

6,000,000           182.2         927      113.9

7,000,000           212.5         976      119.9

8,000,000           242.8       1,021      125.5

9,000,000           273.3       1,061      130.4

10,000,000           303.6       1,100      135.1

In airships of their present capacity, in order to obtain the greatest

amount of lift possible, lightness of construction has been of

paramount importance.  With this object in view duralumin has been

used, and complicated girders built up to obtain strength without

increase of weight.  In a large ship with a considerable gain in lift,

steel will probably be employed with a simpler form of girder work.  In

that way cheapness of construction will be effected together with

increased rapidity of output, and in addition the strength of the whole

structure should be increased.

The rigid airship of 10,000,000 cubic feet capacity will have a

disposable lift of over 200 tons available for fuel, crew, passengers,

and merchandise in such proportions as are desired. The endurance of

such a ship at a cruising speed of 45 miles per hour will be in the

neighbourhood of three weeks, with a maximum speed of 70 to 80 miles

per hour, and a "ceiling" of some 30,000 feet can be reached.  This

will give a range of over 20,000 miles, or very nearly a complete

circuit of the globe.

For commercial purposes the possibilities of such a craft are enormous,

and the uses to which it could be put are manifestly of great

importance.  Urgent mails and passengers could be transported from

England to America in under half the time at present taken by the

steamship routes, and any city in the world could be reached from

London in a fortnight.

In the event of war in the future, which may be waged with a nation

situated at a greater distance from this country than was Germany,

aircraft Of long endurance will be necessary both for scouting in

conjunction with our fleets and convoy duties.  The British Empire is

widely scattered, and large tracts of ocean lie between the various

colonies, all of which will require protection for the safe-guarding of

our merchant shipping.  The provision of a force of these large

airships will greatly add to the security of our out-lying dominions.

We have now reached a point where it is incumbent on us to face certain

difficulties which beset the airship of large dimensions, and which are

always magnified by its detractors.  Firstly, there is the expense of

sheds in which to house it; secondly, the large number of trained

personnel to assist in landing and handling it when on the ground;

thirdly, the risks attendant on the weather--for the airship is still

considered the general public as a fair-weather craft; and fourthly,

though this is principally in connection with its efficiency for

military purposes, its vulnerability.  We will deal with the four

difficulties enumerated under these headings seriatim, and endeavour to

show to what extent they may be surmounted if not entirely removed.

The solution of the first two problems may be summed up in two words:

"mooring out"; on the success of this it is considered that the whole

future of airships for commercial purposes rests. It will be essential

that in every country which the airship visits on its voyages, one

large central station is established for housing and repairs.  The

position of such a station is dependent on good weather conditions and

the best railway facilities possible.  In all respects this station

will be comparable to a dry dock for surface vessels.  The airship will

be taken into the shed for overhaul of hull structure, renewing of

gasbags or outer cover, and in short to undergo a periodical refit.

The cost of a shed capable of housing two rigid airships, even at the

present time, should not greatly exceed L500,000. This sum, though

considerable, is but a small item compared with the cost of

constructing docks to accommodate the Atlantic liner, and when once

completed the cost of maintenance is small when weighed against the

amount annually expended in dredging and making good the wear and tear

of a dock.

Apart from these occasional visits to a shed, the airship, in the

ordinary way at the end of a voyage, will pick up its moorings as does

the big steamer, and land its passengers and cargo, at the same time

replenishing its supplies of fuel, gas, provisions, etc., while minor

repairs to the machinery can be carried out as she rides in the air.

A completely satisfactory solution of the mooring problem for the rigid

airship has yet to reach its consummation.  We saw in the previous

# chapter how, in the case of small non-rigids, they were sheltered in

berths cut into woods or belts of trees, but for the rigid airship

something more secure and less at the mercy of the elements is required.

At the present moment three systems of mooring are in an experimental

stage: one, known as "the single-wire system," is now practically

acknowledged to fall short of perfection; the second, "the three-wire

system," and the third, "mooring to a mast," both have their champions,

but it is probable that the last will be the one finally chosen, and

when thoroughly tried out with its imperfections eliminated will

satisfy the most exacting critics.

The single-wire system is at the same time the simplest and most

obvious method which suggests itself, and means that the ship is

secured by a wire cable attached to a suitable point in the ship and

led to some fixed point on the ground.  It has been found that an

airship secured in this way requires constant attention, and that

steering is always necessary to render her steady in the air.

Considerable improvement is obtained if a dragging weight is added to

the wire, as it tends to check to a considerable extent lateral motion

of the bow of the ship.

The three-wire system is an adaptation and an improvement on the one

previously mentioned.  In this case the mooring point of the ship is

attached to three long wire cables, which, when raised in the air, form

a pyramid to the head of which the ship is attached.  These wires are

led to bollards which form in plan an equilateral triangle.  The lift

of the ship raises these wires off the ground, and if the ship is

trimmed up by the bows she will be found to resist the action of the

wind.  A rigid airship moored out by this method remained in the open

for a considerable time and rendered the future of this experiment most

hopeful.  It was resolved to continue these experiments by adding a

subsidiary system of wires with running blocks, the whole wiring to

form a polygon revolving round a fixed centre.  The disadvantages of

this method appear to be rather serious.  It seems that great

difficulty will always be found in picking up these moorings in a high

wind, and though this also applies to the method with the mast, the

initial obstacles do not appear to be so great.  A powerful engine

driving a winch will be necessary to raise these heavy wires from the

ground, although of course the lift of the airship will assist in this.

Secondly, the lowering of passengers and cargo will not be easy as the

ship will not be rigidly secured.  This, however, can probably be

managed when experiments have reached a further stage, and at present

the system may be said to present distinct possibilities.

The third system, that of mooring to a mast, possesses several features

peculiar to itself, and not embraced by the other two, which should

secure it prolonged investigations.  The system is by no means new and

has been tried from time to time for several years, but since the

question of mooring in the open has been so ventilated and is now

considered of such vital importance, these experiments have been

continued, and in less spasmodic fashion than in the past.  In a trial

with a small non-rigid airship some months ago a signal success was

achieved.  The ship remained attached to a mast in open country with no

protection whatsoever for six weeks in two of the worst months of the

year.  During this period two men only were required to look after the

ship, which experienced gales in which the force of the wind rose to 52

miles per hour, and not the slightest damage was sustained.

Two or three methods of attaching the airship to the mast have been

proposed, but the one which appears to be most practical is to attach

the extreme bow point of the ship to some form of cap, in which the

nose of the ship will fit, and will revolve round the top of the mast

in accordance with the direction of the wind.

For large airships, employed as passenger and commerce carriers, we can

imagine the mast advanced a stage further, and transformed into a tower

with a revolving head.  Incorporated in this tower will be a lift for

passengers and luggage, pipes also will be led to the summit through

which both gas and water can be pumped into the ship.  With the airship

rigidly held at the head of such a structure all the difficulties of

changing crews, embarking and disembarking passengers, shipping and

discharging cargo and also refuelling, vanish at once.  Assuming the

mooring problem solved with success, and we feel correct in this

assumption, the first two of our difficulties automatically disappear.

Sheds will only be necessary as repair depots and will not be

extensively required, all intermediate stopping places being provided

with masts and necessary arrangements for taking in gas, etc.  At these

intermediate stations the number of men employed will be comparatively

speaking few.  At the depots or repair stations the number must, of

course, be considerably increased, but the provision of an enormous

handling party will not be necessary. At present large numbers of men

are only required to take a large airship in or out of a shed when the

wind is blowing in a direction across the shed; when these conditions

prevail the airship will, unless compelled by accident or other

unforeseen circumstances, remain moored out in the open until the

direction of the wind has changed.

Mechanical traction will also help effectually in handling airships on

the ground, and the difficulty of taking them in and out of sheds has

always been unduly magnified.  The provision of track rails and

travellers to which the guys of the ship can be attached, as is the

practice in Germany, will tend to eliminate the source of trouble.

We must now consider the effect that weather will have on the big

airship.  In the past it has been a great handicap owing to the short

hours of endurance, with the resulting probability of the ship having

to land before the wind dropped and being wrecked in consequence.  Bad

weather will not endanger the big airship in flight, and its endurance

will be such that, should it encounter bad weather, it will be able to

wait for a lull to land. Meteorological forecasts have now reached a

high state of efficiency, and it should be possible for ample warnings

to be received of depressions to be met with during a voyage, and these

will be avoided by the airship flying round them.  In the northern

hemisphere, depressions generally travel from west to east and

invariably rotate in a counter-clockwise direction with the wind on the

south side blowing from the west and on the north side blowing from the

east.  Going west, the airship would fly to the north of a depression

to take advantage of the wind circulating round the edge, and going

east the southern course would be taken.

Lastly, the vulnerability of the airship must be taken into account.

Hydrogen is, as everyone knows, most highly inflammable when mixed with

air.  The public still feels uncomfortable misgivings at the close

proximity of an immense volume of gas to a number of running engines.

It may be said that the danger of disaster due to the gas catching fire

is for peace flying to all intents and   purposes negligible.  At the

risk of being thought hackneyed we must point out a fact which has

appeared in every discussion of the kind, namely, that British airships

flew during the war some 21 million miles, and there is only one case

of an airship catching fire in the air.  This was during a trial flight

in a purely experimental ship, and the cause which was afterwards

discovered has been completely eliminated.

For airships employed for military purposes this danger, due to the use

of incendiary bullets, rockets and various other munitions evolved for

their destruction, still exists.

Owing to its ceiling, rate of climb and speed, which we take to be from

70 to 80 miles per hour in the airship of the future, the airship may

be regarded as comparatively safe against attack from the ordinary type

of seaplane.  The chief danger to be apprehended is attack from small

scouting seaplanes, possessing great speed and the power to climb to a

great height, or from aeroplanes launched from the decks of ships.  If,

however, the airship is fitted to carry several small scout aeroplanes

of high efficiency in the manner described in the previous chapter, it

will probably be able to defend itself sufficiently to enable it to

climb to a great height and thus make good its escape.

The airship, moreover, will be more or less immune from such dangers if

the non-inflamable gas, known as "C" gas, becomes sufficiently cheap to

be used for inflating airships.  In the past the expense of this gas

has rendered its use absolutely prohibitive,  but it is believed that

it can be produced in the United States for such a figure as will make

it compare favourably with hydrogen.

The navigation of an airship during these long voyages proposed will

present no difficulty whatever.  The airship, as opposed to the

aeroplane, is reasonably steady in the air and the ordinary naval

instruments can be used.  In addition, "directional" wireless

telegraphy will prove of immense assistance. The method at present in

use is to call up simultaneously two land stations which, knowing their

own distance apart, and reading the direction of the call, plot a

triangle on a chart which fixes the position of the airship.  This

position is then transmitted by wireless to the airship.  In the future

the airship itself will carry its own directional apparatus, with which

it will be able to judge the direction of a call received from a single

land station and plot its own position on a chart.

We have so far confined our attention to the utilization of airships

for transport of passengers, mails and goods, but there appear to be

other fields of activity which can be exploited in times of peace.  The

photographic work carried out by aeroplanes during the war on the

western front and in Syria and Mesopotamia has shown the value of

aerial photography for map making and preliminary surveys of virgin

country.  Photography of broken country and vast tracks of forest can

be much more easily undertaken from an airship than an aeroplane, on

account of its power to hover for prolonged periods over any given area

and its greater powers of endurance.  For exploring the unmapped

regions of the Amazon or the upper reaches of the Chinese rivers the

airship offers unbounded facilities.  Another scope  suggested by the

above is searching for pearl-oyster beds, sunken treasure, and

assisting in salvage operations.  Owing to the clearness of the water

in tropical regions, objects can be located at a great depth when

viewed from the air, and it is imagined that an airship will be of

great assistance in searching for likely places.  Sponges and coral are

also obtained by diving, and here the airship's co-operation will be of

value.  Small ships such as the S.S. Zero would be ideal craft for

these and similar operations.

The mine patrol, as maintained by airships during the war, encourages

the opinion that a systematic search for icebergs in the northern

Atlantic might be carried out by airships during certain months of the

year.  As is well known, icebergs are a source of great danger to

shipping in these waters during the late spring and summer; if the

situation becomes bad the main shipping routes are altered and a

southerly course is taken which adds considerably to the length of the

voyage.  The proposal put forward is that during these months as

continuous a patrol as possible should be carried out over these waters.

The airship employed could be based in Newfoundland and the method of

working would be very similar to anti-submarine patrol.  Fixes could be

obtained from D.F. stations and warnings issued by wireless telegraphy.

Ice is chiefly found within five hundred miles of the coast of

Newfoundland, so that this work would come within the scope of the N.S.

airship.  The knowledge that reliable information concerning the presence

of ice will always be to hand would prove of inestimable value to the

captains of Atlantic liners, and would also relieve the shipping companies

and the public of great anxiety.

There are possibly many other uses to which airships can be put such as

the policing of wide stretches of desert country as in Arabia and the

Soudan.  The merits of all of these will doubtless be considered in due

course and there for the present we must leave them.

Finally, a few words must be written regarding the means to be adopted

in introducing the airship into the realms of commerce. As we said at

the beginning of the chapter it is not likely that the formation of a

company to exploit airships only will at the present moment appeal to

business men.  Airships are very costly and are still in their infancy,

which means that the premiums demanded for their insurance must of

necessity be enormous.  One suggestion is to place a reasonable scheme

before the great shipping companies in case they will care to find the

necessary capital and form subsidiary companies.

Another suggestion is that the Government should make arrangements to

subsidize commercial airships.  The subsidy might take the form of

insuring them.  If the burden of insurance is taken off their

shoulders, it is considered feasible to promote companies which will

give an adequate return for capital invested.  The Government could

also give a financial guarantee if mails are carried, in the same

manner as is done by shipping companies.

In return for this the Government could at the outbreak of hostilities

commandeer all or any of the airships for war purposes and so save the

number to be kept in commission.

By this means the Government will have a large number of highly-trained

and efficient personnel to call upon when the emergency arises, in the

same way as the fleet can call upon the R.N.R.  This system appears to

be the best in every respect, and it cannot be denied that in the long

run it would be the most economical for the country.

The airship has now arrived at the parting of the ways, and at this

point we must leave it.  The flying in war has been concluded, the

flying in peace has not yet commenced. It seems a far cry to the dark

days of 1914, when we only possessed two airships of utility, the one

manufactured in France, the other in Germany, while to-day we have

built the mighty airship which can fly to America and back.  We are now

at the dawn of a new period of reconstruction and progress, and during

this period many wonderful things will happen.  Not the least of these

will be the development of the airship.

*** END OF THE PROJECT GUTENBERG EBOOK BRITISH AIRSHIPS, PAST, PRESENT, AND FUTURE ***

Updated editions will replace the previous one—the old editions will

be renamed.

Creating the works from print editions not protected by U.S. copyright

law means that no one owns a United States copyright in these works,

so the Foundation (and you!) can copy and distribute it in the United

States without permission and without paying copyright

royalties. Special rules, set forth in the General Terms of Use part

of this license, apply to copying and distributing Project

Gutenberg™ electronic works to protect the PROJECT GUTENBERG™

concept and trademark. Project Gutenberg is a registered trademark,

and may not be used if you charge for an eBook, except by following

the terms of the trademark license, including paying royalties for use

of the Project Gutenberg trademark. If you do not charge anything for

copies of this eBook, complying with the trademark license is very

easy. You may use this eBook for nearly any purpose such as creation

of derivative works, reports, performances and research. Project

Gutenberg eBooks may be modified and printed and given away—you may

do practically ANYTHING in the United States with eBooks not protected

by U.S. copyright law. Redistribution is subject to the trademark

license, especially commercial redistribution.

START: FULL LICENSE

THE FULL PROJECT GUTENBERG LICENSE

PLEASE READ THIS BEFORE YOU DISTRIBUTE OR USE THIS WORK

To protect the Project Gutenberg™ mission of promoting the free

distribution of electronic works, by using or distributing this work

(or any other work associated in any way with the phrase “Project

Gutenberg”), you agree to comply with all the terms of the Full

Project Gutenberg™ License available with this file or online at

www.gutenberg.org/license.

Section 1. General Terms of Use and Redistributing Project Gutenberg™

electronic works

1.A. By reading or using any part of this Project Gutenberg™

electronic work, you indicate that you have read, understand, agree to

and accept all the terms of this license and intellectual property

(trademark/copyright) agreement. If you do not agree to abide by all

the terms of this agreement, you must cease using and return or

destroy all copies of Project Gutenberg™ electronic works in your

possession. If you paid a fee for obtaining a copy of or access to a

Project Gutenberg™ electronic work and you do not agree to be bound

by the terms of this agreement, you may obtain a refund from the person

or entity to whom you paid the fee as set forth in paragraph 1.E.8.

1.B. “Project Gutenberg” is a registered trademark. It may only be

used on or associated in any way with an electronic work by people who

agree to be bound by the terms of this agreement. There are a few

things that you can do with most Project Gutenberg™ electronic works

even without complying with the full terms of this agreement. See

paragraph 1.C below. There are a lot of things you can do with Project

Gutenberg™ electronic works if you follow the terms of this

agreement and help preserve free future access to Project Gutenberg™

electronic works. See paragraph 1.E below.

1.C. The Project Gutenberg Literary Archive Foundation (“the

Foundation” or PGLAF), owns a compilation copyright in the collection

of Project Gutenberg™ electronic works. Nearly all the individual

works in the collection are in the public domain in the United

States. If an individual work is unprotected by copyright law in the

United States and you are located in the United States, we do not

claim a right to prevent you from copying, distributing, performing,

displaying or creating derivative works based on the work as long as

all references to Project Gutenberg are removed. Of course, we hope

that you will support the Project Gutenberg™ mission of promoting

free access to electronic works by freely sharing Project Gutenberg™

works in compliance with the terms of this agreement for keeping the

Project Gutenberg™ name associated with the work. You can easily

comply with the terms of this agreement by keeping this work in the

same format with its attached full Project Gutenberg™ License when

you share it without charge with others.

1.D. The copyright laws of the place where you are located also govern

what you can do with this work. Copyright laws in most countries are

in a constant state of change. If you are outside the United States,

check the laws of your country in addition to the terms of this

agreement before downloading, copying, displaying, performing,

distributing or creating derivative works based on this work or any

other Project Gutenberg™ work. The Foundation makes no

representations concerning the copyright status of any work in any

country other than the United States.

1.E. Unless you have removed all references to Project Gutenberg:

1.E.1. The following sentence, with active links to, or other

immediate access to, the full Project Gutenberg™ License must appear

prominently whenever any copy of a Project Gutenberg™ work (any work

on which the phrase “Project Gutenberg” appears, or with which the

phrase “Project Gutenberg” is associated) is accessed, displayed,

performed, viewed, copied or distributed:

This eBook is for the use of anyone anywhere in the United States and most

other parts of the world at no cost and with almost no restrictions

whatsoever. You may copy it, give it away or re-use it under the terms

of the Project Gutenberg License included with this eBook or online

at www.gutenberg.org. If you

are not located in the United States, you will have to check the laws

of the country where you are located before using this eBook.

1.E.2. If an individual Project Gutenberg™ electronic work is

derived from texts not protected by U.S. copyright law (does not

contain a notice indicating that it is posted with permission of the

copyright holder), the work can be copied and distributed to anyone in

the United States without paying any fees or charges. If you are

redistributing or providing access to a work with the phrase “Project

Gutenberg” associated with or appearing on the work, you must comply

either with the requirements of paragraphs 1.E.1 through 1.E.7 or

obtain permission for the use of the work and the Project Gutenberg™

trademark as set forth in paragraphs 1.E.8 or 1.E.9.

1.E.3. If an individual Project Gutenberg™ electronic work is posted

with the permission of the copyright holder, your use and distribution

must comply with both paragraphs 1.E.1 through 1.E.7 and any

additional terms imposed by the copyright holder. Additional terms

will be linked to the Project Gutenberg™ License for all works

posted with the permission of the copyright holder found at the

beginning of this work.

1.E.4. Do not unlink or detach or remove the full Project Gutenberg™

License terms from this work, or any files containing a part of this

work or any other work associated with Project Gutenberg™.

1.E.5. Do not copy, display, perform, distribute or redistribute this

electronic work, or any part of this electronic work, without

prominently displaying the sentence set forth in paragraph 1.E.1 with

active links or immediate access to the full terms of the Project

Gutenberg™ License.

1.E.6. You may convert to and distribute this work in any binary,

compressed, marked up, nonproprietary or proprietary form, including

any word processing or hypertext form. However, if you provide access

to or distribute copies of a Project Gutenberg™ work in a format

other than “Plain Vanilla ASCII” or other format used in the official

version posted on the official Project Gutenberg™ website

(www.gutenberg.org), you must, at no additional cost, fee or expense

to the user, provide a copy, a means of exporting a copy, or a means

of obtaining a copy upon request, of the work in its original “Plain

Vanilla ASCII” or other form. Any alternate format must include the

full Project Gutenberg™ License as specified in paragraph 1.E.1.

1.E.7. Do not charge a fee for access to, viewing, displaying,

performing, copying or distributing any Project Gutenberg™ works

unless you comply with paragraph 1.E.8 or 1.E.9.

1.E.8. You may charge a reasonable fee for copies of or providing

access to or distributing Project Gutenberg™ electronic works

provided that:

• You pay a royalty fee of 20% of the gross profits you derive from

the use of Project Gutenberg™ works calculated using the method

you already use to calculate your applicable taxes. The fee is owed

to the owner of the Project Gutenberg™ trademark, but he has

agreed to donate royalties under this paragraph to the Project

Gutenberg Literary Archive Foundation. Royalty payments must be paid

within 60 days following each date on which you prepare (or are

legally required to prepare) your periodic tax returns. Royalty

payments should be clearly marked as such and sent to the Project

Gutenberg Literary Archive Foundation at the address specified in

Section 4, “Information about donations to the Project Gutenberg

Literary Archive Foundation.”

• You provide a full refund of any money paid by a user who notifies

you in writing (or by e-mail) within 30 days of receipt that s/he

does not agree to the terms of the full Project Gutenberg™

License. You must require such a user to return or destroy all

copies of the works possessed in a physical medium and discontinue

all use of and all access to other copies of Project Gutenberg™

works.

• You provide, in accordance with paragraph 1.F.3, a full refund of

any money paid for a work or a replacement copy, if a defect in the

electronic work is discovered and reported to you within 90 days of

receipt of the work.

• You comply with all other terms of this agreement for free

distribution of Project Gutenberg™ works.

1.E.9. If you wish to charge a fee or distribute a Project

Gutenberg™ electronic work or group of works on different terms than

are set forth in this agreement, you must obtain permission in writing

from the Project Gutenberg Literary Archive Foundation, the manager of

the Project Gutenberg™ trademark. Contact the Foundation as set

forth in Section 3 below.

1.F.

1.F.1. Project Gutenberg volunteers and employees expend considerable

effort to identify, do copyright research on, transcribe and proofread

works not protected by U.S. copyright law in creating the Project

Gutenberg™ collection. Despite these efforts, Project Gutenberg™

electronic works, and the medium on which they may be stored, may

contain “Defects,” such as, but not limited to, incomplete, inaccurate

or corrupt data, transcription errors, a copyright or other

intellectual property infringement, a defective or damaged disk or

other medium, a computer virus, or computer codes that damage or

cannot be read by your equipment.

1.F.2. LIMITED WARRANTY, DISCLAIMER OF DAMAGES - Except for the “Right

of Replacement or Refund” described in paragraph 1.F.3, the Project

Gutenberg Literary Archive Foundation, the owner of the Project

Gutenberg™ trademark, and any other party distributing a Project

Gutenberg™ electronic work under this agreement, disclaim all

liability to you for damages, costs and expenses, including legal

fees. YOU AGREE THAT YOU HAVE NO REMEDIES FOR NEGLIGENCE, STRICT

LIABILITY, BREACH OF WARRANTY OR BREACH OF CONTRACT EXCEPT THOSE

PROVIDED IN PARAGRAPH 1.F.3. YOU AGREE THAT THE FOUNDATION, THE

TRADEMARK OWNER, AND ANY DISTRIBUTOR UNDER THIS AGREEMENT WILL NOT BE

LIABLE TO YOU FOR ACTUAL, DIRECT, INDIRECT, CONSEQUENTIAL, PUNITIVE OR

INCIDENTAL DAMAGES EVEN IF YOU GIVE NOTICE OF THE POSSIBILITY OF SUCH

DAMAGE.

1.F.3. LIMITED RIGHT OF REPLACEMENT OR REFUND - If you discover a

defect in this electronic work within 90 days of receiving it, you can

receive a refund of the money (if any) you paid for it by sending a

written explanation to the person you received the work from. If you

received the work on a physical medium, you must return the medium

with your written explanation. The person or entity that provided you

with the defective work may elect to provide a replacement copy in

lieu of a refund. If you received the work electronically, the person

or entity providing it to you may choose to give you a second

opportunity to receive the work electronically in lieu of a refund. If

the second copy is also defective, you may demand a refund in writing

without further opportunities to fix the problem.

1.F.4. Except for the limited right of replacement or refund set forth

in paragraph 1.F.3, this work is provided to you ‘AS-IS’, WITH NO

OTHER WARRANTIES OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT

LIMITED TO WARRANTIES OF MERCHANTABILITY OR FITNESS FOR ANY PURPOSE.

1.F.5. Some states do not allow disclaimers of certain implied

warranties or the exclusion or limitation of certain types of

damages. If any disclaimer or limitation set forth in this agreement

violates the law of the state applicable to this agreement, the

agreement shall be interpreted to make the maximum disclaimer or

limitation permitted by the applicable state law. The invalidity or

unenforceability of any provision of this agreement shall not void the

remaining provisions.

1.F.6. INDEMNITY - You agree to indemnify and hold the Foundation, the

trademark owner, any agent or employee of the Foundation, anyone

providing copies of Project Gutenberg™ electronic works in

accordance with this agreement, and any volunteers associated with the

production, promotion and distribution of Project Gutenberg™

electronic works, harmless from all liability, costs and expenses,

including legal fees, that arise directly or indirectly from any of

the following which you do or cause to occur: (a) distribution of this

or any Project Gutenberg™ work, (b) alteration, modification, or

additions or deletions to any Project Gutenberg™ work, and (c) any

Defect you cause.

Section 2. Information about the Mission of Project Gutenberg™

Project Gutenberg™ is synonymous with the free distribution of

electronic works in formats readable by the widest variety of

computers including obsolete, old, middle-aged and new computers. It

exists because of the efforts of hundreds of volunteers and donations

from people in all walks of life.

Volunteers and financial support to provide volunteers with the

assistance they need are critical to reaching Project Gutenberg™’s

goals and ensuring that the Project Gutenberg™ collection will

remain freely available for generations to come. In 2001, the Project

Gutenberg Literary Archive Foundation was created to provide a secure

and permanent future for Project Gutenberg™ and future

generations. To learn more about the Project Gutenberg Literary

Archive Foundation and how your efforts and donations can help, see

Sections 3 and 4 and the Foundation information page at www.gutenberg.org.

Section 3. Information about the Project Gutenberg Literary Archive Foundation

The Project Gutenberg Literary Archive Foundation is a non-profit

501(c)(3) educational corporation organized under the laws of the

state of Mississippi and granted tax exempt status by the Internal

Revenue Service. The Foundation’s EIN or federal tax identification

number is 64-6221541. Contributions to the Project Gutenberg Literary

Archive Foundation are tax deductible to the full extent permitted by

U.S. federal laws and your state’s laws.

The Foundation’s business office is located at 809 North 1500 West,

Salt Lake City, UT 84116, (801) 596-1887. Email contact links and up

to date contact information can be found at the Foundation’s website

and official page at www.gutenberg.org/contact

Section 4. Information about Donations to the Project Gutenberg

Literary Archive Foundation

Project Gutenberg™ depends upon and cannot survive without widespread

public support and donations to carry out its mission of

increasing the number of public domain and licensed works that can be

freely distributed in machine-readable form accessible by the widest

array of equipment including outdated equipment. Many small donations

($1 to $5,000) are particularly important to maintaining tax exempt

status with the IRS.

The Foundation is committed to complying with the laws regulating

charities and charitable donations in all 50 states of the United

States. Compliance requirements are not uniform and it takes a

considerable effort, much paperwork and many fees to meet and keep up

with these requirements. We do not solicit donations in locations

where we have not received written confirmation of compliance. To SEND

DONATIONS or determine the status of compliance for any particular state

visit www.gutenberg.org/donate.

While we cannot and do not solicit contributions from states where we

have not met the solicitation requirements, we know of no prohibition

against accepting unsolicited donations from donors in such states who

approach us with offers to donate.

International donations are gratefully accepted, but we cannot make

any statements concerning tax treatment of donations received from

outside the United States. U.S. laws alone swamp our small staff.

Please check the Project Gutenberg web pages for current donation

methods and addresses. Donations are accepted in a number of other

ways including checks, online payments and credit card donations. To

donate, please visit: www.gutenberg.org/donate.

Section 5. General Information About Project Gutenberg™ electronic works

Professor Michael S. Hart was the originator of the Project

Gutenberg™ concept of a library of electronic works that could be

freely shared with anyone. For forty years, he produced and

distributed Project Gutenberg™ eBooks with only a loose network of

volunteer support.

Project Gutenberg™ eBooks are often created from several printed

editions, all of which are confirmed as not protected by copyright in

the U.S. unless a copyright notice is included. Thus, we do not

necessarily keep eBooks in compliance with any particular paper

edition.

Most people start at our website which has the main PG search

facility: www.gutenberg.org.

This website includes information about Project Gutenberg™,

including how to make donations to the Project Gutenberg Literary

Archive Foundation, how to help produce our new eBooks, and how to

subscribe to our email newsletter to hear about new eBooks.