# Credits: Produced

Charles Keller, and David Widger

﻿The Project Gutenberg eBook of Flying Machines: Construction and Operation

This ebook is for the use of anyone anywhere in the United States and

most other parts of the world at no cost and with almost no restrictions

whatsoever. You may copy it, give it away or re-use it under the terms

of the Project Gutenberg License included with this ebook or online

at www.gutenberg.org. If you are not located in the United States,

you will have to check the laws of the country where you are located

before using this eBook.

Title: Flying Machines: Construction and Operation

Author: William J. Jackman

Octave Chanute

Thomas Herbert Russell

Release date: May 1, 1997 [eBook #907]

Most recently updated: July 26, 2008

Language: English

Credits: Produced by Charles Keller, and David Widger

*** START OF THE PROJECT GUTENBERG EBOOK FLYING MACHINES: CONSTRUCTION AND OPERATION ***

Produced by Charles Keller

FLYING MACHINE: CONSTRUCTION AND OPERATION

By W.J. Jackman and Thos. H. Russell

A Practical Book Which Shows, in Illustrations, Working Plans and Text,

How to Build and Navigate the Modern Airship.

W.J. JACKMAN, M.E., Author of "A B C of the Motorcycle," "Facts for

Motorists," etc. etc.

and

THOS. H. RUSSELL, A.M., M.E., Charter Member of the Aero Club of

Illinois, Author of "History of the Automobile," "Motor Boats:

Construction and Operation," etc. etc.

With Introductory Chapter By Octave Chanute, C.E., President Aero Club

of Illinois

1912

PREFACE.

This book is written for the guidance of the novice in aviation--the

man who seeks practical information as to the theory, construction and

operation of the modern flying machine. With this object in view the

wording is intentionally plain and non-technical. It contains some

propositions which, so far as satisfying the experts is concerned, might

doubtless be better stated in technical terms, but this would defeat the

main purpose of its preparation. Consequently, while fully aware of its

shortcomings in this respect, the authors have no apologies to make.

In the stating of a technical proposition so it may be clearly

understood by people not versed in technical matters it becomes

absolutely necessary to use language much different from that which an

expert would employ, and this has been done in this volume.

No man of ordinary intelligence can read this book without obtaining

a clear, comprehensive knowledge of flying machine construction and

operation. He will learn, not only how to build, equip, and manipulate

an aeroplane in actual flight, but will also gain a thorough

understanding of the principle upon which the suspension in the air of

an object much heavier than the air is made possible.

This latter feature should make the book of interest even to those who

have no intention of constructing or operating a flying machine. It will

enable them to better understand and appreciate the performances of the

daring men like the Wright brothers, Curtiss, Bleriot, Farman, Paulhan,

Latham, and others, whose bold experiments have made aviation an

actuality.

For those who wish to engage in the fascinating pastime of construction

and operation it is intended as a reliable, practical guide.

It may be well to explain that the sub-headings in the articles by Mr.

Chanute were inserted by the authors without his knowledge. The purpose

of this was merely to preserve uniformity in the typography of the book.

This explanation is made in justice to Mr. Chanute.

THE AUTHORS.

IN MEMORIAM.

Octave Chanute, "the father of the modern flying machine," died at his

home in Chicago on November 23, 1910, at the age of 72 years. His last

work in the interest of aviation was to furnish the introductory chapter

to the first edition of this volume, and to render valuable assistance

in the handling of the various subjects. He even made the trip from his

home to the office of the publishers one inclement day last spring,

to look over the proofs of the book and, at his suggestion, several

important changes were made. All this was "a labor of love" on Mr.

Chanute's part. He gave of his time and talents freely because he was

enthusiastic in the cause of aviation, and because he knew the authors

of this book and desired to give them material aid in the preparation of

the work--a favor that was most sincerely appreciated.

The authors desire to make acknowledgment of many courtesies in the way

of valuable advice, information, etc., extended by Mr. Octave Chanute,

# C. E., Mr. E. L. Jones, Editor of Aeronautics, and the publishers of,

the New England Automobile Journal and Fly.

CONTENTS

# Chapter

# I.    Evolution of the Two-Surface Flying Machine

Introductory Chapter by Octave Chanute, C. E.

# II.   Theory Development and Use

Origin of the Aeroplane--Developments by Chanute

and the Wrights--Practical Uses and Limits.

# III.  Mechanical Bird Action

What the Motor Does--Puzzle in Bird Soaring.

# IV.   Various Forms of Flying Machines

Helicopters, Ornithopters and Aeroplanes--

Monoplanes, Biplanes and Triplanes.

# V.    Constructing a Gliding Machine

Plans and Materials Required--Estimate of Cost--

Sizes and Preparation of Various Parts--Putting the

Parts Together

# VI.   Learning to Fly

How to Use the Glider--Effect of Body Movements

--Rules for Beginners--Safest Place to Glide.

# VII.  Putting On the Rudder

Its Construction, Application and Use.

# VIII. The Real Flying Machine

Surface Area Required--Proper Size of Frame and

Auxiliaries--Installation of Motor--Cost of

Constructing Machine.

# IX.   Selection of the Motor

Essential Features--Multiplicity of Cylinders--Power

Required--Kind and Action of Propellers--Placing

of the Motor

# X.    Proper Dimensions of Machines

Figuring Out the Details--How to Estimate Load

Capacity--Distribution of the Weight--Measurements

of Leading Machines.

# XI.   Plane and Rudder Control

Various Methods in Use--Wheels and Hand and

Foot Levers

# XII.  How to Use the Machine

Rules of Leading Aviators--Rising from the Ground

--Reasonable Altitude--Preserving Equilibrium--

Learning to Steer.

# XIII. Peculiarities of Aeroplane Power

Pressure of the Wind--How to Determine Upon

Power--Why Speed Is Required--Bird find Flying

Machine Areas.

# XIV.  About Wind Currents, Etc.

Uncertainty of Direct Force--Trouble With Gusty

Currents--Why Bird Action Is Imitated.

# XV.   The Element of Danger

Risk Small Under Proper Conditions--Two Fields

of Safety--Lessons in Recent Accidents.

# XVI.  Radical Changes Being Made

Results of Recent Experiments--New Dimensions

--Increased Speed--The One Governing Rule.

# XVII. Some of the New Designs

Automatic Control of Plane Stability--Inventor

Herring's Devices--Novel Ideas of Students.

# XVIII. Demand for Flying Machines

Wonderful Results in a Year--Factories Over-

crowded with Orders.

# XIX.   Law of the Airship

Rights of Property Owners--Some Legal

Peculiarities--Danger of Trespass.

# XX.     Soaring Flight

# XXI.    Flying Machines vs. Balloons

# XXII.   Problems of Aerial Flight

# XXIII.  Amateurs May Use Wright Patents

# XXIV.   Hints on Propeller Construction

# XXV.    New Motors and Devices

# XXVI.   Monoplanes, Triplanes, Multiplanes

# XXVII.  Records of Various Kinds

FLYING MACHINES: CONSTRUCTION and OPERATION

# CHAPTER I. EVOLUTION OF TWO-SURFACE FLYING MACHINE.

By Octave Chanute.

# I am asked to set forth the development of the "two-surface" type of

flying machine which is now used with modifications by Wright Brothers,

Farman, [1] Delagrange, Herring and others.

This type originated with Mr. F. H. Wenham, who patented it in

England in 1866 (No. 1571), taking out provisional papers only. In the

abridgment of British patent Aeronautical Specifications (1893) it is

described as follows:

"Two or more aeroplanes are arranged one above the other, and support a

framework or car containing the motive power. The aeroplanes are made of

silk or canvas stretched on a frame by wooden rods or steel ribs. When

manual power is employed the body is placed horizontally, and oars or

propellers are actuated by the arms or legs.

"A start may be obtained by lowering the legs and running down hill or

the machine may be started from a moving carriage. One or more screw

propellers may be applied for propelling when steam power is employed."

On June 27, 1866, Mr. Wenham read before the "Aeronautical Society of

Great Britain," then recently organized, the ablest paper ever presented

to that society, and thereby breathed into it a spirit which has

continued to this day. In this paper he described his observations of

birds, discussed the laws governing flight as to the surfaces and power

required both with wings and screws, and he then gave an account of his

own experiments with models and with aeroplanes of sufficient size to

carry the weight of a man.

Second Wenham Aeroplane.

His second aeroplane was sixteen feet from tip to tip. A trussed spar at

the bottom carried six superposed bands of thin holland fabric fifteen

inches wide, connected with vertical webs of holland two feet apart,

thus virtually giving a length of wing of ninety-six feet and one

hundred and twenty square feet of supporting surface. The man was placed

horizontally on a base board beneath the spar. This apparatus when tried

in the wind was found to be unmanageable by reason of the fluttering

motions of the fabric, which was insufficiently stiffened with crinoline

steel, but Mr. Wenham pointed out that this in no way invalidated

the principle of the apparatus, which was to obtain large supporting

surfaces without increasing unduly the leverage and consequent weight of

spar required, by simply superposing the surfaces.

This principle is entirely sound and it is surprising that it is,

to this day, not realized by those aviators who are hankering for

monoplanes.

Experiments by Stringfellow.

The next man to test an apparatus with superposed surfaces was Mr.

Stringfellow, who, becoming much impressed with Mr. Wenham's proposal,

produced a largish model at the exhibition of the Aeronautical Society

in 1868. It consisted of three superposed surfaces aggregating 28 square

feet and a tail of 8 square feet more. The weight was under 12 pounds

and it was driven by a central propeller actuated by a steam engine

overestimated at one-third of a horsepower. It ran suspended to a wire

on its trials but failed of free flight, in consequence of defective

equilibrium. This apparatus has since been rebuilt and is now in the

National Museum of the Smithsonian Institution at Washington. Linfield's

Unsuccessful Efforts.

In 1878 Mr. Linfield tested an apparatus in England consisting of a

cigar-shaped car, to which was attached on each side frames five feet

square, containing each twenty-five superposed planes of stretched and

varnished linen eighteen inches wide, and only two inches apart, thus

reminding one of a Spanish donkey with panniers. The whole weighed two

hundred and forty pounds. This was tested by being mounted on a flat car

behind a locomotive going 40 miles an hour. When towed by a line fifteen

feet long the apparatus rose only a little from the car and exhibited

such unstable equilibrium that the experiment was not renewed. The lift

was only about one-third of what it would have been had the planes been

properly spaced, say their full width apart, instead of one-ninth as

erroneously devised.

Renard's "Dirigible Parachute."

In 1889 Commandant Renard, the eminent superintendent of the French

Aeronautical Department, exhibited at the Paris Exposition of that year,

an apparatus experimented with some years before, which he termed a

"dirigible parachute." It consisted of an oviform body to which were

pivoted two upright slats carrying above the body nine long superposed

flat blades spaced about one-third of their width apart. When this

apparatus was properly set at an angle to the longitudinal axis of the

body and dropped from a balloon, it travelled back against the wind for

a considerable distance before alighting. The course could be varied

by a rudder. No practical application seems to have been made of this

device by the French War Department, but Mr. J. P. Holland, the

inventor of the submarine boat which bears his name, proposed in 1893

an arrangement of pivoted framework attached to the body of a flying

machine which combines the principle of Commandant Renard with the

curved blades experimented with by Mr. Phillips, now to be noticed, with

the addition of lifting screws inserted among the blades.

Phillips Fails on Stability Problem.

In 1893 Mr. Horatio Phillips, of England, after some very interesting

experiments with various wing sections, from which he deduced

conclusions as to the shape of maximum lift, tested an apparatus

resembling a Venetian blind which consisted of fifty wooden slats of

peculiar shape, 22 feet long, one and a half inches wide, and two inches

apart, set in ten vertical upright boards. All this was carried upon a

body provided with three wheels. It weighed 420 pounds and was driven

at 40 miles an hour on a wooden sidewalk by a steam engine of nine

horsepower which actuated a two-bladed screw. The lift was satisfactory,

being perhaps 70 pounds per horsepower, but the equilibrium was quite

bad and the experiments were discontinued. They were taken up again in

1904 with a similar apparatus large enough to carry a passenger, but the

longitudinal equilibrium was found to be defective. Then in 1907 a new

machine was tested, in which four sets of frames, carrying similar

sets of slat "sustainers" were inserted, and with this arrangement the

longitudinal stability was found to be very satisfactory. The whole

apparatus, with the operator, weighed 650 pounds. It flew about 200

yards when driven by a motor of 20 to 22 h.p. at 30 miles an hour,

thus exhibiting a lift of about 32 pounds per h.p., while it will be

remembered that the aeroplane of Wright Brothers exhibits a lifting

capacity of 50 pounds to the h.p.

Hargrave's Kite Experiments.

After experimenting with very many models and building no less than

eighteen monoplane flying model machines, actuated by rubber, by

compressed air and by steam, Mr. Lawrence Hargrave, of Sydney, New South

Wales, invented the cellular kite which bears his name and made it known

in a paper contributed to the Chicago Conference on Aerial Navigation

in 1893, describing several varieties. The modern construction is well

known, and consists of two cells, each of superposed surfaces with

vertical side fins, placed one behind the other and connected by a

rod or frame. This flies with great steadiness without a tail. Mr.

Hargrave's idea was to use a team of these kites, below which he

proposed to suspend a motor and propeller from which a line would be

carried to an anchor in the ground. Then by actuating the propeller the

whole apparatus would move forward, pick up the anchor and fly away. He

said: "The next step is clear enough, namely, that a flying machine with

acres of surface can be safely got under way or anchored and hauled to

the ground by means of the string of kites."

The first tentative experiments did not result well and emphasized the

necessity for a light motor, so that Mr. Hargrave has since been engaged

in developing one, not having convenient access to those which have been

produced by the automobile designers and builders.

Experiments With Glider Model.

And here a curious reminiscence may be indulged in. In 1888 the present

writer experimented with a two-cell gliding model, precisely similar to

a Hargrave kite, as will be confirmed by Mr. Herring. It was frequently

tested by launching from the top of a three-story house and glided

downward very steadily in all sorts of breezes, but the angle of descent

was much steeper than that of birds, and the weight sustained per square

foot was less than with single cells, in consequence of the lesser

support afforded by the rear cell, which operated upon air already set

in motion downward by the front cell, so nothing more was done with

it, for it never occurred to the writer to try it as a kite and he thus

missed the distinction which attaches to Hargrave's name.

Sir Hiram Maxim also introduced fore and aft superposed surfaces in his

wondrous flying machine of 1893, but he relied chiefly for the lift upon

his main large surface and this necessitated so many guys, to prevent

distortion, as greatly to increase the head resistance and this,

together with the unstable equilibrium, made it evident that the design

of the machine would have to be changed.

How Lilienthal Was Killed.

In 1895, Otto Lilienthal, the father of modern aviation, the man to

whose method of experimenting almost all present successes are due,

after making something like two thousand glides with monoplanes, added

a superposed surface to his apparatus and found the control of it much

improved. The two surfaces were kept apart by two struts or vertical

posts with a few guy wires, but the connecting joints were weak and

there was nothing like trussing. This eventually cost his most useful

life. Two weeks before that distressing loss to science, Herr Wilhelm

Kress, the distinguished and veteran aviator of Vienna, witnessed a

number of glides by Lilienthal with his double-decked apparatus. He

noticed that it was much wracked and wobbly and wrote to me after the

accident: "The connection of the wings and the steering arrangement were

very bad and unreliable. I warned Herr Lilienthal very seriously. He

promised me that he would soon put it in order, but I fear that he did

not attend to it immediately."

In point of fact, Lilienthal had built a new machine, upon a different

principle, from which he expected great results, and intended to make

but very few more flights with the old apparatus. He unwisely made one

too many and, like Pilcher, was the victim of a distorted apparatus.

Probably one of the joints of the struts gave way, the upper surface

blew back and Lilienthal, who was well forward on the lower surface, was

pitched headlong to destruction.

Experiments by the Writer.

In 1896, assisted by Mr. Herring and Mr. Avery, I experimented with

several full sized gliding machines, carrying a man. The first was a

Lilienthal monoplane which was deemed so cranky that it was discarded

after making about one hundred glides, six weeks before Lilienthal's

accident. The second was known as the multiple winged machine and

finally developed into five pairs of pivoted wings, trussed together at

the front and one pair in the rear. It glided at angles of descent of 10

or 11 degrees or of one in five, and this was deemed too steep. Then

Mr. Herring and myself made computations to analyze the resistances. We

attributed much of them to the five front spars of the wings and on

a sheet of cross-barred paper I at once drew the design for a new

three-decked machine to be built by Mr. Herring.

Being a builder of bridges, I trussed these surfaces together, in order

to obtain strength and stiffness. When tested in gliding flight the

lower surface was found too near the ground. It was taken off and the

remaining apparatus now consisted of two surfaces connected together

by a girder composed of vertical posts and diagonal ties, specifically

known as a "Pratt truss." Then Mr. Herring and Mr. Avery together

devised and put on an elastic attachment to the tail. This machine

proved a success, it being safe and manageable. Over 700 glides were

made with it at angles of descent of 8 to 10 degrees, or one in six to

one in seven.

First Proposed by Wenham.

The elastic tail attachment and the trussing of the connecting frame of

the superposed wings were the only novelties in this machine, for the

superposing of the surfaces had first been proposed by Wenham, but in

accordance with the popular perception, which bestows all the credit

upon the man who adds the last touch making for success to the labors

of his predecessors, the machine has since been known by many persons as

the "Chanute type" of gliders, much to my personal gratification.

It has since been improved in many ways. Wright Brothers, disregarding

the fashion which prevails among birds, have placed the tail in front

of their apparatus and called it a front rudder, besides placing the

operator in horizontal position instead of upright, as I did; and also

providing a method of warping the wings to preserve equilibrium. Farman

and Delagrange, under the very able guidance and constructive work of

Voisin brothers, then substituted many details, including a box tail for

the dart-like tail which I used. This may have increased the resistance,

but it adds to the steadiness. Now the tendency in France seems to be to

go back to the monoplane.

Monoplane Idea Wrong.

The advocates of the single supporting surface are probably mistaken. It

is true that a single surface shows a greater lift per square foot than

superposed surfaces for a given speed, but the increased weight due to

leverage more than counterbalances this advantage by requiring heavy

spars and some guys. I believe that the future aeroplane dynamic flier

will consist of superposed surfaces, and, now that it has been found

that by imbedding suitably shaped spars in the cloth the head resistance

may be much diminished, I see few objections to superposing three, four

or even five surfaces properly trussed, and thus obtaining a compact,

handy, manageable and comparatively light apparatus. [2]

# CHAPTER II. THEORY, DEVELOPMENT, AND USE.

While every craft that navigates the air is an airship, all airships are

not flying machines. The balloon, for instance, is an airship, but it is

not what is known among aviators as a flying machine. This latter term

is properly used only in referring to heavier-than-air machines which

have no gas-bag lifting devices, and are made to really fly by the

application of engine propulsion.

Mechanical Birds.

All successful flying machines--and there are a number of them--are

based on bird action. The various designers have studied bird flight

and soaring, mastered its technique as devised by Nature, and the modern

flying machine is the result. On an exaggerated, enlarged scale the

machines which are now navigating the air are nothing more nor less than

mechanical birds.

Origin of the Aeroplane.

Octave Chanute, of Chicago, may well be called "the developer of the

flying machine." Leaving balloons and various forms of gas-bags out

of consideration, other experimenters, notably Langley and Lilienthal,

antedated him in attempting the navigation of the air on aeroplanes,

or flying machines, but none of them were wholly successful, and it

remained for Chanute to demonstrate the practicability of what was then

called the gliding machine. This term was adopted because the apparatus

was, as the name implies, simply a gliding machine, being without motor

propulsion, and intended solely to solve the problem of the best form of

construction. The biplane, used by Chanute in 1896, is still the basis

of most successful flying machines, the only radical difference being

that motors, rudders, etc., have been added.

Character of Chanute's Experiments.

It was the privilege of the author of this book to be Mr. Chanute's

guest at Millers, Indiana, in 1896, when, in collaboration with Messrs.

Herring and Avery, he was conducting the series of experiments which

have since made possible the construction of the modern flying machine

which such successful aviators as the Wright brothers and others are

now using. It was a wild country, much frequented by eagles, hawks, and

similar birds. The enthusiastic trio, Chanute, Herring and Avery, would

watch for hours the evolutions of some big bird in the air, agreeing

in the end on the verdict, "When we master the principle of that bird's

soaring without wing action, we will have come close to solving the

problem of the flying machine."

Aeroplanes of various forms were constructed by Mr. Chanute with the

assistance of Messrs. Herring and Avery until, at the time of the

writer's visit, they had settled upon the biplane, or two-surface

machine. Mr. Herring later equipped this with a rudder, and made other

additions, but the general idea is still the basis of the Wright,

Curtiss, and other machines in which, by the aid of gasolene motors,

long flights have been made.

Developments by the Wrights.

In 1900 the Wright brothers, William and Orville, who were then in

the bicycle business in Dayton, Ohio, became interested in Chanute's

experiments and communicated with him. The result was that the Wrights

took up Chanute's ideas and developed them further, making many

additions of their own, one of which was the placing of a rudder in

front, and the location of the operator horizontally on the machine,

thus diminishing by four-fifths the wind resistance of the man's

body. For three years the Wrights experimented with the glider before

venturing to add a motor, which was not done until they had thoroughly

mastered the control of their movements in the air.

Limits of the Flying Machine.

In the opinion of competent experts it is idle to look for a commercial

future for the flying machine. There is, and always will be, a limit to

its carrying capacity which will prohibit its employment for passenger

or freight purposes in a wholesale or general way. There are some, of

course, who will argue that because a machine will carry two people

another may be constructed that will carry a dozen, but those who make

this contention do not understand the theory of weight sustentation in

the air; or that the greater the load the greater must be the lifting

power (motors and plane surface), and that there is a limit to these--as

will be explained later on--beyond which the aviator cannot go.

Some Practical Uses.

At the same time there are fields in which the flying machine may be

used to great advantage. These are:

Sports--Flying machine races or flights will always be popular by reason

of the element of danger. It is a strange, but nevertheless a true

proposition, that it is this element which adds zest to all sporting

events.

Scientific--For exploration of otherwise inaccessible regions such as

deserts, mountain tops, etc.

Reconnoitering--In time of war flying machines may be used to advantage

to spy out an enemy's encampment, ascertain its defenses, etc.

# CHAPTER III. MECHANICAL BIRD ACTION

In order to understand the theory of the modern flying machine one must

also understand bird action and wind action. In this connection the

following simple experiment will be of interest:

Take a circular-shaped bit of cardboard, like the lid of a hat box, and

remove the bent-over portion so as to have a perfectly flat surface with

a clean, sharp edge. Holding the cardboard at arm's length, withdraw

your hand, leaving the cardboard without support. What is the result?

The cardboard, being heavier than air, and having nothing to sustain

it, will fall to the ground. Pick it up and throw it, with considerable

force, against the wind edgewise. What happens? Instead of falling to

the ground, the cardboard sails along on the wind, remaining afloat so

long as it is in motion. It seeks the ground, by gravity, only as the

motion ceases, and then by easy stages, instead of dropping abruptly as

in the first instance.

Here we have a homely, but accurate illustration of the action of the

flying machine. The motor does for the latter what the force of your arm

does for the cardboard--imparts a motion which keeps it afloat. The only

real difference is that the motion given by the motor is continuous and

much more powerful than that given by your arm. The action of the latter

is limited and the end of its propulsive force is reached within a

second or two after it is exerted, while the action of the motor is

prolonged.

Another Simple Illustration.

Another simple means of illustrating the principle of flying machine

operation, so far as sustentation and the elevation and depression of

the planes is concerned, is explained in the accompanying diagram.

A is a piece of cardboard about 2 by 3 inches in size. B is a piece of

paper of the same size pasted to one edge of A. If you bend the paper

to a curve, with convex side up and blow across it as shown in Figure

C, the paper will rise instead of being depressed. The dotted lines show

that the air is passing over the top of the curved paper and yet, no

matter how hard you may blow, the effect will be to elevate the paper,

despite the fact that the air is passing over, instead of under the

curved surface.

In Figure D we have an opposite effect. Here the paper is in a curve

exactly the reverse of that shown in Figure C, bringing the concave side

up. Now if you will again blow across the surface of the card the action

of the paper will be downward--it will be impossible to make it rise.

The harder you blow the greater will be the downward movement.

Principle In General Use.

This principle is taken advantage of in the construction of all

successful flying machines. Makers of monoplanes and biplanes alike

adhere to curved bodies, with the concave surface facing downward.

Straight planes were tried for a time, but found greatly lacking in the

power of sustentation. By curving the planes, and placing the concave

surface downward, a sort of inverted bowl is formed in which the air

gathers and exerts a buoyant effect. Just what the ratio of the curve

should be is a matter of contention. In some instances one inch to the

foot is found to be satisfactory; in others this is doubled, and there

are a few cases in which a curve of as much as 3 inches to the foot has

been used.

Right here it might be well to explain that the word "plane" applied to

flying machines of modern construction is in reality a misnomer. Plane

indicates a flat, level surface. As most successful flying machines have

curved supporting surfaces it is clearly wrong to speak of "planes," or

"aeroplanes." Usage, however, has made the terms convenient and, as they

are generally accepted and understood by the public, they are used in

like manner in this volume.

Getting Under Headway.

A bird, on first rising from the ground, or beginning its flight from

a tree, will flap its wings to get under headway. Here again we have

another illustration of the manner in which a flying machine gets under

headway--the motor imparts the force necessary to put the machine into

the air, but right here the similarity ceases. If the machine is to be

kept afloat the motor must be kept moving. A flying machine will not

sustain itself; it will not remain suspended in the air unless it is

under headway. This is because it is heavier than air, and gravity draws

it to the ground.

Puzzle in Bird Soaring.

But a bird, which is also heavier than air, will remain suspended, in a

calm, will even soar and move in a circle, without apparent movement

of its wings. This is explained on the theory that there are generally

vertical columns of air in circulation strong enough to sustain a bird,

but much too weak to exert any lifting power on a flying machine, It is

easy to understand how a bird can remain suspended when the wind is

in action, but its suspension in a seeming dead calm was a puzzle

to scientists until Mr. Chanute advanced the proposition of vertical

columns of air.

Modeled Closely After Birds.

So far as possible, builders of flying machines have taken what may

be called "the architecture" of birds as a model. This is readily

noticeable in the form of construction. When a bird is in motion its

wings (except when flapping) are extended in a straight line at right

angles to its body. This brings a sharp, thin edge against the air,

offering the least possible surface for resistance, while at the same

time a broad surface for support is afforded by the flat, under side of

the wings. Identically the same thing is done in the construction of the

flying machine.

Note, for instance, the marked similarity in form as shown in the

illustration in Chapter II. Here A is the bird, and B the general

outline of the machine. The thin edge of the plane in the latter is

almost a duplicate of that formed by the outstretched wings of the bird,

while the rudder plane in the rear serves the same purpose as the bird's

tail.

# CHAPTER IV. VARIOUS FORMS OF FLYING MACHINES.

There are three distinct and radically different forms of flying

machines. These are:

Aeroplanes, helicopters and ornithopers.

Of these the aeroplane takes precedence and is used almost exclusively

by successful aviators, the helicopters and ornithopers having been

tried and found lacking in some vital features, while at the same

time in some respects the helicopter has advantages not found in the

aeroplane.

What the Helicopter Is.

The helicopter gets its name from being fitted with vertical propellers

or helices (see illustration) by the action of which the machine is

raised directly from the ground into the air. This does away with the

necessity for getting the machine under a gliding headway before

it floats, as is the case with the aeroplane, and consequently the

helicopter can be handled in a much smaller space than is required for

an aeroplane. This, in many instances, is an important advantage, but it

is the only one the helicopter possesses, and is more than overcome

by its drawbacks. The most serious of these is that the helicopter is

deficient in sustaining capacity, and requires too much motive power.

Form of the Ornithopter.

The ornithopter has hinged planes which work like the wings of a bird.

At first thought this would seem to be the correct principle, and most

of the early experimenters conducted their operations on this line. It

is now generally understood, however, that the bird in soaring is in

reality an aeroplane, its extended wings serving to sustain, as well as

propel, the body. At any rate the ornithoper has not been successful in

aviation, and has been interesting mainly as an ingenious toy. Attempts

to construct it on a scale that would permit of its use by man in actual

aerial flights have been far from encouraging.

Three Kinds of Aeroplanes.

There are three forms of aeroplanes, with all of which more or less

success has been attained. These are:

The monoplane, a one-surfaced plane, like that used by Bleriot.

The biplane, a two-surfaced plane, now used by the Wrights, Curtiss,

Farman, and others.

The triplane, a three-surfaced plane This form is but little used,

its only prominent advocate at present being Elle Lavimer, a Danish

experimenter, who has not thus far accomplished much.

Whatever of real success has been accomplished in aviation may be

credited to the monoplane and biplane, with the balance in favor of

the latter. The monoplane is the more simple in construction and, where

weight-sustaining capacity is not a prime requisite, may probably be

found the most convenient. This opinion is based on the fact that the

smaller the surface of the plane the less will be the resistance offered

to the air, and the greater will be the speed at which the machine

may be moved. On the other hand, the biplane has a much greater plane

surface (double that of a monoplane of the same size) and consequently

much greater weight-carrying capacity.

Differences in Biplanes.

While all biplanes are of the same general construction so far as the

main planes are concerned, each aviator has his own ideas as to the

"rigging."

Wright, for instance, places a double horizontal rudder in front, with

a vertical rudder in the rear. There are no partitions between the

main planes, and the bicycle wheels used on other forms are replaced by

skids.

Voisin, on the contrary, divides the main planes with vertical

partitions to increase stability in turning; uses a single-plane

horizontal rudder in front, and a big box-tail with vertical rudder at

the rear; also the bicycle wheels.

Curtiss attaches horizontal stabilizing surfaces to the upper plane;

has a double horizontal rudder in front, with a vertical rudder

and horizontal stabilizing surfaces in rear. Also the bicycle wheel

alighting gear.

# CHAPTER V. CONSTRUCTING A GLIDING MACHINE.

First decide upon the kind of a machine you want--monoplane, biplane,

or triplane. For a novice the biplane will, as a rule, be found the

most satisfactory as it is more compact and therefore the more easily

handled. This will be easily understood when we realize that the surface

of a flying machine should be laid out in proportion to the amount of

weight it will have to sustain. The generally accepted rule is that 152

square feet of surface will sustain the weight of an average-sized man,

say 170 pounds. Now it follows that if these 152 square feet of surface

are used in one plane, as in the monoplane, the length and width of this

plane must be greater than if the same amount of surface is secured by

using two planes--the biplane. This results in the biplane being more

compact and therefore more readily manipulated than the monoplane, which

is an important item for a novice.

Glider the Basis of Success.

Flying machines without motors are called gliders. In making a flying

machine you first construct the glider. If you use it in this form it

remains a glider. If you install a motor it becomes a flying machine.

You must have a good glider as the basis of a successful flying machine.

It will be well for the novice, the man who has never had any experience

as an aviator, to begin with a glider and master its construction and

operation before he essays the more pretentious task of handling a

fully-equipped flying machine. In fact, it is essential that he should

do so.

Plans for Handy Glider.

A glider with a spread (advancing edge) of 20 feet, and a breadth or

depth of 4 feet, will be about right to begin with. Two planes of this

size will give the 152 square yards of surface necessary to sustain a

man's weight. Remember that in referring to flying machine measurements

"spread" takes the place of what would ordinarily be called "length,"

and invariably applies to the long or advancing edge of the machine

which cuts into the air. Thus, a glider is spoken of as being 20 feet

spread, and 4 feet in depth. So far as mastering the control of the

machine is concerned, learning to balance one's self in the air, guiding

the machine in any desired direction by changing the position of the

body, etc., all this may be learned just as readily, and perhaps more

so, with a 20-foot glider than with a larger apparatus.

Kind of Material Required.

There are three all-important features in flying machine construction,

viz.: lightness, strength and extreme rigidity. Spruce is the wood

generally used for glider frames. Oak, ash and hickory are all stronger,

but they are also considerably heavier, and where the saving of weight

is essential, the difference is largely in favor of spruce. This will be

seen in the following table:

Weight       Tensile          Compressive

per cubic ft.   Strength           Strength

Wood         in lbs.    lbs. per sq. in.   lbs. per sq in.

Hickory           53           12,000          8,500

Oak               50           12,000          9,000

Ash               38           12,000          6,000

Walnut            38            8,000          6,000

Spruce            25            8,000          5,000

Pine              25            5,000          4,500

Considering the marked saving in weight spruce has a greater percentage

of tensile strength than any of the other woods. It is also easier to

find in long, straight-grained pieces free from knots, and it is this

kind only that should be used in flying machine construction.

You will next need some spools or hanks of No. 6 linen shoe

thread, metal sockets, a supply of strong piano wire, a quantity of

closely-woven silk or cotton cloth, glue, turnbuckles, varnish, etc.

Names of the Various Parts.

The long strips, four in number, which form the front and rear edges of

the upper and lower frames, are called the horizontal beams. These are

each 20 feet in length. These horizontal beams are connected by upright

strips, 4 feet long, called stanchions. There are usually 12 of these,

six on the front edge, and six on the rear. They serve to hold the upper

plane away from the lower one. Next comes the ribs. These are 4 feet in

length (projecting for a foot over the rear beam), and while intended

principally as a support to the cloth covering of the planes, also

tend to hold the frame together in a horizontal position just as the

stanchions do in the vertical. There are forty-one of these ribs,

twenty-one on the upper and twenty on the lower plane. Then come the

struts, the main pieces which join the horizontal beams. All of these

parts are shown in the illustrations, reference to which will make the

meaning of the various names clear.

Quantity and Cost of Material.

For the horizontal beams four pieces of spruce, 20 feet long, 1 1/2

inches wide and 3/4 inch thick are necessary. These pieces must be

straight-grain, and absolutely free from knots. If it is impossible to

obtain clear pieces of this length, shorter ones may be spliced, but

this is not advised as it adds materially to the weight. The twelve

stanchions should be 4 feet long and 7/8 inch in diameter and rounded

in form so as to offer as little resistance as possible to the wind. The

struts, there are twelve of them, are 3 feet long by 11/4 x 1/2 inch.

For a 20-foot biplane about 20 yards of stout silk or unbleached muslin,

of standard one yard width, will be needed. The forty-one ribs are

each 4 feet long, and 1/2 inch square. A roll of No. 12 piano wire,

twenty-four sockets, a package of small copper tacks, a pot of glue, and

similar accessories will be required. The entire cost of this material

should not exceed $20. The wood and cloth will be the two largest

items, and these should not cost more than $10. This leaves $10 for the

varnish, wire, tacks, glue, and other incidentals. This estimate is

made for cost of materials only, it being taken for granted that the

experimenter will construct his own glider. Should the services of a

carpenter be required the total cost will probably approximate $60 or

$70.

Application of the Rudders.

The figures given also include the expense of rudders, but the details

of these have not been included as the glider is really complete without

them. Some of the best flights the writer ever saw were made by Mr. A.

M. Herring in a glider without a rudder, and yet there can be no doubt

that a rudder, properly proportioned and placed, especially a rear

rudder, is of great value to the aviator as it keeps the machine with

its head to the wind, which is the only safe position for a novice. For

initial educational purposes, however, a rudder is not essential as the

glides will, or should, be made on level ground, in moderate, steady

wind currents, and at a modest elevation. The addition of a rudder,

therefore, may well be left until the aviator has become reasonably

expert in the management of his machine.

Putting the Machine Together.

Having obtained the necessary material, the first move is to have the

rib pieces steamed and curved. This curve may be slight, about 2 inches

for the 4 feet. While this is being done the other parts should be

carefully rounded so the square edges will be taken off. This may be

done with sand paper. Next apply a coat of shellac, and when dry rub

it down thoroughly with fine sand paper. When the ribs are curved treat

them in the same way.

Lay two of the long horizontal frame pieces on the floor 3 feet apart.

Between these place six of the strut pieces. Put one at each end, and

each 4 1/2 feet put another, leaving a 2-foot space in the center. This

will give you four struts 4 1/2 feet apart, and two in the center 2 feet

apart, as shown in the illustration. This makes five rectangles. Be sure

that the points of contact are perfect, and that the struts are exactly

at right angles with the horizontal frames. This is a most important

feature because if your frame "skews" or twists you cannot keep it

straight in the air. Now glue the ends of the struts to the frame

pieces, using plenty of glue, and nail on strips that will hold the

frame in place while the glue is drying. The next day lash the joints

together firmly with the shoe thread, winding it as you would to mend a

broken gun stock, and over each layer put a coating of glue. This done,

the other frame pieces and struts may be treated in the same way, and

you will thus get the foundations for the two planes.

Another Way of Placing Struts.

In the machines built for professional use a stronger and more certain

form of construction is desired. This is secured by the placing the

struts for the lower plane under the frame piece, and those for the

upper plane over it, allowing them in each instance to come out flush

with the outer edges of the frame pieces. They are then securely

fastened with a tie plate or clamp which passes over the end of the

strut and is bound firmly against the surface of the frame piece by the

eye bolts of the stanchion sockets.

Placing the Rib Pieces.

Take one of the frames and place on it the ribs, with the arched side

up, letting one end of the ribs come flush with the front edge of the

forward frame, and the other end projecting about a foot beyond the rear

frame. The manner of fastening the ribs to the frame pieces is optional.

In some cases they are lashed with shoe thread, and in others clamped

with a metal clamp fastened with 1/2-inch wood screws. Where clamps and

screws are used care should be taken to make slight holes in the wood

with an awl before starting the screws so as to lessen any tendency to

split the wood. On the top frame, twenty-one ribs placed one foot apart

will be required. On the lower frame, because of the opening left for

the operator's body, you will need only twenty.

Joining the Two Frames.

The two frames must now be joined together. For this you will need

twenty-four aluminum or iron sockets which may be purchased at a

foundry or hardware shop. These sockets, as the name implies, provide

a receptacle in which the end of a stanchion is firmly held, and have

flanges with holes for eye-bolts which hold them firmly to the frame

pieces, and also serve to hold the guy wires. In addition to these

eye-bolt holes there are two others through which screws are fastened

into the frame pieces. On the front frame piece of the bottom plane

place six sockets, beginning at the end of the frame, and locating them

exactly opposite the struts. Screw the sockets into position with wood

screws, and then put the eye-bolts in place. Repeat the operation on the

rear frame. Next put the sockets for the upper plane frame in place.

You are now ready to bring the two planes together. Begin by inserting

the stanchions in the sockets in the lower plane. The ends may need a

little rubbing with sandpaper to get them into the sockets, but care

must be taken to have them fit snugly. When all the stanchions are in

place on the lower plane, lift the upper plane into position, and fit

the sockets over the upper ends of the stanchions.

Trussing with Guy Wires.

The next move is to "tie" the frame together rigidly by the aid of guy

wires. This is where the No. 12 piano wire comes in. Each rectangle

formed by the struts and stanchions with the exception of the small

center one, is to be wired separately as shown in the illustration. At

each of the eight corners forming the rectangle the ring of one of the

eye-bolts will be found. There are two ways of doing this "tieing," or

trussing. One is to run the wires diagonally from eye-bolt to eye-bolt,

depending upon main strength to pull them taut enough, and then twist

the ends so as to hold. The other is to first make a loop of wire

at each eye-bolt, and connect these loops to the main wires with

turn-buckles. This latter method is the best, as it admits of the

tension being regulated by simply turning the buckle so as to draw the

ends of the wire closer together. A glance at the illustration will make

this plain, and also show how the wires are to be placed. The proper

degree of tension may be determined in the following manner:

After the frame is wired place each end on a saw-horse so as to lift the

entire frame clear of the work-shop floor. Get under it, in the center

rectangle and, grasping the center struts, one in each hand, put your

entire weight on the structure. If it is properly put together it will

remain rigid and unyielding. Should it sag ever so slightly the tension

of the wires must be increased until any tendency to sag, no matter how

slight it may be, is overcome.

Putting on the Cloth.

We are now ready to put on the cloth covering which holds the air and

makes the machine buoyant. The kind of material employed is of small

account so long as it is light, strong, and wind-proof, or nearly so.

Some aviators use what is called rubberized silk, others prefer balloon

cloth. Ordinary muslin of good quality, treated with a coat of light

varnish after it is in place, will answer all the purposes of the

amateur.

Cut the cloth into strips a little over 4 feet in length. As you have

20 feet in width to cover, and the cloth is one yard wide, you will need

seven strips for each plane, so as to allow for laps, etc. This will

give you fourteen strips. Glue the end of each strip around the front

horizontal beams of the planes, and draw each strip back, over the ribs,

tacking the edges to the ribs as you go along, with small copper or

brass tacks. In doing this keep the cloth smooth and stretched tight.

Tacks should also be used in addition to the glue, to hold the cloth to

the horizontal beams.

Next, give the cloth a coat of varnish on the clear, or upper side, and

when this is dry your glider will be ready for use.

Reinforcing the Cloth.

While not absolutely necessary for amateur purposes, reinforcement

of the cloth, so as to avoid any tendency to split or tear out from

wind-pressure, is desirable. One way of doing this is to tack narrow

strips of some heavier material, like felt, over the cloth where it laps

on the ribs. Another is to sew slips or pockets in the cloth itself and

let the ribs run through them. Still another method is to sew 2-inch

strips (of the same material as the cover) on the cloth, placing them

about one yard apart, but having them come in the center of each piece

of covering, and not on the laps where the various pieces are joined.

Use of Armpieces.

Should armpieces be desired, aside from those afforded by the center

struts, take two pieces of spruce, 3 feet long, by 1 x 1 3/4 inches, and

bolt them to the front and rear beams of the lower plane about 14 inches

apart. These will be more comfortable than using the struts, as the

operator will not have to spread his arms so much. In using the struts

the operator, as a rule, takes hold of them with his hands, while with

the armpieces, as the name implies, he places his arms over them, one of

the strips coming under each armpit.

Frequently somebody asks why the ribs should be curved. The answer is

easy. The curvature tends to direct the air downward toward the rear

and, as the air is thus forced downward, there is more or less of an

impact which assists in propelling the aeroplane upwards.

# CHAPTER VI. LEARNING TO FLY.

Don't be too ambitious at the start. Go slow, and avoid unnecessary

risks. At its best there is an element of danger in aviation which

cannot be entirely eliminated, but it may be greatly reduced and

minimized by the use of common sense.

Theoretically, the proper way to begin a glide is from the top of an

incline, facing against the wind, so that the machine will soar until

the attraction of gravitation draws it gradually to the ground. This is

the manner in which experienced aviators operate, but it must be kept in

mind that these men are experts. They understand air currents, know how

to control the action and direction of their machines by shifting the

position of their bodies, and by so doing avoid accidents which would be

unavoidable by a novice.

Begin on Level Ground.

Make your first flights on level ground, having a couple of men to

assist you in getting the apparatus under headway. Take your position in

the center rectangle, back far enough to give the forward edges of the

glider an inclination to tilt upward very slightly. Now start and run

forward at a moderately rapid gait, one man at each end of the glider

assisting you. As the glider cuts into the air the wind will catch under

the uplifted edges of the curved planes, and buoy it up so that it will

rise in the air and take you with it. This rise will not be great, just

enough to keep you well clear of the ground. Now project your legs a

little to the front so as to shift the center of gravity a trifle and

bring the edges of the glider on an exact level with the atmosphere.

This, with the momentum acquired in the start, will keep the machine

moving forward for some distance.

Effect of Body Movements.

When the weight of the body is slightly back of the center of gravity

the edges of the advancing planes are tilted slightly upward. The glider

in this position acts as a scoop, taking in the air which, in turn,

lifts it off the ground. When a certain altitude is reached--this varies

with the force of the wind--the tendency to a forward movement is

lost and the glider comes to the ground. It is to prolong the forward

movement as much as possible that the operator shifts the center of

gravity slightly, bringing the apparatus on an even keel as it were by

lowering the advancing edges. This done, so long as there is momentum

enough to keep the glider moving, it will remain afloat.

If you shift your body well forward it will bring the front edges of

the glider down, and elevate the rear ones. In this way the air will be

"spilled" out at the rear, and, having lost the air support or buoyancy,

the glider comes down to the ground. A few flights will make any

ordinary man proficient in the control of his apparatus by his body

movements, not only as concerns the elevating and depressing of the

advancing edges, but also actual steering. You will quickly learn,

for instance, that, as the shifting of the bodily weight backwards

and forwards affects the upward and downward trend of the planes, so a

movement sideways--to the left or the right--affects the direction in

which the glider travels.

Ascends at an Angle.

In ascending, the glider and flying machine, like the bird, makes an

angular, not a vertical flight. Just what this angle of ascension may be

is difficult to determine. It is probable and in fact altogether likely,

that it varies with the force of the wind, weight of the rising body,

power of propulsion, etc. This, in the language of physicists, is the

angle of inclination, and, as a general thing, under normal conditions

(still air) should be put down as about one in ten, or 5 3/4 degrees.

This would be an ideal condition, but it has not, as vet been reached.

The force of the wind affects the angle considerably, as does also the

weight and velocity of the apparatus. In general practice the angle

varies from 23 to 45 degrees. At more than 45 degrees the supporting

effort is overcome by the resistance to forward motion.

Increasing the speed or propulsive force, tends to lessen the angle at

which the machine may be successfully operated because it reduces the

wind pressure. Most of the modern flying machines are operated at an

angle of 23 degrees, or less.

Maintaining an Equilibrium.

Stable equilibrium is one of the main essentials to successful flight,

and this cannot be preserved in an uncertain, gusty wind, especially by

an amateur. The novice should not attempt a glide unless the conditions

are just right. These conditions are: A clear, level space, without

obstructions, such as trees, etc., and a steady wind of not exceeding

twelve miles an hour. Always fly against the wind.

When a reasonable amount of proficiency in the handling of the machine

on level ground has been acquired the field of practice may be changed

to some gentle slope. In starting from a slope it will be found easier

to keep the machine afloat, but the experience at first is likely to be

very disconcerting to a man of less than iron nerve. As the glider sails

away from the top of the slope the distance between him and the ground

increases rapidly until the aviator thinks he is up a hundred miles

in the air. If he will keep cool, manipulate his apparatus so as to

preserve its equilibrium, and "let nature take its course," he will come

down gradually and safely to the ground at a considerable distance

from the starting place. This is one advantage of starting from an

elevation--your machine will go further.

But, if the aviator becomes "rattled"; if he loses control of his

machine, serious results, including a bad fall with risk of death,

are almost certain. And yet this practice is just as necessary as the

initial lessons on level ground. When judgment is used, and "haste made

slowly," there is very little real danger. While experimenting with

gliders the Wrights made flights innumerable under all sorts of

conditions and never had an accident of any kind.

Effects of Wind Currents.

The larger the machine the more difficult it will be to control its

movements in the air, and yet enlargement is absolutely necessary as

weight, in the form of motor, rudder, etc., is added.

Air currents near the surface of the ground are diverted by every

obstruction unless the wind is blowing hard enough to remove the

obstruction entirely. Take, for instance, the case of a tree or shrub,

in a moderate wind of from ten to twelve miles an hour. As the wind

strikes the tree it divides, part going to one side and part going to

the other, while still another part is directed upward and goes over

the top of the obstruction. This makes the handling of a glider on

an obstructed field difficult and uncertain. To handle a glider

successfully the place of operation should be clear and the wind

moderate and steady. If it is gusty postpone your flight. In this

connection it will be well to understand the velocity of the wind, and

what it means as shown in the following table:

Miles per hour Feet per second     Pressure per sq. foot

10                14.7                .492

25                36.7                3.075

50                73.3               12.300

100              146.6               49.200

Pressure of wind increases in proportion to the square of the velocity.

Thus wind at 10 miles an hour has four times the pressure of wind at

5 miles an hour. The greater this pressure the large and heavier the

object which can be raised. Any boy who has had experience in flying

kites can testify to this, High winds, however, are almost invariably

gusty and uncertain as to direction, and this makes them dangerous for

aviators. It is also a self-evident fact that, beyond a certain stage,

the harder the wind blows the more difficult it is to make headway

against it.

Launching Device for Gliders.

On page 195 will be found a diagram of the various parts of a launcher

for gliders, designed and patented by Mr. Octave Chanute. In describing

this invention in Aeronautics, Mr. Chanute says:

"In practicing, the track, preferably portable, is generally laid in

the direction of the existing wind and the car, preferably a

light platform-car, is placed on the track. The truck carrying

the winding-drum and its motor is placed to windward a suitable

distance--say from two hundred to one thousand feet--and is firmly

blocked or anchored in line with the portable track, which is preferably

80 or 100 feet in length. The flying or gliding machine to be launched

with its operator is placed on the platform-car at the leeward end of

the portable track. The line, which is preferably a flexible combination

wire-and-cord cable, is stretched between the winding-drum on the track

and detachably secured to the flying or gliding machine, preferably by

means of a trip-hoop, or else held in the hand of the operator, so that

the operator may readily detach the same from the flying-machine when

the desired height is attained."

How Glider Is Started.

"Then upon a signal given by the operator the engineer at the motor

puts it into operation, gradually increasing the speed until the line

is wound upon the drum at a maximum speed of, say, thirty miles an

hour. The operator of the flying-machine, whether he stands upright and

carries it on his shoulders, or whether he sits or lies down prone upon

it, adjusts the aeroplane or carrying surfaces so that the wind shall

strike them on the top and press downward instead of upward until the

platform-car under action of the winding-drum and line attains the

required speed.

"When the operator judges that his speed is sufficient, and this depends

upon the velocity of the wind as well as that of the car moving against

the wind, he quickly causes the front of the flying-machine to tip

upward, so that the relative wind striking on the under side of the

planes or carrying surfaces shall lift the flying machine into the air.

It then ascends like a kite to such height as may be desired by the

operator, who then trips the hook and releases the line from the

machine."

What the Operator Does.

"The operator being now free in the air has a certain initial velocity

imparted by the winding-drum and line and also a potential energy

corresponding to his height above the ground. If the flying or gliding

machine is provided with a motor, he can utilize that in his further

flight, and if it is a simple gliding machine without motor he can make

a descending flight through the air to such distance as corresponds to

the velocity acquired and the height gained, steering meanwhile by the

devices provided for that purpose.

"The simplest operation or maneuver is to continue the flight straight

ahead against the wind; but it is possible to vary this course to the

right or left, or even to return in downward flight with the wind to

the vicinity of the starting-point. Upon nearing the ground the operator

tips upward his carrying-surfaces and stops his headway upon the

cushion of increased air resistance so caused. The operator is in no way

permanently fastened to his machine, and the machine and the operator

simply rest upon the light platform-car, so that the operator is free

to rise with the machine from the car whenever the required initial

velocity is attained.

Motor For the Launcher.

"The motor may be of any suitable kind or construction, but is

preferably an electric or gasolene motor. The winding-drum is furnished

with any suitable or customary reversing-guide to cause the line to

wind smoothly and evenly upon the drum. The line is preferably a cable

composed of flexible wire and having a cotton or other cord core to

increase its flexibility. The line extends from the drum to the flying

or gliding machine. Its free end may, if desired, be grasped and held

by the operator until the flying-machine ascends to the desired height,

when by simply letting go of the line the operator may continue his

flight free. The line, however, is preferably connected to the flying

or gliding machine directly by a trip-hook having a handle or trip lever

within reach of the operator, so that when he ascends to the required

height he may readily detach the line from the flying or gliding

machine."

# CHAPTER VII. PUTTING ON THE RUDDER.

Gliders as a rule have only one rudder, and this is in the rear. It

tends to keep the apparatus with its head to the wind. Unlike the rudder

on a boat it is fixed and immovable. The real motor-propelled flying

machine, generally has both front and rear rudders manipulated by wire

cables at the will of the operator.

Allowing that the amateur has become reasonably expert in the

manipulation of the glider he should, before constructing an actual

flying machine, equip his glider with a rudder.

Cross Pieces for Rudder Beam.

To do this he should begin by putting in a cross piece, 2 feet long by

1/4 x 3/4 inches between the center struts, in the lower plane. This may

be fastened to the struts with bolts or braces. The former method is

preferable. On this cross piece, and on the rear frame of the plane

itself, the rudder beam is clamped and bolted. This rudder beam is 8

feet 11 inches long. Having put these in place duplicate them in exactly

the same manner and dimensions from the upper frame The cross pieces on

which the ends of the rudder beams are clamped should be placed about

one foot in advance of the rear frame beam.

The Rudder Itself.

The next step is to construct the rudder itself. This consists of two

sections, one horizontal, the other vertical. The latter keeps

the aeroplane headed into the wind, while the former keeps it

steady--preserves the equilibrium.

The rudder beams form the top and bottom frames of the vertical rudder.

To these are bolted and clamped two upright pieces, 3 feet, 10 inches

in length, and 3/4 inch in cross section. These latter pieces are placed

about two feet apart. This completes the framework of the vertical

rudder. See next page (59).

For the horizontal rudder you will require two strips 6 feet long, and

four 2 feet long. Find the exact center of the upright pieces on the

vertical rudder, and at this spot fasten with bolts the long pieces of

the horizontal, placing them on the outside of the vertical strips. Next

join the ends of the horizontal strips with the 2-foot pieces, using

small screws and corner braces. This done you will have two of the

2-foot pieces left. These go in the center of the horizontal frame,

"straddling" the vertical strips, as shown in the illustration.

The framework is to be covered with cloth in the same manner as the

planes. For this about ten yards will be needed.

Strengthening the Rudder.

To ensure rigidity the rudder must be stayed with guy wires. For this

purpose the No. 12 piano wire is the best. Begin by running two of these

wires from the top eye-bolts of stanchions 3 and 4, page 37, to rudder

beam where it joins the rudder planes, fastening them at the bottom.

Then run two wires from the top of the rudder beam at the same point,

to the bottom eye-bolts of the same stanchions. This will give you

four diagonal wires reaching from the rudder beam to the top and bottom

planes of the glider. Now, from the outer ends of the rudder frame run

four similar diagonal wires to the end of the rudder beam where it rests

on the cross piece. You will then have eight truss wires strengthening

the connection of the rudder to the main body of the glider.

The framework of the rudder planes is then to be braced in the same way,

which will take eight more wires, four for each rudder plane. All the

wires are to be connected at one end with turn-buckles so the tension

may be regulated as desired.

In forming the rudder frame it will be well to mortise the corners, tack

them together with small nails, and then put in a corner brace in the

inside of each joint. In doing this bear in mind that the material to be

thus fastened is light, and consequently the lightest of nails, screws,

bolts and corner pieces, etc., is necessary.

# CHAPTER VIII. THE REAL FLYING MACHINE.

We will now assume that you have become proficient enough to warrant an

attempt at the construction of a real flying machine--one that will not

only remain suspended in the air at the will of the operator, but make

respectable progress in whatever direction he may desire to go. The

glider, it must be remembered, is not steerable, except to a limited

extent, and moves only in one direction--against the wind. Besides this

its power of flotation--suspension in the air--is circumscribed.

Larger Surface Area Required.

The real flying machine is the glider enlarged, and equipped with

motor and propeller. The first thing to do is to decide upon the size

required. While a glider of 20 foot spread is large enough to sustain a

man it could not under any possible conditions, be made to rise with the

weight of the motor, propeller and similar equipment added. As the load

is increased so must the surface area of the planes be increased.

Just what this increase in surface area should be is problematical as

experienced aviators disagree, but as a general proposition it may be

placed at from three to four times the area of a 20-foot glider. [3]

Some Practical Examples.

The Wrights used a biplane 41 feet in spread, and 6 1/2 ft. deep. This,

for the two planes, gives a total surface area of 538 square feet,

inclusive of auxiliary planes. This sustains the engine equipment,

operator, etc., a total weight officially announced at 1,070 pounds. It

shows a lifting capacity of about two pounds to the square foot of plane

surface, as against a lifting capacity of about 1/2 pound per square

foot of plane surface for the 20-foot glider. This same Wright machine

is also reported to have made a successful flight, carrying a total load

of 1,100 pounds, which would be over two pounds for each square foot of

surface area, which, with auxiliary planes, is 538 square feet.

To attain the same results in a monoplane, the single surface would

have to be 60 feet in spread and 9 feet deep. But, while this is the

mathematical rule, Bleriot has demonstrated that it does not always hold

good. On his record-breaking trip across the English channel, July 25th,

1909, the Frenchman was carried in a monoplane 24 1/2 feet in spread,

and with a total sustaining surface of 150 1/2 square feet. The total

weight of the outfit, including machine, operator and fuel sufficient

for a three-hour run, was only 660 pounds. With an engine of (nominally)

25 horsepower the distance of 21 miles was covered in 37 minutes.

Which is the Best?

Right here an established mathematical quantity is involved. A small

plane surface offers less resistance to the air than a large one and

consequently can attain a higher rate of speed. As explained further

on in this chapter speed is an important factor in the matter of

weight-sustaining capacity. A machine that travels one-third faster

than another can get along with one-half the surface area of the latter

without affecting the load. See the closing paragraph of this chapter on

this point. In theory the construction is also the simplest, but this is

not always found to be so in practice. The designing and carrying

into execution of plans for an extensive area like that of a monoplane

involves great skill and cleverness in getting a framework that will be

strong enough to furnish the requisite support without an undue excess

of weight. This proposition is greatly simplified in the biplane and,

while the speed attained by the latter may not be quite so great as that

of the monoplane, it has much larger weight-carrying capacity.

Proper Sizes For Frame.

Allowing that the biplane form is selected the construction may be

practically identical with that of the 20-foot glider described in

# Chapter V., except as to size and elimination of the armpieces. In

size the surface planes should be about twice as large as those of

the 20-foot glider, viz: 40 feet spread instead of 20, and 6 feet deep

instead of 3. The horizontal beams, struts, stanchions, ribs, etc.,

should also be increased in size proportionately.

While care in the selection of clear, straight-grained timber is

important in the glider, it is still more important in the construction

of a motor-equipped flying machine as the strain on the various parts

will be much greater.

How to Splice Timbers.

It is practically certain that you will have to resort to splicing the

horizontal beams as it will be difficult, if not impossible, to find

40-foot pieces of timber totally free from knots and worm holes, and of

straight grain.

If splicing is necessary select two good 20-foot pieces, 3 inches wide

and 1 1/2 inches thick, and one 10-foot long, of the same thickness and

width. Plane off the bottom sides of the 10-foot strip, beginning about

two feet back from each end, and taper them so the strip will be about

3/4 inch thick at the extreme ends. Lay the two 20-foot beams end to

end, and under the joint thus made place the 10-foot strip, with the

planed-off ends downward. The joint of the 20-foot pieces should be

directly in the center of the 10-foot piece. Bore ten holes (with a

1/4-inch augur) equi-distant apart through the 20-foot strips and the

10-foot strip under them. Through these holes run 1/4-inch stove bolts

with round, beveled heads. In placing these bolts use washers top and

bottom, one between the head and the top beam, and the other between the

bottom beam and the screw nut which holds the bolt. Screw the nuts down

hard so as to bring the two beams tightly together, and you will have a

rigid 40-foot beam.

Splicing with Metal Sleeves.

An even better way of making a splice is by tonguing and grooving the

ends of the frame pieces and enclosing them in a metal sleeve, but

it requires more mechanical skill than the method first named. The

operation of tonguing and grooving is especially delicate and calls for

extreme nicety of touch in the handling of tools, but if this dexterity

is possessed the job will be much more satisfactory than one done with a

third timber.

As the frame pieces are generally about 1 1/2 inch in diameter, the

tongue and the groove into which the tongue fits must be correspondingly

small. Begin by sawing into one side of one of the frame pieces about

4 inches back from the end. Make the cut about 1/2 inch deep. Then turn

the piece over and duplicate the cut. Next saw down from the end

to these cuts. When the sawed-out parts are removed you will have a

"tongue" in the end of the frame timber 4 inches long and 1/2 inch

thick. The next move is to saw out a 5/8-inch groove in the end of the

frame piece which is to be joined. You will have to use a small chisel

to remove the 5/8-inch bit. This will leave a groove into which the

tongue will fit easily.

Joining the Two Pieces.

Take a thin metal sleeve--this is merely a hollow tube of aluminum

or brass open at each end--8 inches long, and slip it over either the

tongued or grooved end of one of the frame timbers. It is well to have

the sleeve fit snugly, and this may necessitate a sand-papering of the

frame pieces so the sleeve will slip on.

Push the sleeve well back out of the way. Cover the tongue thoroughly

with glue, and also put some on the inside of the groove. Use plenty

of glue. Now press the tongue into the groove, and keep the ends firmly

together until the glue is thoroughly dried. Rub off the joint lightly

with sand-paper to remove any of the glue which may have oozed out, and

slip the sleeve into place over the joint. Tack the sleeve in position

with small copper tacks, and you will have an ideal splice.

The same operation is to be repeated on each of the four frame pieces.

Two 20-foot pieces joined in this way will give a substantial frame, but

when suitable timber of this kind can not be had, three pieces, each 6

feet 11 inches long, may be used. This would give 20 feet 9 inches, of

which 8 inches will be taken up in the two joints, leaving the frame 20

feet 1 inch long.

Installation of Motor.

Next comes the installation of the motor. The kinds and efficiency of

the various types are described in the following chapter (IX). All we

are interested in at this point is the manner of installation. This

varies according to the personal ideas of the aviator. Thus one man puts

his motor in the front of his machine, another places it in the center,

and still another finds the rear of the frame the best. All get

good results, the comparative advantages of which it is difficult

to estimate. Where one man, as already explained, flies faster than

another, the one beaten from the speed standpoint has an advantage in

the matter of carrying weight, etc.

The ideas of various well-known aviators as to the correct placing of

motors may be had from the following:

Wrights--In rear of machine and to one side.

Curtiss--Well to rear, about midway between upper and lower planes.

Raich--In rear, above the center.

Brauner-Smith--In exact center of machine.

Van Anden--In center.

Herring-Burgess--Directly behind operator.

Voisin--In rear, and on lower plane.

Bleriot--In front.

R. E. P.--In front.

The One Chief Object.

An even distribution of the load so as to assist in maintaining the

equilibrium of the machine, should be the one chief object in deciding

upon the location of the motor. It matters little what particular spot

is selected so long as the weight does not tend to overbalance the

machine, or to "throw it off an even keel." It is just like loading

a vessel, an operation in which the expert seeks to so distribute

the weight of the cargo as to keep the vessel in a perfectly upright

position, and prevent a "list" or leaning to one side. The more evenly

the cargo is distributed the more perfect will be the equilibrium of the

vessel and the better it can be handled. Sometimes, when not properly

stowed, the cargo shifts, and this at once affects the position of the

craft. When a ship "lists" to starboard or port a preponderating weight

of the cargo has shifted sideways; if bow or stern is unduly depressed

it is a sure indication that the cargo has shifted accordingly. In

either event the handling of the craft becomes not only difficult, but

extremely hazardous. Exactly the same conditions prevail in the handling

of a flying machine.

Shape of Machine a Factor.

In placing the motor you must be governed largely by the shape and

construction of the flying machine frame. If the bulk of the weight

of the machine and auxiliaries is toward the rear, then the natural

location for the motor will be well to the front so as to counterbalance

the excess in rear weight. In the same way if the preponderance of the

weight is forward, then the motor should be placed back of the center.

As the propeller blade is really an integral part of the motor, the

latter being useless without it, its placing naturally depends upon the

location selected for the motor.

Rudders and Auxiliary Planes.

Here again there is great diversity of opinion among aviators as to

size, location and form. The striking difference of ideas in this

respect is well illustrated in the choice made by prominent makers as

follows:

Voisin--horizontal rudder, with two wing-like planes, in front; box-like

longitudinal stability plane in rear, inside of which is a vertical

rudder.

Wright--large biplane horizontal rudder in front at considerable

distance--about 10 feet--from the main planes; vertical biplane rudder

in rear; ends of upper and lower main planes made flexible so they may

be moved.

Curtiss--horizontal biplane rudder, with vertical damping plane between

the rudder planes about 10 feet in front of main planes; vertical rudder

in rear; stabilizing planes at each end of upper main plane.

Bleriot--V-shaped stabilizing fin, projecting from rear of plane, with

broad end outward; to the broad end of this fin is hinged a vertical

rudder; horizontal biplane rudder, also in rear, under the fin.

These instances show forcefully the wide diversity of opinion existing

among experienced aviators as to the best manner of placing the rudders

and stabilizing, or auxiliary planes, and make manifest how hopeless

would be the task of attempting to select any one form and advise its

exclusive use.

Rudder and Auxiliary Construction.

The material used in the construction of the rudders and auxiliary

planes is the same as that used in the main planes--spruce for the

framework and some kind of rubberized or varnished cloth for the

covering. The frames are joined and wired in exactly the same manner

as the frames of the main planes, the purpose being to secure the same

strength and rigidity. Dimensions of the various parts depend upon the

plan adopted and the size of the main plane.

No details as to exact dimensions of these rudders and auxiliary planes

are obtainable. The various builders, while willing enough to supply

data as to the general measurements, weight, power, etc., of their

machines, appear to have overlooked the details of the auxiliary parts,

thinking, perhaps, that these were of no particular import to the

general public. In the Wright machine, the rear horizontal and front

vertical rudders may be set down as being about one-quarter (probably a

little less) the size of the main supporting planes.

Arrangement of Alighting Gear.

Most modern machines are equipped with an alighting gear, which not

only serves to protect the machine and aviator from shock or injury in

touching the ground, but also aids in getting under headway. All the

leading makes, with the exception of the Wright, are furnished with a

frame carrying from two to five pneumatic rubber-tired bicycle wheels.

In the Curtiss and Voisin machines one wheel is placed in front and two

in the rear. In the Bleriot and other prominent machines the reverse is

the rule--two wheels in front and one in the rear. Farman makes use of

five wheels, one in the extreme rear, and four, arranged in pairs, a

little to the front of the center of the main lower plane.

In place of wheels the Wright machine is equipped with a skid-like

device consisting of two long beams attached to the lower plane by

stanchions and curving up far in front, so as to act as supports to the

horizontal rudder.

Why Wood Is Favored.

A frequently asked question is: "Why is not aluminum, or some similar

metal, substituted for wood." Wood, particularly spruce, is preferred

because, weight considered, it is much stronger than aluminum, and this

is the lightest of all metals. In this connection the following table

will be of interest:

Compressive

Weight      Tensile Strength         Strength

per cubic foot    per sq. inch         per sq. inch

Material        in lbs.           in lbs.              in lbs.

Spruce....    25               8,000                5,000

Aluminum         162              16,000              ......

Brass (sheet)    510              23,000               12,000

Steel (tool)     490             100,000               40,000

Copper (sheet)   548              30,000               40,000

As extreme lightness, combined with strength, especially tensile

strength, is the great essential in flying-machine construction, it can

be readily seen that the use of metal, even aluminum, for the framework,

is prohibited by its weight. While aluminum has double the strength

of spruce wood it is vastly heavier, and thus the advantage it has in

strength is overbalanced many times by its weight. The specific gravity

of aluminum is 2.50; that of spruce is only 0.403.

Things to Be Considered.

In laying out plans for a flying machine there are five important points

which should be settled upon before the actual work of construction is

started. These are:

First--Approximate weight of the machine when finished and equipped.

Second--Area of the supporting surface required.

Third--Amount of power that will be necessary to secure the desired

speed and lifting capacity.

Fourth--Exact dimensions of the main framework and of the auxiliary

parts.

Fifth--Size, speed and character of the propeller.

In deciding upon these it will be well to take into consideration

the experience of expert aviators regarding these features as given

elsewhere. (See Chapter X.)

Estimating the Weights Involved.

In fixing upon the probable approximate weight in advance of

construction much, of course, must be assumed. This means that it will

be a matter of advance estimating. If a two-passenger machine is to be

built we will start by assuming the maximum combined weight of the two

people to be 350 pounds. Most of the professional aviators are lighter

than this. Taking the medium between the weights of the Curtiss and

Wright machines we have a net average of 850 pounds for the framework,

motor, propeller, etc. This, with the two passengers, amounts to 1,190

pounds. As the machines quoted are in successful operation it will be

reasonable to assume that this will be a safe basis to operate on.

What the Novice Must Avoid.

This does not mean, however, that it will be safe to follow these

weights exactly in construction, but that they will serve merely as

a basis to start from. Because an expert can turn out a machine,

thoroughly equipped, of 850 pounds weight, it does not follow that a

novice can do the same thing. The expert's work is the result of years

of experience, and he has learned how to construct frames and motor

plants of the utmost lightness and strength.

It will be safer for the novice to assume that he can not duplicate the

work of such men as Wright and Curtiss without adding materially to the

gross weight of the framework and equipment minus passengers.

How to Distribute the Weight.

Let us take 1,030 pounds as the net weight of the machine as against the

same average in the Wright and Curtiss machines. Now comes the question

of distributing this weight between the framework, motor, and other

equipment. As a general proposition the framework should weigh about

twice as much as the complete power plant (this is for amateur work).

The word "framework" indicates not only the wooden frames of the main

planes, auxiliary planes, rudders, etc., but the cloth coverings as

well--everything in fact except the engine and propeller.

On the basis named the framework would weigh 686 pounds, and the power

plant 344. These figures are liberal, and the results desired may be

obtained well within them as the novice will learn as he makes progress

in the work.

Figuring on Surface Area.

It was Prof. Langley who first brought into prominence in connection

with flying machine construction the mathematical principle that the

larger the object the smaller may be the relative area of support. As

explained in Chapter XIII, there are mechanical limits as to size which

it is not practical to exceed, but the main principle remains in effect.

Take two aeroplanes of marked difference in area of surface. The larger

will, as a rule, sustain a greater weight in relative proportion to

its area than the smaller one, and do the work with less relative

horsepower. As a general thing well-constructed machines will average

a supporting capacity of one pound for every one-half square foot of

surface area. Accepting this as a working rule we find that to sustain

a weight of 1,200 pounds--machine and two passengers--we should have 600

square feet of surface.

Distributing the Surface Area.

The largest surfaces now in use are those of the Wright, Voisin and

Antoinette machines--538 square feet in each. The actual sustaining

power of these machines, so far as known, has never been tested to the

limit; it is probable that the maximum is considerably in excess of what

they have been called upon to show. In actual practice the average is a

little over one pound for each one-half square foot of surface area.

Allowing that 600 square feet of surface will be used, the next question

is how to distribute it to the best advantage. This is another important

matter in which individual preference must rule. We have seen how the

professionals disagree on this point, some using auxiliary planes

of large size, and others depending upon smaller auxiliaries with an

increase in number so as to secure on a different plan virtually the

same amount of surface.

In deciding upon this feature the best thing to do is to follow the

plans of some successful aviator, increasing the area of the auxiliaries

in proportion to the increase in the area of the main planes. Thus, if

you use 600 square feet of surface where the man whose plans you

are following uses 500, it is simply a matter of making your planes

one-fifth larger all around.

The Cost of Production.

Cost of production will be of interest to the amateur who essays to

construct a flying machine. Assuming that the size decided upon is

double that of the glider the material for the framework, timber, cloth,

wire, etc., will cost a little more than double. This is because it must

be heavier in proportion to the increased size of the framework, and

heavy material brings a larger price than the lighter goods. If we allow

$20 as the cost of the glider material it will be safe to put down

the cost of that required for a real flying machine framework at $60,

provided the owner builds it himself.

As regards the cost of motor and similar equipment it can only be said

that this depends upon the selection made. There are some reliable

aviation motors which may be had as low as $500, and there are others

which cost as much as $2,000.

Services of Expert Necessary.

No matter what kind of a motor may be selected the services of an expert

will be necessary in its proper installation unless the amateur has

considerable genius in this line himself. As a general thing $25 should

be a liberal allowance for this work. No matter how carefully the engine

may be placed and connected it will be largely a matter of luck if it is

installed in exactly the proper manner at the first attempt. The chances

are that several alterations, prompted by the results of trials, will

have to be made. If this is the case the expert's bill may readily run

up to $50. If the amateur is competent to do this part of the work the

entire item of $50 may, of course, be cut out.

As a general proposition a fairly satisfactory flying machine, one that

will actually fly and carry the operator with it, may be constructed for

$750, but it will lack the better qualities which mark the higher priced

machines. This computation is made on the basis of $60 for material, $50

for services of expert, $600 for motor, etc., and an allowance of $40

for extras.

No man who has the flying machine germ in his system will be long

satisfied with his first moderate price machine, no matter how well it

may work. It's the old story of the automobile "bug" over again. The man

who starts in with a modest $1,000 automobile invariably progresses by

easy stages to the $4,000 or $5,000 class. The natural tendency is to

want the biggest and best attainable within the financial reach of the

owner.

It's exactly the same way with the flying machine convert. The more

proficient he becomes in the manipulation of his car, the stronger

becomes the desire to fly further and stay in the air longer than the

rest of his brethren. This necessitates larger, more powerful, and more

expensive machines as the work of the germ progresses.

Speed Affects Weight Capacity.

Don't overlook the fact that the greater speed you can attain the

smaller will be the surface area you can get along with. If a machine

with 500 square feet of sustaining surface, traveling at a speed of

40 miles an hour, will carry a weight of 1,200 pounds, we can cut the

sustaining surface in half and get along with 250 square feet, provided

a speed of 60 miles an hour can be obtained. At 100 miles an hour only

80 square feet of surface area would be required. In both instances the

weight sustaining capacity will remain the same as with the 500 square

feet of surface area--1,200 pounds.

One of these days some mathematical genius will figure out this problem

with exactitude and we will have a dependable table giving the maximum

carrying capacity of various surface areas at various stated speeds,

based on the dimensions of the advancing edges. At present it is largely

a matter of guesswork so far as making accurate computation goes.

Much depends upon the shape of the machine, and the amount of surface

offering resistance to the wind, etc.

# CHAPTER IX. SELECTION OF THE MOTOR.

Motors for flying machines must be light in weight, of great strength,

productive of extreme speed, and positively dependable in action.

It matters little as to the particular form, or whether air or water

cooled, so long as the four features named are secured. There are at

least a dozen such motors or engines now in use. All are of the gasolene

type, and all possess in greater or lesser degree the desired qualities.

Some of these motors are:

Renault--8-cylinder, air-cooled; 50 horse power; weight 374 pounds.

Fiat--8-cylinder, air-cooled; 50 horse power; weight 150 pounds.

Farcot--8-cylinder, air-cooled; from 30 to 100 horse power, according to

bore of cylinders; weight of smallest, 84 pounds.

R. E. P.--10-cylinder, air-cooled; 150 horse power; weight 215 pounds.

Gnome--7 and 14 cylinders, revolving type, air-cooled; 50 and 100 horse

power; weight 150 and 300 pounds.

Darracq--2 to 14 cylinders, water cooled; 30 to 200 horse power; weight

of smallest 100 pounds.

Wright--4-cylinder, water-cooled; 25 horse power; weight 200 pounds.

Antoinette--8 and 16-cylinder, water-cooled; 50 and 100 horse power;

weight 250 and 500 pounds.

E. N. V.--8-cylinder, water-cooled; from 30 to 80 horse power, according

to bore of cylinder; weight 150 to 400 pounds.

Curtiss--8-cylinder, water-cooled; 60 horse power; weight 300 pounds.

Average Weight Per Horse Power.

It will be noticed that the Gnome motor is unusually light, being about

three pounds to the horse power produced, as opposed to an average of 4

1/2 pounds per horse power in other makes. This result is secured by

the elimination of the fly-wheel, the engine itself revolving, thus

obtaining the same effect that would be produced by a fly-wheel. The

Farcot is even lighter, being considerably less than three pounds per

horse power, which is the nearest approach to the long-sought engine

equipment that will make possible a complete flying machine the total

weight of which will not exceed one pound per square foot of area.

How Lightness Is Secured.

Thus far foreign manufacturers are ahead of Americans in the production

of light-weight aerial motors, as is evidenced by the Gnome and Farcot

engines, both of which are of French make. Extreme lightness is made

possible by the use of fine, specially prepared steel for the cylinders,

thus permitting them to be much thinner than if ordinary forms of steel

were used. Another big saving in weight is made by substituting what are

known as "auto lubricating" alloys for bearings. These alloys are made

of a combination of aluminum and magnesium.

Still further gains are made in the use of alloy steel tubing instead of

solid rods, and also by the paring away of material wherever it can be

done without sacrificing strength. This plan, with the exclusive use of

the best grades of steel, regardless of cost, makes possible a marked

reduction in weight.

Multiplicity of Cylinders.

Strange as it may seem, multiplicity of cylinders does not always add

proportionate weight. Because a 4-cylinder motor weighs say 100 pounds,

it does not necessarily follow that an 8-cylinder equipment will weigh

200 pounds. The reason of this will be plain when it is understood

that many of the parts essential to a 4-cylinder motor will fill the

requirements of an 8-cylinder motor without enlargement or addition.

Neither does multiplying the cylinders always increase the horsepower

proportionately. If a 4-cylinder motor is rated at 25 horsepower it is

not safe to take it for granted that double the number of cylinders

will give 50 horsepower. Generally speaking, eight cylinders, the bore,

stroke and speed being the same, will give double the power that can be

obtained from four, but this does not always hold good. Just why this

exception should occur is not explainable by any accepted rule.

Horse Power and Speed.

Speed is an important requisite in a flying-machine motor, as the

velocity of the aeroplane is a vital factor in flotation. At

first thought, the propeller and similar adjuncts being equal, the

inexperienced mind would naturally argue that a 50-horsepower engine

should produce just double the speed of one of 25-horsepower. That

this is a fallacy is shown by actual performances. The Wrights, using a

25-horsepower motor, have made 44 miles an hour, while Bleriot, with a

50-horsepower motor, has a record of a short-distance flight at the rate

of 52 miles an hour. The fact is that, so far as speed is concerned,

much depends upon the velocity of the wind, the size and shape of the

aeroplane itself, and the size, shape and gearing of the propeller. The

stronger the wind is blowing the easier it will be for the aeroplane

to ascend, but at the same time the more difficult it will be to make

headway against the wind in a horizontal direction. With a strong head

wind, and proper engine force, your machine will progress to a certain

extent, but it will be at an angle. If the aviator desired to keep

on going upward this would be all right, but there is a limit to the

altitude which it is desirable to reach--from 100 to 500 feet for

experts--and after that it becomes a question of going straight ahead.

Great Waste of Power.

One thing is certain--even in the most efficient of modern aerial motors

there is a great loss of power between the two points of production and

effect. The Wright outfit, which is admittedly one of the most effective

in use, takes one horsepower of force for the raising and propulsion of

each 50 pounds of weight. This, for a 25-horsepower engine, would give

a maximum lifting capacity of 1250 pounds. It is doubtful if any of

the higher rated motors have greater efficiency. As an 8-cylinder motor

requires more fuel to operate than a 4-cylinder, it naturally follows

that it is more expensive to run than the smaller motor, and a normal

increase in capacity, taking actual performances as a criterion, is

lacking. In other words, what is the sense of using an 8-cylinder motor

when one of 4 cylinders is sufficient?

What the Propeller Does.

Much of the efficiency of the motor is due to the form and gearing of

the propeller. Here again, as in other vital parts of flying-machine

mechanism, we have a wide divergence of opinion as to the best form. A

fish makes progress through the water by using its fins and tail; a

bird makes its way through the air in a similar manner by the use of its

wings and tail. In both instances the motive power comes from the body

of the fish or bird.

In place of fins or wings the flying machine is equipped with a

propeller, the action of which is furnished by the engine. Fins and

wings have been tried, but they don't work.

While operating on the same general principle, aerial propellers are

much larger than those used on boats. This is because the boat

propeller has a denser, more substantial medium to work in (water), and

consequently can get a better "hold," and produce more propulsive force

than one of the same size revolving in the air. This necessitates the

aerial propellers being much larger than those employed for marine

purposes. Up to this point all aviators agree, but as to the best form

most of them differ.

Kinds of Propellers Used.

One of the most simple is that used by Curtiss. It consists of two

pear-shaped blades of laminated wood, each blade being 5 inches wide

at its extreme point, tapering slightly to the shaft connection. These

blades are joined at the engine shaft, in a direct line. The propeller

has a pitch of 5 feet, and weighs, complete, less than 10 pounds. The

length from end to end of the two blades is 6 1/2 feet.

Wright uses two wooden propellers, in the rear of his biplane, revolving

in opposite directions. Each propeller is two-bladed.

Bleriot also uses a two-blade wooden propeller, but it is placed in

front of his machine. The blades are each about 3 1/2 feet long and have

an acute "twist."

Santos-Dumont uses a two-blade wooden propeller, strikingly similar to

the Bleriot.

On the Antoinette monoplane, with which good records have been made, the

propeller consists of two spoon-shaped pieces of metal, joined at the

engine shaft in front, and with the concave surfaces facing the machine.

The propeller on the Voisin biplane is also of metal, consisting of two

aluminum blades connected by a forged steel arm.

Maximum thrust, or stress--exercise of the greatest air-displacing

force--is the object sought. This, according to experts, is best

obtained with a large propeller diameter and reasonably low speed. The

diameter is the distance from end to end of the blades, which on the

largest propellers ranges from 6 to 8 feet. The larger the blade surface

the greater will be the volume of air displaced, and, following this,

the greater will be the impulse which forces the aeroplane ahead. In all

centrifugal motion there is more or less tendency to disintegration in

the form of "flying off" from the center, and the larger the revolving

object is the stronger is this tendency. This is illustrated in the many

instances in which big grindstones and fly-wheels have burst from being

revolved too fast. To have a propeller break apart in the air would

jeopardize the life of the aviator, and to guard against this it has

been found best to make its revolving action comparatively slow.

Besides this the slow motion (it is only comparatively slow) gives the

atmosphere a chance to refill the area disturbed by one propeller blade,

and thus have a new surface for the next blade to act upon.

Placing of the Motor.

As on other points, aviators differ widely in their ideas as to the

proper position for the motor. Wright locates his on the lower plane,

midway between the front and rear edges, but considerably to one side of

the exact center. He then counter-balances the engine weight by placing

his seat far enough away in the opposite direction to preserve the

center of gravity. This leaves a space in the center between the motor

and the operator in which a passenger may be carried without disturbing

the equilibrium.

Bleriot, on the contrary, has his motor directly in front and preserves

the center of gravity by taking his seat well back, this, with the

weight of the aeroplane, acting as a counter-balance.

On the Curtiss machine the motor is in the rear, the forward seat of the

operator, and weight of the horizontal rudder and damping plane in front

equalizing the engine weight.

No Perfect Motor as Yet.

Engine makers in the United States, England, France and Germany are all

seeking to produce an ideal motor for aviation purposes. Many of the

productions are highly creditable, but it may be truthfully said that

none of them quite fill the bill as regards a combination of the minimum

of weight with the maximum of reliable maintained power. They are all,

in some respects, improvements upon those previously in use, but the

great end sought for has not been fully attained.

One of the motors thus produced was made by the French firm of Darracq

at the suggestion of Santos Dumont, and on lines laid down by him.

Santos Dumont wanted a 2-cylinder horizontal motor capable of developing

30 horsepower, and not exceeding 4 1/2 pounds per horsepower in weight.

There can be no question as to the ability and skill of the Darracq

people, or of their desire to produce a motor that would bring new

credit and prominence to the firm. Neither could anything radically

wrong be detected in the plans. But the motor, in at least one important

requirement, fell short of expectations.

It could not be depended upon to deliver an energy of 30 horsepower

continuously for any length of time. Its maximum power could be secured

only in "spurts."

This tends to show how hard it is to produce an ideal motor for aviation

purposes. Santos Dumont, of undoubted skill and experience as an

aviator, outlined definitely what he wanted; one of the greatest

designers in the business drew the plans, and the famous house of

Darracq bent its best energies to the production. But the desired end

was not fully attained.

Features of Darracq Motor.

Horizontal motors were practically abandoned some time ago in favor of

the vertical type, but Santos Dumont had a logical reason for reverting

to them. He wanted to secure a lower center of gravity than would be

possible with a vertical engine. Theoretically his idea was correct as

the horizontal motor lies flat, and therefore offers less resistance to

the wind, but it did not work out as desired.

At the same time it must be admitted that this Darracq motor is a marvel

of ingenuity and exquisite workmanship. The two cylinders, having a bore

of 5 1-10 inches and a stroke of 4 7-10 inches, are machined out of a

solid bar of steel until their weight is only 8 4-5 pounds complete.

The head is separate, carrying the seatings for the inlet and exhaust

valves, is screwed onto the cylinder, and then welded in position. A

copper water-jacket is fitted, and it is in this condition that the

weight of 8 4-5 pounds is obtained.

On long trips, especially in regions where gasolene is hard to get,

the weight of the fuel supply is an important feature in aviation. As a

natural consequence flying machine operators favor the motor of greatest

economy in gasolene consumption, provided it gives the necessary power.

An American inventor, Ramsey by name, is working on a motor which

is said to possess great possibilities in this line. Its distinctive

features include a connecting rod much shorter than usual, and a crank

shaft located the length of the crank from the central axis of the

cylinder. This has the effect of increasing the piston stroke, and also

of increasing the proportion of the crank circle during which effective

pressure is applied to the crank.

Making the connecting rod shorter and leaving the crank mechanism the

same would introduce excessive cylinder friction. This Ramsey overcomes

by the location of his crank shaft. The effect of the long piston stroke

thus secured, is to increase the expansion of the gases, which in turn

increases the power of the engine without increasing the amount of fuel

used.

Propeller Thrust Important.

There is one great principle in flying machine propulsion which must

not be overlooked. No matter how powerful the engine may be unless the

propeller thrust more than overcomes the wind pressure there can be no

progress forward. Should the force of this propeller thrust and that of

the wind pressure be equal the result is obvious. The machine is at a

stand-still so far as forward progress is concerned and is deprived of

the essential advancing movement.

Speed not only furnishes sustentation for the airship, but adds to the

stability of the machine. An aeroplane which may be jerky and uncertain

in its movements, so far as equilibrium is concerned, when moving at

a slow gait, will readily maintain an even keel when the speed is

increased.

Designs for Propeller Blades.

It is the object of all men who design propellers to obtain the maximum

of thrust with the minimum expenditure of engine energy. With this

purpose in view many peculiar forms of propeller blades have been

evolved. In theory it would seem that the best effects could be secured

with blades so shaped as to present a thin (or cutting) edge when they

come out of the wind, and then at the climax of displacement afford a

maximum of surface so as to displace as much air as possible. While

this is the form most generally favored there are others in successful

operation.

There is also wide difference in opinion as to the equipment of the

propeller shaft with two or more blades. Some aviators use two and some

four. All have more or less success. As a mathematical proposition it

would seem that four blades should give more propulsive force than two,

but here again comes in one of the puzzles of aviation, as this result

is not always obtained.

Difference in Propeller Efficiency.

That there is a great difference in propeller efficiency is made readily

apparent by the comparison of effects produced in two leading makes of

machines--the Wright and the Voisin.

In the former a weight of from 1,100 to 1,200 pounds is sustained and

advance progress made at the rate of 40 miles an hour and more, with

half the engine speed of a 25 horse-power motor. This would be a

sustaining capacity of 48 pounds per horsepower. But the actual capacity

of the Wright machine, as already stated, is 50 pounds per horsepower.

The Voisin machine, with aviator, weighs about 1,370 pounds, and is

operated with a so-horsepower motor. Allowing it the same speed as the

Wright we find that, with double the engine energy, the lifting capacity

is only 27 1/2 pounds per horsepower. To what shall we charge this

remarkable difference? The surface of the planes is exactly the same in

both machines so there is no advantage in the matter of supporting area.

Comparison of Two Designs.

On the Wright machine two wooden propellers of two blades each (each

blade having a decided "twist") are used. As one 25 horsepower motor

drives both propellers the engine energy amounts to just one-half of

this for each, or 12 1/2 horsepower. And this energy is utilized at

one-half the normal engine speed.

On the Voisin a radically different system is employed. Here we have

one metal two-bladed propeller with a very slight "twist" to the blade

surfaces. The full energy of a 50-horsepower motor is utilized.

Experts Fail to Agree.

Why should there be such a marked difference in the results obtained?

Who knows? Some experts maintain that it is because there are two

propellers on the Wright machine and only one on the Voisin, and

consequently double the propulsive power is exerted. But this is not a

fair deduction, unless both propellers are of the same size. Propulsive

power depends upon the amount of air displaced, and the energy put into

the thrust which displaces the air.

Other experts argue that the difference in results may be traced to the

difference in blade design, especially in the matter of "twist."

The fact is that propeller results depend largely upon the nature of

the aeroplanes on which they are used. A propeller, for instance,

which gives excellent results on one type of aeroplane, will not work

satisfactorily on another.

There are some features, however, which may be safely adopted in

propeller selection. These are: As extensive a diameter as possible;

blade area 10 to 15 per cent of the area swept; pitch four-fifths of

the diameter; rotation slow. The maximum of thrust effort will be thus

obtained.

# CHAPTER X. PROPER DIMENSIONS OF MACHINES.

In laying out plans for a flying machine the first thing to decide upon

is the size of the plane surfaces. The proportions of these must be

based upon the load to be carried. This includes the total weight of

the machine and equipment, and also the operator. This will be a rather

difficult problem to figure out exactly, but practical approximate

figures may be reached.

It is easy to get at the weight of the operator, motor and propeller,

but the matter of determining, before they are constructed, what the

planes, rudders, auxiliaries, etc., will weigh when completed is an

intricate proposition. The best way is to take the dimensions of some

successful machine and use them, making such alterations in a minor way

as you may desire.

Dimensions of Leading Machines.

In the following tables will be found the details as to surface area,

weight, power, etc., of the nine principal types of flying machines

which are now prominently before the public:

MONOPLANES.

Surface area    Spread in     Depth in

Make          Passengers     sq. feet      linear feet  linear

feet

Santos-Dumont.. 1           110             16.0         26.0

Bleriot..... 1           150.6           24.6         22.0

R. E. P..... 1           215             34.1         28.9

Bleriot..... 2           236             32.9         23.0

Antoinette.... 2           538             41.2         37.9

No. of                  Weight Without

Propeller

Make         Cylinders   Horse Power       Operator

Diameter

Santos-Dumont.. 2          30                250            5.0

Bleriot..... 3          25                680            6.9

R. E. P..... 7          35                900            6.6

Bleriot..... 7          50              1,240            8.1

Antoinette... 8          50              1,040            7.2

BIPLANES.

Surface Area       Spread in      Depth

in

Make      Passengers       sq. feet        linear feet    linear

feet

Curtiss... 2               258             29.0

28.7

Wright.... 2               538             41.0

30.7

Farman.... 2               430             32.9

39.6

Voisin.... 2               538             37.9

39.6

No. of                     Weight Without

Propeller

Make       Cylinders      Horse Power      Operator

Diameter

Curtiss... 8               50               600          6.0

Wright.... 4               25             1,100          8.1

Farman.... 7               50             1,200          8.9

Voisin.... 8               50             1,200          6.6

In giving the depth dimensions the length over all--from the extreme

edge of the front auxiliary plane to the extreme tip of the rear is

stated. Thus while the dimensions of the main planes of the Wright

machine are 41 feet spread by 6 1/2 feet in depth, the depth over all is

30.7.

Figuring Out the Details.

With this data as a guide it should be comparatively easy to decide

upon the dimensions of the machine required. In arriving at the maximum

lifting capacity the weight of the operator must be added. Assuming this

to average 170 pounds the method of procedure would be as follows:

Add the weight of the operator to the weight of the complete machine.

The new Wright machine complete weighs 900 pounds. This, plus 170, the

weight of the operator, gives a total of 1,070 pounds. There are 538

square feet of supporting surface, or practically one square foot of

surface area to each two pounds of load.

There are some machines, notably the Bleriot, in which the supporting

power is much greater. In this latter instance we find a surface area of

150 1/2 square feet carrying a load of 680 plus 170, or an aggregate of

850 pounds. This is the equivalent of five pounds to the square foot.

This ratio is phenomenally large, and should not be taken as a guide by

amateurs.

The Matter of Passengers.

These deductions are based on each machine carrying one passenger, which

is admittedly the limit at present of the monoplanes like those operated

for record-making purposes by Santos-Dumont and Bleriot. The biplanes,

however, have a two-passenger capacity, and this adds materially to the

proportion of their weight-sustaining power as compared with the surface

area. In the following statement all the machines are figured on the

one-passenger basis. Curtiss and Wright have carried two passengers on

numerous occasions, and an extra 170 pounds should therefore be added to

the total weight carried, which would materially increase the capacity.

Even with the two-passenger load the limit is by no means reached,

but as experiments have gone no further it is impossible to make more

accurate figures.

Average Proportions of Load.

It will be interesting, before proceeding to lay out the dimension

details, to make a comparison of the proportion of load effect with

the supporting surfaces of various well-known machines. Here are the

figures:

Santos-Dumont--A trifle under four pounds per square foot.

Bleriot--Five pounds.

R. E. P.--Five pounds.

Antoinette--About two and one-quarter pounds.

Curtiss--About two and one-half pounds.

Wright--Two and one-quarter pounds.

Farman--A trifle over three pounds.

Voisin--A little under two and one-half pounds.

Importance of Engine Power.

While these figures are authentic, they are in a way misleading, as the

important factor of engine power is not taken into consideration. Let us

recall the fact that it is the engine power which keeps the machine in

motion, and that it is only while in motion that the machine will remain

suspended in the air. Hence, to attribute the support solely to the

surface area is erroneous. True, that once under headway the planes

contribute largely to the sustaining effect, and are absolutely

essential in aerial navigation--the motor could not rise without

them--still, when it comes to a question of weight-sustaining power, we

must also figure on the engine capacity.

In the Wright machine, in which there is a lifting capacity of

approximately 2 1/4 pounds to the square foot of surface area, an engine

of only 25 horsepower is used. In the Curtiss, which has a lifting

capacity of 2 1/2 pounds per square foot, the engine is of 50

horsepower. This is another of the peculiarities of aerial construction

and navigation. Here we have a gain of 1/4 pound in weight-lifting

capacity with an expenditure of double the horsepower. It is this

feature which enables Curtiss to get along with a smaller surface area

of supporting planes at the expense of a big increase in engine power.

Proper Weight of Machine.

As a general proposition the most satisfactory machine for amateur

purposes will be found to be one with a total weight-sustaining power of

about 1,200 pounds. Deducting 170 pounds as the weight of the operator,

this will leave 1,030 pounds for the complete motor-equipped machine,

and it should be easy to construct one within this limit. This implies,

of course, that due care will be taken to eliminate all superfluous

weight by using the lightest material compatible with strength and

safety.

This plan will admit of 686 pounds weight in the frame work, coverings,

etc., and 344 for the motor, propeller, etc., which will be ample. Just

how to distribute the weight of the planes is a matter which must be

left to the ingenuity of the builder.

Comparison of Bird Power.

There is an interesting study in the accompanying illustration. Note

that the surface area of the albatross is much smaller than that of the

vulture, although the wing spread is about the same. Despite this the

albatross accomplishes fully as much in the way of flight and soaring as

the vulture. Why? Because the albaboss is quicker and more powerful in

action. It is the application of this same principle in flying machines

which enables those of great speed and power to get along with less

supporting surface than those of slower movement.

Measurements of Curtiss Machine.

Some idea of framework proportion may be had from the following

description of the Curtiss machine. The main planes have a spread

(width) of 29 feet, and are 4 1/2 feet deep. The front double surface

horizontal rudder is 6x2 feet, with an area of 24 square feet. To the

rear of the main planes is a single surface horizontal plane 6x2 feet,

with an area of 12 square feet. In connection with this is a vertical

rudder 2 1/2 feet square. Two movable ailerons, or balancing planes, are

placed at the extreme ends of the upper planes. These are 6x2 feet,

and have a combined area of 24 square feet. There is also a triangular

shaped vertical steadying surface in connection with the front rudder.

Thus we have a total of 195 square feet, but as the official figures are

258, and the size of the triangular-shaped steadying surface is unknown,

we must take it for granted that this makes up the difference. In

the matter of proportion the horizontal double-plane rudder is about

one-tenth the size of the main plane, counting the surface area of

only one plane, the vertical rudder one-fortieth, and the ailerons

one-twentieth.

# CHAPTER XI. PLANE AND RUDDER CONTROL.

Having constructed and equipped your machine, the next thing is to

decide upon the method of controlling the various rudders and auxiliary

planes by which the direction and equilibrium and ascending and

descending of the machine are governed.

The operator must be in position to shift instantaneously the position

of rudders and planes, and also to control the action of the motor. This

latter is supposed to work automatically and as a general thing does

so with entire satisfaction, but there are times when the supply of

gasolene must be regulated, and similar things done. Airship navigation

calls for quick action, and for this reason the matter of control is an

important one--it is more than important; it is vital.

Several Methods of Control.

Some aviators use a steering wheel somewhat after the style of that used

in automobiles, and by this not only manipulate the rudder planes, but

also the flow of gasolene. Others employ foot levers, and still others,

like the Wrights, depend upon hand levers.

Curtiss steers his aeroplane by means of a wheel, but secures the

desired stabilizing effect with an ingenious jointed chair-back. This is

so arranged that by leaning toward the high point of his wing planes the

aeroplane is restored to an even keel. The steering post of the wheel is

movable backward and forward, and by this motion elevation is obtained.

The Wrights for some time used two hand levers, one to steer by and warp

the flexible tips of the planes, the other to secure elevation. They

have now consolidated all the functions in one lever. Bleriot also uses

the single lever control.

Farman employs a lever to actuate the rudders, but manipulates the

balancing planes by foot levers.

Santos-Dumont uses two hand levers with which to steer and elevate,

but manipulates the planes by means of an attachment to the back of his

outer coat.

Connection With the Levers.

No matter which particular method is employed, the connection between

the levers and the object to be manipulated is almost invariably by

wire. For instance, from the steering levers (or lever) two wires

connect with opposite sides of the rudder. As a lever is moved so as to

draw in the right-hand wire the rudder is drawn to the right and vice

versa. The operation is exactly the same as in steering a boat. It

is the same way in changing the position of the balancing planes. A

movement of the hands or feet and the machine has changed its course,

or, if the equilibrium is threatened, is back on an even keel.

Simple as this seems it calls for a cool head, quick eye, and steady

hand. The least hesitation or a false movement, and both aviator and

craft are in danger.

Which Method is Best?

It would be a bold man who would attempt to pick out any one of these

methods of control and say it was better than the others. As in other

sections of aeroplane mechanism each method has its advocates who dwell

learnedly upon its advantages, but the fact remains that all the various

plans work well and give satisfaction.

What the novice is interested in knowing is how the control is effected,

and whether he has become proficient enough in his manipulation of it to

be absolutely dependable in time of emergency. No amateur should attempt

a flight alone, until he has thoroughly mastered the steering and plane

control. If the services and advice of an experienced aviator are not to

be had the novice should mount his machine on some suitable supports so

it will be well clear of the ground, and, getting into the operator's

seat, proceed to make himself well acquainted with the operation of the

steering wheel and levers.

Some Things to Be Learned.

He will soon learn that certain movements of the steering gear produce

certain effects on the rudders. If, for instance, his machine is

equipped with a steering wheel, he will find that turning the wheel to

the right turns the aeroplane in the same direction, because the tiller

is brought around to the left. In the same way he will learn that a

given movement of the lever throws the forward edge of the main plane

upward, and that the machine, getting the impetus of the wind under the

concave surfaces of the planes, will ascend. In the same way it will

quickly become apparent to him that an opposite movement of the lever

will produce an opposite effect--the forward edges of the planes will be

lowered, the air will be "spilled" out to the rear, and the machine will

descend.

The time expended in these preliminary lessons will be well spent. It

would be an act of folly to attempt to actually sail the craft without

them.

# CHAPTER XII. HOW TO USE THE MACHINE.

It is a mistaken idea that flying machines must be operated at extreme

altitudes. True, under the impetus of handsome prizes, and the incentive

to advance scientific knowledge, professional aviators have ascended to

considerable heights, flights at from 500 to 1,500 feet being now

common with such experts as Farman, Bleriot, Latham, Paulhan, Wright and

Curtiss. The altitude record at this time is about 4,165 feet, held by

Paulhan.

One of the instructions given by experienced aviators to pupils, and for

which they insist upon implicit obeyance, is: "If your machine gets more

than 30 feet high, or comes closer to the ground than 6 feet, descend at

once." Such men as Wright and Curtiss will not tolerate a violation of

this rule. If their instructions are not strictly complied with they

decline to give the offender further lessons.

Why This Rule Prevails.

There is good reason for this precaution. The higher the altitude the

more rarefied (thinner) becomes the air, and the less sustaining power

it has. Consequently the more difficult it becomes to keep in suspension

a given weight. When sailing within 30 feet of the ground sustentation

is comparatively easy and, should a fall occur, the results are not

likely to be serious. On the other hand, sailing too near the ground

is almost as objectionable in many ways as getting up too high. If the

craft is navigated too close to the ground trees, shrubs, fences and

other obstructions are liable to be encountered. There is also the

handicap of contrary air currents diverted by the obstructions referred

to, and which will be explained more fully further on.

How to Make a Start.

Taking it for granted that the beginner has familiarized himself with

the manipulation of the machine, and especially the control mechanism,

the next thing in order is an actual flight. It is probable that his

machine will be equipped with a wheeled alighting gear, as the skids

used by the Wrights necessitate the use of a special starting track.

In this respect the wheeled machine is much easier to handle so far as

novices are concerned as it may be easily rolled to the trial grounds.

This, as in the case of the initial experiments, should be a clear,

reasonably level place, free from trees, fences, rocks and similar

obstructions with which there may be danger of colliding.

The beginner will need the assistance of three men. One of these should

take his position in the rear of the machine, and one at each end. On

reaching the trial ground the aviator takes his seat in the machine and,

while the men at the ends hold it steady the one in the rear assists in

retaining it until the operator is ready. In the meantime the aviator

has started his motor. Like the glider the flying machine, in order to

accomplish the desired results, should be headed into the wind.

When the Machine Rises.

Under the impulse of the pushing movement, and assisted by the motor

action, the machine will gradually rise from the ground--provided it

has been properly proportioned and put together, and everything is in

working order. This is the time when the aviator requires a cool head,

At a modest distance from the ground use the control lever to bring the

machine on a horizontal level and overcome the tendency to rise. The

exact manipulation of this lever depends upon the method of control

adopted, and with this the aviator is supposed to have thoroughly

familiarized himself as previously advised in Chapter XI.

It is at this juncture that the operator must act promptly, but with the

perfect composure begotten of confidence. One of the great drawbacks in

aviation by novices is the tendency to become rattled, and this is much

more prevalent than one might suppose, even among men who, under other

conditions, are cool and confident in their actions.

There is something in the sensation of being suddenly lifted from the

ground, and suspended in the air that is disconcerting at the start,

but this will soon wear off if the experimenter will keep cool. A few

successful flights no matter how short they may be, will put a lot of

confidence into him.

Make Your Flights Short.

Be modest in your initial flights. Don't attempt to match the records

of experienced men who have devoted years to mastering the details of

aviation. Paulhan, Farman, Bleriot, Wright, Curtiss, and all the rest of

them began, and practiced for years, in the manner here described, being

content to make just a little advancement at each attempt. A flight of

150 feet, cleanly and safely made, is better as a beginning than one of

400 yards full of bungling mishaps.

And yet these latter have their uses, provided the operator is of a

discerning mind and can take advantage of them as object lessons. But,

it is not well to invite them. They will occur frequently enough under

the most favorable conditions, and it is best to have them come later

when the feeling of trepidation and uncertainty as to what to do has

worn off.

Above all, don't attempt to fly too high. Keep within a reasonable

distance from the ground--about 25 or 30 feet. This advice is not given

solely to lessen the risk of serious accident in case of collapse, but

mainly because it will assist to instill confidence in the operator.

It is comparatively easy to learn to swim in shallow water, but the

knowledge that one is tempting death in deep water begets timidity.

Preserving the Equilibrium.

After learning how to start and stop, to ascend and descend, the next

thing to master is the art of preserving equilibrium, the knack of

keeping the machine perfectly level in the air--on an "even keel," as

a sailor would say. This simile is particularly appropriate as all

aviators are in reality sailors, and much more daring ones than those

who course the seas. The latter are in craft which are kept afloat by

the buoyancy of the water, whether in motion or otherwise and, so long

as normal conditions prevail, will not sink. Aviators sail the air in

craft in which constant motion must be maintained in order to ensure

flotation.

The man who has ridden a bicycle or motorcycle around curves at anything

like high speed, will have a very good idea as to the principle of

maintaining equilibrium in an airship. He knows that in rounding curves

rapidly there is a marked tendency to change the direction of the motion

which will result in an upset unless he overcomes it by an inclination

of his body in an opposite direction. This is why we see racers lean

well over when taking the curves. It simply must be done to preserve the

equilibrium and avoid a spill.

How It Works In the Air.

If the equilibrium of an airship is disturbed to an extent which

completely overcomes the center of gravity it falls according to the

location of the displacement. If this displacement, for instance, is at

either end the apparatus falls endways; if it is to the front or rear,

the fall is in the corresponding direction.

Owing to uncertain air currents--the air is continually shifting and

eddying, especially within a hundred feet or so of the earth--the

equilibrium of an airship is almost constantly being disturbed to some

extent. Even if this disturbance is not serious enough to bring on

a fall it interferes with the progress of the machine, and should

be overcome at once. This is one of the things connected with aerial

navigation which calls for prompt, intelligent action.

Frequently, when the displacement is very slight, it may be overcome,

and the craft immediately righted by a mere shifting of the operator's

body. Take, for illustration, a case in which the extreme right end

of the machine becomes lowered a trifle from the normal level. It is

possible to bring it back into proper position by leaning over to the

left far enough to shift the weight to the counter-balancing point. The

same holds good as to minor front or rear displacements.

When Planes Must Be Used.

There are other displacements, however, and these are the most frequent,

which can be only overcome by manipulation of the stabilizing planes.

The method of procedure depends upon the form of machine in use. The

Wright machine, as previously explained, is equipped with plane ends

which are so contrived as to admit of their being warped (position

changed) by means of the lever control. These flexible tip planes move

simultaneously, but in opposite directions. As those on one end rise,

those on the other end fall below the level of the main plane. By this

means air is displaced at one point, and an increased amount secured in

another.

This may seem like a complicated system, but its workings are simple

when once understood. It is by the manipulation or warping of these

flexible tips that transverse stability is maintained, and any tendency

to displacement endways is overcome. Longitudinal stability is governed

by means of the front rudder.

Stabilizing planes of some form are a feature, and a necessary feature,

on all flying machines, but the methods of application and manipulation

vary according to the individual ideas of the inventors. They all tend,

however, toward the same end--the keeping of the machine perfectly level

when being navigated in the air.

When to Make a Flight.

A beginner should never attempt to make a flight when a strong wind is

blowing. The fiercer the wind, the more likely it is to be gusty and

uncertain, and the more difficult it will be to control the machine.

Even the most experienced and daring of aviators find there is a limit

to wind speed against which they dare not compete. This is not because

they lack courage, but have the sense to realize that it would be silly

and useless.

The novice will find a comparatively still day, or one when the wind is

blowing at not to exceed 15 miles an hour, the best for his experiments.

The machine will be more easily controlled, the trip will be safer, and

also cheaper as the consumption of fuel increases with the speed of the

wind against which the aeroplane is forced.

# CHAPTER XIII. PECULIARITIES OF AIRSHIP POWER.

As a general proposition it takes much more power to propel an airship

a given number of miles in a certain time than it does an automobile

carrying a far heavier load. Automobiles with a gross load of 4,000

pounds, and equipped with engines of 30 horsepower, have travelled

considerable distances at the rate of 50 miles an hour. This is an

equivalent of about 134 pounds per horsepower. For an average modern

flying machine, with a total load, machine and passengers, of 1,200

pounds, and equipped with a 50-horsepower engine, 50 miles an hour

is the maximum. Here we have the equivalent of exactly 24 pounds per

horsepower. Why this great difference?

No less an authority than Mr. Octave Chanute answers the question in a

plain, easily understood manner. He says:

"In the case of an automobile the ground furnishes a stable support;

in the case of a flying machine the engine must furnish the support and

also velocity by which the apparatus is sustained in the air."

Pressure of the Wind.

Air pressure is a big factor in the matter of aeroplane horsepower.

Allowing that a dead calm exists, a body moving in the atmosphere

creates more or less resistance. The faster it moves, the greater is

this resistance. Moving at the rate of 60 miles an hour the resistance,

or wind pressure, is approximately 50 pounds to the square foot of

surface presented. If the moving object is advancing at a right angle

to the wind the following table will give the horsepower effect of the

resistance per square foot of surface at various speeds.

Horse Power

Miles per Hour    per sq. foot

10             0.013

15             0 044

20             0.105

25             0.205

30             0.354

40             0.84

50             1.64

60             2.83

80             6.72

100            13.12

While the pressure per square foot at 60 miles an hour, is only 1.64

horsepower, at 100 miles, less than double the speed, it has increased

to 13.12 horsepower, or exactly eight times as much. In other words the

pressure of the wind increases with the square of the velocity. Wind at

10 miles an hour has four times more pressure than wind at 5 miles an

hour.

How to Determine Upon Power.

This element of air resistance must be taken into consideration in

determining the engine horsepower required. When the machine is under

headway sufficient to raise it from the ground (about 20 miles an hour),

each square foot of surface resistance, will require nearly nine-tenths

of a horsepower to overcome the wind pressure, and propel the machine

through the air. As shown in the table the ratio of power required

increases rapidly as the speed increases until at 60 miles an hour

approximately 3 horsepower is needed.

In a machine like the Curtiss the area of wind-exposed surface is about

15 square feet. On the basis of this resistance moving the machine at 40

miles an hour would require 12 horsepower. This computation covers only

the machine's power to overcome resistance. It does not cover the power

exerted in propelling the machine forward after the air pressure is

overcome. To meet this important requirement Mr. Curtiss finds it

necessary to use a 50-horsepower engine. Of this power, as has been

already stated, 12 horsepower is consumed in meeting the wind pressure,

leaving 38 horsepower for the purpose of making progress.

The flying machine must move faster than the air to which it is opposed.

Unless it does this there can be no direct progress. If the two forces

are equal there is no straight-ahead advancement. Take, for sake of

illustration, a case in which an aeroplane, which has developed a speed

of 30 miles an hour, meets a wind velocity of equal force moving in an

opposite direction. What is the result? There can be no advance because

it is a contest between two evenly matched forces. The aeroplane stands

still. The only way to get out of the difficulty is for the operator to

wait for more favorable conditions, or bring his machine to the ground

in the usual manner by manipulation of the control system.

Take another case. An aeroplane, capable of making 50 miles an hour in a

calm, is met by a head wind of 25 miles an hour. How much progress does

the aeroplane make? Obviously it is 25 miles an hour over the ground.

Put the proposition in still another way. If the wind is blowing harder

than it is possible for the engine power to overcome, the machine will

be forced backward.

Wind Pressure a Necessity.

While all this is true, the fact remains that wind pressure, up to

a certain stage, is an absolute necessity in aerial navigation. The

atmosphere itself has very little real supporting power, especially if

inactive. If a body heavier than air is to remain afloat it must move

rapidly while in suspension.

One of the best illustrations of this is to be found in skating over

thin ice. Every school boy knows that if he moves with speed he may

skate or glide in safety across a thin sheet of ice that would not

begin to bear his weight if he were standing still. Exactly the same

proposition obtains in the case of the flying machine.

The non-technical reason why the support of the machine becomes easier

as the speed increases is that the sustaining power of the atmosphere

increases with the resistance, and the speed with which the object is

moving increases this resistance. With a velocity of 12 miles an hour

the weight of the machine is practically reduced by 230 pounds. Thus, if

under a condition of absolute calm it were possible to sustain a weight

of 770 pounds, the same atmosphere would sustain a weight of 1,000

pounds moving at a speed of 12 miles an hour. This sustaining power

increases rapidly as the speed increases. While at 12 miles the

sustaining power is figured at 230 pounds, at 24 miles it is four times

as great, or 920 pounds.

Supporting Area of Birds.

One of the things which all producing aviators seek to copy is the

motive power of birds, particularly in their relation to the area of

support. Close investigation has established the fact that the larger

the bird the less is the relative area of support required to secure a

given result. This is shown in the following table:

Supporting

Weight       Surface       Horse      area

Bird         in lbs.     in sq. feet   power     per lb.

Pigeon         1.00      0.7           0.012     0.7

Wild Goose     9.00      2.65          0.026     0.2833

Buzzard        5.00      5.03          0.015     1.06

Condor        17.00      9.85          0.043     0.57

So far as known the condor is the largest of modern birds. It has a wing

stretch of 10 feet from tip to tip, a supporting area of about 10 square

feet, and weighs 17 pounds. It. is capable of exerting perhaps 1-30

horsepower. (These figures are, of course, approximate.) Comparing the

condor with the buzzard with a wing stretch of 6 feet, supporting area

of 5 square feet, and a little over 1-100 horsepower, it may be seen

that, broadly speaking, the larger the bird the less surface area

(relatively) is needed for its support in the air.

Comparison With Aeroplanes.

If we compare the bird figures with those made possible by the

development of the aeroplane it will be readily seen that man has made a

wonderful advance in imitating the results produced by nature. Here are

the figures:

Supporting

Weight      Surface       Horse      area

Machine         in lbs.    in sq. feet    power     per lb.

Santos-Dumont..  350      110.00        30        0.314

Bleriot.....     700      150.00        25        0.214

Antoinette.... 1,200      538.00        50        0.448

Curtiss.....     700      258.00        60        0.368

Wright.....[4] 1,100      538.00        25        0.489

Farman......   1,200      430.00        50        0.358

Voisin......   1,200      538.00        50        0.448

While the average supporting surface is in favor of the aeroplane, this

is more than overbalanced by the greater amount of horsepower required

for the weight lifted. The average supporting surface in birds is about

three-quarters of a square foot per pound. In the average aeroplane it

is about one-half square foot per pound. On the other hand the average

aeroplane has a lifting capacity of 24 pounds per horsepower, while the

buzzard, for instance, lifts 5 pounds with 15-100 of a horsepower.

If the Wright machine--which has a lifting power of 50 pounds per

horsepower--should be alone considered the showing would be much more

favorable to the aeroplane, but it would not be a fair comparison.

More Surface, Less Power.

Broadly speaking, the larger the supporting area the less will be the

power required. Wright, by the use of 538 square feet of supporting

surface, gets along with an engine of 25 horsepower. Curtiss, who uses

only 258 square feet of surface, finds an engine of 50 horsepower is

needed. Other things, such as frame, etc., being equal, it stands

to reason that a reduction in the area of supporting surface will

correspondingly reduce the weight of the machine. Thus we have the

Curtiss machine with its 258 square feet of surface, weighing only 600

pounds (without operator), but requiring double the horsepower of

the Wright machine with 538 square feet of surface and weighing 1,100

pounds. This demonstrates in a forceful way the proposition that the

larger the surface the less power will be needed.

But there is a limit, on account of its bulk and awkwardness in

handling, beyond which the surface area cannot be enlarged. Otherwise

it might be possible to equip and operate aeroplanes satisfactorily with

engines of 15 horsepower, or even less.

The Fuel Consumption Problem.

Fuel consumption is a prime factor in the production of engine power.

The veriest mechanical tyro knows in a general way that the more power

is secured the more fuel must be consumed, allowing that there is no

difference in the power-producing qualities of the material used. But

few of us understand just what the ratio of increase is, or how it is

caused. This proposition is one of keen interest in connection with

aviation.

Let us cite a problem which will illustrate the point quoted: Allowing

that it takes a given amount of gasolene to propel a flying machine a

given distance, half the way with the wind, and half against it, the

wind blowing at one-half the speed of the machine, what will be the

increase in fuel consumption?

Increase of Thirty Per Cent.

On the face of it there would seem to be no call for an increase as the

resistance met when going against the wind is apparently offset by the

propulsive force of the wind when the machine is travelling with

it. This, however, is called faulty reasoning. The increase in fuel

consumption, as figured by Mr. F. W. Lanchester, of the Royal Society of

Arts, will be fully 30 per cent over the amount required for a similar

operation of the machine in still air. If the journey should be made at

right angles to the wind under the same conditions the increase would be

15 per cent.

In other words Mr. Lanchester maintains that the work done by the motor

in making headway against the wind for a certain distance calls for more

engine energy, and consequently more fuel by 30 per cent, than is saved

by the helping force of the wind on the return journey.

# CHAPTER XIV. ABOUT WIND CURRENTS, ETC.

One of the first difficulties which the novice will encounter is the

uncertainty of the wind currents. With a low velocity the wind, some

distance away from the ground, is ordinarily steady. As the velocity

increases, however, the wind generally becomes gusty and fitful in its

action. This, it should be remembered, does not refer to the velocity of

the machine, but to that of the air itself.

In this connection Mr. Arthur T. Atherholt, president of the Aero

Club of Pennsylvania, in addressing the Boston Society of Scientific

Research, said:

"Probably the whirlpools of Niagara contain no more erratic currents

than the strata of air which is now immediately above us, a fact hard to

realize on account of its invisibility."

Changes In Wind Currents.

While Mr. Atherholt's experience has been mainly with balloons it is all

the more valuable on this account, as the balloons were at the mercy of

the wind and their varying directions afforded an indisputable guide as

to the changing course of the air currents. In speaking of this he said:

"In the many trips taken, varying in distance traversed from twenty-five

to 900 miles, it was never possible except in one instance to maintain

a straight course. These uncertain currents were most noticeable in

the Gordon-Bennett race from St. Louis in 1907. Of the nine aerostats

competing in that event, eight covered a more or less direct course

due east and southeast, whereas the writer, with Major Henry B. Hersey,

first started northwest, then north, northeast, east, east by south,

and when over the center of Lake Erie were again blown northwest

notwithstanding that more favorable winds were sought for at altitudes

varying from 100 to 3,000 meters, necessitating a finish in Canada

nearly northeast of the starting point.

"These nine balloons, making landings extending from Lake Ontario,

Canada, to Virginia, all started from one point within the same hour.

"The single exception to these roving currents occurred on October 21st,

of last year (1909) when, starting from Philadelphia, the wind shifted

more than eight degrees, the greatest variation being at the lowest

altitudes, yet at no time was a height of over a mile reached.

"Throughout the entire day the sky was overcast, with a thermometer

varying from fifty-seven degrees at 300 feet to forty-four degrees,

Fahrenheit at 5,000 feet, at which altitude the wind had a velocity of

43 miles an hour, in clouds of a cirro-cumulus nature, a landing finally

being made near Tannersville, New York, in the Catskill mountains, after

a voyage of five and one-half hours.

"I have no knowledge of a recorded trip of this distance and duration,

maintained in practically a straight line from start to finish."

This wind disturbance is more noticeable and more difficult to contend

with in a balloon than in a flying machine, owing to the bulk and

unwieldy character of the former. At the same time it is not conducive

to pleasant, safe or satisfactory sky-sailing in an aeroplane. This is

not stated with the purpose of discouraging aviation, but merely that

the operator may know what to expect and be prepared to meet it.

Not only does the wind change its horizontal course abruptly and without

notice, but it also shifts in a vertical direction, one second blowing

up, and another down. No man has as yet fathomed the why and wherefore

of this erratic action; it is only known that it exists.

The most stable currents will be found from 50 to 100 feet from the

earth, provided the wind is not diverted by such objects as trees,

rocks, etc. That there are equally stable currents higher up is true,

but they are generally to be found at excessive altitudes.

How a Bird Meets Currents.

Observe a bird in action on a windy day and you will find it continually

changing the position of its wings. This is done to meet the varying

gusts and eddies of the air so that sustentation may be maintained and

headway made. One second the bird is bending its wings, altering the

angle of incidence; the next it is lifting or depressing one wing at a

time. Still again it will extend one wing tip in advance of the other,

or be spreading or folding, lowering or raising its tail.

All these motions have a meaning, a purpose. They assist the bird in

preserving its equilibrium. Without them the bird would be just as

helpless in the air as a human being and could not remain afloat.

When the wind is still, or comparatively so, a bird, having secured the

desired altitude by flight at an angle, may sail or soar with no wing

action beyond an occasional stroke when it desires to advance. But, in a

gusty, uncertain wind it must use its wings or alight somewhere.

Trying to Imitate the Bird.

Writing in _Fly_, Mr. William E. White says:

"The bird's flight suggests a number of ways in which the equilibrium

of a mechanical bird may be controlled. Each of these methods of control

may be effected by several different forms of mechanism.

"Placing the two wings of an aeroplane at an angle of three to five

degrees to each other is perhaps the oldest way of securing lateral

balance. This way readily occurs to anyone who watches a sea gull

soaring. The theory of the dihedral angle is that when one wing is

lifted by a gust of wind, the air is spilled from under it; while

the other wing, being correspondingly depressed, presents a greater

resistance to the gust and is lifted restoring the balance. A fixed

angle of three to five degrees, however, will only be sufficient for

very light puffs of wind and to mount the wings so that the whole

wing may be moved to change the dihedral angle presents mechanical

difficulties which would be better avoided.

"The objection of mechanical impracticability applies to any plan

to preserve the balance by shifting weight or ballast. The center of

gravity should be lower than the center of the supporting surfaces,

but cannot be made much lower. It is a common mistake to assume that

complete stability will be secured by hanging the center of gravity very

low on the principle of the parachute. An aeroplane depends upon rapid

horizontal motion for its support, and if the center of gravity be far

below the center of support, every change of speed or wind pressure will

cause the machine to turn about its center of gravity, pitching forward

and backward dangerously.

Preserving Longitudinal Balance.

"The birds maintain longitudinal, or fore and aft balance, by elevating

or depressing their tails. Whether this action is secured in an

aeroplane by means of a horizontal rudder placed in the rear, or by

deflecting planes placed in front of the main planes, the principle is

evidently the same. A horizontal rudder placed well to the rear as in

the Antoinette, Bleriot or Santos-Dumont monoplanes, will be very much

safer and steadier than the deflecting planes in front, as in the Wright

or Curtiss biplanes, but not so sensitive or prompt in action.

"The natural fore and aft stability is very much strengthened by placing

the load well forward. The center of gravity near the front and a

tail or rudder streaming to the rear secures stability as an arrow is

balanced by the head and feathering. The adoption of this principle

makes it almost impossible for the aeroplane to turn over.

The Matter of Lateral Balance.

"All successful aeroplanes thus far have maintained lateral balance by

the principle of changing the angle of incidence of the wings.

"Other ways of maintaining the lateral balance, suggested by observation

of the flight of birds are--extending the wing tips and spilling the air

through the pinions; or, what is the same thing, varying the area of the

wings at their extremities.

"Extending the wing tips seems to be a simple and effective solution of

the problem. The tips may be made to swing outward upon a vertical axis

placed at the front edge of the main planes; or they may be hinged to

the ends of the main plane so as to be elevated or depressed through

suitable connections by the aviator; or they may be supported from a

horizontal axis parallel with the ends of the main planes so that they

may swing outward, the aviator controlling both tips through one lever

so that as one tip is extended the other is retracted.

"The elastic wing pinions of a bird bend easily before the wind,

permitting the gusts to glance off, but presenting always an even and

efficient curvature to the steady currents of the air."

High Winds Threaten Stability.

To ensure perfect stability, without control, either human or automatic,

it is asserted that the aeroplane must move faster than the wind is

blowing. So long as the wind is blowing at the rate of 30 miles an hour,

and the machine is traveling 40 or more, there will be little trouble as

regards equilibrium so far as wind disturbance goes, provided the wind

blows evenly and does not come in gusts or eddying currents. But when

conditions are reversed--when the machine travels only 30 miles an hour

and the wind blows at the rate of 50, look out for loss of equilibrium.

One of the main reasons for this is that high winds are rarely steady;

they seldom blow for any length of time at the same speed. They are

usually "gusty," the gusts being a momentary movement at a higher speed.

Tornadic gusts are also formed by the meeting of two opposing currents,

causing a whirling motion, which makes stability uncertain. Besides, it

is not unusual for wind of high speed to suddenly change its direction

without warning.

Trouble With Vertical Columns.

Vertical currents--columns of ascending air--are frequently encountered

in unexpected places and have more or less tendency, according to their

strength, to make it difficult to keep the machine within a reasonable

distance from the ground.

These vertical currents are most generally noticeable in the vicinity

of steep cliffs, or deep ravines. In such instances they are usually of

considerable strength, being caused by the deflection of strong winds

blowing against the face of the cliffs. This deflection exerts a back

pressure which is felt quite a distance away from the point of origin,

so that the vertical current exerts an influence in forcing the machine

upward long before the cliff is reached.

# CHAPTER XV. THE ELEMENT OF DANGER.

That there is an element of danger in aviation is undeniable, but it is

nowhere so great as the public imagines. Men are killed and injured in

the operation of flying machines just as they are killed and injured

in the operation of railways. Considering the character of aviation the

percentage of casualties is surprisingly small.

This is because the results following a collapse in the air are very

much different from what might be imagined. Instead of dropping to the

ground like a bullet an aeroplane, under ordinary conditions will, when

anything goes wrong, sail gently downward like a parachute, particularly

if the operator is cool-headed and nervy enough to so manipulate the

apparatus as to preserve its equilibrium and keep the machine on an even

keel.

Two Fields of Safety.

At least one prominent aviator has declared that there are two fields

of safety--one close to the ground, and the other well up in the air. In

the first-named the fall will be a slight one with little chance of the

operator being seriously hurt. From the field of high altitude the the

descent will be gradual, as a rule, the planes of the machine serving to

break the force of the fall. With a cool-headed operator in control the

aeroplane may be even guided at an angle (about 1 to 8) in its descent

so as to touch the ground with a gliding motion and with a minimum of

impact.

Such an experience, of course, is far from pleasant, but it is by no

means so dangerous as might appear. There is more real danger in falling

from an elevation of 75 or 100 feet than there is from 1,000 feet, as

in the former case there is no chance for the machine to serve as a

parachute--its contact with the ground comes too quickly.

Lesson in Recent Accidents.

Among the more recent fatalities in aviation are the deaths of Antonio

Fernandez and Leon Delagrange. The former was thrown to the ground by a

sudden stoppage of his motor, the entire machine seeming to collapse. It

is evident there were radical defects, not only in the motor, but in the

aeroplane framework as well. At the time of the stoppage it is

estimated that Fernandez was up about 1,500 feet, but the machine got

no opportunity to exert a parachute effect, as it broke up immediately.

This would indicate a fatal weakness in the structure which, under

proper testing, could probably have been detected before it was used in

flight.

It is hard to say it, but Delagrange appears to have been culpable to

great degree in overloading his machine with a motor equipment much

heavier than it was designed to sustain. He was 65 feet up in the air

when the collapse occurred, resulting in his death. As in the case of

Fernandez common-sense precaution would doubtless have prevented the

fatality.

Aviation Not Extra Hazardous.

All told there have been, up to the time of this writing (April, 1910),

just five fatalities in the history of power-driven aviation. This is

surprisingly low when the nature of the experiments, and the fact that

most of the operators were far from having extended experience, is taken

into consideration. Men like the Wrights, Curtiss, Bleriot, Farman,

Paulhan and others, are now experts, but there was a time, and it was

not long ago, when they were unskilled. That they, with numerous

others less widely known, should have come safely through their many

experiments would seem to disprove the prevailing idea that aviation is

an extra hazardous pursuit.

In the hands of careful, quick-witted, nervy men the sailing of an

airship should be no more hazardous than the sailing of a yacht. A

vessel captain with common sense will not go to sea in a storm, or

navigate a weak, unseaworthy craft. Neither should an aviator attempt to

sail when the wind is high and gusty, nor with a machine which has not

been thoroughly tested and found to be strong and safe.

Safer Than Railroading.

Statistics show that some 12,000 people are killed and 72,000 injured

every year on the railroads of the United States. Come to think it over

it is small wonder that the list of fatalities is so large. Trains

are run at high speeds, dashing over crossings at which collisions are

liable to occur, and over bridges which often collapse or are swept away

by floods. Still, while the number of casualties is large, the actual

percentage is small considering the immense number of people involved.

It is so in aviation. The number of casualties is remarkably small in

comparison with the number of flights made. In the hands of competent

men the sailing of an airship should be, and is, freer from risk of

accident than the running of a railway train. There are no rails

to spread or break, no bridges to collapse, no crossings at which

collisions may occur, no chance for some sleepy or overworked employee

to misunderstand the dispatcher's orders and cause a wreck.

Two Main Causes of Trouble.

The two main causes of trouble in an airship leading to disaster may

be attributed to the stoppage of the motor, and the aviator becoming

rattled so that he loses control of his machine. Modern ingenuity is

fast developing motors that almost daily become more and more reliable,

and experience is making aviators more and more self-confident in their

ability to act wisely and promptly in cases of emergency. Besides this

a satisfactory system of automatic control is in a fair way of being

perfected.

Occasionally even the most experienced and competent of men in all

callings become careless and by foolish action invite disaster. This

is true of aviators the same as it is of railroaders, men who work in

dynamite mills, etc. But in nearly every instance the responsibility

rests with the individual; not with the system. There are some men

unfitted by nature for aviation, just as there are others unfitted to be

railway engineers.

# CHAPTER XVI. RADICAL CHANGES BEING MADE.

Changes, many of them extremely radical in their nature, are continually

being made by prominent aviators, and particularly those who have won

the greatest amount of success. Wonderful as the results have been

few of the aviators are really satisfied. Their successes have merely

spurred them on to new endeavors, the ultimate end being the development

of an absolutely perfect aircraft.

Among the men who have been thus experimenting are the Wright Brothers,

who last year (1909) brought out a craft totally different as regards

proportions and weight from the one used the preceding year. One marked

result was a gain of about 3 1/2 miles an hour in speed.

Dimensions of 1908 Machine.

The 1908 model aeroplane was 40 by 29 feet over all. The carrying

surfaces, that is, the two aerocurves, were 40 by 6 feet, having a

parabolical curve of one in twelve. With about 70 square feet of surface

in the rudders, the total surface given was about 550 square feet.

The engine, which is the invention of the Wright brothers, weighed,

approximately, 200 pounds, and gave about 25 horsepower at 1,400

revolutions per minute. The total weight of the aeroplane, exclusive of

passenger, but inclusive of engine, was about 1,150 pounds. This result

showed a lift of a fraction over 2 1/4 pounds to the square foot of

carrying surface. The speed desired was 40 miles an hour, but the

machine was found to make only a scant 39 miles an hour. The upright

struts were about 7/8-inch thick, the skids, 2 1/2 by 1 1/4 inches

thick.

Dimensions of 1909 Machine.

The 1909 aeroplane was built primarily for greater speed, and relatively

heavier; to be less at the mercy of the wind. This result was obtained

as follows: The aerocurves, or carrying surfaces, were reduced in

dimensions from 40 by 6 feet to 36 by 5 1/2 feet, the curve remaining

the same, one in twelve. The upright struts were cut from seven-eighths

inch to five-eighths inch, and the skids from two and one-half by one

and one-quarter to two and one-quarter by one and three-eighths inches.

This result shows that there were some 81 square feet of carrying

surface missing over that of last year's model. and some 25 pounds loss

of weight. Relatively, though, the 1909 model aeroplane, while actually

25 pounds lighter, is really some 150 pounds heavier in the air than the

1908 model, owing to the lesser square feet of carrying surface.

Some of the Results Obtained.

Reducing the carrying surfaces from 6 to 5 1/2 feet gave two

results--first, less carrying capacity; and, second, less head-on

resistance, owing to the fact that the extent of the parabolic curve

in the carrying surfaces was shortened. The "head-on" resistance is

the retardance the aeroplane meets in passing through the air, and is

counted in square feet. In the 1908 model the curve being one in twelve

and 6 feet deep, gave 6 inches of head-on resistance. The plane being

40 feet spread, gave 6 inches by 40 feet, or 20 square feet of head-on

resistance. Increasing this figure by a like amount for each plane, and

adding approximately 10 square feet for struts, skids and wiring, we

have a total of approximately, 50 square feet of surface for "head-on"

resistance.

In the 1909 aeroplane, shortening the curve 6 inches at the parabolic

end of the curve took off 1 inch of head-on resistance. Shortening the

spread of the planes took off between 3 and 4 square feet of head-on

resistance. Add to this the total of 7 square feet, less curve surface

and about 1 square foot, less wire and woodwork resistance, and we

have a grand total of, approximately, 12 square feet of less "head-on"

resistance over the 1908 model.

Changes in Engine Action.

The engine used in 1909 was the same one used in 1908, though some minor

changes were made as improvements; for instance, a make and break spark

was used, and a nine-tooth, instead of a ten-tooth magneto gear-wheel

was used. This increased the engine revolutions per minute from 1,200 to

1,400, and the propeller revolutions per minute from 350 to 371, giving

a propeller thrust of, approximately, 170 foot pounds instead of 153, as

was had last year.

More Speed and Same Capacity.

One unsatisfactory feature of the 1909 model over that of 1908,

apparently, was the lack of inherent lateral stability. This was caused

by the lesser surface and lesser extent of curvatures at the portions

of the aeroplane which were warped. This defect did not show so plainly

after Mr. Orville Wright had become fully proficient in the handling of

the new machine, and with skillful management, the 1909 model aeroplane

will be just as safe and secure as the other though it will take a

little more practice to get that same degree of skill.

To sum up: The aeroplane used in 1909 was 25 pounds lighter, but really

about 150 pounds heavier in the air, had less head-on resistance, and

greater propeller thrust. The speed was increased from about 39 miles

per hour to 42 1/2 miles per hour. The lifting capacity remained about

the same, about 450 pounds capacity passenger-weight, with the 1908

machine. In this respect, the loss of carrying surface was compensated

for by the increased speed.

During the first few flights it was plainly demonstrated that it would

need the highest skill to properly handle the aeroplane, as first one

end and then the other would dip and strike the ground, and either tear

the canvas or slew the aeroplane around and break a skid.

Wrights Adopt Wheeled Gears.

In still another important respect the Wrights, so far as the output

of one of their companies goes, have made a radical change. All the

aeroplanes turned out by the Deutsch Wright Gesellschaft, according to

the German publication, _Automobil-Welt_, will hereafter be equipped

with wheeled running gears and tails. The plan of this new machine is

shown in the illustration on page 145. The wheels are three in number,

and are attached one to each of the two skids, just under the front edge

of the planes, and one forward of these, attached to a cross-member. It

is asserted that with these wheels the teaching of purchasers to operate

the machines is much simplified, as the beginners can make short flights

on their own account without using the starting derrick.

This is a big concession for the Wrights to make, as they have hitherto

adhered stoutly to the skid gear. While it is true they do not control

the German company producing their aeroplanes, yet the nature of their

connection with the enterprise is such that it may be taken for granted

no radical changes in construction would be made without their approval

and consent.

Only Three Dangerous Rivals.

Official trials with the 1909 model smashed many records and leave the

Wright brothers with only three dangerous rivals in the field, and with

basic patents which cover the curve, warp and wing-tip devices found on

all the other makes of aeroplanes. These three rivals are the Curtiss

and Voisin biplane type and the Bleriot monoplane pattern.

The Bleriot monoplane is probably the most dangerous rival, as this make

of machine has a record of 54 miles per hour, has crossed the English

channel, and has lifted two passengers besides the operator. The

latest type of this machine only weighs 771.61 pounds complete, without

passengers, and will lift a total passenger weight of 462.97 pounds,

which is a lift of 5.21 pounds to the square foot. This is a better

result than those published by the Wright brothers, the best noted being

4.25 pounds per square foot.

Other Aviators at Work.

The Wrights, however, are not alone in their efforts to promote the

efficiency of the flying machine. Other competent inventive aviators,

notably Curtiss, Voisin, Bleriot and Farman, are close after them. The

Wrights, as stated, have a marked advantage in the possession of

patents covering surface plane devices which have thus far been found

indispensable in flying machine construction. Numerous law suits growing

out of alleged infringements of these patents have been started, and

others are threatened. What effect these actions will have in deterring

aviators in general from proceeding with their experiments remains to be

seen.

In the meantime the four men named--Curtiss, Voisin, Bleriot and

Farman--are going ahead regardless of consequences, and the inventive

genius of each is so strong that it is reasonable to expect some

remarkable developments in the near future.

Smallest of Flying Machines.

To Santos Dumont must be given the credit of producing the smallest

practical flying machine yet constructed. True, he has done nothing

remarkable with it in the line of speed, but he has demonstrated the

fact that a large supporting surface is not an essential feature.

This machine is named "La Demoiselle." It is a monoplane of the dihedral

type, with a main plane on each side of the center. These main

planes are of 18 foot spread, and nearly 6 1/2 feet in depth, giving

approximately 115 feet of surface area. The total weight is 242

pounds, which is 358 pounds less than any other machine which has been

successfully used. The total depth from front to rear is 26 feet.

The framework is of bamboo, strengthened and held taut with wire guys.

Have One Rule in Mind.

In this struggle for mastery in flying machine efficiency all the

contestants keep one rule in mind, and this is:

"The carrying capacity of an aeroplane is governed by the peripheral

curve of its carrying surfaces, plus the speed; and the speed

is governed by the thrust of the propellers, less the 'head-on'

resistance."

Their ideas as to the proper means of approaching the proposition

may, and undoubtedly are, at variance, but the one rule in solving the

problem of obtaining the greatest carrying capacity combined with the

greatest speed, obtains in all instances.

# CHAPTER XVII. SOME OF THE NEW DESIGNS.

Spurred on by the success attained by the more experienced and better

known aviators numerous inventors of lesser fame are almost daily

producing practical flying machines varying radically in construction

from those now in general use.

One of these comparatively new designs is the Van Anden biplane, made

by Frank Van Anden of Islip, Long Island, a member of the New York

Aeronautic Society. While his machine is wholly experimental, many

successful short flights were made with it last fall (1909). One flight,

made October 19th, 1909, is of particular interest as showing the

practicability of an automatic stabilizing device installed by the

inventor. The machine was caught in a sudden severe gust of wind and

keeled over, but almost immediately righted itself, thus demonstrating

in a most satisfactory manner the value of one new attachment.

Features of Van Anden Model.

In size the surfaces of the main biplane are 26 feet in spread, and 4

feet in depth from front to rear. The upper and lower planes are 4 feet

apart. Silkolene coated with varnish is used for the coverings. Ribs

(spruce) are curved one inch to the foot, the deepest part of the curve

(4 inches) being one foot back from the front edge of the horizontal

beam. Struts (also of spruce, as is all the framework) are elliptical in

shape. The main beams are in three sections, nearly half round in form,

and joined by metal sleeves.

There is a two-surface horizontal rudder, 2x2x4 feet, in front. This

is pivoted at its lateral center 8 feet from the front edge of the main

planes. In the rear is another two-surface horizontal rudder 2x2x2 1/2

feet, pivoted in the same manner as the front one, 15 feet from the rear

edges of the main planes.

Hinged to the rear central strut of the rear rudder is a vertical rudder

2 feet high by 3 feet in length.

The Method of Control.

In the operation of these rudders--both front and rear--and the

elevation and depression of the main planes, the Curtiss system is

employed. Pushing the steering-wheel post outward depresses the front

edges of the planes, and brings the machine downward; pulling the

steering-wheel post inward elevates the front edges of the planes and

causes the machine to ascend.

Turning the steering wheel itself to the right swings the tail rudder to

the left, and the machine, obeying this like a boat, turns in the same

direction as the wheel is turned. By like cause turning the wheel to the

left turns the machine to the left.

Automatic Control of Wings.

There are two wing tips, each of 6 feet spread (length) and 2 feet from

front to rear. These are hinged half way between the main surfaces to

the two outermost rear struts. Cables run from these to an automatic

device working with power from the engine, which automatically operates

the tips with the tilting of the machine. Normally the wing tips are

held horizontal by stiff springs introduced in the cables outside of the

device.

It was the successful working of this device which righted the Van

Anden craft when it was overturned in the squall of October 19th, 1909.

Previous to that occurrence Mr. Van Anden had looked upon the device as

purely experimental, and had admitted that he had grave uncertainty as

to how it would operate in time of emergency. He is now quoted as being

thoroughly satisfied with its practicability. It is this automatic

device which gives the Van Anden machine at least one distinctively new

feature.

While on this subject it will not be amiss to add that Mr. Curtiss does

not look kindly on automatic control. "I would rather trust to my own

action than that of a machine," he says. This is undoubtedly good

logic so far as Mr. Curtiss is concerned, but all aviators are not so

cool-headed and resourceful.

Motive Power of Van Anden.

A 50-horsepower "H-F" water cooled motor drives a laminated wood

propeller 6 feet in diameter, with a 17 degree pitch at the extremities,

increasing toward the hub. The rear end of the motor is about 6 inches

back from the rear transverse beam and the engine shaft is in a direct

line with the axes of the two horizontal rudders. An R. I. V. ball

bearing carries the shaft at this point. Flying, the motor turns at

about 800 revolutions per minute, delivering 180 pounds pull. A test of

the motor running at 1,200 showed a pull of 250 pounds on the scales.

Still Another New Aeroplane.

Another new aeroplane is that produced by A. M. Herring (an old-timer)

and W. S. Burgess, under the name of the Herring-Burgess. This is also

equipped with an automatic stability device for maintaining the balance

transversely. The curvature of the planes is also laid out on new

lines. That this new plan is effective is evidenced by the fact that the

machine has been elevated to an altitude of 40 feet by using one-half

the power of the 30-horsepower motor.

The system of rudder and elevation control is very simple. The aviator

sits in front of the lower plane, and extending his arms, grasps two

supports which extend down diagonally in front. On the under side of

these supports just beneath his fingers are the controls which operate

the vertical rudder, in the rear. Thus, if he wishes to turn to the

right, he presses the control under the fingers of his right hand; if to

the left, that under the fingers of his left hand. The elevating rudder

is operated by the aviator's right foot, the control being placed on a

foot-rest.

Motor Is Extremely Light.

Not the least notable feature of the craft is its motor. Although

developing, under load, 30-horsepower, or that of an ordinary

automobile, it weighs, complete, hardly 100 pounds. Having occasion to

move it a little distance for inspection, Mr. Burgess picked it up and

walked off with it--cylinders, pistons, crankcase and all, even the

magneto, being attached. There are not many 30-horsepower engines which

can be so handled. Everything about it is reduced to its lowest terms

of simplicity, and hence, of weight. A single camshaft operates not

only all of the inlet and exhaust valves, but the magneto and gear water

pump, as well. The motor is placed directly behind the operator, and the

propeller is directly mounted on the crankshaft.

This weight of less than 100 pounds, it must be remembered, is not for

the motor alone; it includes the entire power plant equipment.

The "thrust" of the propeller is also extraordinary, being between 250

and 260 pounds. The force of the wind displacement is strong enough to

knock down a good-sized boy as one youngster ascertained when he got

behind the propeller as it was being tested. He was not only knocked

down but driven for some distance away from the machine. The propeller

has four blades which are but little wider than a lath.

Machine Built by Students.

Students at the University of Pennsylvania, headed by Laurence J. Lesh,

a protege of Octave Chanute, have constructed a practical aeroplane of

ordinary maximum size, in which is incorporated many new ideas. The most

unique of these is to be found in the steering gear, and the provision

made for the accommodation of a pupil while taking lessons under an

experienced aviator.

Immediately back of the aviator is an extra seat and an extra steering

wheel which works in tandem style with the front wheel. By this

arrangement a beginner may be easily and quickly taught to have

perfect control of the machine. These tandem wheels are also handy for

passengers who may wish to operate the car independently of one another,

it being understood, of course, that there will be no conflict of

action.

Frame Size and Engine Power.

The frame has 36 feet spread and measures 35 feet from the front edge to

the end of the tail in the rear. It is equipped with two rear propellers

operated by a Ramsey 8-cylinder motor of 50 horsepower, placed

horizontally across the lower plane, with the crank shaft running clear

through the engine.

The "Pennsylvania I" is the first two-propeller biplane chainless

car, this scheme having been adopted in order to avoid the crossing of

chains. The lateral control is by a new invention by Octave Chanute

and Laurence J. Lesh, for which Lesh is now applying for a patent. The

device was worked out before the Wright brothers' suit was begun, and is

said to be superior to the Wright warping or the Curtiss ailerons. The

landing device is also new in design. This aeroplane will weigh about

1,500 pounds, and will carry fuel for a flight of 150 miles, and it is

expected to attain a speed of at least 45 miles an hour.

There are others, lots of them, too numerous in fact to admit of mention

in a book of this size.

# CHAPTER XVIII. DEMAND FOR FLYING MACHINES.

As a commercial proposition the manufacture and sale of motor-equipped

aeroplanes is making much more rapid advance than at first obtained in

the similar handling of the automobile. Great, and even phenomenal,

as was the commercial development of the motor car, that of the flying

machine is even greater. This is a startling statement, but it is fully

warranted by the facts.

It is barely more than a year ago (1909) that attention was seriously

attracted to the motor-equipped aeroplane as a vehicle possible of

manipulation by others than professional aviators. Up to that time

such actual flights as were made were almost exclusively with the sole

purpose of demonstrating the practicability of the machine, and the

merits of the ideas as to shape, engine power, etc., of the various

producers.

Results of Bleriot's Daring.

It was not until Bleriot flew across the straits of Dover on July 25th,

1909, that the general public awoke to a full realization of the fact

that it was possible for others than professional aviators to indulge

in aviation. Bleriot's feat was accepted as proof that at last

an absolutely new means of sport, pleasure and research, had been

practically developed, and was within the reach of all who had the

inclination, nerve and financial means to adopt it.

From this event may be dated the birth of the modern flying machine into

the world of business. The automobile was taken up by the general public

from the very start because it was a proposition comparatively easy of

demonstration. There was nothing mysterious or uncanny in the fact that

a wheeled vehicle could be propelled on solid, substantial roads by

means of engine power. And yet it took (comparatively speaking) a long

time to really popularize the motor car.

Wonderful Results in a Year.

Men of large financial means engaged in the manufacture of automobiles,

and expended fortunes in attracting public attention to them through the

medium of advertisements, speed and road contests, etc. By these means

a mammoth business has been built up, but bringing this business to its

present proportions required years of patient industry and indomitable

pluck.

At this writing, less than a year from the day when Bleriot crossed the

channel, the actual sales of flying machines outnumber the actual sales

of automobiles in the first year of their commercial development. This

may appear incredible, but it is a fact as statistics will show.

In this connection we should take into consideration the fact that up to

a year ago there was no serious intention of putting flying machines

on the market; no preparations had been made to produce them on a

commercial scale; no money had been expended in advertisements with a

view to selling them.

Some of the Actual Results.

Today flying machines are being produced on a commercial basis, and

there is a big demand for them. The people making them are overcrowded

with orders. Some of the producers are already making arrangements to

enlarge their plants and advertise their product for sale the same as

is being done with automobiles, while a number of flying machine motor

makers are already promoting the sale of their wares in this way.

Here are a few actual figures of flying machine sales made by the more

prominent producers since July 25th, 1909.

Santos Dumont, 90 machines; Bleriot, 200; Farman, 130;

Clemenceau-Wright, 80; Voisin, 100; Antoinette, 100. Many of these

orders have been filled by delivery of the machines, and in others the

construction work is under way.

The foregoing are all of foreign make. In this country Curtiss and the

Wrights are engaged in similar work, but no actual figures of their

output are obtainable.

Larger Plants Are Necessary.

And this situation exists despite the fact that none of the producers

are really equipped with adequate plants for turning out their

machines on a modern, business-like basis. The demand was so sudden and

unexpected that it found them poorly prepared to meet it. This, however,

is now being remedied by the erection of special plants, the enlargement

of others, and the introduction of new machinery and other labor-saving

conveniences.

Companies, with large capitalization, to engage in the exclusive

production of airships are being organized in many parts of the world.

One notable instance of this nature is worth quoting as illustrative

of the manner in which the production of flying machines is being

commercialized. This is the formation at Frankfort, Germany, of the

Flugmaschine Wright, G. m. b. H., with a capital of $119,000, the

Krupps, of Essen, being interested.

Prices at Which Machines Sell.

This wonderful demand from the public has come notwithstanding the fact

that the machines, owing to lack of facilities for wholesale production,

are far from being cheap. Such definite quotations as are made are on

the following basis:

Santos Dumont--List price $1,000, but owing to the rush of orders agents

are readily getting from $1,300 to $1,500. This is the smallest machine

made.

Bleriot--List price $2,500. This is for the cross-channel type, with

Anzani motor.

Antoinette--List price from $4,000 to $5,000, according to size.

Wright--List price $5,600.

Curtiss--List price $5,000.

There is, however, no stability in prices as purchasers are almost

invariably ready to pay a considerable premium to facilitate delivery.

The motor is the most expensive part of the flying machine. Motor

prices range from $500 to $2,000, this latter amount being asked for the

Curtiss engine.

Systematic Instruction of Amateurs.

In addition to the production of flying machines many of the experienced

aviators are making a business of the instruction of amateurs. Curtiss

and the Wrights in this country have a number of pupils, as have also

the prominent foreigners. Schools of instruction are being opened in

various parts of the world, not alone as private money-making ventures,

but in connection with public educational institutions. One of these

latter is to be found at the University of Barcelona, Spain.

The flying machine agent, the man who handles the machines on a

commission, has also become a known quantity, and will soon be as

numerous as his brother of the automobile. The sign "John Bird, agent

for Skimmer's Flying Machine," is no longer a curiosity.

Yes, the Airship Is Here.

From all of which we may well infer that the flying machine in practical

form has arrived, and that it is here to stay. It is no exaggeration to

say that the time is close at hand when people will keep flying machines

just as they now keep automobiles, and that pleasure jaunts will be

fully as numerous and popular. With the important item of practicability

fully demonstrated, "Come, take a trip in my airship," will have more

real significance than now attaches to the vapid warblings of the

vaudeville vocalist.

As a further evidence that the airship is really here, and that its

presence is recognized in a business way, the action of life and

accident insurance companies is interesting. Some of them are

reconstructing their policies so as to include a special waiver of

insurance by aviators. Anything which compels these great corporations

to modify their policies cannot be looked upon as a mere curiosity or

toy.

It is some consolation to know that the movement in this direction is

not thus far widespread. Moreover it is more than probable that the

competition for business will eventually induce the companies to act

more liberally toward aviators, especially as the art of aviation

advances.

# CHAPTER XIX. LAW OF THE AIRSHIP.

Successful aviation has evoked some peculiar things in the way of legal

action and interpretation of the law.

It is well understood that a man's property cannot be used without his

consent. This is an old established principle in common law which holds

good today.

The limits of a man's property lines, however, have not been so well

understood by laymen. According to eminent legal authorities such as

Blackstone, Littleton and Coke, the "fathers of the law," the owner of

realty also holds title above and below the surface, and this theory is

generally accepted without question by the courts.

Rights of Property Owners.

In other words the owner of realty also owns the sky above it without

limit as to distance. He can dig as deep into his land, or go as high

into the air as he desires, provided he does not trespass upon or injure

similar rights of others.

The owner of realty may resist by force, all other means having failed,

any trespass upon, or invasion of his property. Other people, for

instance, may not enter upon it, or over or under it, without his

express permission and consent. There is only one exception, and this is

in the case of public utility corporations such as railways which,

under the law of eminent domain, may condemn a right of way across the

property of an obstinate owner who declines to accept a fair price for

the privilege.

Privilege Sharply Confined.

The law of eminent domain may be taken advantage of only by corporations

which are engaged in serving the public. It is based upon the principle

that the advancement and improvement of a community is of more

importance and carries with it more rights than the interests of the

individual owner. But even in cases where the right of eminent domain is

exercised there can be no confiscation of the individual's property.

Exercising the right of eminent domain is merely obtaining by public

purchase what is held to be essential to the public good, and which

cannot be secured by private purchase. When eminent domain proceedings

are resorted to the court appoints appraisers who determine upon the

value of the property wanted, and this value (in money) is paid to the

owner.

How It Affects Aviation.

It should be kept in mind that this privilege of the "right of eminent

domain" is accorded only to corporations which are engaged in serving

the public. Individuals cannot take advantage of it. Thus far all

aviation has been conducted by individuals; there are no flying machine

or airship corporations regularly engaged in the transportation of

passengers, mails or freight.

This leads up to the question "What would happen if realty owners

generally, or in any considerable numbers, should prohibit the

navigation of the air above their holdings?" It is idle to say such

a possibility is ridiculous--it is already an actuality in a few

individual instances.

One property owner in New Jersey, a justice of the peace, maintains a

large sign on the roof of his house warning aviators that they must not

trespass upon his domain. That he is acting well within his rights in

doing this is conceded by legal authorities.

Hard to Catch Offenders.

But, suppose the alleged trespass is committed, what is the property

owner going to do about it? He must first catch the trespasser and this

would be a pretty hard job. He certainly could not overtake him, unless

he kept a racing aeroplane for this special purpose. It would be

equally difficult to identify the offender after the offense had been

committed, even if he were located, as aeroplanes carry no license

numbers.

Allowing that the offender should be caught the only recourse of the

realty owner is an action for damages. He may prevent the commission of

the offense by force if necessary, but after it is committed he can only

sue for damages. And in doing this he would have a lot of trouble.

Points to Be Proven.

One of the first things the plaintiff would be called upon to prove

would be the elevation of the machine. If it were reasonably close to

the ground there would, of course, be grave risk of damage to fences,

shrubbery, and other property, and the court would be justified in

holding it to be a nuisance that should be suppressed.

If, on the other hand; the machine was well up in the air, but going

slowly, or hovering over the plaintiff's property, the court might be

inclined to rule that it could not possibly be a nuisance, but right

here the court would be in serious embarrassment. By deciding that it

was not a nuisance he would virtually override the law against invasion

of a man's property without his consent regardless of the nature of the

invasion. By the same decision he would also say in effect that, if one

flying machine could do this a dozen or more would have equal right to

do the same thing. While one machine hovering over a certain piece of

property may be no actual nuisance a dozen or more in the same position

could hardly be excused.

Difficult to Fix Damages.

Such a condition would tend to greatly increase the risk of accident,

either through collision, or by the carelessness of the aviators in

dropping articles which might cause damages to the people or property

below. In such a case it would undoubtedly be a nuisance, and in

addition to a fine, the offender would also be liable for the damages.

Taking it for granted that no actual damage is done, and the owner

merely sues on account of the invasion of his property, how is the

amount of compensation to be fixed upon? The owner has lost nothing; no

# part of his possessions has been taken away; nothing has been injured

or destroyed; everything is left in exactly the same condition as before

the invasion. And yet, if the law is strictly interpreted, the offender

is liable.

Right of Way for Airships.

Somebody has suggested the organization of flying-machine corporations

as common carriers, which would give them the right of eminent domain

with power to condemn a right of way. But what would they condemn? There

is nothing tangible in the air. Railways in condemning a right of way

specify tangible property (realty) within certain limits. How would an

aviator designate any particular right of way through the air a certain

number of feet in width, and a certain distance from the ground?

And yet, should the higher courts hold to the letter of the law and

decide that aviators have no right to navigate their craft over private

property, something will have to be done to get them out of the dilemma,

as aviation is too far advanced to be discarded. Fortunately there is

little prospect of any widespread antagonism among property owners so

long as aviators refrain from making nuisances of themselves.

Possible Solution Offered.

One possible solution is offered and that is to confine the path of

airships to the public highways so that nobody's property rights would

be invaded. In addition, as a matter of promoting safety for both

operators and those who may happen to be beneath the airships as they

pass over a course, adoption of the French rules are suggested. These

are as follows:

Aeroplanes, when passing, must keep to the right, and pass at a distance

of at least 150 feet. They are free from this rule when flying at

altitudes of more than 100 feet. Every machine when flying at night or

during foggy weather must carry a green light on the right, and a red

light on the left, and a white headlight on the front.

These are sensible rules, but may be improved upon by the addition of a

signal system of some kind, either horn, whistle or bell.

Responsibility of Aviators.

Mr. Jay Carver Bossard, in recent numbers of _Fly_, brings out some

curious and interesting legal points in connection with aviation, among

which are the following:

"Private parties who possess aerial craft, and desire to operate the

same in aerial territory other than their own, must obtain from land

owners special permission to do so, such permission to be granted only

by agreement, founded upon a valid consideration. Otherwise, passing

over another's land will in each instance amount to a trespass.

"Leaving this highly technical side of the question, let us turn to

another view: the criminal and tort liability of owners and operators to

airship passengers. If A invites B to make an ascension with him in his

machine, and B, knowing that A is merely an enthusiastic amateur and

far from being an expert, accepts and is through A's innocent negligence

injured, he has no grounds for recovery. But if A contracts with B, to

transport him from one place to another, for a consideration, and B is

injured by the poor piloting of A, A would be liable to B for damages

which would result. Now in order to safeguard such people as B, curious

to the point of recklessness, the law will have to require all airship

operators to have a license, and to secure this license airship pilots

will have to meet certain requirements. Here again is a question. Who

is going to say whether an applicant is competent to pilot a balloon or

airship?

Fine for an Aeronaut.

"An aeroplane while maneuvering is suddenly caught by a treacherous gale

and swept to the ground. A crowd of people hasten over to see if the

aeronaut is injured, and in doing so trample over Tax-payer Smith's

garden, much to the detriment of his growing vegetables and flowers. Who

is liable for the damages? Queer as it may seem, a case very similar to

this was decided in 1823, in the New York supreme court, and it was held

that the aeronaut was liable upon the following grounds: 'To render one

man liable in trespass for the acts of others, it must appear either

that they acted in concert, or that the act of the one, ordinarily and

naturally produced the acts of the others, Ascending in a balloon is not

an unlawful act, but it is certain that the aeronaut has no control

over its motion horizontally, but is at the sport of the wind, and is

to descend when and how he can. His reaching the earth is a matter of

hazard. If his descent would according to the circumstances draw a crowd

of people around him, either out of curiosity, or for the purpose

of rescuing him from a perilous situation, all this he ought to have

foreseen, and must be responsible for.'

Air Not Really Free.

"The general belief among people is, that the air is free. Not only free

to breathe and enjoy, but free to travel in, and that no one has any

definite jurisdiction over, or in any part of it. Now suppose this were

made a legal doctrine. Would a murder perpetrated above the clouds have

to go unpunished? Undoubtedly. For felonies committed upon the high seas

ample provision is made for their punishment, but new provisions will

have to be made for crimes committed in the air.

Relations of Owner and Employee.

"It is a general rule of law that a master is bound to provide

reasonably safe tools, appliances and machines for his servant. How this

rule is going to be applied in cases of aeroplanes, remains to be seen.

The aeroplane owner who hires a professional aeronaut, that is, one who

has qualified as an expert, owes him very little legal duty to supply

him with a perfect aeroplane. The expert is supposed to know as much

regarding the machine as the owner, if not more, and his acceptance of

his position relieves the owner from liability. When the owner hires an

amateur aeronaut to run the aeroplane, and teaches him how to manipulate

it, even though the prescribed manner of manipulation will make flight

safe, nevertheless if the machine is visibly defective, or known to be

so, any injury which results to the aeronaut the owner is liable for.

As to Aeroplane Contracts.

"At the present time there are many orders being placed with aeroplane

manufacturing companies. There are some unique questions to be raised

here under the law of contract. It is an elementary principle of law

that no one can be compelled to complete a contract which in itself is

impossible to perform. For instance, a contract to row a boat across

the Atlantic in two weeks, for a consideration, could never be enforced

because it is within judicial knowledge that such an undertaking is

beyond human power. Again, contracts formed for the doing of acts

contrary to nature are never enforcible, and here is where our

difficulty comes in. Is it possible to build a machine or species of

craft which will transport a person or goods through the air? The courts

know that balloons are practical; that is, they know that a bag

filled with gas has a lifting power and can move through the air at an

appreciable height. Therefore, a contract to transport a person in such

manner is a good contract, and the conditions being favorable could

undoubtedly be enforced. But the passengers' right of action for injury

would be very limited.

No Redress for Purchasers.

"In the case of giving warranties on aeroplanes, we have yet to see just

what a court is going to say. It is easy enough for a manufacturer to

guarantee to build a machine of certain dimensions and according to

certain specifications, but when he inserts a clause in the contract to

the effect that the machine will raise itself from the surface of the

earth, defy the laws of gravity, and soar in the heavens at the will of

the aviator, he is to say the least contracting to perform a miracle.

"Until aeroplanes have been made and accepted as practical, no court

will force a manufacturer to turn out a machine guaranteed to fly. So

purchasers can well remember that if their machines refuse to fly they

have no redress against the maker, for he can always say, 'The industry

is still in its experimental stage.' In contracting for an engine no

builder will guarantee that the particular engine will successfully

operate the aeroplane. In fact he could never be forced to live up to

such an agreement, should he agree to a stipulation of that sort. The

best any engine maker will guarantee is to build an engine according to

specifications."

# CHAPTER XX. SOARING FLIGHT.

By Octave Chanute.

[5] There is a wonderful performance daily exhibited in southern climes

and occasionally seen in northerly latitudes in summer, which has

never been thoroughly explained. It is the soaring or sailing flight

of certain varieties of large birds who transport themselves on rigid,

unflapping wings in any desired direction; who in winds of 6 to 20

miles per hour, circle, rise, advance, return and remain aloft for hours

without a beat of wing, save for getting under way or convenience in

various maneuvers. They appear to obtain from the wind alone all the

necessary energy, even to advancing dead against that wind. This feat is

so much opposed to our general ideas of physics that those who have

not seen it sometimes deny its actuality, and those who have only

occasionally witnessed it subsequently doubt the evidence of their own

eyes. Others, who have seen the exceptional performances, speculate on

various explanations, but the majority give it up as a sort of "negative

gravity."

Soaring Power of Birds.

The writer of this paper published in the "Aeronautical Annual" for 1896

and 1897 an article upon the sailing flight of birds, in which he gave

a list of the authors who had described such flight or had advanced

theories for its explanation, and he passed these in review. He also

described his own observations and submitted some computations to

account for the observed facts. These computations were correct as

far as they went, but they were scanty. It was, for instance, shown

convincingly by analysis that a gull weighing 2.188 pounds, with a total

supporting surface of 2.015 square feet, a maximum body cross-section

of 0.126 square feet and a maximum cross-section of wing edges of 0.098

square feet, patrolling on rigid wings (soaring) on the weather side of

a steamer and maintaining an upward angle or attitude of 5 degrees to 7

degrees above the horizon, in a wind blowing 12.78 miles an hour, which

was deflected upward 10 degrees to 20 degrees by the side of the steamer

(these all being carefully observed facts), was perfectly sustained at

its own "relative speed" of 17.88 miles per hour and extracted from

the upward trend of the wind sufficient energy to overcome all the

resistances, this energy amounting to 6.44 foot-pounds per second.

Great Power of Gulls.

It was shown that the same bird in flapping flight in calm air, with an

attitude or incidence of 3 degrees to 5 degrees above the horizon and

a speed of 20.4 miles an hour was well sustained and expended 5.88

foot-pounds per second, this being at the rate of 204 pounds sustained

per horsepower. It was stated also that a gull in its observed

maneuvers, rising up from a pile head on unflapping wings, then plunging

forward against the wind and subsequently rising higher than his

starting point, must either time his ascents and descents exactly with

the variations in wind velocities, or must meet a wind billow rotating

on a horizontal axis and come to a poise on its crest, thus availing of

an ascending trend.

But the observations failed to demonstrate that the variations of the

wind gusts and the movements of the bird were absolutely synchronous,

and it was conjectured that the peculiar shape of the soaring wing of

certain birds, as differentiated from the flapping wing, might, when

experimented upon, hereafter account for the performance.

Mystery to be Explained.

These computations, however satisfactory they were for the speed of

winds observed, failed to account for the observed spiral soaring of

buzzards in very light winds and the writer was compelled to confess:

"Now, this spiral soaring in steady breezes of 5 to 10 miles per hour

which are apparently horizontal, and through which the bird maintains an

average speed of about 20 miles an hour, is the mystery to be explained.

It is not accounted for, quantitatively, by any of the theories which

have been advanced, and it is the one performance which has led some

observers to claim that it was done through 'aspiration.' i, e., that

a bird acted upon by a current, actually drew forward into that current

against its exact direction of motion."

Buzzards Soar in Dead Calm.

A still greater mystery was propounded by the few observers who asserted

that they had seen buzzards soaring in a dead calm, maintaining their

elevation and their speed. Among these observers was Mr. E. C. Huffaker,

at one time assistant experimenter for Professor Langley. The writer

believed and said then that he must in some way have been mistaken, yet,

to satisfy himself, he paid several visits to Mr. Huffaker, in Eastern

Tennessee and took along his anemometer. He saw quite a number of

buzzards sailing at a height of 75 to 100 feet in breezes measuring 5

or 6 miles an hour at the surface of the ground, and once he saw one

buzzard soaring apparently in a dead calm.

The writer was fairly baffled. The bird was not simply gliding,

utilizing gravity or acquired momentum, he was actually circling

horizontally in defiance of physics and mathematics. It took two years

and a whole series of further observations to bring those two sciences

into accord with the facts.

Results of Close Observations.

Curiously enough the key to the performance of circling in a light wind

or a dead calm was not found through the usual way of gathering human

knowledge, i. e., through observations and experiment. These had failed

because I did not know what to look for. The mystery was, in fact,

solved by an eclectic process of conjecture and computation, but once

these computations indicated what observations should be made, the

results gave at once the reasons for the circling of the birds, for

their then observed attitude, and for the necessity of an independent

initial sustaining speed before soaring began. Both Mr. Huffaker and

myself verified the data many times and I made the computations.

These observations disclosed several facts:

1st.--That winds blowing five to seventeen miles per hour frequently had

rising trends of 10 degrees to 15 degrees, and that upon occasions when

there seemed to be absolutely no wind, there was often nevertheless a

local rising of the air estimated at a rate of four to eight miles or

more per hour. This was ascertained by watching thistledown, and rising

fogs alongside of trees or hills of known height. Everyone will readily

realize that when walking at the rate of four to eight miles an hour in

a dead calm the "relative wind" is quite inappreciable to the senses and

that such a rising air would not be noticed.

2nd.--That the buzzard, sailing in an apparently dead horizontal calm,

progressed at speeds of fifteen to eighteen miles per hour, as measured

by his shadow on the ground. It was thought that the air was then

possibly rising 8.8 feet per second, or six miles per hour.

3rd.--That when soaring in very light winds the angle of incidence of

the buzzards was negative to the horizon--i. e., that when seen coming

toward the eye, the afternoon light shone on the back instead of on the

breast, as would have been the case had the angle been inclined above

the horizon.

4th.--That the sailing performance only occurred after the bird had

acquired an initial velocity of at least fifteen or eighteen miles per

hour, either by industrious flapping or by descending from a perch.

An Interesting Experiment.

5th.--That the whole resistance of a stuffed buzzard, at a negative

angle of 3 degrees in a current of air of 15.52 miles per hour, was 0.27

pounds. This test was kindly made for the writer by Professor A. F. Zahm

in the "wind tunnel" of the Catholic University at Washington, D. C.,

who, moreover, stated that the resistance of a live bird might be less,

as the dried plumage could not be made to lie smooth.

This particular buzzard weighed in life 4.25 pounds, the area of his

wings and body was 4.57 square feet, the maximum cross-section of

his body was 0.110 square feet, and that of his wing edges when fully

extended was 0.244 square feet.

With these data, it became surprisingly easy to compute the performance

with the coefficients of Lilienthal for various angles of incidence

and to demonstrate how this buzzard could soar horizontally in a dead

horizontal calm, provided that it was not a vertical calm, and that the

air was rising at the rate of four or six miles per hour, the lowest

observed, and quite inappreciable without actual measuring.

Some Data on Bird Power.

The most difficult case is purposely selected. For if we assume that the

bird has previously acquired an initial minimum speed of seventeen miles

an hour (24.93 feet per second, nearly the lowest measured), and that

the air was rising vertically six miles an hour (8.80 feet per second),

then we have as the trend of the "relative wind" encountered:

6

-- = 0.353, or the tangent of 19 degrees 26'.

17

which brings the case into the category of rising wind effects. But the

bird was observed to have a negative angle to the horizon of about 3

degrees, as near as could be guessed, so that his angle of incidence to

the "relative wind" was reduced to 16 degrees 26'.

The relative speed of his soaring was therefore:

Velocity = square root of (17 squared + 6 squared) = 18.03 miles per

hour.

At this speed, using the Langley co-efficient recently practically

confirmed by the accurate experiments of Mr. Eiffel, the air pressure

would be:

18.03 squared X 0.00327 = 1.063 pounds per square foot.

If we apply Lilienthal's co-efficients for an angle of 6 degrees 26', we

have for the force in action:

Normal: 4.57 X 1.063 X 0.912 = 4.42 pounds.

Tangential: 4.57 X 1.063 X 0.074 = - 0.359 pounds,

which latter, being negative, is a propelling force.

Results Astonish Scientists.

Thus we have a bird weighing 4.25 pounds not only thoroughly supported,

but impelled forward by a force of 0.359 pounds, at seventeen miles

per hour, while the experiments of Professor A. F. Zahm showed that the

resistance at 15.52 miles per hour was only 0.27 pounds,

17 squared

or 0.27 X ------- = 0.324 pounds, at seventeen miles an

15.52 squared

hour.

These are astonishing results from the data obtained, and they lead to

the inquiry whether the energy of the rising air is sufficient to make

up the losses which occur by reason of the resistance and friction of

the bird's body and wings, which, being rounded, do not encounter air

pressures in proportion to their maximum cross-section.

We have no accurate data upon the co-efficients to apply and estimates

made by myself proved to be much smaller than the 0.27 pounds resistance

measured by Professor Zahm, so that we will figure with the latter as

modified. As the speed is seventeen miles per hour, or 24.93 feet per

second, we have for the work:

Work done, 0.324 X 24.93 = 8.07 foot pounds per second.

Endorsed by Prof. Marvin.

Corresponding energy of rising air is not sufficient at four miles per

hour. This amounts to but 2.10 foot pounds per second, but if we assume

that the air was rising at the rate of seven miles per hour (10.26 feet

per second), at which the pressure with the Langley coefficient would be

0.16 pounds per square foot, we have on 4.57 square feet for energy of

rising air: 4.57 X 0.16 X 10.26 = 7.50 foot pounds per second, which

is seen to be still a little too small, but well within the limits of

error, in view of the hollow shape of the bird's wings, which receive

greater pressure than the flat planes experimented upon by Langley.

These computations were chiefly made in January, 1899, and were

communicated to a few friends, who found no fallacy in them, but thought

that few aviators would understand them if published. They were then

submitted to Professor C. F. Marvin of the Weather Bureau, who is well

known as a skillful physicist and mathematician. He wrote that they

were, theoretically, entirely sound and quantitatively, probably, as

accurate as the present state of the measurements of wind pressures

permitted. The writer determined, however, to withhold publication until

the feat of soaring flight had been performed by man, partly because he

believed that, to ensure safety, it would be necessary that the machine

should be equipped with a motor in order to supplement any deficiency in

wind force.

Conditions Unfavorable for Wrights.

The feat would have been attempted in 1902 by Wright brothers if the

local circumstances had been more favorable. They were experimenting on

"Kill Devil Hill," near Kitty Hawk, N. C. This sand hill, about 100

feet high, is bordered by a smooth beach on the side whence come the

sea breezes, but has marshy ground at the back. Wright brothers were

apprehensive that if they rose on the ascending current of air at the

front and began to circle like the birds, they might be carried by the

descending current past the back of the hill and land in the marsh.

Their gliding machine offered no greater head resistance in proportion

than the buzzard, and their gliding angles of descent are practically as

favorable, but the birds performed higher up in the air than they.

Langley's Idea of Aviation.

Professor Langley said in concluding his paper upon "The Internal Work

of the Wind":

"The final application of these principles to the art of aerodromics

seems, then, to be, that while it is not likely that the perfected

aerodrome will ever be able to dispense altogether with the ability

to rely at intervals on some internal source of power, it will not be

indispensable that this aerodrome of the future shall, in order to go

any distance--even to circumnavigate the globe without alighting--need

to carry a weight of fuel which would enable it to perform this journey

under conditions analogous to those of a steamship, but that the fuel

and weight need only be such as to enable it to take care of itself in

exceptional moments of calm."

Now that dynamic flying machines have been evolved and are being brought

under control, it seems to be worth while to make these computations and

the succeeding explanations known, so that some bold man will attempt

the feat of soaring like a bird. The theory underlying the performance

in a rising wind is not new, it has been suggested by Penaud and others,

but it has attracted little attention because the exact data and

the maneuvers required were not known and the feat had not yet been

performed by a man. The puzzle has always been to account for the

observed act in very light winds, and it is hoped that by the present

selection of the most difficult case to explain--i. e., the soaring in a

dead horizontal calm--somebody will attempt the exploit.

Requisites for Soaring Flights.

The following are deemed to be the requisites and maneuvers to master

the secrets of soaring flight:

1st--Develop a dynamic flying machine weighing about one pound per

square foot of area, with stable equilibrium and under perfect control,

capable of gliding by gravity at angles of one in ten (5 3/4 degrees) in

still air.

2nd.--Select locations where soaring birds abound and occasions where

rising trends of gentle winds are frequent and to be relied on.

3rd.--Obtain an initial velocity of at least 25 feet per second before

attempting to soar.

4th.--So locate the center of gravity that the apparatus shall assume a

negative angle, fore and aft, of about 3 degrees.

Calculations show, however, that sufficient propelling force may still

exist at 0 degrees, but disappears entirely at +4 degrees.

5th.--Circle like the bird. Simultaneously with the steering, incline

the apparatus to the side toward which it is desired to turn, so that

the centrifugal force shall be balanced by the centripetal force. The

amount of the required inclination depends upon the speed and on the

radius of the circle swept over.

6th.--Rise spirally like the bird. Steer with the horizontal rudder, so

as to descend slightly when going with the wind and to ascend when going

against the wind. The bird circles over one spot because the rising

trends of wind are generally confined to small areas or local chimneys,

as pointed out by Sir H. Maxim and others.

7th.--Once altitude is gained, progress may be made in any direction by

gliding downward by gravity.

The bird's flying apparatus and skill are as yet infinitely superior

to those of man, but there are indications that within a few years the

latter may evolve more accurately proportioned apparatus and obtain

absolute control over it.

It is hoped, therefore, that if there be found no radical error in the

above computations, they will carry the conviction that soaring flight

is not inaccessible to man, as it promises great economies of motive

power in favorable localities of rising winds.

The writer will be grateful to experts who may point out any mistake

committed in data or calculations, and will furnish additional

information to any aviator who may wish to attempt the feat of soaring.

# CHAPTER XXI. FLYING MACHINES VS. BALLOONS.

While wonderful success has attended the development of the dirigible

(steerable) balloon the most ardent advocates of this form of aerial

navigation admit that it has serious drawbacks. Some of these may be

described as follows:

Expense and Other Items.

Great Initial Expense.--The modern dirigible balloon costs a fortune.

The Zeppelin, for instance, costs more than $100,000 (these are official

figures).

Expense of Inflation.--Gas evaporates rapidly, and a balloon must

be re-inflated, or partially re-inflated, every time it is used. The

Zeppelin holds 460,000 cubic feet of gas which, even at $1 per thousand,

would cost $460.

Difficulty of Obtaining Gas.--If a balloon suddenly becomes deflated, by

accident or atmospheric conditions, far from a source of gas supply, it

is practically worthless. Gas must be piped to it, or the balloon carted

to the gas house--an expensive proceeding in either event.

Lack of Speed and Control.

Lack of Speed.--Under the most favorable conditions the maximum speed

of a balloon is 30 miles an hour. Its great bulk makes the high speed

attained by flying machines impossible.

Difficulty of Control.--While the modern dirigible balloon is readily

handled in calm or light winds, its bulk makes it difficult to control

in heavy winds.

The Element of Danger.--Numerous balloons have been destroyed by

lightning and similar causes. One of the largest of the Zeppelins was

thus lost at Stuttgart in 1908.

Some Balloon Performances.

It is only a matter of fairness to state that, under favorable

conditions, some very creditable records have been made with modern

balloons, viz:

November 23d, 1907, the French dirigible Patrie, travelled 187 miles in

6 hours and 45 minutes against a light wind. This was a little over 28

miles an hour.

The Clement-Bayard, another French machine, sold to the Russian

government, made a trip of 125 miles at a rate of 27 miles an hour.

Zeppelin No. 3, carrying eight passengers, and having a total lifting

capacity of 5,500 pounds of ballast in addition to passengers, weight

of equipment, etc., was tested in October, 1906, and made 67 miles in 2

hours and 17 minutes, about 30 miles an hour.

These are the best balloon trips on record, and show forcefully the

limitations of speed, the greatest being not over 30 miles an hour.

Speed of Flying Machines.

Opposed to the balloon performances we have flying machine trips (of

authentic records) as follows:

Bleriot--monoplane--in 1908--52 miles an hour.

Delagrange--June 22, 1908--10 1/2 miles in 16 minutes, approximately 42

miles an hour.

Wrights--October, 1905--the machine was then in its infancy--24 miles

in 38 minutes, approximately 44 miles an hour. On December 31, 1908, the

Wrights made 77 miles in 2 hours and 20 minutes.

Lambert, a pupil of the Wrights, and using a Wright biplane, on October

18, 1909, covered 29.82 miles in 49 minutes and 39 seconds, being at

the rate of 36 miles an hour. This flight was made at a height of 1,312

feet.

Latham--October 21, 1909--made a short flight, about 11 minutes, in

the teeth of a 40 mile gale, at Blackpool, Eng. He used an Antoniette

monoplane, and the official report says: "This exhibition of nerve,

daring and ability is unparalled in the history of aviation."

Farman--October 20, 1909--was in the air for 1 hour, 32 min., 16

seconds, travelling 47 miles, 1,184 yards, a duration record for

England.

Paulhan--January 18, 1901--47 1/2 miles at the rate of 45 miles an hour,

maintaining an altitude of from 1,000 to 2,000 feet.

Expense of Producing Gas.

Gas is indispensable in the operation of dirigible balloons, and gas

is expensive. Besides this it is not always possible to obtain it in

sufficient quantities even in large cities, as the supply on hand is

generally needed for regular customers. Such as can be had is either

water or coal gas, neither of which is as efficient in lifting power as

hydrogen.

Hydrogen is the lightest and consequently the most buoyant of all known

gases. It is secured commercially by treating zinc or iron with dilute

sulphuric or hydrochloric acid. The average cost may be safely placed

at $10 per 1,000 feet so that, to inflate a balloon of the size of the

Zeppelin, holding 460,000 cubic feet, would cost $4,600.

Proportions of Materials Required.

In making hydrogen gas it is customary to allow 20 per cent for loss

between the generation and the introduction of the gas into the balloon.

Thus, while the formula calls for iron 28 times heavier than the weight

of the hydrogen required, and acid 49 times heavier, the real quantities

are 20 per cent greater. Hydrogen weighs about 0.09 ounce to the cubic

foot. Consequently if we need say 450,000 cubic feet of gas we must have

2,531.25 pounds in weight. To produce this, allowing for the 20 percent

loss, we must have 35 times its weight in iron, or over 44 tons. Of acid

it would take 60 times the weight of the gas, or nearly 76 tons.

In Time of Emergency.

These figures are appalling, and under ordinary conditions would be

prohibitive, but there are times when the balloon operator, unable to

obtain water or coal gas, must foot the bills. In military maneuvers,

where the field of operation is fixed, it is possible to furnish

supplies of hydrogen gas in portable cylinders, but on long trips

where sudden leakage or other cause makes descent in an unexpected spot

unavoidable, it becomes a question of making your own hydrogen gas or

deserting the balloon. And when this occurs the balloonist is up against

another serious proposition--can he find the necessary zinc or iron? Can

he get the acid?

Balloons for Commercial Use.

Despite all this the balloon has its uses. If there is to be such a

thing as aerial navigation in a commercial way--the carrying of freight

and passengers--it will come through the employment of such monster

balloons as Count Zeppelin is building. But even then the carrying

capacity must of necessity be limited. The latest Zeppelin creation,

a monster in size, is 450 feet long, and 42 1/2 feet in diameter. The

dimensions are such as to make all other balloons look like pigmies;

even many ocean-going steamers are much smaller, and yet its passenger

capacity is very small. On its 36-hour flight in May, 1909, the

Zeppelin, carried only eight passengers. The speed, however, was quite

respectable, 850 miles being covered in the 36 hours, a trifle over 23

miles an hour. The reserve buoyancy, that is the total lifting capacity

aside from the weight of the airship and its equipment, is estimated at

three tons.

# CHAPTER XXII. PROBLEMS OF AERIAL FLIGHT.

In a lecture before the Royal Society of Arts, reported in Engineering,

F. W. Lanchester took the position that practical flight was not the

abstract question which some apparently considered it to be, but a

problem in locomotive engineering. The flying machine was a locomotive

appliance, designed not merely to lift a weight, but to transport it

elsewhere, a fact which should be sufficiently obvious. Nevertheless one

of the leading scientific men of the day advocated a type in which

this, the main function of the flying machine, was overlooked. When

the machine was considered as a method of transport, the vertical screw

type, or helicopter, became at once ridiculous. It had, nevertheless,

many advocates who had some vague and ill-defined notion of subsequent

motion through the air after the weight was raised.

Helicopter Type Useless.

When efficiency of transport was demanded, the helicopter type was

entirely out of court. Almost all of its advocates neglected the effect

of the motion of the machine through the air on the efficiency of the

vertical screws. They either assumed that the motion was so slow as not

to matter, or that a patch of still air accompanied the machine in its

flight. Only one form of this type had any possibility of success. In

this there were two screws running on inclined axles--one on each

side of the weight to be lifted. The action of such inclined screw was

curious, and in a previous lecture he had pointed out that it was almost

exactly the same as that of a bird's wing. In high-speed racing craft

such inclined screws were of necessity often used, but it was at

a sacrifice of their efficiency. In any case the efficiency of the

inclined-screw helicopter could not compare with that of an aeroplane,

and that type might be dismissed from consideration so soon as

efficiency became the ruling factor of the design.

Must Compete With Locomotive.

To justify itself the aeroplane must compete, in some regard or other,

with other locomotive appliances, performing one or more of the purposes

of locomotion more efficiently than existing systems. It would be no use

unless able to stem air currents, so that its velocity must be greater

than that of the worst winds liable to be encountered. To illustrate the

limitations imposed on the motion of an aeroplane by wind velocity, Mr.

Lanchester gave the diagrams shown in Figs. 1 to 4. The circle in each

case was, he said, described with a radius equal to the speed of the

aeroplane in still air, from a center placed "down-wind" from the

aeroplane by an amount equal to the velocity of the wind.

Fig. 1 therefore represented the case in which the air was still, and

in this case the aeroplane represented by _A_ had perfect liberty of

movement in any direction

In Fig. 2 the velocity of the wind was half that of the aeroplane, and

the latter could still navigate in any direction, but its speed against

the wind was only one-third of its speed with the wind.

In Fig. 3 the velocity of the wind was equal to that of the aeroplane,

and then motion against the wind was impossible; but it could move to

any point of the circle, but not to any point lying to the left of the

tangent _A_ _B_. Finally, when the wind had a greater speed than the

aeroplane, as in Fig. 4, the machine could move only in directions

limited by the tangents _A_ _C_ and _A_ _D_.

Matter of Fuel Consumption.

Taking the case in which the wind had a speed equal to half that of the

aeroplane, Mr. Lanchester said that for a given journey out and home,

down wind and back, the aeroplane would require 30 per cent more fuel

than if the trip were made in still air; while if the journey was made

at right angles to the direction of the wind the fuel needed would be 15

per cent more than in a calm. This 30 per cent extra was quite a heavy

enough addition to the fuel; and to secure even this figure it was

necessary that the aeroplane should have a speed of twice that of the

maximum wind in which it was desired to operate the machine. Again, as

stated in the last lecture, to insure the automatic stability of the

machine it was necessary that the aeroplane speed should be largely in

excess of that of the gusts of wind liable to be encountered.

Eccentricities of the Wind.

There was, Mr. Lanchester said, a loose connection between the average

velocity of the wind and the maximum speed of the gusts. When the

average speed of the wind was 40 miles per hour, that of the gusts might

be equal or more. At one moment there might be a calm or the direction

of the wind even reversed, followed, the next moment, by a violent gust.

About the same minimum speed was desirable for security against gusts as

was demanded by other considerations. Sixty miles an hour was the least

figure desirable in an aeroplane, and this should be exceeded as much as

possible. Actually, the Wright machine had a speed of 38 miles per hour,

while Farman's Voisin machine flew at 45 miles per hour.

Both machines were extremely sensitive to high winds, and the speaker,

in spite of newspaper reports to the contrary, had never seen either

flown in more than a gentle breeze. The damping out of the oscillations

of the flight path, discussed in the last lecture, increased with the

fourth power of the natural velocity of flight, and rapid damping

formed the easiest, and sometimes the only, defense against dangerous

oscillations. A machine just stable at 35 miles per hour would have

reasonably rapid damping if its speed were increased to 60 miles per

hour.

Thinks Use Is Limited.

It was, the lecturer proceeded, inconceivable that any very extended use

should be made of the aeroplane unless the speed was much greater than

that of the motor car. It might in special cases be of service,

apart from this increase of speed, as in the exploration of countries

destitute of roads, but it would have no general utility. With an

automobile averaging 25 to 35 miles per hour, almost any part of Europe,

Russia excepted, was attainable in a day's journey. A flying machine

of but equal speed would have no advantages, but if the speed could be

raised to 90 or 100 miles per hour, the whole continent of Europe

would become a playground, every part being within a daylight flight of

Berlin. Further, some marine craft now had speeds of 40 miles per hour,

and efficiently to follow up and report movements of such vessels an

aeroplane should travel at 60 miles per hour at least. Hence from

all points of view appeared the imperative desirability of very high

velocities of flight. The difficulties of achievement were, however,

great.

Weight of Lightest Motors.

As shown in the first lecture of his course, the resistance to motion

was nearly independent of the velocity, so that the total work done in

transporting a given weight was nearly constant. Hence the question of

fuel economy was not a bar to high velocities of flight, though should

these become excessive, the body resistance might constitute a large

proportion of the total. The horsepower required varied as the

velocity, so the factor governing the maximum velocity of flight was

the horsepower that could be developed on a given weight. At present the

weight per horsepower of feather-weight motors appeared to range from

2 1/4 pounds up to 7 pounds per brake horsepower, some actual figures

being as follows:

Antoinette........ 5 lbs.

Fiat.............. 3 lbs.

Gnome....... Under 3 lbs.

Metallurgic....... 8 lbs.

Renault........... 7 lbs.

Wright.............6 lbs.

Automobile engines, on the other hand, commonly weighed 12 pounds to 13

pounds per brake horsepower.

For short flights fuel economy was of less importance than a saving

in the weight of the engine. For long flights, however, the case

was different. Thus, if the gasolene consumption was 1/2 pound per

horsepower hour, and the engine weighed 3 pounds per brake horsepower,

the fuel needed for a six-hour flight would weigh as much as the engine,

but for half an hour's flight its weight would be unimportant.

Best Means of Propulsion.

The best method of propulsion was by the screw, which acting in air

was subject to much the same conditions as obtained in marine work.

Its efficiency depended on its diameter and pitch and on its position,

whether in front of or behind the body propelled. From this theory

of dynamic support, Mr. Lanchester proceeded, the efficiency of each

element of a screw propeller could be represented by curves such as were

given in his first lecture before the society, and from these curves the

over-all efficiency of any proposed propeller could be computed, by mere

inspection, with a fair degree of accuracy. These curves showed that the

tips of long-bladed propellers were inefficient, as was also the portion

of the blade near the root. In actual marine practice the blade from

boss to tip was commonly of such a length that the over-all efficiency

was 95 per cent of that of the most efficient element of it.

Advocates Propellers in Rear.

From these curves the diameter and appropriate pitch of a screw could

be calculated, and the number of revolutions was then fixed. Thus, for a

speed of 80 feet per second the pitch might come out as 8 feet, in which

case the revolutions would be 600 per minute, which might, however, be

too low for the motor. It was then necessary either to gear down the

propeller, as was done in the Wright machine, or, if it was decided to

drive it direct, to sacrifice some of the efficiency of the propeller.

An analogous case arose in the application of the steam turbine to the

propulsion of cargo boats, a problem as yet unsolved. The propeller

should always be aft, so that it could abstract energy from the wake

current, and also so that its wash was clear of the body propelled. The

best possible efficiency was about 70 per cent, and it was safe to rely

upon 66 per cent.

Benefits of Soaring Flight.

There was, Mr. Lanchester proceeded, some possibility of the aeronaut

reducing the power needed for transport by his adopting the principle of

soaring flight, as exemplified by some birds. There were, he continued,

two different modes of soaring flight. In the one the bird made use of

the upward current of air often to be found in the neighborhood of steep

vertical cliffs. These cliffs deflected the air upward long before it

actually reached the cliff, a whole region below being thus the seat of

an upward current. Darwin has noted that the condor was only to be found

in the neighborhood of such cliffs. Along the south coast also the gulls

made frequent use of the up currents due to the nearly perpendicular

chalk cliffs along the shore.

In the tropics up currents were also caused by temperature differences.

Cumulus clouds, moreover, were nearly always the terminations of such

up currents of heated air, which, on cooling by expansion in the upper

regions, deposited their moisture as fog. These clouds might, perhaps,

prove useful in the future in showing the aeronaut where up currents

were to be found. Another mode of soaring flight was that adopted by

the albatross, which took advantage of the fact that the air moved

in pulsations, into which the bird fitted itself, being thus able to

extract energy from the wind. Whether it would be possible for the

aeronaut to employ a similar method must be left to the future to

decide.

Main Difficulties in Aviation.

In practical flight difficulties arose in starting and in alighting.

There was a lower limit to the speed at which the machine was stable,

and it was inadvisable to leave the ground till this limit was attained.

Similarly, in alighting it was inexpedient to reduce the speed below the

limit of stability. This fact constituted a difficulty in the adoption

of high speeds, since the length of run needed increased in proportion

to the square of the velocity. This drawback could, however, be

surmounted by forming starting and alighting grounds of ample size.

He thought it quite likely in the future that such grounds would be

considered as essential to the flying machine as a seaport was to an

ocean-going steamer or as a road was to the automobile.

Requisites of Flying Machine.

Flying machines were commonly divided into monoplanes and biplanes,

according as they had one or two supporting surfaces. The distinction

was not, however, fundamental. To get the requisite strength some

form of girder framework was necessary, and it was a mere question of

convenience whether the supporting surface was arranged along both

the top and the bottom of this girder, or along the bottom only. The

framework adopted universally was of wood braced by ties of pianoforte

wire, an arrangement giving the stiffness desired with the least

possible weight. Some kind of chassis was also necessary.

# CHAPTER XXIII. AMATEURS MAY USE WRIGHT PATENTS.

Owing to the fact that the Wright brothers have enjoined a number of

professional aviators from using their system of control, amateurs have

been slow to adopt it. They recognize its merits, and would like to use

the system, but have been apprehensive that it might involve them in

litigation. There is no danger of this, as will be seen by the following

statement made by the Wrights:

What Wright Brothers Say.

"Any amateur, any professional who is not exhibiting for money, is at

liberty to use our patented devices. We shall be glad to have them do

so, and there will be no interference on our part, by legal action, or

otherwise. The only men we proceed against are those who, without our

permission, without even asking our consent, coolly appropriate the

results of our labors and use them for the purpose of making money.

Curtiss, Delagrange, Voisin, and all the rest of them who have used our

devices have done so in money-making exhibitions. So long as there

is any money to be made by the use of the products of our brains, we

propose to have it ourselves. It is the only way in which we can get

any return for the years of patient work we have given to the problem of

aviation. On the other hand, any man who wants to use these devices for

the purpose of pleasure, or the advancement of science, is welcome to do

so, without money and without price. This is fair enough, is it not?"

Basis of the Wright Patents.

In a flying machine a normally flat aeroplane having lateral marginal

portions capable of movement to different positions above or below the

normal plane of the body of the aeroplane, such movement being about

an axis transverse to the line of flight, whereby said lateral marginal

portions may be moved to different angles relatively to the normal

plane of the body of the aeroplane, so as to present to the atmosphere

different angles of incidence, and means for so moving said lateral

marginal portions, substantially as described.

Application of vertical struts near the ends having flexible joints.

Means for simultaneously imparting such movement to said lateral

portions to different angles relatively to each other.

Refers to the movement of the lateral portions on the same side to the

same angle.

Means for simultaneously moving vertical rudder so as to present to

the wind that side thereof nearest the side of the aeroplane having the

smallest angle of incidence.

Lateral stability is obtained by warping the end wings by moving the

lever at the right hand of the operator, connection being made by wires

from the lever to the wing tips. The rudder may also be curved or warped

in similar manner by lever action.

Wrights Obtain an Injunction.

In January, 1910, Judge Hazel, of the United States Circuit Court,

granted a preliminary injunction restraining the Herring-Curtiss

Co., and Glenn H. Curtiss, from manufacturing, selling, or using for

exhibition purposes the machine known as the Curtiss aeroplane. The

injunction was obtained on the ground that the Curtiss machine is an

infringement upon the Wright patents in the matter of wing warping and

rudder control.

It is not the purpose of the authors to discuss the subject pro or con.

Such discussion would have no proper place in a volume of this kind. It

is enough to say that Curtiss stoutly insists that his machine is not

an infringement of the Wright patents, although Judge Hazel evidently

thinks differently.

What the Judge Said.

In granting the preliminary injunction the judge said:

"Defendants claim generally that the difference in construction of their

apparatus causes the equilibrium or lateral balance to be maintained and

its aerial movement secured upon an entirely different principle from

that of complainant; the defendants' aeroplanes are curved, firmly

attached to the stanchions and hence are incapable of twisting or

turning in any direction; that the supplementary planes or so-called

rudders are secured to the forward stanchion at the extreme lateral ends

of the planes and are adjusted midway between the upper and lower

planes with the margins extending beyond the edges; that in moving the

supplementary planes equal and uniform angles of incidence are presented

as distinguished from fluctuating angles of incidence. Such claimed

functional effects, however, are strongly contradicted by the expert

witness for complainant.

Similar to Plan of Wrights.

"Upon this contention it is sufficient to say that the affidavits for

the complainant so clearly define the principle of operation of the

flying machines in question that I am reasonably satisfied that there

is a variableness of the angle of incidence in the machine of defendants

which is produced when a supplementary plane on one side is tilted

or raised and the other stimultaneously tilted or lowered. I am also

satisfied that the rear rudder is turned by the operator to the side

having the least angle of incidence and that such turning is done at the

time the supplementary planes are raised or depressed to prevent tilting

or upsetting the machine. On the papers presented I incline to the view,

as already indicated, that the claims of the patent in suit should be

broadly construed; and when given such construction, the elements of

the Wright machine are found in defendants' machine performing the

same functional result. There are dissimilarities in the defendants'

structure--changes of form and strengthening of parts--which may be

improvements, but such dissimilarities seem to me to have no bearing

upon the means adopted to preserve the equilibrium, which means are the

equivalent of the claims in suit and attain an identical result.

Variance From Patent Immaterial.

"Defendants further contend that the curved or arched surfaces of the

Wright aeroplanes in commercial use are departures from the patent,

which describes 'substantially flat surfaces,' and that such a

construction would be wholly impracticable. The drawing, Fig. 3,

however, attached to the specification, shows a curved line inward of

the aeroplane with straight lateral edges, and considering such drawing

with the terminology of the specification, the slight arching of the

surface is not thought a material departure; at any rate, the patent in

issue does not belong to the class of patents which requires narrowing

to the details of construction."

"June Bug" First Infringement.

Referring to the matter of priority, the judge said:

"Indeed, no one interfered with the rights of the patentees by

constructing machines similar to theirs until in July, 1908, when

Curtiss exhibited a flying machine which he called the 'June Bug.' He

was immediately notified by the patentees that such machine with its

movable surfaces at the tips of wings infringed the patent in suit, and

he replied that he did not intend to publicly exhibit the machine for

profit, but merely was engaged in exhibiting it for scientific purposes

as a member of the Aerial Experiment Association. To this the patentees

did not object. Subsequently, however, the machine, with supplementary

planes placed midway between the upper and lower aeroplanes, was

publicly exhibited by the defendant corporation and used by Curtiss in

aerial flights for prizes and emoluments. It further appears that the

defendants now threaten to continue such use for gain and profit, and to

engage in the manufacture and sale of such infringing machines, thereby

becoming an active rival of complainant in the business of constructing

flying machines embodying the claims in suit, but such use of the

infringing machines it is the duty of this court, on the papers

presented, to enjoin.

"The requirements in patent causes for the issuance of an injunction

pendente lite--the validity of the patent, general acquiescence by the

public and infringement by the defendants--are so reasonably clear that

# I believe if not probable the complainant may succeed at final hearing,

and therefore, status quo should be preserved and a preliminary

injunction granted.

"So ordered."

Points Claimed By Curtiss.

That the Herring-Curtiss Co. will appeal is a certainty. Mr. Emerson R.

Newell, counsel for the company, states its case as follows:

"The Curtiss machine has two main supporting surfaces, both of which are

curved * * * and are absolutely rigid at all times and cannot be moved,

warped or distorted in any manner. The front horizontal rudder is used

for the steering up or down, and the rear vertical rudder is used only

for steering to the right or left, in the same manner as a boat is

steered by its rudder. The machine is provided at the rear with a fixed

horizontal surface, which is not present in the machine of the patent,

and which has a distinct advantage in the operation of defendants'

machine, as will be hereafter discussed.

Does Not Warp Main Surface.

"Defendants' machine does not use the warping of the main supporting

surfaces in restoring the lateral equilibrium, but has two comparatively

small pivoted balancing surfaces or rudders. When one end of the machine

is tipped up or down from the normal, these planes may be thrown in

opposite directions by the operator, and so steer each end of the

machine up or down to its normal level, at which time tension upon them

is released and they are moved back by the pressure of the wind to their

normal position.

Rudder Used Only For Steering.

"When defendants' balancing surfaces are moved they present equal angles

of incidence to the normal rush of air and equal resistances, at each

side of the machine, and there is therefore no tendency to turn around a

vertical axis as is the case of the machine of the patent, consequently

no reason or necessity for turning the vertical rear rudder in

defendants' machine to counteract any such turning tendency. At any

rate, whatever may be the theories in regard to this matter, the fact is

that the operator of defendants' machine does not at any time turn his

vertical rudder to counteract any turning tendency clue to the side

balancing surfaces, but only uses it to steer the machine the same as a

boat is steered."

Aero Club Recognizes Wrights.

The Aero Club of America has officially recognized the Wright patents.

This course was taken following a conference held April 9th, 1910,

participated in by William Wright and Andrew Freedman, representing the

Wright Co., and the Aero Club's committee, of Philip T. Dodge, W. W.

Miller, L. L. Gillespie, Wm. H. Page and Cortlandt F. Bishop.

At this meeting arrangements were made by which the Aero Club recognizes

the Wright patents and will not give its section to any open meet

where the promoters thereof have not secured a license from the Wright

Company.

The substance of the agreement was that the Aero Club of America

recognizes the rights of the owners of the Wright patents under

the decisions of the Federal courts and refuses to countenance the

infringement of those patents as long as these decisions remain in

force.

In the meantime, in order to encourage aviation, both at home and

abroad, and in order to permit foreign aviators to take part in aviation

contests in this country it was agreed that the Aero Club of America, as

the American representative of the International Aeronautic Federation,

should approve only such public contests as may be licensed by the

Wright Company and that the Wright Company, on the other hand, should

encourage the holding of open meets or contests where ever approved as

aforesaid by the Aero Club of America by granting licenses to promoters

who make satisfactory arrangements with the company for its compensation

for the use of its patents. At such licensed meet any machine of any

make may participate freely without securing any further license or

permit. The details and terms of all meets will be arranged by the

committee having in charge the interests of both organizations.

# CHAPTER XXIV. HINTS ON PROPELLER CONSTRUCTION.

Every professional aviator has his own ideas as to the design of

the propeller, one of the most important features of flying-machine

construction. While in many instances the propeller, at a casual glance,

may appear to be identical, close inspection will develop the fact

that in nearly every case some individual idea of the designer has been

incorporated. Thus, two propellers of the two-bladed variety, while of

the same general size as to length and width of blade, will vary greatly

as to pitch and "twist" or curvature.

What the Designers Seek.

Every designer is seeking for the same result--the securing of the

greatest possible thrust, or air displacement, with the least possible

energy.

The angles of any screw propeller blade having a uniform or true pitch

change gradually for every increased diameter. In order to give a

reasonably clear explanation, it will be well to review in a primary way

some of the definitions or terms used in connection with and applied to

screw propellers.

Terms in General Use.

Pitch.--The term "pitch," as applied to a screw propeller, is the

theoretical distance through which it would travel without slip in one

revolution, and as applied to a propeller blade it is the angle at

which the blades are set so as to enable them to travel in a spiral path

through a fixed distance theoretically without slip in one revolution.

Pitch speed.--The term "pitch speed" of a screw propeller is the speed

in feet multiplied by the number of revolutions it is caused to make

in one minute of time. If a screw propeller is revolved 600 times

per minute, and if its pitch is 7 ft., then the pitch speed of such a

propeller would be 7x600 revolutions, or 4200 ft. per minute.

Uniform pitch.--A true pitch screw propeller is one having its blades

formed in such a manner as to enable all of its useful portions, from

the portion nearest the hub to its outer portion, to travel at a uniform

pitch speed. Or, in other words, the pitch is uniform when the projected

area of the blade is parallel along its full length and at the same time

representing a true sector of a circle.

All screw propellers having a pitch equal to their diameters have the

same angle for their blades at their largest diameter.

When Pitch Is Not Uniform.

A screw propeller not having a uniform pitch, but having the same angle

for all portions of its blades, or some arbitrary angle not a true

pitch, is distinguished from one having a true pitch in the variation of

the pitch speeds that the various portions of its blades are forced to

travel through while traveling at its maximum pitch speed.

On this subject Mr. R. W. Jamieson says in Aeronautics:

"Take for example an 8-foot screw propeller having an 8-foot pitch at

its largest diameter. If the angle is the same throughout its entire

blade length, then all the porions of its blades approaching the hub

from its outer portion would have a gradually decreasing pitch. The

2-foot portion would have a 2-foot pitch; the 3-foot portion a 3-foot

pitch, and so on to the 8-foot portion which would have an 8-foot pitch.

When this form of propeller is caused to revolve, say 500 r.p.m., the

8-foot portion would have a calculated pitch speed of 8 feet by 500

revolutions, or 4,000 feet per min.; while the 2-foot portion would have

a calculated pitch speed of 500 revolutions by 2 feet, or 1,000 feet per

minute.

Effect of Non-Uniformity.

"Now, as all of the portions of this type of screw propeller must travel

at some pitch speed, which must have for its maximum a pitch speed

in feet below the calculated pitch speed of the largest diameter, it

follows that some portions of its blades would perform useful work

while the action of the other portions would be negative--resisting

the forward motion of the portions having a greater pitch speed. The

portions having a pitch speed below that at which the screw is traveling

cease to perform useful work after their pitch speed has been exceeded

by the portions having a larger diameter and a greater pitch speed.

"We might compare the larger and smaller diameter portions of this form

of screw propeller, to two power-driven vessels connected with a line,

one capable of traveling 20 miles per hour, the other 10 miles per hour.

It can be readily understood that the boat capable of traveling 10 miles

per hour would have no useful effect to help the one traveling 20 miles

per hour, as its action would be such as to impose a dead load upon the

latter's progress."

The term "slip," as applied to a screw propeller, is the distance

between its calculated pitch speed and the actual distance it travels

through under load, depending upon the efficiency and proportion of its

blades and the amount of load it has to carry.

The action of a screw propeller while performing useful work might be

compared to a nut traveling on a threaded bolt; little resistance is

offered to its forward motion while it spins freely without load, but

give it a load to carry; then it will take more power to keep up its

speed; if too great a load is applied the thread will strip, and so

it is with a screw propeller gliding spirally on the air. A propeller

traveling without load on to new air might be compared to the nut

traveling freely on the bolt. It would consume but little power and it

would travel at nearly its calculated pitch speed, but give it work to

do and then it will take power to drive it.

There is a reaction caused from the propeller projecting air backward

when it slips, which, together with the supporting effect of the blades,

combine to produce useful work or pull on the object to be carried.

A screw propeller working under load approaches more closely to its

maximum efficiency as it carries its load with a minimum amount of slip,

or nearing its calculated pitch speed.

Why Blades Are Curved.

It has been pointed out by experiment that certain forms of curved

surfaces as applied to aeroplanes will lift more per horse power, per

unit of square foot, while on the other hand it has been shown that a

flat surface will lift more per horse power, but requires more area of

surface to do it.

As a true pitch screw propeller is virtually a rotating aeroplane, a

curved surface may be advantageously employed when the limit of size

prevents using large plane surfaces for the blades.

Care should be exercised in keeping the chord of any curve to be used

for the blades at the proper pitch angle, and in all cases propeller

blades should be made rigid so as to preserve the true angle and not be

distorted by centrifugal force or from any other cause, as flexibility

will seriously affect their pitch speed and otherwise affect their

efficiency.

How to Determine Angle.

To find the angle for the proper pitch at any point in the diameter of

a propeller, determine the circumference by multiplying the diameter by

3.1416, which represent by drawing a line to scale in feet. At the end

of this line draw another line to represent the desired pitch in feet.

Then draw a line from the point representing the desired pitch in feet

to the beginning of the circumference line. For example:

If the propeller to be laid out is 7 feet in diameter, and is to have

a 7-foot pitch, the circumference will be 21.99 feet. Draw a diagram

representing the circumference line and pitch in feet. If this diagram

is wrapped around a cylinder the angle line will represent a true thread

7 feet in diameter and 7 feet long, and the angle of the thread will be

17 3/4 degrees.

Relation of Diameter to Circumference.

Since the areas of circles decrease as the diameter lessens, it follows

that if a propeller is to travel at a uniform pitch speed, the volume of

its blade displacement should decrease as its diameter becomes less, so

as to occupy a corresponding relation to the circumferences of larger

diameters, and at the same time the projected area of the blade must be

parallel along its full length and should represent a true sector of a

circle.

Let us suppose a 7-foot circle to be divided into 20 sectors, one of

which represents a propeller blade. If the pitch is to be 7 feet, then

the greatest depth of the angle would be 1/20 part of the pitch, or 4

2/10 inch. If the line representing the greatest depth of the angle is

kept the same width as it approaches the hub, the pitch will be uniform.

If the blade is set at an angle so its projected area is 1/20 part of

the pitch, and if it is moved through 20 divisions for one revolution,

it would have a travel of 7 feet.

# CHAPTER XXV. NEW MOTORS AND DEVICES.

Since the first edition of this book was printed, early in 1910, there

has been a remarkable advance in the construction of aeroplane motors,

which has resulted in a wonderful decrease in the amount of surface area

from that formerly required. Marked gain in lightness and speed of

the motor has enabled aviators to get along, in some instances, with

one-quarter of the plane supporting area previously used. The first

Wright biplane, propelled by a motor of 25 h.p., productive of a fair

average speed of 30 miles an hour, had a plane surface of 538 square

feet. Now, by using a specially designed motor of 65 h. p., capable

of developing a speed of from 70 to 80 miles an hour, the Wrights are

enabled to successfully navigate a machine the plane area of which is

about 130 square feet. This apparatus is intended to carry only one

person (the operator). At Belmont Park, N. Y., the Wrights demonstrated

that the small-surfaced biplane is much faster, easier to manage in the

hands of a skilled manipulator, and a better altitude climber than the

large and cumbersome machines with 538 square feet of surface heretofore

used by them.

In this may be found a practical illustration of the principle that

increased speed permits of a reduction in plane area in mathematical

ratio to the gain in speed. The faster any object can be made to move

through the air, the less will be the supporting surface required to

sustain a given weight. But, there is a limit beyond which the plane

surface cannot be reduced with safety. Regard must always be had to

the securing of an ample sustaining surface so that in case of motor

stoppage there will be sufficient buoyancy to enable the operator to

descend safely.

The baby Wright used at the Belmont Park (N. Y.) aviation meet in the

fall of 1910, had a plane length of 19 feet 6 inches, and an extreme

breadth of 21 feet 6 inches, with a total surface area of 146 square

feet. It was equipped with a new Wright 8-cylinder motor of 60 h. p.,

and two Wright propellers of 8 feet 6 inches diameter and 500 r. p. m.

It was easily the fastest machine at the meet. After the tests, Wilbur

Wright said:

"It is our intention to put together a machine with specially designed

propellers, specially designed gears and a motor which will give us 65

horsepower at least. We will then be able, after some experimental work

we are doing now, to send forth a machine that will make a new speed

record."

In the new Wright machines the front elevating planes for up-and-down

control have been eliminated, and the movements of the apparatus are now

regulated solely by the rear, or "tail" control.

A Powerful Light Motor.

Another successful American aviation motor is the aeromotor,

manufactured by the Detroit Aeronautic Construction. Aeromotors are made

in four models as follows:

Model 1.--4-cylinder, 30-40 h. p., weight 200 pounds.

Model 2.--4-cylinder, (larger stroke and bore) 40-50 h. p., weight 225

pounds.

Model 3.--6-cylinder. 50-60 h. p., weight 210 pounds.

Model 4.--6-cylinder, 60-75 h. p., weight 275 pounds.

This motor is of the 4-cycle, vertical, water-cooled type. Roberts

Aviation Motor.

One of the successful aviation motors of American make, is that produced

by the Roberts Motor Co., of Sandusky, Ohio. It is designed by E. W.

Roberts, M. E., who was formerly chief assistant and designer for Sir

Hiram Maxim, when the latter was making his celebrated aeronautical

experiments in England in 1894-95. This motor is made in both the

4- and 6-cylinder forms. The 4-cylinder motor weighs complete with Bosch

magneto and carbureter 165 pounds, and will develop 40 actual brake

h. p. at 1,000 r. p. m., 46 h. p. at 1,200 and 52 h. p. at 1,400. The

6-cylinder weighs 220 pounds and will develop 60 actual brake h. p. at

1,000 r. p. m., 69 h. p. at 1,200 and 78 h. p. at 1,500.

Extreme lightness has been secured by doing away with all superfluous

parts, rather than by a shaving down of materials to a dangerous

thinness. For example, there is neither an intake or exhaust manifold on

the motor. The distributing valve forms a part of the crankcase as

does the water intake, and the gear pump. Magnalium takes the place of

aluminum in the crankcase, because it is not only lighter but stronger

and can be cast very thin. The crankshaft is 2 1/2-inch diameter with a

2 1/4-inch hole, and while it would be strong enough in ordinary 40

per cent carbon steel it is made of steel twice the strength of that

customarily employed. Similar care has been exercised on other parts and

the result is a motor weighing 4 pounds per h. p.

The Rinek Motor.

The Rinek aviation motor, constructed by the Rinek Aero Mfg. Co., of

Easton, Pa., is another that is meeting with favor among aviators. Type

B-8 is an 8-cylinder motor, the cylinders being set at right angles,

on a V-shaped crank case. It is water cooled, develops 50-60 h. p., the

minimum at 1,220 r. p. m., and weighs 280 pounds with all accessories.

Type B-4, a 4-cylinder motor, develops 30 h. p. at 1,800 r. p. m., and

weighs 130 pounds complete. The cylinders in both motors are made of

cast iron with copper water jackets.

The Overhead Camshaft Boulevard.

The overhead camshaft Boulevard is still another form of aviation motor

which has been favorably received. This is the product of the Boulevard

Engine Co., of St. Louis. It is made with 4 and 8 cylinders. The former

develops 30-35 h. p. at 1,200 r. p. m., and weighs 130 pounds. The

8-cylinder motor gives 60-70 h. p. at 1,200 r. p. m., and weighs 200

pounds. Simplicity of construction is the main feature of this motor,

especially in the manipulation of the valves.

# CHAPTER XXVI. MONOPLANES, TRIPLANES, MULTIPLANES.

Until recently, American aviators had not given serious attention to

any form of flying machines aside from biplanes. Of the twenty-one

monoplanes competing at the International meet at Belmont Park, N. Y.,

in November, 1910, only three makes were handled by Americans. Moissant

and Drexel navigated Bleriot machines, Harkness an Antoinette, and Glenn

Curtiss a single decker of his own construction. On the other hand the

various foreign aviators who took part in the meet unhesitatingly gave

preference to monoplanes.

Whatever may have been the cause of this seeming prejudice against

the monoplane on the part of American air sailors, it is slowly being

overcome. When a man like Curtiss, who has attained great success with

biplanes, gives serious attention to the monoplane form of construction

and goes so far as to build and successfully operate a single surface

machine, it may be taken for granted that the monoplane is a fixture in

this country.

Dimensions of Monoplanes.

The makes, dimensions and equipment of the various monoplanes used at

Belmont Park are as follows:

Bleriot--(Moissant, operator)--plane length 23 feet, extreme breadth 28

feet, surface area 160 square feet, 7-cylinder, 50 h. p. Gnome engine,

Chauviere propeller, 7 feet 6 inches diameter, 1,200 r. p. m.

Bleriot--(Drexel, operator)--exactly the same as Moissant's machine.

Antoinette--(Harkness, operator)--plane length 42 feet, extreme breadth

46 feet, surface area 377 square feet, Emerson 6-cylinder, 50 h. p.

motor, Antoinette propeller, 7 feet 6 inches diameter, 1,200 r. p. m.

Curtiss--(Glenn H. Curtiss, operator)--plane length 25 feet, extreme

breadth 26 feet, surface area 130 square feet, Curtiss 8-cylinder, 60 h.

p. motor, Paragon propeller, 7 feet in diameter, 1,200 r. p. m.

With one exception Curtiss had the smallest machine of any of those

entering into competition. The smallest was La Demoiselle, made by

Santos-Dumont, the proportions of which were: plane length 20 feet,

extreme breadth 18 feet, surface area 100 square feet, Clement-Bayard

2-cylinder, 30 h. p. motor, Chauviere propeller, 6 feet 6 inches in

diameter, 1,100 r. p. m.

Winnings Made with Monoplanes.

Operators of monoplanes won a fair share of the cash prizes. They won

$30,283 out of a total of $63,250, to say nothing about Grahame-White's

winnings. The latter won $13,600, but part of his winning flights were

made in a Bleriot monoplane, and part in a Farman machine. Aside from

Grahame-White the winnings were divided as follows: Moissant (Bleriot)

$13,350; Latham (Antoinette) $8,183; Aubrun (Bleriot) $2,400; De Lesseps

(Bleriot) $2,300; Drexel (Bleriot) $1,700; Radley (Bleriot) $1,300;

Simon (Bleriot) $750; Andemars (Clement-Bayard) $100; Barrier (Bleriot)

$100.

Out of a total of $30,283, operators of Bleriot machines won $21,900,

again omitting Grahame-White's share. If the winnings with monoplane and

biplane could be divided so as to show the amount won with each type

of machine the credit side of the Bleriot account would be materially

enlarged.

The Most Popular Monoplanes.

While the number of successful monoplanes is increasing rapidly, and

there is some feature of advantage in nearly all the new makes, interest

centers chiefly in the Santos-Dumont, Antoinette and Bleriot machines.

This is because more has been accomplished with them than with any of

the others, possibly because they have had greater opportunities.

For the guidance of those who may wish to build a machine of the

monoplane type after the Santos-Dumont or Bleriot models, the following

details will be found useful.

Santos-Dumont--The latest production of this maker is called the "No. 20

Baby." It is of 18 feet spread, and 20 feet over all in depth. It stands

4 feet 2 inches in height, not counting the propeller. When this latter

is in a vertical position the extreme height of the machine is 7 feet

5 inches. It is strictly a one-man apparatus. The total surface area

is 115 square feet. The total weight of the monoplane with engine and

propeller is 352 pounds. Santos-Dumont weighs 110 pounds, so the entire

weight carried while in flight is 462 pounds, or about 3.6 pounds per

square foot of surface.

Bamboo is used in the construction of the body frame, and also for the

frame of the tail. The body frame consists of three bamboo poles about

2 inches in diameter at the forward end and tapering to about 1 inch at

the rear. These poles are jointed with brass sockets near the rear of

the main plane so they may be taken apart easily for convenience in

housing or transportation. The main plane is built upon four transverse

spars of ash, set at a slight dihedral angle, two being placed on each

side of the central bamboo. These spars are about 2 inches wide by 1

1/8-inch deep for a few feet each side of the center of the machine, and

from there taper down to an inch in depth at the center bamboo, and at

their outer ends, but the width remains the same throughout their entire

length. The planes are double surfaced with silk and laced above and

below the bamboo ribs which run fore and aft under the main spars and

terminate in a forked clip through which a wire is strung for lacing on

the silk. The tail consists of a horizontal and vertical surface placed

on a universal joint about 10 feet back of the rear edge of the main

plane. Both of these surfaces are flat and consist of a silk covering

stretched upon bamboo ribs. The horizontal surface is 6 feet 5 inches

across, and 4 feet 9 inches from front to back. The vertical surface

is of the same width (6 feet 5 inches) but is only 3 feet 7 inches

from front to back. All the details of construction are shown in the

accompanying illustration.

Power is furnished by a very light (110 pounds) Darracq motor, of the

double-opposed-cylinder type. It has a bore of 4.118 inches, and stroke

of 4.724 inches, runs at 1,800 r. p. m., and with a 6 1/2-foot propeller

develops a thrust of 242 1/2 pounds when the monoplane is held steady.

Bleriot--No. XI, the latest of the Bleriot productions, and the greatest

record maker of the lot, is 28 feet in spread of main plane, and depth

of 6 feet in largest part. This would give a main surface of 168 square

feet, but as the ends of the plane are sharply tapered from the rear,

the actual surface is reduced to 150 square feet. Projecting from

the main frame is an elongated tail (shown in the illustration) which

carries the horizontal and vertical rudders. The former is made in three

sections. The center piece is 6 feet 1 inch in spread, and 2 feet 10

inches in depth, containing 17 square feet of surface. The end sections,

which are made movable for warping purposes, are each 2 feet 10 inches

square, the combined surface area in the entire horizontal rudder

being 33 square feet. The vertical rudder contains 4 1/2 square feet of

surface, making the entire supporting area 187 1/2 square feet.

From the outer end of the propeller shaft in front to the extreme rear

edge of the vertical rudder, the machine is 25 feet deep. Deducting

the 6-foot depth of the main plane leaves 19 feet as the length of the

rudder beam and rudders. The motor equipment consists of a 3-cylinder,

air-cooled engine of about 30 h. p. placed at the front end of the body

frame, and carrying on its crankshaft a two-bladed propeller 6 feet 8

inches in diameter. The engine speed is about 1,250 r. p. m. at which

the propeller develops a thrust of over 200 pounds.

The Bleriot XI complete weighs 484 pounds, and with operator and fuel

supply ready for a 25- or 30-mile flight, 715 pounds. One peculiarity of

the Bleriot construction is that, while the ribs of the main plane are

curved, there is no preliminary bending of the pieces as in other forms

of construction. Bleriot has his rib pieces cut a little longer than

required and, by springing them into place, secures the necessary

curvature. A good view of the Bleriot plane framework is given on page

63.

Combined Triplane and Biplane.

At Norwich, Conn., the Stebbins-Geynet Co., after several years of

experiment, has begun the manufacture of a combination triplane and

biplane machine. The center plane, which is located about midway

between the upper and lower surfaces, is made removable. The change

from triplane to biplane, or vice versa, may be readily made in a few

minutes. The constructors claim for this type of air craft a large

supporting surface area with the minimum of dimensions in planes.

Although this machine has only 24-foot spread and is only 26 feet over

all, its total amount of supporting area is 400 square feet; weight, 600

pounds in flying order, and lifting capacity approximately 700 pounds

more.

The frame is made entirely of a selected grade of Oregon spruce,

finished down to a smooth surface and varnished. All struts are

fish-shaped and set in aluminum sockets, which are bolted to top and

lower beams with special strong bolts of small diameter. The middle

plane is set inside the six uprights and held in place by aluminum

castings. A flexible twisted seven-strand wire cable and Stebbins-Geynet

turnbuckles are used for trussing.

The top plane is in three sections, laced together. It has a 24-foot

spread and is 7 feet in depth. The middle plane is in two sections each

of 7 1/2 feet spread and 6 feet in depth. The center ends of the middle

plane sections do not come within 5 feet of joining, this open space

being left for the engine. The bottom plane is of 16 feet spread and 5

feet in depth. It will thus be seen that the planes overhang one another

in depth, the bottom one being the smallest in this respect. The planes

are set at an angle of 9 degrees, and there is a clear space of 3 1/2

feet between each, making the total distance from the bottom to the top

plane a trifle over 7 feet. The total supporting surface in the main

planes is 350 square feet. By arranging the three plane surfaces at

an angle as described and varying their size, the greatest amount of

lifting area is secured above the center of gravity, and the greatest

weight carried below.

The ribs are made of laminated spruce, finished down to 1/2x3/4-inch

cross section dimensions, with a curvature of about 1 in 20, and

fastened to the beams with special aluminum castings. Number 2 Naiad

aeroplane cloth is used in covering the planes, with pockets sewn in for

the ribs.

Two combination elevating rudders are set up well in front, each having

18 square feet of supporting area. These rudders are arranged to work

in unison, independently, or in opposite directions. In the Model B

machine, there are also two small rear elevating rudders, which work in

unison with the front rudders. One vertical rudder of 10 square feet is

suspended in the rear of a small stationary horizontal plane in Model A,

while the vertical rudder on Model B is only 6 square feet in size. The

elevating rudders are arranged so as to act as stabilizing planes when

the machine is in flight. The wing tips are held in place with a special

two-piece casting which forms a hinge, and makes a quick detachable

joint. Wing tips are also used in balancing.

Model A is equipped with a Cameron 25-30 h. p., 4-cylinder, air-cooled

motor. On Model B a Holmes rotary 7-cylinder motor of 4x4-inch bore and

stroke is used.

Positive control is secured by use of the Stebbins-Geynet "auto-control"

system. A pull or push movement operates the elevating rudders, while

the balancing is done by means of side movements or slight turns. The

rear vertical rudder is manipulated by means of a foot lever.

New Cody Biplane.

Among the comparatively new biplanes is one constructed by Willard F.

Cody, of London, Eng., the principal distinctive feature of which is an

automatic control which works independently of the hand levers. For the

other control a long lever carrying a steering wheel furnishes all the

necessary control movements, there being no footwork at all. The lever

is universally jointed and when moved fore and aft operates the two

ailerons as if they were one; when the shaft is rotated it moves the

tail as a whole. The horizontal tail component is immovable. When the

lever is moved from side to side it works not only the ailerons and

the independent elevators, but also through a peculiar arrangement, the

vertical rear rudder as well.

The spread of the planes is 46 feet 6 inches and the width 6 feet 6

inches. The ailerons jut out 1 foot 6 inches on each side of the machine

and are 13 feet 6 inches long. The cross-shaped tail is supported by an

outrigger composed of two long bamboos and of this the vertical plane

is 9 feet by 4 feet, while the horizontal plane is 8 feet by 4 feet. The

over-all length of the machine is 36 feet. The lifting surface is 857

square feet. It will weigh, with a pilot, 1,450 pounds. The distance

between the main planes is 8 feet 6 inches, which is a rather notable

feature in this flyer.

The propeller has a diameter of 11 feet and 2 inches with a 13-foot

6-inch pitch; it is driven at 560 revolutions by a chain, and the gear

reduction between the chain and propeller shaft is two to one.

The machine from elevator to tail plane bristles in original points. The

hump in the ribs has been cut away entirely, so that although the plane

is double surfaced, the surfaces are closest together at a point which

approximates the center of pressure. The plane is practically of two

stream-line forms, of which one is the continuation of the other.

This construction, claims the inventor, will give increased lift, and

decreased head resistance. The trials substantiate this, as the angle of

incidence in flying is only about one in twenty-six.

The ribs in the main planes are made of strips of silver spruce one-half

by one-half inch, while those in the ailerons are solid and one-fourth

inch thick. In the main planes the fabric is held down with thin wooden

fillets. Cody's planes are noted for their neatness, rigidity and

smoothness. Pegamoid fabric is used throughout.

Pressey Automatic Control.

Another ingenious system of automatic control has been perfected by Dr.

J. B. Pressey, of Newport News, Va. The aeroplane is equipped with a

manually operated, vertical rudder, (3), at the stern, and a horizontal,

manually operated, front control, (4), in front. At the ends of the main

plane, and about midway between the upper and lower sections thereof,

there are supplemental planes, (5).

In connection with these supplemental planes (5), there is employed a

gravity influenced weight, the aviator in his seat, for holding them in

a horizontal, or substantially horizontal, position when the main plane

is traveling on an even keel; and for causing them to tip when the main

plane dips laterally, to port or starboard, the planes (5) having

a lifting effect upon the depressed end of the main plane, and a

depressing effect upon the lifted end of the main plane, so as to

correct such lateral dip of the main plane, and restore it to an even

keel. To the forward, upper edge of planes (5) connection is made by

means of rod (13) to one arm of a bellcrank lever, (14) the latter being

pivotally mounted upon a fore and aft pin (15), supported from the main

plane; and the other arms of the port and starboard bellcrank levers

(16), are connected by rod (17), which has an eye (18), for receiving

the segmental rod (19), secured to and projecting from cross bar on seat

supporting yoke (7). When, therefore, the main plane tips downwardly on

the starboard side, the rod (17) will be moved bodily to starboard, and

the starboard balancing plane (5) will be inclined so as to raise its

forward edge and depress its rear edge, while, at the same time, the

port balancing plane (5), will be inclined so as to depress its forward

edge, and raise its rear edge, thereby causing the starboard balancing

plane to exert a lifting effect, and the port balancing plane to exert a

depressing effect upon the main plane, with the result of restoring the

main plane to an even keel, at which time the balancing planes (5), will

have resumed their normal, horizontal position.

When the main plane dips downwardly on the port side, a reverse action

takes place, with the like result of restoring the main plane to an even

keel. In order to correct forward and aft dip of the main plane, fore

and aft balancing planes (20) and (23) are provided. These planes are

carried by transverse rock shafts, which may be pivotally mounted in

any suitable way, upon structures carried by main plane. In the present

instance, the forward balancing plane is pivotally mounted in extensions

(21) of the frame (22) which carries the forward, manually operated,

horizontal ascending and descending plane

It is absolutely necessary, in making a turn with an aeroplane, if that

turn is to be made in safety, that the main plane shall be inclined, or

"banked," to a degree proportional to the radius of the curve and to the

speed of the aeroplane. Each different curve, at the same speed, demands

a different inclination, as is also demanded by each variation in speed

in rounding like curves. This invention gives the desired result with

absolute certainty.

The Sellers' Multiplane.

Another innovation is a multiplane, or four-surfaced machine, built and

operated by M. B. Sellers, formerly of Grahn, Ky., but now located at

Norwood, Ga. Aside from the use of four sustaining surfaces, the novelty

in the Sellers machine lies in the fact that it is operated successfully

with an 8 h. p. motor, which is the smallest yet used in actual flight.

In describing his work, Mr. Sellers says his purpose has been to develop

the efficiency of the surfaces to a point where flight may be obtained

with the minimum of power and, judging by the results accomplished,

he has succeeded. In a letter written to the authors of this book, Mr.

Sellers says:

"I dislike having my machine called a quadruplane, because the number of

planes is immaterial; the distinctive feature being the arrangement

of the planes in steps; a better name would be step aeroplane, or step

plane.

"The machine as patented, comprises two or more planes arranged in step

form, the highest being in front. The machine I am now using has four

planes 3 ft. x 18 ft.; total about 200 square feet; camber (arch) 1 in

16.

"The vertical keel is for lateral stability; the rudder for direction.

This is the first machine (so far as I know) to have a combination

of wheels and runners or skids (Oct. 1908). The wheels rise up

automatically when the machine leaves the ground, so that it may alight

on the runners.

"A Duthirt & Chalmers 2-cylinder opposed, 3 1/8-inch engine was used

first, and several hundred short flights were made. The engine gave

four brake h. p., which was barely sufficient for continued flight. The

aeroplane complete with this engine weighed 78 pounds. The engine now

used is a Bates 3 5/8-inch, 2-cylinder opposed, showing 8 h. p., and

apparently giving plenty of power. The weight of aeroplane with this

engine is now 110 pounds. Owing to poor grounds only short flights have

been made, the longest to date (Dec. 31, 1910) being about 1,000 feet.

"In building the present machine, my object was to produce a safe, slow,

light, and small h. p. aeroplane, a purpose which I have accomplished."

# CHAPTER XXVII. 1911 AEROPLANE RECORDS.

THE WORLD AT LARGE.

Greatest Speed Per Hour, Whatever Length of Flight, Aviator Alone--E.

Nieuport, Mourmelon, France, June 21, Nieuport Machine, 82.72 miles;

with one passenger, E. Nieuport, Moumlelon, France, June 12, Nieuport

Machine, 67.11 miles; with two passengers, E. Nieuport, Mourmelon,

France, March 9, Nieuport Machine, 63.91 miles; with three passengers,

G. Busson, Rheims, France, March 10, Deperdussin Machine, 59.84 miles;

with four passengers, G. Busson, Rheims, France, March 10, Deperdussin

Machine, 54.21 miles.

Greatest Distance Aviator Alone--G. Fourny, no stops, Buc, France,

September 2, M. Farman Machine, 447.01 miles; E. Helen, three stops,

Etampes, France, September 8, Nieuport Machine, 778.45 miles; with

one passenger, Lieut. Bier, Austria, October 2, Etrich Machine, 155.34

miles; with two passengers, Lieut. Bier, Austria, October 4, Etrich

Machine, 69.59 miles; with three passengers, G. Busson, Rheims, France,

March 10, Deperdussin Machine, 31.06 miles; with four passengers, G.

Busson, Rheims, France, March 10, Deperdussin Machine, 15.99 miles.

Greatest Duration Aviator Alone--G. Fourny, no stops, Buc, France,

September 2, M. Farman Machine, 11 hours, 1 minute, 29 seconds, E.

Helen, three stops, Etampes, France, September 8, Nieuport Machine, 14

hours, 7 minutes, 50 seconds, 13 hours, 17 minutes net time; with one

passenger, Suvelack, Johannisthal, Germany, December 8, 4 hours, 23

minutes; with two passengers, T. de W. Milling, Nassau Boulevard, New

York, September 26, Burgess-Wright Machine, 1 hour, 54 minutes, 42 3-5

seconds; with three passengers, Warchalowski, Wiener-Neustadt, Aust.,

October 30, 45 minutes, 46 seconds; with four passengers, G. Busson,

Rheims, France, March 10, Deperdussin Machine, 17 minutes, 28 1-5

seconds.

Greatest Altitude Aviator Alone--Garros, St. Malo, France, September

4, Bleriot Machine, 13,362 feet; with one passenger, Prevost, Courcy,

France, December 2, 9,840 feet; with two passengers, Lieut. Bier,

Austria, Etrich Machine, 4,010 feet.

AMERICAN RECORDS.

Greatest Speed Per Hour, Whatever Length of Flight, Aviator Alone--A.

Leblanc, Belmont Park, N. Y., October 29, Bleriot Machine, 67.87 miles;

with one passenger, C. Grahame-White, Squantum, Mass., September 4,

Nieuport Machine, 63.23 miles; with two passengers, T. O. M. Sopwith,

Chicago, Ill., August 15, Wright Machine, 34.96 miles.

Greatest Distance Aviator Alone--St. Croix Johnstone, Mineola, N. Y.,

July 27, Moisant (Bleriot Type) Machine, 176.23 miles.

Greatest Duration Aviator Alone--Howard W. Gill, Kinloch, Mo., October

19, Wright Machine, 4 hours, 16 minutes, 35 seconds; with one passenger,

G. W. Beatty, Chicago, Ill., August 19, Wright Machine, 3 hours, 42

minutes, 22 1-5 seconds; with two passengers, T. de W. Milling, Nassau

Boulevard, N. Y., September 26, Burgess-Wright Machine, 1 hour, 54

minutes, 42 3-5 seconds.

Greatest Altitude Aviator Alone--L. Beachy, Chicago, Ill., August 20,

Curtiss Machine, 11,642 feet; with one passenger, C. Grahame-White,

Nassau Boulevard, N. Y., September 30, Nieuport Machine, 3,347 feet.

Weight Carrying--P. O. Parmelee, Chicago, III., August 19, Wright

Machine, 458 lbs.

AVIATION DEVELOPMENT.

The wonderful progress made in the science of aviation during the

year 1911 far surpasses any twelve months' advancement recorded. The

advancement has not been confined to any country or continent, since

every part of the world is taking its part in aviation history making.

The rapidly increasing interest in aviation has brought forth schools

for the instruction of flying in both the old and new world, and

licensed air pilots before they receive their sanctions from the

governing aero clubs of their country are required to pass an extremely

trying examination in actual flights. Exhibition flights and races

were common in all parts of the world during 1911, and touring aviators

visited India, China, Japan, South Africa, Australia and South America,

giving exhibitions and instruction.

Europe was the scene of a number of cross-country races in which entries

ranging from ten to twenty aviators flew from city to city around a

given circuit, which in some instances exceeded 1,000 miles in distance.

Cross-country flights with and without passengers became so common that

those of less than two hours' duration attracted little attention.

There were fewer attempts at high altitude soaring, although the world's

record in this department of aviation was bettered several times. In

place of these high flights, the aviators devoted more attention to

speed, duration and spectacular manoeuvres, which appeared to satisfy

the spectators. The prize money won during 1911 exceeded $1,000,000, but

owing to the increased number of aviators the individual winnings were

not as large as in 1910.

It is estimated that within the past twelve months more than 300,000

miles have been covered in aeroplane flights and more than seven

thousand persons, classed either as aviators or passengers, taken up

into the air. The aeroplane of today ranges through monoplane, biplane,

triplane and even quadraplane, and more than two hundred types of these

machines are in use.

Aeroplanes are becoming a factor of international commerce. The records

of the Bureau of Statistics show that more than $50,000 worth of

aeroplanes were imported into, and exported from, the United States in

the months of July, August and September, 1911. The Bureau of Statistics

only began the maintenance of a separate record of this comparatively

new article of commerce with the opening of the fiscal year 1911-12.

Two of the prominent developments of 1911 were the introduction of

the hydro-aeroplane and the motorless glider experiments of the

Wright brothers at Killdevil Hills, N. C., where during the two weeks'

experiments numerous flights with and against the wind were made,

culminating in the establishing of a record by Orville Wright on October

25, 1911, when in a 52-mile per hour blow he reached an elevation of 225

feet and remained in the air 10 minutes and 34 seconds. The search

for the secret of automatic stability still continues, and though some

remarkable progress has been made the solution has not yet been reached.

NOTABLE CROSS-COUNTRY FLIGHTS OF 1911.

One of the important features of 1911 in aviation was the rapid increase

in the number and distance of cross-country flights made either for the

purpose of exhibition, testing, instruction or pleasure. Flights between

cities in almost every country of the world became common occurrences.

So great was the number that only those of more than ordinary importance

because of speed, distance or duration are recorded. The flights of

Harry N. Atwood from Boston to Washington and from St. Louis to New

York, and C. P. Rodgers from New York to Los Angeles were the most

important events of the kind in this country. The St Louis to New York

flight was a distance by air route, 1,266 miles. Duration of flight, 12

days. Net flying time, 28 hours 53 minutes. Average daily flight, 105.5

miles. Average speed, 43.9 miles per hour.

Transcontinental Flight of Calbraith P. Rodgers.--All world records

for cross-country flying were broken during the New York to Los Angeles

flight of Calbraith P. Rodgers, who left Sheepshead Bay, N. Y., on

Sunday, September 17, 1911, and completed his flight to the Pacific

Coast on Sunday, November 5, at Pasadena, Cal. Rodgers flew a Wright

biplane, and during his long trip the machine was repeatedly repaired,

so great was the strain of the long journey in the air. Rodgers is

estimated to have covered 4,231 miles, although the actual route as

mapped out was but 4,017 miles. Elapsed time to Pasadena, Cal., 49 days;

actual time in the air, 4,924 minutes, equivalent to 3 days 10 hours 4

minutes; average speed approximating 51 miles per hour. Rodgers' longest

flight in one day was from Sanderson to Sierra Blanca, Texas, on October

28, when he covered 231 miles. On November 12, Rodgers fell at Compton,

Cal., and was badly injured, causing a delay of 28 days.

European Circuit Race.--Started from Paris on June 18, 1911. Distance,

1,073 miles, via Paris to Liege; Liege to Spa to Liege; Liege to

Utrecht, Holland; Utrecht to Brussels, Belgium; Brussels to Roubaix;

Roubaix to Calais; Calais to London; London to Calais and Calais to

Paris. Three aeronauts were killed either at the start or shortly after

the race was in progress. They were Capt. Princetau, M. Le Martin and

M. Lendron. Three others were injured by falls. Seven hundred thousand

spectators witnessed the start from the aviation field at Vincennes,

near Paris. There were more than forty starters, of which eight

finished. The winner, Lieut. Jean Conneau, who flies under the name of

"Andre Beaumont," completed the circuit on July 7; his actual net flying

time for the distance being 58h. 38m. 4-5s.

Circuit of England Race--1,010 Miles in Five Sections.--

Start, July 22. Finish, July 26. Prize, $50,000. Twenty-eight entries

and eighteen starters. Seventeen finished the first section from

Brooklands to Hendon, a distance of twenty miles. Five reached

Edinburgh, the second section, a distance of 343 miles, and four

completed the entire circuit.

Paris to Madrid Race.--This race was started at the Paris aviation

held at Issy-les-Moulineaux on Sunday, May 21. There were twenty-one

entrants, and fully 300,000 spectators gathered to witness the initial

flight of the aerial races. The race was divided into three stages as

follows: Paris to Angouleme, 248 miles; Angouleme to St. Sebastian, 208

miles, and from St. Sebastian to Madrid, 386 miles, a total distance

of 842 miles. After three of the entrants had safely left the field,

Aviator Train lost control of his plane, and in falling struck and

killed M. Berteaux, the French Minister of War, and seriously injured

Premier Monis. The accident caused the withdrawal of all but six of the

original entrants, and of these but one finished. The race called for

a flight over the Pyrenees Mountains, and Vedrines, the winner, had to

rise to a height of more than 7,000 feet to pass the mountain barrier

near Somosierra Pass. Both Vedrines and Gibert, another competitor, were

attacked by eagles during the latter stages of the flight. Vedrines,

who started from Paris on Monday, May 22, finished the long and perilous

race at 8:06 a. m. Friday, May 26. Vedrines net flying time, all

controls and enforced stops subtracted, was 14h. 55m. 18s. The various

prizes to the winner aggregated $30,000.

The Paris-Rome-Turin Race.--The conditions of this race called for a

flight between the cities of Paris, Rome and Turin, covering a distance

of 1,300 miles. The aviators were permitted by the rules to alight

whenever and wherever they desired and the time limit was set from

May 28 to June 15. A prize of $100,000 was offered the winner, but the

contest was never finished, as one after another the aviators dropped

out until Frey fell near Roncigilione, France, breaking both arms and

legs and unofficially ending the contest. There were twenty-one entries

and twelve actual starters.

International Speed Cup Race.--The third annual international James

Gordon Bennett speed cup race was held at Eastchurch, England, on July

1, 1911, and for the second time was won by an American aviator, C. T.

Weymann, in a French racing aeroplane. The distance was 150 kilometres

equivalent to 94 miles, and the winner's time of 1h. 11m. 36s. showed an

average speed of 78.77 miles per hour. The first race was held in 1909

and was won by Glenn Curtiss, who flew the twenty kilometres (12.4

miles) in 15 minutes 50 2-5 seconds at an average speed of 47 miles per

hour. In 1910 the winner was Grahame-White, who covered 100 kilometres

(62 miles) at Belmont Park, L. I., in 60 minutes 47 3-5 seconds, an

average speed of 61.3 miles per hour. In the 1911 race there were six

starters: three from France, two from Great Britain and one from the

United States.

Milan to Turin to Milan Race.--This race which was started from Milan,

Italy, on October 29, was restricted to Italian aviators and had six

starters. The distance was approximately 177 miles and won by Manissero

in a Bleriot machine in 3h. 16m. 2 4-5s.

New York to Philadelphia Race.--The first intercity aeroplane race ever

held in the United States was started from New York City on August

5, and finished in Philadelphia the same day. The prize of $5,000 was

offered by a commercial concern with stores in the two cities: Three

entrants competed from the Curtiss Exhibition Company. The distance was

approximately 83 miles and won by L. Beachey in a Curtiss machine in 1h.

50m. at an average speed of 45 miles per hour.

Tri-State Race.--The tri-state race was the feature event of the Harvard

Aviation Society meet held at Squantum, Mass., August 26 to September

6. It was held Labor Day, September 4, over a course of 174 miles, from

Boston to Nashua to Worcester to Providence to Boston. Four competitors

started, of which two finished, the winner, E. Ovington, in a Bleriot

machine. Ovington's net flying time, 3h. 6m. 22 1-5s. Winner's prize,

$10,000.

AEROPLANES AND DIRIGIBLE BALLOONS IN WARFARE.

Wonderful progress has been made in the development of the aeroplane in

this country and in Europe since 1903, and within the last two or three

years the leading powers of the world have entered upon extensive tests

and experiments to determine its availability and usefulness in land and

naval warfare.

At the present time all the great powers are building or purchasing

aeroplanes on an extensive scale. They have established government

schools for the instruction of their army and navy officers and for

experimental work. So-called "Airship Fleets" have been constructed and

placed in commission as auxiliaries to the armies and navies. The fleets

of France and Germany are about equal and are larger by far than those

of any of the other powers. The length of the dirigibles composing these

fleets runs from 150 to 500 feet; they are equipped with engines of from

50 to 500 horse-power, with a rate of speed ranging from 20 to 30 miles

per hour. Their approximate range is from 200 to 900 miles; the longest

actual run (made by the Zeppelin II, Germany) is 800 miles.

A British naval airship, one of the largest yet built, was completed

last summer. It has cost over $200,000, and it was in course of

designing and construction two years. It is 510 feet long; can carry 22

persons, and has a lift of 21 tons.

The relative value of the dirigible balloon and the aeroplane in actual

war is yet to be determined. The dirigible is considered to be the

safer, yet several large balloons of this class in Germany and France

have met with disaster, involving loss of lives. The capacity of the

dirigible for longer flights and its superior facilities for carrying

apparatus and operators for wireless telegraphy are distinct advantages.

There has not yet been much opportunity to test the airship in actual

warfare. The aeroplane has been used by the Italians in Tripoli for

scouting and reconnoitering and is said to have justified expectations.

On several occasions the Italian military aviators followed the

movements of the enemy, in one instance as far as forty miles inland. At

the time of the attack by the Turks a skillful aeroplane reconnaissance

revealed the approach of a large Turkish force, believed to be at the

time sixty miles away in the mountains.

Aeroplanes and airships, as they exist today, would doubtless render

very valuable service in a time of war, both over land and water, in

scouting, reconnoitering, carrying dispatches, and as some experts

believe, in locating submarines and mines placed by the enemy in

channels of exits from ports. A "coast aeroplane" could fly out 30 or 40

miles from land, and rising to a great height, descry any hostile ships

on the distant horizon, observe their number, strength, formation and

direction, and return within two hours with a report to obtain which

would require several swift torpedo-boat destroyers and a much greater

time. The question as to whether it would be practicable to bombard an

enemy on land or sea with explosive bombs dropped or discharged from

flying machines or airships, is one which is much discussed but hardly

yet determined.

Aeroplanes have been constructed with floats in the place of runners and

several attempts have been made, in some cases successfully, to light

with them on and to rise from the water. Mr. Curtiss did this at San

Francisco, in January, 1911. Attempts have also been made with the

aeroplane to alight on and to take flight from the deck of a warship.

Toward the end of 1910 Aviator Ely flew to land from the cruiser

Birmingham, and in January, 1911, he flew from land and alighted on the

cruiser Pennsylvania. But in these cases special arrangements were made

which would be hardly practicable in a time of actual war.

In November, 1911, a test was made at Newport, R. I., by Lieut. Rodgers,

of the navy, of a "hydro-areoplane" as an auxiliary to a battleship. The

idea of the test was to alight alongside of the ship, hoist the machine

aboard, put out to sea and launch the machine again with the use of a

crane. Lieut. Rodgers came down smoothly alongside the Ohio, his machine

was easily drawn aboard with a crane, and the Ohio steamed down to

the open sea, where it was blowing half a gale. But, owing to the

misjudgment of the ship's headway, one of the wings of the machine when

it struck the water after being released from the crane, went under the

water and was snapped off. Lieut. Rodgers was convinced that this method

was too risky and that some other must be devised.

# CHAPTER XXVIII. GLOSSARY OF AERONAUTICAL TERMS.

Aerodrome.--Literally a machine that runs in the air. Aerofoil.--The

advancing transverse section of an aeroplane.

Aeroplane.--A flying machine of the glider pattern, used in

contra-distinction to a dirigible balloon.

Aeronaut.--A person who travels in the air.

Aerostat.--A machine sustaining weight in the air. A balloon is an

aerostat.

Aerostatic.--Pertaining to suspension in the air; the art of aerial

navigation.

Ailerons.--Small stabilizing planes attached to the main planes to

assist in preserving equilibrium.

Angle of Incidence.--Angle formed by making comparison with a

perpendicular line or body.

Angle of Inclination.--Angle at which a flying machine rises. This

angle, like that of incidence, is obtained by comparison with an

upright, or perpendicular line.

Auxiliary Planes.--Minor plane surfaces, used in conjunction with the

main planes for stabilizing purposes.

Biplane.--A flying-machine of the glider type with two surface planes.

Blade Twist.--The angle of twist or curvature on a propeller blade.

Cambered.--Curve or arch in plane, or wing from port to starboard.

Chassis.--The under framework of a flying machine; the framework of the

lower plane.

Control.--System by which the rudders and stabilizing planes are

manipulated.

Dihedral.--Having two sides and set at an angle, like dihedral planes,

or dihedral propeller blades.

Dirigible.--Obedient to a rudder; something that may be steered or

directed.

Helicopter.--Flying machine the lifting power of which is furnished by

vertical propellers.

Lateral Curvature.--Parabolic form in a transverse direction.

Lateral Equilibrium or Stability.--Maintenance of the machine on an even

keel transversely. If the lateral equilibrium is perfect the extreme

ends of the machine will be on a dead level.

Longitudinal Equilibrium or Stability.--Maintenance of the machine on an

even keel from front to rear.

Monoplane.--Flying machine with one supporting, or surface plane.

Multiplane.--Flying machine with more than three surface planes.

Ornithopter.--Flying machine with movable bird-like wings.

Parabolic Curves.--Having the form of a parabola--a conic section.

Pitch of Propeller Blade.--See "Twist."

Ribs.--The pieces over which the cloth covering is stretched.

Spread.--The distance from end to end of the main surface; the

transverse dimension.

Stanchions.--Upright pieces connecting the upper and lower frames.

Struts.--The pieces which hold together longitudinally the main frame

beams.

Superposed.--Placed one over another.

Surface Area.--The amount of cloth-covered supporting surface which

furnishes the sustaining quality.

Sustentation.--Suspension in the air. Power of sustentation; the quality

of sustaining a weight in the air.

Triplane.--Flying machine with three surface planes.

Thrust of Propeller.--Power with which the blades displace the air.

Width.--The distance from the front to the rear edge of a flying

machine.

Wind Pressure.--The force exerted by the wind when a body is moving

against it. There is always more or less wind pressure, even in a calm.

Wing Tips.--The extreme ends of the main surface planes. Sometimes these

are movable parts of the main planes, and sometimes separate auxiliary

planes.

Footnotes:

[Footnote 1: Now dead.]

[Footnote 2: Aeronautics.]

[Footnote 3: See Chapter XXV.]

[Footnote 4: The Wrights' new machine weighs only 900 pounds.]

[Footnote 5: Aeronautics.]

*** END OF THE PROJECT GUTENBERG EBOOK FLYING MACHINES: CONSTRUCTION AND OPERATION ***

Updated editions will replace the previous one—the old editions will

be renamed.

Creating the works from print editions not protected by U.S. copyright

law means that no one owns a United States copyright in these works,

so the Foundation (and you!) can copy and distribute it in the United

States without permission and without paying copyright

royalties. Special rules, set forth in the General Terms of Use part

of this license, apply to copying and distributing Project

Gutenberg™ electronic works to protect the PROJECT GUTENBERG™

concept and trademark. Project Gutenberg is a registered trademark,

and may not be used if you charge for an eBook, except by following

the terms of the trademark license, including paying royalties for use

of the Project Gutenberg trademark. If you do not charge anything for

copies of this eBook, complying with the trademark license is very

easy. You may use this eBook for nearly any purpose such as creation

of derivative works, reports, performances and research. Project

Gutenberg eBooks may be modified and printed and given away—you may

do practically ANYTHING in the United States with eBooks not protected

by U.S. copyright law. Redistribution is subject to the trademark

license, especially commercial redistribution.

START: FULL LICENSE

THE FULL PROJECT GUTENBERG LICENSE

PLEASE READ THIS BEFORE YOU DISTRIBUTE OR USE THIS WORK

To protect the Project Gutenberg™ mission of promoting the free

distribution of electronic works, by using or distributing this work

(or any other work associated in any way with the phrase “Project

Gutenberg”), you agree to comply with all the terms of the Full

Project Gutenberg™ License available with this file or online at

www.gutenberg.org/license.

Section 1. General Terms of Use and Redistributing Project Gutenberg™

electronic works

1.A. By reading or using any part of this Project Gutenberg™

electronic work, you indicate that you have read, understand, agree to

and accept all the terms of this license and intellectual property

(trademark/copyright) agreement. If you do not agree to abide by all

the terms of this agreement, you must cease using and return or

destroy all copies of Project Gutenberg™ electronic works in your

possession. If you paid a fee for obtaining a copy of or access to a

Project Gutenberg™ electronic work and you do not agree to be bound

by the terms of this agreement, you may obtain a refund from the person

or entity to whom you paid the fee as set forth in paragraph 1.E.8.

1.B. “Project Gutenberg” is a registered trademark. It may only be

used on or associated in any way with an electronic work by people who

agree to be bound by the terms of this agreement. There are a few

things that you can do with most Project Gutenberg™ electronic works

even without complying with the full terms of this agreement. See

paragraph 1.C below. There are a lot of things you can do with Project

Gutenberg™ electronic works if you follow the terms of this

agreement and help preserve free future access to Project Gutenberg™

electronic works. See paragraph 1.E below.

1.C. The Project Gutenberg Literary Archive Foundation (“the

Foundation” or PGLAF), owns a compilation copyright in the collection

of Project Gutenberg™ electronic works. Nearly all the individual

works in the collection are in the public domain in the United

States. If an individual work is unprotected by copyright law in the

United States and you are located in the United States, we do not

claim a right to prevent you from copying, distributing, performing,

displaying or creating derivative works based on the work as long as

all references to Project Gutenberg are removed. Of course, we hope

that you will support the Project Gutenberg™ mission of promoting

free access to electronic works by freely sharing Project Gutenberg™

works in compliance with the terms of this agreement for keeping the

Project Gutenberg™ name associated with the work. You can easily

comply with the terms of this agreement by keeping this work in the

same format with its attached full Project Gutenberg™ License when

you share it without charge with others.

1.D. The copyright laws of the place where you are located also govern

what you can do with this work. Copyright laws in most countries are

in a constant state of change. If you are outside the United States,

check the laws of your country in addition to the terms of this

agreement before downloading, copying, displaying, performing,

distributing or creating derivative works based on this work or any

other Project Gutenberg™ work. The Foundation makes no

representations concerning the copyright status of any work in any

country other than the United States.

1.E. Unless you have removed all references to Project Gutenberg:

1.E.1. The following sentence, with active links to, or other

immediate access to, the full Project Gutenberg™ License must appear

prominently whenever any copy of a Project Gutenberg™ work (any work

on which the phrase “Project Gutenberg” appears, or with which the

phrase “Project Gutenberg” is associated) is accessed, displayed,

performed, viewed, copied or distributed:

This eBook is for the use of anyone anywhere in the United States and most

other parts of the world at no cost and with almost no restrictions

whatsoever. You may copy it, give it away or re-use it under the terms

of the Project Gutenberg License included with this eBook or online

at www.gutenberg.org. If you

are not located in the United States, you will have to check the laws

of the country where you are located before using this eBook.

1.E.2. If an individual Project Gutenberg™ electronic work is

derived from texts not protected by U.S. copyright law (does not

contain a notice indicating that it is posted with permission of the

copyright holder), the work can be copied and distributed to anyone in

the United States without paying any fees or charges. If you are

redistributing or providing access to a work with the phrase “Project

Gutenberg” associated with or appearing on the work, you must comply

either with the requirements of paragraphs 1.E.1 through 1.E.7 or

obtain permission for the use of the work and the Project Gutenberg™

trademark as set forth in paragraphs 1.E.8 or 1.E.9.

1.E.3. If an individual Project Gutenberg™ electronic work is posted

with the permission of the copyright holder, your use and distribution

must comply with both paragraphs 1.E.1 through 1.E.7 and any

additional terms imposed by the copyright holder. Additional terms

will be linked to the Project Gutenberg™ License for all works

posted with the permission of the copyright holder found at the

beginning of this work.

1.E.4. Do not unlink or detach or remove the full Project Gutenberg™

License terms from this work, or any files containing a part of this

work or any other work associated with Project Gutenberg™.

1.E.5. Do not copy, display, perform, distribute or redistribute this

electronic work, or any part of this electronic work, without

prominently displaying the sentence set forth in paragraph 1.E.1 with

active links or immediate access to the full terms of the Project

Gutenberg™ License.

1.E.6. You may convert to and distribute this work in any binary,

compressed, marked up, nonproprietary or proprietary form, including

any word processing or hypertext form. However, if you provide access

to or distribute copies of a Project Gutenberg™ work in a format

other than “Plain Vanilla ASCII” or other format used in the official

version posted on the official Project Gutenberg™ website

(www.gutenberg.org), you must, at no additional cost, fee or expense

to the user, provide a copy, a means of exporting a copy, or a means

of obtaining a copy upon request, of the work in its original “Plain

Vanilla ASCII” or other form. Any alternate format must include the

full Project Gutenberg™ License as specified in paragraph 1.E.1.

1.E.7. Do not charge a fee for access to, viewing, displaying,

performing, copying or distributing any Project Gutenberg™ works

unless you comply with paragraph 1.E.8 or 1.E.9.

1.E.8. You may charge a reasonable fee for copies of or providing

access to or distributing Project Gutenberg™ electronic works

provided that:

• You pay a royalty fee of 20% of the gross profits you derive from

the use of Project Gutenberg™ works calculated using the method

you already use to calculate your applicable taxes. The fee is owed

to the owner of the Project Gutenberg™ trademark, but he has

agreed to donate royalties under this paragraph to the Project

Gutenberg Literary Archive Foundation. Royalty payments must be paid

within 60 days following each date on which you prepare (or are

legally required to prepare) your periodic tax returns. Royalty

payments should be clearly marked as such and sent to the Project

Gutenberg Literary Archive Foundation at the address specified in

Section 4, “Information about donations to the Project Gutenberg

Literary Archive Foundation.”

• You provide a full refund of any money paid by a user who notifies

you in writing (or by e-mail) within 30 days of receipt that s/he

does not agree to the terms of the full Project Gutenberg™

License. You must require such a user to return or destroy all

copies of the works possessed in a physical medium and discontinue

all use of and all access to other copies of Project Gutenberg™

works.

• You provide, in accordance with paragraph 1.F.3, a full refund of

any money paid for a work or a replacement copy, if a defect in the

electronic work is discovered and reported to you within 90 days of

receipt of the work.

• You comply with all other terms of this agreement for free

distribution of Project Gutenberg™ works.

1.E.9. If you wish to charge a fee or distribute a Project

Gutenberg™ electronic work or group of works on different terms than

are set forth in this agreement, you must obtain permission in writing

from the Project Gutenberg Literary Archive Foundation, the manager of

the Project Gutenberg™ trademark. Contact the Foundation as set

forth in Section 3 below.

1.F.

1.F.1. Project Gutenberg volunteers and employees expend considerable

effort to identify, do copyright research on, transcribe and proofread

works not protected by U.S. copyright law in creating the Project

Gutenberg™ collection. Despite these efforts, Project Gutenberg™

electronic works, and the medium on which they may be stored, may

contain “Defects,” such as, but not limited to, incomplete, inaccurate

or corrupt data, transcription errors, a copyright or other

intellectual property infringement, a defective or damaged disk or

other medium, a computer virus, or computer codes that damage or

cannot be read by your equipment.

1.F.2. LIMITED WARRANTY, DISCLAIMER OF DAMAGES - Except for the “Right

of Replacement or Refund” described in paragraph 1.F.3, the Project

Gutenberg Literary Archive Foundation, the owner of the Project

Gutenberg™ trademark, and any other party distributing a Project

Gutenberg™ electronic work under this agreement, disclaim all

liability to you for damages, costs and expenses, including legal

fees. YOU AGREE THAT YOU HAVE NO REMEDIES FOR NEGLIGENCE, STRICT

LIABILITY, BREACH OF WARRANTY OR BREACH OF CONTRACT EXCEPT THOSE

PROVIDED IN PARAGRAPH 1.F.3. YOU AGREE THAT THE FOUNDATION, THE

TRADEMARK OWNER, AND ANY DISTRIBUTOR UNDER THIS AGREEMENT WILL NOT BE

LIABLE TO YOU FOR ACTUAL, DIRECT, INDIRECT, CONSEQUENTIAL, PUNITIVE OR

INCIDENTAL DAMAGES EVEN IF YOU GIVE NOTICE OF THE POSSIBILITY OF SUCH

DAMAGE.

1.F.3. LIMITED RIGHT OF REPLACEMENT OR REFUND - If you discover a

defect in this electronic work within 90 days of receiving it, you can

receive a refund of the money (if any) you paid for it by sending a

written explanation to the person you received the work from. If you

received the work on a physical medium, you must return the medium

with your written explanation. The person or entity that provided you

with the defective work may elect to provide a replacement copy in

lieu of a refund. If you received the work electronically, the person

or entity providing it to you may choose to give you a second

opportunity to receive the work electronically in lieu of a refund. If

the second copy is also defective, you may demand a refund in writing

without further opportunities to fix the problem.

1.F.4. Except for the limited right of replacement or refund set forth

in paragraph 1.F.3, this work is provided to you ‘AS-IS’, WITH NO

OTHER WARRANTIES OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT

LIMITED TO WARRANTIES OF MERCHANTABILITY OR FITNESS FOR ANY PURPOSE.

1.F.5. Some states do not allow disclaimers of certain implied

warranties or the exclusion or limitation of certain types of

damages. If any disclaimer or limitation set forth in this agreement

violates the law of the state applicable to this agreement, the

agreement shall be interpreted to make the maximum disclaimer or

limitation permitted by the applicable state law. The invalidity or

unenforceability of any provision of this agreement shall not void the

remaining provisions.

1.F.6. INDEMNITY - You agree to indemnify and hold the Foundation, the

trademark owner, any agent or employee of the Foundation, anyone

providing copies of Project Gutenberg™ electronic works in

accordance with this agreement, and any volunteers associated with the

production, promotion and distribution of Project Gutenberg™

electronic works, harmless from all liability, costs and expenses,

including legal fees, that arise directly or indirectly from any of

the following which you do or cause to occur: (a) distribution of this

or any Project Gutenberg™ work, (b) alteration, modification, or

additions or deletions to any Project Gutenberg™ work, and (c) any

Defect you cause.

Section 2. Information about the Mission of Project Gutenberg™

Project Gutenberg™ is synonymous with the free distribution of

electronic works in formats readable by the widest variety of

computers including obsolete, old, middle-aged and new computers. It

exists because of the efforts of hundreds of volunteers and donations

from people in all walks of life.

Volunteers and financial support to provide volunteers with the

assistance they need are critical to reaching Project Gutenberg™’s

goals and ensuring that the Project Gutenberg™ collection will

remain freely available for generations to come. In 2001, the Project

Gutenberg Literary Archive Foundation was created to provide a secure

and permanent future for Project Gutenberg™ and future

generations. To learn more about the Project Gutenberg Literary

Archive Foundation and how your efforts and donations can help, see

Sections 3 and 4 and the Foundation information page at www.gutenberg.org.

Section 3. Information about the Project Gutenberg Literary Archive Foundation

The Project Gutenberg Literary Archive Foundation is a non-profit

501(c)(3) educational corporation organized under the laws of the

state of Mississippi and granted tax exempt status by the Internal

Revenue Service. The Foundation’s EIN or federal tax identification

number is 64-6221541. Contributions to the Project Gutenberg Literary

Archive Foundation are tax deductible to the full extent permitted by

U.S. federal laws and your state’s laws.

The Foundation’s business office is located at 809 North 1500 West,

Salt Lake City, UT 84116, (801) 596-1887. Email contact links and up

to date contact information can be found at the Foundation’s website

and official page at www.gutenberg.org/contact

Section 4. Information about Donations to the Project Gutenberg

Literary Archive Foundation

Project Gutenberg™ depends upon and cannot survive without widespread

public support and donations to carry out its mission of

increasing the number of public domain and licensed works that can be

freely distributed in machine-readable form accessible by the widest

array of equipment including outdated equipment. Many small donations

($1 to $5,000) are particularly important to maintaining tax exempt

status with the IRS.

The Foundation is committed to complying with the laws regulating

charities and charitable donations in all 50 states of the United

States. Compliance requirements are not uniform and it takes a

considerable effort, much paperwork and many fees to meet and keep up

with these requirements. We do not solicit donations in locations

where we have not received written confirmation of compliance. To SEND

DONATIONS or determine the status of compliance for any particular state

visit www.gutenberg.org/donate.

While we cannot and do not solicit contributions from states where we

have not met the solicitation requirements, we know of no prohibition

against accepting unsolicited donations from donors in such states who

approach us with offers to donate.

International donations are gratefully accepted, but we cannot make

any statements concerning tax treatment of donations received from

outside the United States. U.S. laws alone swamp our small staff.

Please check the Project Gutenberg web pages for current donation

methods and addresses. Donations are accepted in a number of other

ways including checks, online payments and credit card donations. To

donate, please visit: www.gutenberg.org/donate.

Section 5. General Information About Project Gutenberg™ electronic works

Professor Michael S. Hart was the originator of the Project

Gutenberg™ concept of a library of electronic works that could be

freely shared with anyone. For forty years, he produced and

distributed Project Gutenberg™ eBooks with only a loose network of

volunteer support.

Project Gutenberg™ eBooks are often created from several printed

editions, all of which are confirmed as not protected by copyright in

the U.S. unless a copyright notice is included. Thus, we do not

necessarily keep eBooks in compliance with any particular paper

edition.

Most people start at our website which has the main PG search

facility: www.gutenberg.org.

This website includes information about Project Gutenberg™,

including how to make donations to the Project Gutenberg Literary

Archive Foundation, how to help produce our new eBooks, and how to

subscribe to our email newsletter to hear about new eBooks.